| Joint Support and Enabling Service Headquarters Stationing / Infrastructure Branch Hardthöhe Fontainengraben 150 53123 Bonn |  |
| --- | --- |

Statement of Requirements

(SOR ZAP 101)
Functional area

“Materiel storage site- ZAPALSKIAI”

(MatLgrNdlg ZAP)

Long-term Infrastructure of the Bundeswehr
as Part of Stationing a DEU Brigade in Lithuania

| Facility: | MatNdlg ZAP |
| --- | --- |
| Location: Prepared by: | ZAP in LTU DEU Joint Support and Enabling Service Headquarters, Planning / Organisation Division, Stationing / Infrastructure Branch KdoSKBPlgOrgStatInfra@bundeswehr.org Tel. (Bw mil.): 90 - 3402 - 1976 Tel. (civ.): +49 (0) 2285504 – 1976 |

Annexes:

- 4_1_01_00 space requirement_MatNdlg ZAP

- 4_1_01_01 RBu Lagerhalle Verpflegung

- 4_1_01_02 RBu Entsorgungs-/Umschlagstelle

- 4_1_01_03 RBu Lagerhalle SBH

- 4_1_01_04 RBu Lagerhalle Sperr- /BauMat

- 4_1_01_05 RBu Freifläche Schüttgut

- 4_1_01_06 RBu Schutzdach Sperr- /BauMat

Table of Contents

1.	Infrastructure Requirements	2

1.1.	Functional area materiel storage site (Mat) ZAP	2

1.1.1.	Introductory note / general and functional description	2

1.1.2.	Space and area requirements	2

1.1.3.	Functional associations	2

# Infrastructure Requirements

## Functional area materiel storage site (Mat) ZAP

## Introductory note / general and functional description

For the construction of demand-oriented infrastructure for a materiel storage site, the infrastructure requirements outlined below are mandatory.

The materiel storage site must be located within a military restricted area.

As of now, there will be no permanent workplaces in the materiel storage site. Based on the logistic organisational procedures, the LogCoy personnel will only be temporarily present at the materiel storage site.

The infrastructural requirements for the materiel storage site consist of storage, parking and staging areas.

## Space and area requirements

The space and area requirements of the functional area “ZAP materiel storage site” are specified in the space and area requirement plan (IAW annex1  4_1_01_00 space requirement_MatNdlg ZAP).

The structural and technical requirements as well as the furnishings are detailed in the room logbooks 1 to 6 (annexes 2-7).

The floor space requirements for the ZAP materiel storage site is described in the SOR parking areas ZAP (SOR ZAP 103).

## Functional associations

In general:

- The functional areas rations storage building with the rations storage hall apron and the disposal/handling point, the POL storage hall with the POL storage hall apron, the storage hall obstacle and construction material with apron, the open area for bulk material and the protective roof storage area for obstacle and construction material are each connected in terms of function. The aprons of the respective buildings serve as a connection between the road network and the respective storage hall. 
The disposal/handling point provides electricity for e. g. temperature-controlled containers, the handling of containers and parking of waste and recycling containers.

- Sanitary facilities are required and, due to the lack of permanent workplaces, will be provided as mobile toilets (2 x).

The schematic representation of the sub-area rations storage building below shows the layout of the required spaces and areas.

Figure 1, schematic representation

| Entsorgung-/Umschlagestelle | Disposal/handling point |
| --- | --- |
| Lagerhalle Verpflegung | Rations storage building |
| Vorfeld | Apron |
| Straße | Road |

In principle, this spatial arrangement complies with demands-based organisational procedures to ensure value-retaining storage, picking, parking and handling and is to be implemented as described, if possible.

- The disposal/handling point, the rations storage building and the apron are to be connected to the road and track network.

- The disposal and handling point requires power connections for the electrical supply of e. g. temperature-controlled containers (see room logbook disposal and handling point).

Room sketch for disposal/handling point

Annex: 4_1_01_02: Disposal/handling point [1,330 m²]

Figure 2, room sketch

| Müllcontainer | Waste container |
| --- | --- |
| Container | Container |
| Thermocontainer | Temperature-controlled container |
| Anschlusspunkt Container | Container connection point |

Room sketch for rations storage building

Annex:4 _1_01_01: Rations storage building (3,949 m²) with apron (295 m²) [4,244m²]

Figure 3, room sketch

| Gebäudeschnitt | Building section |
| --- | --- |
| Hallenvorfeld | Apron |
| Verkehrs- und Rangierfläche | Traffic and manoeuvring area |
| Gabelstapler | Forklift |

The schematic representation of the sub-area POL storage building below shows the layout of the required spaces and areas.

Figure 4, schematic representation

| Lagerhalle Schmier- und Betriebshilfsstoffe | POL storage building |
| --- | --- |
| Vorfeld | Apron |
| Straße | Road |

In principle, this spatial arrangement complies with demands-based organisational procedures to ensure value-retaining storage, picking, parking and handling and is to be implemented as described, if possible.

- The POL storage building and the corresponding apron are to be connected to the road and track network.

- Depending on the type and quantity of hazardous materials stored there, it may be necessary to build further storage areas (to be separated by e. g. fire walls) or additional storage buildings.

Room sketch for POL storage building

Annex: 4_1_01_03: POL storage building (1,166 m²) with apron (150m²) [1,316m²]

Figure 5, room sketch

| SBH Palette | POL pallet |
| --- | --- |
| Gebäudeschnitt | Building section |
| Hallenvorfeld | Apron |
| Verkehrs- und Rangierfläche | Traffic and manoeuvring area |
| Gabelstapler | Forklift |

The schematic representation of the sub-area obstacle and construction material storage building below shows the layout of the required spaces and areas.

Room sketch obstacle and construction material storage building

Annex: 4_1_01_04: Storage building for obstacle and construction material (330 m²) with apron (80 m²) [410 m²]

Figure 6, room sketch

| Gebäudeschnitt | Building section |
| --- | --- |
| Hallenvorfeld | Apron |
| Verkehrs- und Rangierfläche | Traffic and manoeuvring area |
| Gabelstapler | Forklift |

In principle, this spatial arrangement complies with demands-based organisational procedures to ensure value-retaining storage, picking, parking and handling and is to be implemented as described, if possible.

- The storage building for obstacle and construction materials and the corresponding apron are to be connected to the road and track network.

The schematic representation of the sub-area bulk material storage area below shows the layout of the required spaces and areas.

Figure 7, schematic representation

| Lagerfläche Schüttgut | Bulk material storage area |
| --- | --- |
| Seitenstraße | Side road |
| Straße | Road |

Room sketch open area for bulk material

Annex: 4_1_01_05: Open area for bulk material [2,805 m²]

Figure 8, room sketch

| Schüttgut | Bulk material |
| --- | --- |

This area is for the permanent storage of bulk material.

The bulk material must be accessible for trucks from the side (8 m) in order for them to load bulk material when it is needed.

The following room sketch of the sub-area protective roof storage area for obstacle material shows the required space and area requirements.

Room sketch protective roof for obstacle and construction material

Annex: 4_1_01_06: Protective roof for obstacle and construction material				[3,990 m²]

Figure 9, room sketch

| Fahrstreifen | Traffic lane |
| --- | --- |
| Vorfelder | Aprons |
| Schutzdach | Protective roof |

This area is for the permanent storage of obstacle material.

To permit loading activities, the storage area must possess an 8-m-deep apron.