## Room layout plan

| Gebäudeschnitt | Building section |
| --- | --- |
| Hallenvorfeld | Apron |
| Verkehrs- und Rangierfläche | Traffic and manoeuvring area |
| Gabelstapler | Forklift |

Sample image:

## Structural and technical space requirements:

| A | 3,949 m² | Rations storage building | 360 x pallet rack element à 5 m |
| --- | --- | --- | --- |
| B | 295 m² | Apron |  |

Storage building for value-retaining storage of rations.

It is used for processing incoming and outgoing goods and for picking and checking the stored goods.

The stored goods are moved with the help of pallet trucks and class I forklifts (approx. 12.5 kN/m², single-wheel load approx. 16kN).

The height of the upper edge of the finished floor of the storage hall is to be equal to the upper edge of the apron. To ensure that loads can be properly moved, sufficient manoeuvring and traffic areas must be available (3 m). The minimum corridor width is 2.5 m. The apron is to be constructed at a depth of 8 m over the entire width of the building. The storage building is to be secured against pests and vermin.

| Designation | Qty | Specifications | Comments |
| --- | --- | --- | --- |
| Room temperature |  | Min. 2° C, max. 25 °C |  |
| Lighting A B |  | Min. 200 lx Min. 150 lx | LED lamps are to be installed where possible. B (150 lx) for illumination of the loading area / manoeuvring area |
| Power supply A | 3 | Double socket 250 V / AC / 16 A | General electrical supply of the room One double socket each in the vicinity of the gates |
| Window | 0 |  | No direct sunlight |
| Doors A | 3 | Double-leaf door, steel | Width/height: 2 m / 2.5 m |
| Floor A B |  | Concrete floor with epoxy resin coating | A: Anti-static, non-slip, oil-resistant, easy to clean, low-dust. Load-bearing capacity: ≥20 kN/m² to be trafficable with material handling equipment. Point loads caused by shelf systems must be considered. Pallet weight: 830 kg B: Load category 1.0, road surface made of asphalt (IAW Guidelines for the Standardisation of Pavement Structures for Traffic Areas 12/24). In order to avoid surface water entering, appropriate drainage must be planned for in the gate area. |
| Walls/ceilings |  | Clear height: A = 5.40 m | Smooth, light-coloured. Column-free if possible. |
| Ventilation |  | Max. relative humidity 65 % Air-conditioning system (ventilation system) | Dry storage, ventilation system to avoid/reduce substance, humidity and heat loads |
| Fire safety |  |  | Field rations are classified as fire load. Accordingly, a certain distance must be maintained to sources of ignition and hazardous materials. One fire alarm system per storage section is to be installed. |

## Furnishings

| Item | Qty | Comments |
| --- | --- | --- |
| Pallet rack |  | 5 m x 0.8 m x 1.9 m (l/w/h), if possible, height of racks is to be flexibly adjustable. The racks are to be designed in accordance with static requirements/load-bearing capacity. 1 level, load-bearing capacity per rack: 3,320 kg, number of pallet spaces: 4 pallets / pallet rack compartment, 2,873 pallet spaces, spread over 18 rows of racks on two levels (floor + 1 pallet shelf compartment) |
| Fire detection and fire alarm system |  |  |

Side view pallet rack element