## Room layout plan

| Müllcontainer | Waste container |
| --- | --- |
| Container | Container |
| Thermocontainer | Temperature-controlled container |
| Anschlusspunkt Container | Container connection point |

## Structural and technical space requirements:

| A | 1330 m² | Disposal/handling point | Sealed storage area for containers |
| --- | --- | --- | --- |

Sealed storage area for containers as part of the disposal and handling processes.

The height of the upper edge of the finished floor of the storage building is to be equal to the upper edge of the apron. To ensure that loads can be properly moved, sufficient manoeuvring and traffic areas must be available (24 m). The area is to be located directly adjoining the rations storage building.

| Item | Qty | Specifications | Comments |
| --- | --- | --- | --- |
| Lighting |  | At least 150 lx | Illumination of disposal/handling point |
| Power supply | 5 | Two connections each for high-voltage current 400 V / AC / 16 A Two power connections each 250 V / AC / 16 A | General electrical supply of the room One double socket each for power supply of temperature-controlled containers |
| Floors |  | Concrete floor | Oil resistant, easy to clean, Load-bearing capacity: ≥ 800 kN/m² trafficable with trucks and container handlers. Load category 1.3, road surface made of concrete (IAW the Guidelines for the Standardisation of Pavement Structures for Traffic Areas 12/24). |

## Furnishings:

N/a