## Room layout plan

| Gebäudeschnitt | Building section |
| --- | --- |
| Hallenvorfeld | Apron |
| Verkehrs- und Rangierfläche | Traffic and manoeuvring area |
| Gabelstapler | Forklift |

## Structural and technical space requirements

| A | 330 m² | Storage building for obstacle and construction material | 12x pallet rack element, 5 m each |
| --- | --- | --- | --- |
| B | 80 m² | Apron | Space for delivery of goods |

Storage building for obstacle and construction material to ensure value-retaining storage of obstacle and construction material (wood, sandbags, razor wire, HESCOS, etc.). For this purpose, floor and pallet rack storage space is required (see sketch). The height of the upper edge of the finished floor of the storage building is to be equal to the upper edge of the apron. To ensure that loads can be properly moved, sufficient manoeuvring and traffic areas are to be available (3 m). The minimum corridor width is 2.5 m. The apron is to be constructed at a depth of 8 m over the entire width of the building.

| Item | Quantity | Specifications | Remarks |
| --- | --- | --- | --- |
| Room temperature |  | Frost-free |  |
| Lighting A B |  | At least 200 lx At least 150 lx | LED lamps are to be installed where possible. B (150 lx) for the lighting of the loading area / apron |
| Power supply | - | - | No power supply required |
| Window(s) | - | - | No direct sunlight UV irradiation |
| Door A | 1 | Roller door | Width/height: 5 m / 4 m |
| Floor A B |  | Concrete floor | A: Load-bearing capacity: ≥ 20 kN/m² To be trafficable with ground conveyors B: Load category 1.0, Road surface of asphalt (IAW RStO 12/24). |
| Walls/ceilings |  | Clear height: A ≥ 5,40 m | Smooth, light-coloured. Column-free if possible. |
| Ventilation |  | Max. relative humidity 65 % | Dry storage |

## Furnishings:

| Item | Quantity | Remarks |
| --- | --- | --- |
| Pallet rack |  | 5 m x 0.8 m x 1.9 m (length x width x height), if possible, height of racks can be adjusted flexibly. The shelves are to be designed in accordance with static requirements/load-bearing capacity. 1 level, load-bearing capacity per shelf: 3,320 kg, number of pallet storage spaces: 4 pallets / pallet rack compartments, 96 pallet storage spaces in total, distributed in 2 rows of racks on 2 levels (floor + 1 pallet rack compartment) |
| Floor storage area |  | 6 m x 5 m required storage space for timber |

Side view of pallet rack element