German Joint Support and Enabling Service Headquarters Stationing / Infrastructure Branch Hardthöhe Fontainengraben 150, 53123 Bonn

Statement of Requirements

(SOR ZAP 102)

“ZAPALSKIAI Ammunition Storage Site”
Functional Area

(ZAP Ammo Storage Site)

Long-term Infrastructure of the Bundeswehr
as Part of Stationing a DEU Brigade in Lithuania

| Facility: | ZAPALSKIA Materiel Storage Site |  |
| --- | --- | --- |
| Location: | ZAPALSKIAI in LTU |  |
| Prepared by: | DEU Joint Support and Enabling Service Headquarters, Planning / Organisation Division, Stationing / Infrastructure Branch KdoSKBPlgOrgStatInfra@bundeswehr.org |  |
|  | Tel. (Bw mil.): | 90 - 3402 - 1976 |
|  | Tel. (civ.): | +49 (0) 2285504 – 1976 |

Annexes:

- 4_1_02_00 space requirement_MunNdlg

- 4_1_02_01 LTU-MArbH

- 4_1_02_02 LTU-ECM

- 4_1_02_03 RBu Abstellfl munbeladene Kfz

# Infrastructure Requirements

## ZAP ammunition storage site functional area

### General information / functional description / introductory note

For the construction of demand-oriented infrastructure for an ammunition storage site, the infrastructure requirements outlined below are mandatory.

The ammunition storage site must be located within a military restricted area, and should be integrated into the ZAP materiel storage site (SOR ZAP 101), if possible. The ammunition storage site must be furnished with a separate fencing within the materiel storage site. As a matter of principle, it is mandatory for LTU to prepare a security concept in cooperation with DEU officers responsible. The area is subject to a specific theft and sabotage risk and is defined as particularly sensitive installation.

In addition, three ammunition storehouses require increased technical protection due to the storage of ground-to-air guided missiles (MANPADS – man-portable air defence system). This protection is to be defined together with DEU representatives in the planning and implementation phase. The technical as well as the personnel and organisational elements of the protection should result from the aforementioned security concept.

As of now, there will be no permanent workplaces at the ammunition storage site. In accordance with the logistic organisational procedures, the logistic company (-) personnel will only be temporarily at the ammunition storage site, i.e. when actual requirements and their respective mission demand their presence.

The infrastructural requirements for the ammunition storage site consist of storage, parking and maintenance areas.

With regard to quality and quantity, the functional area of the “ZAP Ammunition Storage Site” is described by means of the space and area requirements (in accordance with para 1.1.2).

An overview of the types and scope of ammunition item cycles stored and/or the individual classifications of the safety distance categories to determine/specify safety distances was not yet available at the time these requirements were prepared. This means that, at present, it is only possible to point out the demand for/compliance with safety distances between potential explosion sites (largely ammunition storehouses, ammunition inspection buildings) and the objects protected. This is to be specified afterwards together with DEU representatives in the planning and implementation process.

### Space and area requirements

The space and area requirements of the “ZAP Ammunition Storage Site” functional area are outlined in the Space Requirement (Annex 1). The structural and technical requirements as well as furnishings are detailed in Annexes 2 to 4.

The “Ammunition Storage Site” functional area comprises the following sub-areas:

- ammunition maintenance area in the ammunition inspection building,

- ammunition storage area, and

- parking area for ammunition-loaded vehicles.

The following infrastructure is required for the ammunition storage site:

| Special infrastructure | [m²] |
| --- | --- |
| Ammunition inspection building “LTU – Ammunition Inspection Building” type (Anlage 2 - 4_1_02_01 LTU-MArbH) | 200 |
| Ready-use explosives storehouse type (450 m²) incl. elc. input room (29 m²) “LTU – Standard ECM” (Anlage 3 - 4_1_02_02 LTU-ECM) | 479 |
| Ammunition storehouses type (450 m²) incl. elc. input room (29 m²) “LTU – Standard ECM” (Anlage 3 - 4_1_02_02 LTU-ECM) | 16,765 |
| Parking area for ammunition-loaded vehicles (Anlage 3 - 4_1_02_03 RBu Abstellfl munbeladene Kfz) | 2,508 |
|  | 19,952.0 |

Table 1

### Functional associations

As a matter of principle,

- the infrastructure requirements outlined in Table 1 are potential explosion sites and objects to be protected among one another,

- the infrastructure requirements outlined in Table 1 must be located as a unit in a separately secured area within the materiel storage site.

- The type and quality of the protection should result from the security concept to be prepared. In this context, safety distances to one another and to other protected objects must be considered, unauthorised access is to be prevented by physical and technical protection measures unless a security concept yet to be prepared by LTU in cooperation with DEU officers responsible does not conflict with this. It should be verified as part of preparing the security concept whether “complicating” unauthorised access – possibly combined with its “indication” and the pertinent alerting of guard forces – seems to be feasible as an alternative. It is recommended to compare the intervention times of the guard forces with the resistance time values of the structural elements (walls, doors, windows, etc.).

The following schematic representation of the ammunition storage site functional area shows an example of the arrangement of the spaces and areas required.

Figure 1, schematic representation of the functional area (not to scale)

| Grenzzaun MunNdlg | Fence, ammunition storage site |
| --- | --- |
| Traverse | Barrier |
| Abstellbereich munbeladene Kfz | Parking area for ammunition-loaded vehicles |
| Munitionslagerbereich | Ammunition storage area |
| Straße | Road |
| Munitionsabstellhaus | Ready-use explosives storehouse |
| min. 50 m | min. 50 m |
| Munitionsarbeitshaus | Ammunition inspection building |

As a matter of principle, this spatial arrangement complies with demand-oriented organisational procedures for value-retaining storage, order picking, parking and maintenance, and is to be implemented as described here, if possible and taking into account the local conditions (safety distances, barriers, etc.).

- The ammunition storage area is to be connected to the road and path network. The complete ammunition storage area is to be enclosed with a ring road (see Figure 1).

- The parking area for ammunition-loaded vehicles is to be connected to the road and path network. This requires an access and an exit road (see Figure 1).

- The maintenance area is to be connected to the road and path network. In doing so, it must be ensured that the road is aligned sideways along the longitudinal axis of the building.

- The ready-use explosives storehouse is to be connected to the road and path network. It is to be located in the immediate vicinity of the ammunition inspection building, but at a minimum linear distance of 50 m.

Ammunition inspection building type – LTU “AIB”

- Annex 2 - 4_1_02_01: Munitionsarbeitshaus (Planungsunterlage)	[200 m²]

Figure 2, “Ammunition inspection building” planning document

Ready-use explosives storehouse type – LTU “ECM”

- Annex 3 - 4_1_02_02: Munitionslagerhaus (Planungsunterlage) [450 m² +29 m²]]

Figure 3, “Earth-covered magazine” planning document

Ammunition storehouse type – LTU “ECM”

- Annex 3 - 4_1_02_02: Munitionslagerhaus (Planungsunterlage)	[16.765]

Figure 4, “Earth-covered magazine” planning document

Parking area for ammunition-loaded vehicles

- Annex 4 - 4_1_02_03: Abstellfläche für munitionsbeladene Kfz		[2.508 m²]

Figure 5, “Parking area for ammunition-loaded vehicles” layout plan

| Verkehrswege | Passageways |
| --- | --- |
| Parkfläche | Parking area |

The area is used for temporary parking of ammunition-loaded vehicles and is designed for the parking of 10 vehicles.

The area is accessed and exited via separate road connections.

The area is to be protected against unauthorised access and intrusion in accordance with the definition as laid down in the security concept to be prepared.

The area is classified as a potential explosion site assuming an explosive mass of 25 to. Consequently, safety distances and protective structures (barriers) to objects to be protected must be considered. A lightning protection system in accordance with the protective angle calculation procedure as a minimum is to be considered.