The parking area is used to park ammunition-loaded vehicles.

The parking area must be protected. For this purpose, the area is to be illuminated and monitored

Based on lessons learned with regard to the short-term and high-volume provision of ammunition to requesting units, the parking area is designed for ten ammunition-loaded vehicles.

A 0,50 m wide divider strip is required between the driveways and the fence so that the road can be fully used.

A maximum explosives mass of 25 to is assumed for the parking area and the area is therefore defined as a potential explosion site. Consequently, persons and objects in the vicinity must be protected against potential dangers (fragmentation/debris effects, pressure shock, and thermal effects) caused by the potential explosion site by means of distance and/or a barrier, or potential damage must be reduced to the maximum acceptable level.

Unless otherwise specified in the future security concept, the area must be protected against

- unauthorised access,

- penetration (intrusion).

## Room layout plan

| Verkehrswege | Passageways |
| --- | --- |
| Parkfläche | Parking area |

Examples of barriers (concrete angles, as an alternative):

| Verbindungslinie | Connecting line |
| --- | --- |

| Referenzpunkt | Reference point |
| --- | --- |

## Structural and technical space requirements

| A | 2508 m² | Parking area for ammunition-loaded vehicles | 10x parking space for lorries |
| --- | --- | --- | --- |
| B | approx. 300 linear metres | Protective barrier | Protection of persons and objects against the potential explosion site |

| Item | Quantity | Specifications | Remarks |
| --- | --- | --- | --- |
| B | 1x circumferential | Configurations: gabions, reinforced concrete, masonry, or soil construction materials. Permissible explosives mass 25 to. The exact design depends on the local conditions and must be checked and/or measured on an individual basis. | To protect persons and objects against the potential explosion site of the parking area. Barriers as per local regulations |
| Floor A |  | Concrete or concrete stones | Load class 1,0 |
| Lighting A |  | Artificial outdoor illumination, at night At least 20 lx | As part of the site’s security, it must be possible to illuminate the parking area at night with a minimum of 20 lx. |
| Lightning Protection A |  | The design and scope of as well as the necessity for lightning protection is to be determined as part of a risk analysis in accordance with DIN EN 62305 “Installation of Lightning Protection”. | Lightning protection in conjunction with a lightning protection system of protection class II in accordance with DIN EN 62305-3 |
| Gate | 2 | e.g. roller gates width ≥ 3,6 m height ≥ 2,50 m incl. anti-climbing guard (3 strands of barbed wire) Incl. intrusion-resistant closing and locking technology. Protection class 5 (RC 3, high protection with anti-drill protection) mechanical lock and intrusion-resistant ES 1 class protective fitting incl. a fixed knob. | Observe requirements of the security concept to be prepared |
| Fence A | 1x circumferential | e.g.(double) bar-grate fence or wire-mesh fence incl. anti-climbing guard (3 strands of barbed wire) and alarm sensors. Height incl. anti-climbing guard 2,5 m Construction of the fence sufficiently rigid to reduce/prevent “fluttering” and thus false alarms. Fence and posts protected against corrosion. | Observe requirements of the security concept to be prepared |

## Furnishings

- Camera and video surveillance as required by the security concept

The entire parking area is to be protected by means of the above-mentioned surveillance technology and connected to the guardhouse.

- Induction loops in the roadway

Induction loops, as they are called, are to be integrated into the roadway of the parking area in order to trigger an alert at the guardhouse in case of unauthorised vehicle movements.