## Room layout plan

Fig. 1 Example: Protective roof with front supports and impact protection

Fig. 2 Example: Protective roof without front supports

Manoeuvring area

Manoeuvring area

Protective roof

Protective roof

Traffic lane

Traffic lane

Fig. 3 Example: 7 m x 15 m protective roofs, each with an 8-m deep manoeuvring area and a 4-m wide traffic lane (12 m per side)

Manoeuvring area

Protective roof

Protective roof

Traffic lane

Manoeuvring area

Fig. 4 Example: 7 m x 15 m opposite protective roofs, each with an 8-m deep manoeuvring area, with a 4-m wide traffic lane between the manoeuvring areas (20 m total)

Traffic lane

Manoeuvring area

Protective roof

Fig. 5 Example: 7 m x 15 m protective roof with a 24-m deep manoeuvring area and a 4-m wide traffic lane (28 m total) for heavy equipment transporters (HETs)

Traffic lane

Protective roof

Protective roof

Manoeuvring area

Manoeuvring area

Fig. 6 Example: 7 m x 15 m opposite protective roofs, each with a 14-m deep manoeuvring area, with a 4-m wide traffic lane in between (32 m total) for HETs

## Structural and technical space requirements:

General requirements

- Parking areas must be made of concrete so that they are able to withstand tracked vehicles turning and manoeuvring on them (suitable for use by tanks), no deformation or rutting caused by heavy-duty traffic can occur and contaminants are prevented from seeping into the soil.

- The construction of the road will be based on the Guidelines for the Standardisation of Pavement Structures for Traffic Areas (“Richtlinien für die Standardisierung des Oberbaus von Verkehrsflächen”, RStO). Typically, the road structure will be dimensioned in accordance with predefined load categories (BK). The vehicle with the highest specific ground pressure or axle loads is the 70-ton “Sattelzugmaschine 2” (SaZgM 2) truck tractor with a 70-ton “Sattelanhänger 2” (SAnh 2) semitrailer and a LEOPARD 2 A7V MBT as its load. The total weight is therefore approximately 124 t, the respective load per axle (SaZgM 2 with 4 axles, SAnh 2 with 7 axles) ranges from 9.35 t to 11.8 t.

- Trafficable drain covers must be attached in such a way that they cannot become detached – not even under the turning movements of tracked vehicles.

- Manoeuvring areas must be provided in front of all parking areas.

- Connections for trickle chargers must be provided in parking areas where defence materiel will be parked which, due to its design, may be connected to trickle charging.

- The connections for trickle chargers must be installed in such a way that they are protected from being damaged by manoeuvring vehicles.

- Power connections for air-conditioning/CBRN protective ventilation systems must be provided in the quantity required for these vehicles/devices.

- The equipment needed for trickle charging (cables, connectors, devices) will not be provided by the contractor.

- With regard to tank trucks or vehicles with tanker body, the relevant legal provisions (environmental protection regulations) must be complied with. Light-liquid separators are not required in the parking area of these vehicles as trucks are not considered a fixed installation (unlike a filling station, for example).

Special requirements

- For protective roofs that are installations within the technical area, manoeuvring areas with a depth of 8 m and at least one traffic lane with a width of 4 m are to be provided (12 m total, see Fig. 3).

- In case of opposite protective roofs, there is to be an 8-m deep manoeuvring area in front of each of them, separated by a 4-m wide traffic lane (Fig. 4, 20 m total).

- If an open space or a vehicle hall is located opposite of the protective roof, the depth of the manoeuvring area including traffic lane increases to 25 m total (2 x 10.5 m + 4 m).

- For HETs, a 24-m deep manoeuvring area and a 4-m wide traffic lane are to be provided (Fig. 5, 28 m total).

- In case of opposite protective roofs, there is to be a 14-m deep manoeuvring area for HETs in front of each of the roofs, separated by a 4-m wide traffic lane (Fig. 6, 32 m total).

Height of protective roof

The clear height of the protective roof must be at least 4.25 m in the area where vehicles are parked (upper edge of floor to lowest component of the roof structure or cable bridge for utility supply).

Protective-roof supports

The protective roof is to be designed as an open structure. If the protective roof is fitted with supports at the front, they are to be spaced at least 21 m apart, and an impact protection is to be installed in front of them. For the rear supports, impact protection (e.g. reinforced concrete) must be provided to prevent damage to the supports by manoeuvring vehicles.

Utility supply

The utility supply to every parking bay is to be implemented in such a way that it cannot be damaged by manoeuvring vehicles, e. g. as cable bridge on the protective-roof ceiling or as wall installation on the rear wall of the protective roof. The number of connections for trickle charging and power outlets will depend on the number of vehicles and equipment to be parked under the roof.

Ground under a protective roof

The ground under a protective roof must be sufficiently resistant to turning and manoeuvring tracked/wheeled vehicles as well as their loads. Among other things, it also must prevent contaminants from seeping into the soil and therefore must be made of concrete.

Ground of manoeuvring areas / traffic lanes next to a protective roof

To be adequately resilient to loads from tracked/wheeled vehicles, especially those caused by tracked vehicles turning (turning around on the yaw axis) and the high axle loads of laden HETs, the ground of manoeuvring areas and the adjacent traffic lanes must also be made of concrete. This will also prevent any contaminants from seeping into the soil.

Drains

Trafficable drain covers must be attached in such a way that they cannot become detached – not even under the turning movements of tracked vehicles.

Telephone connection

For each protective roof, a telephone connection with industrial bell is to be provided on one of the two ends.

| Designation | Qty | Specifications | Comments |
| --- | --- | --- | --- |
| Height of protective roof |  | 4.25 m | Minimum vertical clearance |
| Depth of protective roof |  | 15 m |  |
| Supports |  | Open protective-roof structure | Distance between supports at the front > 21 m |
| Impact protection |  | Reinforced concrete | For supports (front and rear) |
| Utility supply |  |  |  |
| Trickle charger socket, 250 V |  | Electrical outlets, type F (CEE 7/3 – double-pole outlets with protective earth contact, AC, 16 A, 250 V, 50 Hz) 16 A 250 V 50Hz) | Number of connections per 7 m x 15 m grid dependent on number of vehicles / equipment |
| Socket, 250 V |  | Electrical outlets, type F (CEE 7/3 – double-pole outlets with protective earth contact, AC, 16 A, 250 V, 50 Hz) 16 A 250 V 50Hz) | Number of connections per 7 m x 15 m grid dependent on number of vehicles / equipment |
| Ground under a protective roof |  | Concrete |  |
| Manoeuvring area / traffic lane |  | Concrete |  |
| Drains |  |  | Secured against unintentional loosening |
| Protective roof lighting |  | At least 15 En Lux | Nominal illuminance, 1x above each parking space |
| Lighting of manoeuvring areas |  | At least 15 En Lux | Nominal illuminance, number and spacing depending on length of protective roof |
| Light switches | 2 | No special requirements | On both ends, located under the protective roof |
| Telephone connection | 1 | Per protective roof | Located on one end, with industrial bell |