Open space is required for vehicles and equipment suitable for being parked in an open space / under a protective roof as well as for all vehicles and equipment for which parking in open spaces is specified.

Other trafficable areas are located at and around tank facilities and grease racks and are required as access paths or parking areas for vehicles and equipment using these installations.

In the area of fixed tank facilities, they must be constructed like a road with a lane on both sides of the gas station island and with a sufficient number of access points.

In the area of grease racks, they must be constructed like a road with a lane that is connected to the road and has a length of > 10 m before and after the grease rack.

## Room layout plan

Traffic lane

Manoeuvring area

Parking area (open space)

Fig. 1 Example: Open space with a grid size of 7 m x 15 m, with an 8-m-deep manoeuvring area and a 4-m-wide traffic lane

Parking area (open space)

Manoeuvring area

Traffic lane

Manoeuvring area

Parking area (open space)

Fig. 2 Example: Opposite open spaces with a grid size of 7 m x 15 m, each with an 8-m-deep manoeuvring area, separated by a 4-m-wide traffic lane

Traffic lane

Manoeuvring area

Parking area (open space)

Fig. 3 Example: Open space with a grid size of 7 m x 15 m, with a 24-m-deep manoeuvring area (for heavy equipment transporters (HETs)) and a 4-m-wide traffic lane

Parking area (open space)

Manoeuvring area

Traffic lane

Manoeuvring area

Parking area (open space)

Fig. 4 Example: Opposite open spaces with a grid size of 7 m x 15 m, each with a 14-m-deep manoeuvring area (HETs), separated by a 4-m-wide traffic lane

## Structural and technical space requirements:

General requirements

- Parking areas must be made of concrete so that they are able to withstand tracked vehicles turning and manoeuvring on them (suitable for use by tanks), no deformation or rutting caused by heavy-duty traffic can occur and contaminants are prevented from seeping into the soil.

- The construction of roads is generally based on the Guidelines for the Standardisation of Pavement Structures for Traffic Areas (Richtlinien für die Standardisierung des Oberbaus von Verkehrsflächen – RStO). Typically, the road structure will be dimensioned in accordance with predefined load categories (Belastungsklassen, BK). Main roads inside facilities, roads in the technical support/maintenance area, aprons/manoeuvring areas and brake testing tracks are subject to the heaviest loads.

- The vehicle with the highest specific ground pressure or axle loads is the 70-ton “Sattelzugmaschine 2” (SaZgM 2) truck tractor with a 70-ton “Sattelanhänger 2” (SAnh 2) semitrailer and a LEOPARD 2 A7V MBT as its load. The total weight is therefore approximately 124 t, the respective load per axle (SaZgM 2 with 4 axles, SAnh 2 with 7 axles) ranges from 9.35 t to 11.8 t.

- Trafficable drain covers must be attached in such a way that they cannot become detached – not even under the turning movements of tracked vehicles.

- Manoeuvring areas must be provided in front of all parking areas.

- Connections for trickle chargers must be provided in parking areas where defence materiel will be parked which, due to its design, may be connected to trickle charging.

- The connections for trickle chargers must be installed in such a way that they are protected from being damaged by manoeuvring vehicles.

- Power connections for air-conditioning/CBRN protective ventilation systems must be provided in the quantity required for these vehicles/devices.

- The equipment needed for trickle charging (cables, connectors, devices) will not be provided by the contractor.

- If electric vehicles are used for official purposes in the facility, a charging station (charging infrastructure) must be provided within the facility. In view of the danger arising from the rechargeable batteries of electric vehicles (battery fire), these charging stations should be located in open spaces. The charging facilities must be protected against unauthorised use.

- With regard to tank trucks or vehicles with tanker body, the relevant legal provisions (environmental protection regulations) must be complied with. Light-liquid separators are not required in the parking area of these vehicles as trucks are not considered a fixed installation (unlike a filling station, for example).

Special requirements

- Dimensions

  - depth:	15 m

- Specifications

  - Concrete structure (see first paragraph under “General requirements”)

  - Connections for trickle charging and power supply

  - Manoeuvring areas with specified depth

  - Traffic lane with a width of at least 4 m

  - Lighting of open spaces (En 15 lx) and manoeuvring areas (En 4 lx)

  - Connection of the open spaces and their manoeuvring areas to the facility’s road network

| Designation | Qty | Specifications | Comments |
| --- | --- | --- | --- |
| Depth of open spaces |  | 15 m |  |
| Utility supply |  |  |  |
| Trickle charger socket, 250 V |  | Electrical outlets, type F (CEE 7/3 – double-pole outlets with protective earth contact, AC, 16 A, 250 V, 50 Hz) 16 A 250 V 50Hz) | Number of connections per 7 m x 15 m grid dependent on number of vehicles / equipment |
| Socket, 250 V |  | Electrical outlets, type F (CEE 7/3 – double-pole outlets with protective earth contact, AC, 16 A, 250 V, 50 Hz) 16 A 250 V 50Hz) | Number of connections per 7 m x 15 m grid dependent on number of vehicles / equipment |
| Ladestationen (Ladeinfrastruktur) Elektrofahrzeuge (E-Fzg) |  |  | Anzahl/ Art Anschlüsse abhängig von Anzahl dienstlicher E-Fzg und Art Ladesäulen (AC 22 kW oder DC/HPC 50 bis 350 kW) |
| Open space ground |  | Concrete |  |
| Drains |  |  | Secured against unintentional loosening |
| Open space lighting Lighting of manoeuvring areas |  | At least 15 En Lux At least 4 En Lux | Nominal illuminance, quantity / spacing dependent on size of open spaces / manoeuvring areas |