## General construction requirements for all functional rooms and offices

| Item | Specifications |
| --- | --- |
| Walls | Neutral, light colours |
| Floor | Floors must be designed, maintained and cleaned in such a way that they can be safely used, taking into account the type of use, the operational conditions and weather influences. They must be barrier-free and level, safe to step on and anti-slip (e.g. linoleum, polyvinyl chloride). |
| Windows | During normal daylight hours, additional artificial lighting should not be required. External sunshades Internal glare protection Internal blinds Mosquito screens The surface area of the windows must be at least 10% of the room’s size. |
| Door | Must be lockable |
| Room temperature | Minimum room temperature +20 °C |
| Lighting | Minimum illuminance: 500 lx |
| IT/power supply at workstations/instructors’ stations | The following IT/power supply must be provided for each computer workstation: 1 x RJ 45 network socket with two connector sockets 1x 250 V triple socket for the power supply of IT equipment 1 x 250 V double socket to connect other office equipment Each office must be provided with a power supply of 250 V with an additional double socket each in the door and window areas. The number of computer workstations varies depending on the requested room size. |

Figure: Single computer workstation:

Electrical outlets

IT-RJ45

Electrical outlets (IT)

| Brüstungskanal | Parapet duct |
| --- | --- |

Figure: Double computer workstation:

## Special building requirements to the room

None

## Furnishings

| Item | Type of office | Furnishings |
| --- | --- | --- |
| 9 m² | Offices | - 2 x computer desk 160 x 80 x 68-76 cm - 2 x office swivel chair; arm rest - 1 x filing cabinet, top and base element, 128 x 40 cm |
| 12 m² | Offices | - 2 x computer desk 160 x 80 x 68-76 cm - 2 x office swivel chair; arm rest - 1 x filing cabinet, top and base element, 128 x 40 cm |
| 15 m² | Offices | - 2 x computer desk 160 x 80 x 68-76 cm - 2 x office swivel chair; arm rest - 1 x filing cabinet, top and base element, 128 x 40 cm |
| 21 m² | Offices | - 3 x computer desk 160 x 80 x 68-76 cm - 3 x office swivel chair; arm rest - 2 x filing cabinet, top and base element, 128 x 40 cm |
| 24 m² | Offices | - 3 x computer desk 160 x 80 x 68-76 cm - 3 x office swivel chair; arm rest - 2 x filing cabinet, top and base element, 128 x 40 cm |
| 27 m² | Offices | - 4 x computer desk 160 x 80 x 68-76 cm - 4 x office swivel chair; arm rest - 4 x filing cabinet, top and base element, 128 x 40 cm |
| 40 m² | Conference room | - 4 x computer desk 160 x 80 x 68-76 cm - 10 x office swivel chair; arm rest - 10 x upholstered chair, cloth cover, stackable - 1 x flip chart stand - 1 x whiteboard, anti-glare 3.00 m x 2.00 m (for projector) - Projector ceiling mount |

The planning and implementation of structured cabling is based on the standard series DIN EN 50173, DIN EN 50174, DIN EN 50310 and DIN EN 50346.

If the IT network of the premises is operated by the Bundeswehr inhouse company BWI GmbH, compliance with the Handbuch IT-LtgN Bw (Bundeswehr IT network manual) with regard to its planning and construction is mandatory.

Colour-coded (mostly red or orange) and provided with a label “Nur für IT” (for IT only) and the electrical circuit number.