## Room layout plan

Example

The kitchenette in the guard building is to be installed in the recreation room.

## Structural and technical space requirements

| Item | Quantity | Specifications | Remarks |
| --- | --- | --- | --- |
| Room temperature |  | At least 18° C to a maximum of 26° C | Adjustable temperature ranges |
| Lighting |  | At least 200 lx |  |
| Power supply | 6 | Type F sockets (CEE 7/3 – double-pole sockets with protective earth contact, AC, 16 A, 250 V, 50 Hz) | 2x double sockets at worktop level 1x double socket on opposite wall 1x electrical outlet next to door for general electrical supply of the room |
| Window |  | Standard size | Sufficient lighting with glare protection |
| Door |  | Interior door (standard size) |  |
| Floor |  | Medium light / medium dark | Abrasion-resistant, suitable for mopping (e. g., linoleum, light grey, marbled/textured) |
| Walls/ceilings |  | Light-coloured paint | Wipeable wall along the worktop, sink and cooktop |
| Ventilation |  |  | Via windows or through kitchen hood |
| Water supply | 1 | Cold/hot water with sink | In staff area with hot-water tank In accommodation area with instantaneous water heater |
| Wastewater disposal | 1 |  | Rinsing water/dishwasher |

## Furnishings:

| Item | Quantity | Remarks |
| --- | --- | --- |
| Kitchen unit | 1 | Approx. 100 cm x approx. 60 cm (width x depth) |
| Worktop | 1 | Washable and water-resistant |
| Sink with drainboard | 1 | Cold/hot water (5-litre open-outlet water heater) |
| Microwave | 1 |  |
| Built-in refrigerator | 1 | Volume approx. 145 litres |
| Dishwasher | 1 | Small |
| Base cabinet | 1 |  |
| Wall-mounted cabinet | 1 |  |
| Waste bin | 3 |  |
| Kitchenette equipment set | 1 |  |
| Paper towel dispenser | 1 |  |
| Soap dispenser | 1 |  |