## Structural and technical space requirements

| Item | Quantity | Specifications | Remarks |
| --- | --- | --- | --- |
| Room height |  | Minimum vertical clearance of 2.50 m |  |
| Mechanical ventilation |  | Ventilation system | Must be designed in such a way that an exhaust air volume flow of 11 m³/(h m²) is achieved. |
| Natural ventilation (window ventilation) |  | Minimum cross section, one-sided ventilation: 0.04 m2 per m2 floor space *cross ventilation: 0.024 m2 per m2 floor space | The values provided are the sum of the supply and exhaust air area. *Air vents in opposite external walls or in an external wall and the ceiling surface. |
| Humidity |  | Mechanical ventilation | An appropriate rate of supply air must be ensured. |
| Doors, windows |  |  | The ventilation of the room must be arranged in a way that prevents draughts when in use. |
| Floor drains |  | Existing floor drains must be equipped with an odour trap. Renewal of the sealing water must be ensured. | If this is not possible, an additional tap (water supply point) must be provided nearby. |
| Wastewater |  | Must be able to drain quickly and as directly as possible |  |
| Floor |  | Must be designed in such a way that moisture cannot penetrate or rise from the ground. | Must be easy to clean. Must be slip-resistant even when wet. |
| Walls |  |  | Must be easy to clean. |
| Lighting |  | Minimum value: Intensity of illumination (E): 200 lx Minimum value: Colour rendering index Ra: 80 | If mirror lighting is used, the minimum level of vertical illumination intensity is to be 500 Ix. |
| Air temperature |  | At least +24oC |  |
| Heating |  |  | Heating systems must be arranged, designed or shielded in such a way that personnel are protected from incidental contact with surfaces that are too hot. |
| Noise protection |  |  | No special requirements |
| Marking |  | Sign on the door |  |

## Furnishings:

| Item | Quantity | Remarks |
| --- | --- | --- |
| Coatrack |  | - Stable coatracks: - a total of at least 100 coat hooks (load-carrying capacity per hook at least 10 kg), - material: metal, colour: black or grey, - height of the coatrack approx. 1.80 m, - a total of at least 100 durable clothes hangers (for jackets and trousers) approx. 45.0 cm wide. |