## Structural and technical space requirements

| Item | Quantity | Specifications | Remarks |
| --- | --- | --- | --- |
| Window(s) |  | Tilt-and-turn windows with the possibility of mechanical ventilation and serving as a secondary rescue route, sufficient supply of daylight, temporary protection against direct sunlight (e.g. external sunscreen) |  |
| Ventilation |  |  | Ventilation generally via windows, additional forced ventilation via wall fan(s) |
| Room temperature |  | At least 21° to a maximum of 26° C | Adjustable temperature ranges |
| Lighting |  | At least 200 lx | Preferably installation of LED lamps |
| Noise Protection |  | Recommended noise level value ≤ 55 dB(A) |  |
| Power supply | 2 | Double socket, type F, 250 V / AC / 16 A. | In kitchen unit |
|  | 2 | Double socket, type F, 250 V / AC / 16 A. | Next to access door |
|  | 2 | Double socket, type F, 250 V / AC / 16 A. | At television set |
| Fire/smoke detection system |  |  |  |
| Door | 1 | Internal door, single-leaf, rebated, 985 mm x 2,110 mm | Barrier-free access |
| Floors |  | Level, easy to clean, elastic (e.g. linoleum or PVC) |  |
| Ceilings |  | Level, smooth, light-coloured |  |
| Walls |  | Level, smooth, light-coloured, easy to clean, wooden guard strips, approx. 20 cm wide at the level of the backrests |  |
| IT port | 3 | Multimedia socket, TV, internet, radio | At television set |

## Furnishings

| Item | Quantity | Remarks |
| --- | --- | --- |
| The following is required for each recreation room: |  |  |
| Table |  | Length: 1.6 m, width: 0.8 m, 1 x per 6 persons |
| Chair | XX | 1 x per person |
| Sink | 1 | Including a small kitchen unit |
| Water boiling appliance | 1 |  |
| Television set |  |  |