| German Joint Support and Enabling Service Headquarters | Bonn, 19 March 2024 |
| --- | --- |

Lead Document

Statement of Requirements (SOR)

Facility SOR

(Short title: LEAD SOR ZAP)

DEU Brigade in Lithuania

ZAPALSKIAI

| Prepared by: | DEU Joint Support and Enabling Service Headquarters, Planning / Organisation Division, Stationing / Infrastructure Branch Hardthöhe Fontainengraben 150 53123 Bonn kdoskbplgorgstatinfra@bundeswehr.org Tel. (Bw mil.): 90 – 3402 – 1976 Tel. (civ.): +49 (0) 228 5504 – 1976 |
| --- | --- |

# SOR Annexes

- 4_ZAP_01_Raumbuch_Verkehrswege_Lg

- 4_ZAP_02_Raumbuch_Einfriedung

- 4_ZAP_03_Raumbuch_Zaunanlage

- 4_ZAP_04_Raumbuch_Hubschrauberlandefläche

- 4_ZAP_05_Raumbuch_Betankungsanlage

- 4_ZAP_06_Raumbuch IT-Leitungsnetz

- 4_ZAP_07_Raumbuch_LZK_Geb_Etagenverteiler

# Stationed units

The following functions are required to be accommodated in the ZAP materiel storage site:

- Materiel storage site

- Ammunition storage site

- Parking areas

- Guard area

In line with current planning, the 39 soldiers belonging to the reduced logistics company (Log Coy (-)) will be stationed at the RUKLA (RUK) facility. Based on actual requirements and their respective mission, elements of that company will temporarily perform their duties on the premises of the materiel storage site.

# Introduction

The Facility SOR as the “Lead/Collective SOR” describes the general structural and constructional requirements for a facility/barracks with regard to

- Development,

- area organisation,

- the structural organisation of certain areas,

- the planned development of the facility,

- traffic facilities, roads, paths, squares and outdoor lighting,

- enclosure of the facility and fence system of the ammunition storage site,

- emergency exit,

- refuelling facility

- IT network of the facility

- the drainage of paved surfaces,

- safety distances of buildings,

- emergency power supply,

- storage of materiel and ammunition,

- commonly used buildings and areas,

- the preventive and defensive fire protection of the facility.

Following the lead of the Facility SOR, the SORs describing all buildings, areas and functions required to ensure materiel storage will be provided subsequently. The facility will be managed by the personnel of the reduced logistics company. All SORs will have room and area logbooks containing a detailed description of the structural and technical requirements.

SOR overview –-total requirements for the ZAP materiel storage site:

| Ser. no. | SOR no. | SOR title |
| --- | --- | --- |
| 1 | SOR ZAP 101 | Materiel storage site |
| 2 | SOR ZAP 102 | Ammunition storage site |
| 3 | SOR ZAP 103 | Parking areas |
| 4 | SOR ZAP 104 a and b | Guard area with main entrance |

# Functional and structural requirements

For further facility and construction planning, particularly in the context of specific infrastructure requirements, it is advised to involve DEU experts. In particular, expertise in the field of ammunition-based firing safety is to be involved. In addition, DEU security experts for requirements relating to physical protection and physical security should be involved.

## Development

Traffic accessibility must be ensured by making the traffic infrastructure suitable for heavy-duty traffic (≥ 12 tons), roadway with 2 lanes (width: 8.5 m), load category 1.8.

## Area organisation

The facility is to be planned as a military restricted area which accommodates all required functional areas (figure 1).

Figure 1, schematic layout of functional areas (not to scale and without protective distances)

| Notausfahrt | Emergency exit |
| --- | --- |
| Lagerhalle Verpflegung mit Vorfeld | Food storage bay with forecourt |
| Entsorgungs- und Umschlagstelle | Disposal and loading point |
| Lagerhalle SBH mit Vorfeld | POL storage bay with forecourt |
| Lagerhalle Sperr-BauMat mit Vorfeld | Bulky construction material storage bay with forecourt |
| Munitionslagerbereich | Ammunition storage area |
| Abstellfläche Munbeladene Kfz | Hardstand for loaded ammunition transport vehicles |
| Traverse | Traverse |
| Munitionsabstellhaus | Ready-use explosives storehouse |
| Munitionsarbeitshaus | Explosives laboratory |
| Grenzzaun MunNdlg | Fence, ammunition storage site |
| Postenhaus | Sentry box |
| Freifläche Schüttgut | Open space for bulk material |
| Betankungsanlage | Refuelling facility |
| Schutzdach Sperr-BauMat | Protective roof for construction obstacle material |
| Wache | Guardhouse |
| Fahrzeugabstellbereich | Vehicle bay |
| Grenzzaun MatNdlg | Fence, materiel storage site |
| Haupteinfahrt | Main entrance |

The facility is characterised by the spatial separation of the material and ammunition storage sites.

The materiel storage site comprises the materiel storage area and the vehicle parking area.

The ammunition storage site comprises the ammunition storage area, the maintenance area and the parking area for loaded ammunition transport vehicles.

For the vehicle parking area, the requirements of both storage sites were combined to take advantage of synergy effects, potential savings in terms of areas to be developed and the related costs as well as simplified property management.

According to the principle of functionality, the general structure of the barracks (from the inside out) will be as follows:

- Guard area with main entrance and exit,

- vehicle parking areas (hangar, protective roof, open space),

- refuelling facility, firefighting water supply, fire protection facilities, if required, technical facilities, utilities,

- material storage area,

- boundary to the internal ammunition storage site (as ‘ammunition and explosives area’),

- ammunition maintenance area,

- hardstand for loaded ammunition transport vehicles,

- ammunition storage area.

All areas of the facility described under items 1 to 8 must be accommodated inside ONE contiguous facility. The different areas are physically separated inside the facility.

Besides considering the local situation of the respective functional areas, quantity distances between potential explosion sites and exposed sites must be defined. In addition, a safe area must be defined for the facility. For this purpose, the Ammunition Safety Officer must be involved in the planning of the site.

As a first concept for the construction layout plan or the quantity distance relations, the following is assumed

- 3x ammunition magazine, earth covered (ECM) (side, roof, rear side), hazard class 1.2 with 2.7 tons explosive quantity (5.38 kg/piece x 500 pieces) per ECM, potential explosion site b1 290 m distance to civilian areas, 225 m to military areas outside the facility, 2 m to other ECMs (to the side), 25 m to other ECMs (behind each other), 135 m to other buildings within the facility, 290 m to the refuelling facility and the materiel storage area

- 32 ammunition magazines, hazard class up to 1.3 with 50 tons explosive quantity per magazines, potential explosion site b1 240 m to civilian/military areas outside the facility, 160 m to military areas within the facility, 2 m to other ammunition magazines to the side), 25 m to other ammunition magazines (behind each other), 240 m to the refuelling facility and the materiel storage area

- explosives workshop (ammunition surveillance) with barricades, hazard class up to 1.2, free-standing, cubic, solid construction with up to 6 tons explosive quantity, potential explosion site d   (depending on the orientation of the exposed sites) up to 135 m distance within the facility and 255 m to military areas outside the facility and 330 m to civilian areas outside the facility, 330 m to the refuelling facility and to the materiel storage area.

- hardstand for loaded ammunition transport vehicles, open spaces with barricades, hazard class 1.2 with up to 25 tons explosive quantity, potential explosion site d  (depending on the orientation of the exposed sites) up to 135 m distance within the facility and up to 425 m outside the facility and 425 m to civilian areas outside the facility, 425 m to the refuelling facility and to the materiel storage area.

The following schematic diagram is intended to indicate the need for quantity distances and to illustrate quantity distance relations (assumption/draft) with resulting area requirements.

Figure 2, schematic layout of functional areas with quantity distances (not to scale)

| Notausfahrt | Emergency exit |
| --- | --- |
| Lagerhalle Verpflegung mit Vorfeld | Food storage bay with forecourt |
| Entsorgungs- und Umschlagstelle | Disposal and loading point |
| Lagerhalle SBH mit Vorfeld | POL storage bay with forecourt |
| Lagerhalle Sperr-BauMat mit Vorfeld | Construction obstacle material storage bay with forecourt |
| Munitionslagerbereich | Ammunition storage are |
| Abstellfläche Munbeladene Kfz | Hardstand for loaded ammunition transport vehicles |
| Traverse | Traverse |
| Munitionsabstellhaus | Ready-use explosives storehouse |
| Munitionsarbeitshaus | Explosives laboratory |
| Grenzzaun MunNdlg | Fence, ammunition storage site |
| Postenhaus | Sentry box |
| Mind. 50 m | at least 50 m |
| Freifläche Schüttgut | Open space for bulk material |
| Betankungsanlage | Refuelling facility |
| Schutzdach Sperr-BauMat | Protective roof for construction obstacle material |
| Wache | Guardhouse |
| Fahrzeugabstellbereich | Vehicle bay |
| Grenzzaun MatNdlg | Fence, materiel storage site |
| Haupteinfahrt | Main entrance |

## Facility development and enlargement and reserved areas

Detailed planning has not yet been completed. Current situation developments must be considered and may lead to further requirements in terms of storage and parking areas. This must be taken into account in the development of the usable areas to be identified by LTU.

As part of the development planning for the facility, 20% of the overall area are to be designated as enlargement and reserved areas to meet future requirements.

## Firefighting water supply and fire brigade movement areas.

Defensive fire protection provisions and the basic planning of the facility are to be drawn up at the same time. The available forces and assets of the responsible fire brigade must be considered when designing buildings, installations and the firefighting water supply. Current defensive fire protection capabilities determine the structural design.

## Traffic facilities, roads, paths, squares and lighting

The respective infrastructure will be described in 4_ZAP_01_Raumbuch_Verkehrswege_Lg. (“Traffic routes” room and area logbook).

## Surface drainage

Drainage comprises all structural and natural ways of keeping or draining surface water from traffic infrastructure.

The following areas can be taken as a rough guideline for the number of road drains:

- Roads: approx. 400 m²/drain

- Parking areas / temporary storage and provisioning areas / forecourts approx. 500 m²/drain

## Safety distances

The described minimum distances are to inhibit or limit any direct outside influence on buildings and physical structures of the facility. Safety distances support military security measures and facilitate the guarding of the facility.

| Number | Building/building type | Minimum distance to the fence |
| --- | --- | --- |
| 1 | Guard buildings and facilities of the guard area | 3 m |
| 2 | HQ buildings and other office buildings | 50 m |
| 2 | Special use buildings (special areas) | 30 m |
| 3 | Storage buildings/spaces of the storage area | 30 m |
| 4 | Fuel storages and tankage facilities | 30 m |
| 5 | Workshop buildings | 30 m |
| 6 | Parking area for vehicles, major equipment; washing hangars, heated hangars and aprons | 30 m |
| 7 | Ammunition storage sites | 35 m |
| 8 | Central supply systems, alternate and emergency power systems air intake openings, shaft heads and emergency exits of protective shelters and communications centres, cable entry shafts for communications centres | 30 m |
| 9 | Transition from an aboveground powerline to underground cables inside the facility | 30 m |
| 10 | Traffic infrastructure inside the facility | 5 m |
| 11 | Parking areas | 5 m |
| 12 | Any other buildings and installations | 10 m |
| 13 | Helicopter landing site | 30 m |

## Fence system - enclosure of materiel storage site and fence system of ammunition storage site

The enclosure of the ZAP materiel storage site is different from the integrated fence system of the ZAP ammunition storage site.

The enclosure and the fence system will be described in „4_ZAP_2_Raumbuch_Einfriedung“ (“Enclosure” room and area logbook) and „4_ZAP_3_Raumbuch_Zaunanlage“ (“Fence system” room and area logbook)

## Emergency exit

An emergency exit is required to enable personnel to quickly leave the facility in case of an alert. The emergency exit must be connected to the public road network. If possible, the emergency exit is to be located in the part of the facility opposite the main entrance.

This results in the following structural and technical requirements:

- Road surface of asphalt or concrete paving blocks, load category 1,0

- Roadway width: 4.00 m,

- Cantilever sliding gate (figure 3):

  - electromechanical drive,

  - opening / closing speed between 0.25 m/s and 0.5 m/s,

  - hand-operated emergency opening function in case of power or drive failure,

  - gate must cover the entire opening (no gaps!),

  - connection to the technical surveillance system (intrusion detection system),

  - certified protection properties and installation for:

    - Resistance to burglary IAW DIN EN 1627,

    - resistance to thrown objects IAW DIN EN 356,

    - resistance to penetration IAW DIN EN 356,

  - possibility of fitting an additional lock or padlock

Figure 3, cantilever sliding gate

## Storage of materiel and ammunition

Infrastructure requirements will be described in „SOR_ZAP_101“ and „SOR_ZAP_102“ .

## Commonly used areas

      - Parking areas for military vehicles

The required parking areas in/under

hangars

protective roofs

open spaces

will be described in „SOR ZAP 1_1_03_Abstellbereiche“.

      - Helicopter landing area

The respective infrastructure will be described in Raumbuch „4_ZAP_04_Raumbuch_ Hubschrauberlandefläche“ (“Helicopter landing site” room and area logbook). A helicopter landing site inside the facility does not require any additional guarding or security and may be used at any time and without much organisational effort (protests outside the facility).

A backup helicopter landing site outside the facility is possible, but it must be in the immediate vicinity of the facility.

      - Set-up area for flagpoles

Erect flagpoles with a height of 8 m are to be placed at prominent locations near the main entrance and inside the facility, e.g. in front of the guard building and the headquarters building. The flagpoles must be of weather-resistant material.

The flagpoles near the main entrance must be protected against flag theft (e.g. cleat cover box with lock).

All flagpoles will be provided with a 0.80 m wide access path.

| - Quantity: | Guard area | 5x |
| --- | --- | --- |
| - Distance/spacing |  | 3.50 m |
| - Height: |  | 8.00 m |
| - Diameter: |  | 0.10 m |

# General requirements for the operation of the facility

The operation of the facility is the responsibility of the host nation LTU.

The utility supply of the facility, of all buildings and areas (power, water, IT, wastewater disposal, drainage) is to cater the entire facility and is NOT part of this requirement.

      - Air conditioning of the guard building

Unless required otherwise, the room climate in these areas should ensure favourable conditions with a maximum temperature of 26 °C. If this cannot be achieved by means of passive air-conditioning, technical measures must be taken.

For room climate requirements for the functional infrastructure refer to the respective SOR and/or room and area logbooks.

      - Power supply

The facility is supplied via the public grid.

As part of the planning steps, the planning office has to define the power requirement.

An uninterruptible emergency power supply (in case of a power failure) is required for the entire security technology and infrastructure (intrusion detection systems, video/camera surveillance, guard building, lighting, ammunition magazines, gates). Emergency power supply is to be guaranteed for at least 36 hours.

In the event of a power supply failure, it is required that power will be restored within 36 hours.

The functionality of fire protection facilities must be ensured in the event of a power failure.

      - IT network/facility IT access node

The planning and implementation of structured cabling is based on the standard series DIN EN 50173, DIN EN 50174, DIN EN 50310 and DIN EN 50346.

If the IT network of the premises is operated by the Bundeswehr inhouse company BWI GmbH, compliance with the Handbuch IT-LtgN Bw (Bundeswehr IT network manual) with regard to its planning and construction is mandatory.

All requirements relating to the IT network, the IT access node and the floor and building distributors of the facility will be described in the annexes

- 4_ZAP_06_Raumbuch_IT-Leitungsnetz (“IT network” room and area logbook)

- 4_ZAP_07_Raumbuch_LZK_Geb_Etagenverteiler (“Building and floor distributor” room and area logbook)

.

In this case, the BWI is responsible for the facility IT access node. It is a sensitive point and must therefore be located inside the facility and protected against sabotage.

The room and area logbooks of the different SORs describe the required IT equipment but do not specify the structural cabling/IT network inside the building.

      - Utilities/building service equipment rooms

The buildings are connected to the utility supply and disposal of the facility via the utilities/building service equipment rooms. This is where the inside connecting lines and operating equipment for the water supply and wastewater disposal, gas or district heating supply, power and IT supply as well as the fire protection facilities are located.

In the utilities room, all utility supply equipment is installed IAW operator requirements to allow for safe operation, maintenance and repair; including metering equipment.

To keep connecting lines short, utilities rooms are preferably located in the basement/on the ground floor of a building facing the street. These rooms are to be dry and accessible from the outside (usually no walk-through rooms), not accessible for unauthorised persons but for recipients and users and the fire brigade at any time.

      - Operational services for the facility

The following services are requested as part of Host Nation Support (HNS):

- Maintenance/servicing of the technical components as well as the associated equipment and devices

- Building maintenance of all objects on site

- Cleaning of offices, common rooms, corridors and sanitary facilities in all buildings (cleaning service)

- Waste disposal:

    - General waste disposal, all buildings

    - Hazardous waste

    - Bulky waste

    - Classified waste material

    - Disposal of recyclable material at the collection point

- Disposal of waste material produced during weapons cleaning

    - MWR area, PX, dining facility, etc.

- Grease separator, dining facility ventilation

- Kitchen waste and reusable materials

- Drinking water supply

- Firefighting water supply

- Wastewater disposal

- Power supply, 250 V/400 V 50 Hz mains voltage

- Building automation

- Fire detection and fire alarm systems (DIN EN 54 series)

- Extinguishing equipment (fire extinguishers, risers)

- Earthing systems, lightning protection systems

- Exercise central responsibility for the operation and assumption of all related traffic safety obligations of the facility, including provision of a central point of contact

- Facility management, including winter maintenance services

      - Other overarching requirements/constraints:

- Barrier-free access routes to the ground floors of all buildings of the facility:

    - No steps or edges

    - No winding access routes – this needs to be resolved by means of area management

    - Identification signs / exterior lighting for buildings

- Building automation (monitoring system for operation)

    - Climate control (temperatures must not be below -20 °C or higher than 40 °C, air humidity must not exceed 65%)

    - Fire detection and fire alarm systems

    - Firefighting equipment (smoke extraction system, possibly additional extinguishing systems)

    - Accommodation and functional buildings are to be equipped with fire protection systems

- Electrical installation / power and emergency power supply of the facility

    - Establishment/operation of the power distribution network

    - Uninterrupted distribution of electric energy to consumers with constant quality

    - Interference suppression

    - Low-voltage and weak-current installations

    - Emergency power supply – medical centre/pharmacy

    - Electrical systems must include a way to de-energise them

    - Locking system controls

- Orientation systems and signposts

    - Escape and rescue routes must be signposted inside the facility

    - Exits, escape and rescue routes must be marked with emergency signs

    - Security lighting is to be installed along escape routes

    - Personnel assembly points

- Compliance with occupational safety and health as well as environmental protection regulations