Traffic routes include all roads and paths of the facility.  Paved areas (such as hardstands) and squares (parade grounds, parking areas) are not included in this document as they are described in separate room and area logbooks.

## Sample images

Fig. 1 Example: Roadway with sidewalk and lighting

Fig. 2 Example: A footpath connecting a sports facility to the road/pathway network of the facility

## Requirements outline

Roads

Roads will be used by all vehicles stationed in the facility. Their purpose is to:

- Connect Bundeswehr facilities to the public road network;

- Connect different areas within a Bundeswehr facility;

- Make buildings, facilities and installations of the site accessible;

- Allow test drives as part of technical inspections in the technical support/maintenance area;

Roadways

Space and area requirements for roadways in Bundeswehr facilities are described in Fig. 4. The decision to have roadways with a width over 4.75 m is taken by the FMoD on a case-by-case basis. Inside the facilities, roads with one or two lanes are required to make buildings, infrastructure and installations accessible.

Roads are to be laid out in such a way that buildings, infrastructure and installations of the facility are made accessible as directly as possible and that the aprons in the technical support/maintenance area can usually be connected at a right angle.

Sidewalks

Sidewalks are for pedestrian traffic only. They must not be used by motor vehicles. They are to be placed along roadways wherever it would be irresponsible for pedestrians to use the road for safety reasons.

Specific requirement for sidewalks (winter maintenance has been taken into account):

| - Sidewalk width: | 1.6 m main and secondary sidewalks |
| --- | --- |

Sentry paths

Sentry paths are to be laid out so as to possibly avoid illuminated areas and areas without vegetation and offer vantage points for observation of the fence.

Specific requirements for sentry paths:

| - Sentry path width: | 0.75 m |
| --- | --- |
| - Vertical clearance above sentry paths: | 2.5 m |

Sentry paths are to be laid out so as to allow guard personnel to move as noiselessly as possible.

Exterior lighting systems

Outdoor lighting fixtures must be fitted and arranged so that their maintenance (servicing, inspection and repair) and the replacement of light bulbs can be performed easily. All lighting systems must use state-of-the-art energy-efficient lamps.

Exterior lighting systems include:

- Road lighting

- Lighting of workplaces, hardstands and parking areas

- Installation defence lighting.

The following guide values for lighting are required:

| Area type | Nominal illuminance EnLux |
| --- | --- |
| Storage space for empty cans | 15 |
| Aprons of storage houses | 15 |
| Transshipment point | 30 |
| Open spaces, storage areas, loading areas | 15 |
| Loading ramps | 100 |
| Pump area of gas stations | 100 |
| Messing facility yard | 15 |
| Guard area (vehicle and identification checks) | 100 |
| Protective roofs | 15 |
| Facility traffic routes | 5 |
| Fence and apron lighting | 4 |

Fig. 3 Example: Nominal illuminance of different areas

Exterior lighting systems should be combined wherever functional and cost-effective; e.g. road, parking area and apron lighting.

Road lighting

The road and parking area lighting of the facility improves traffic safety. It is to be controlled automatically via a daylight control but must also be switchable manually by the guards. Lampposts are to be installed at a distance of at least 1 m from the kerb.

Lighting of workplaces, hardstands and parking areas

The lighting of workplaces, hardstands and parking areas helps prevent accidents. The following areas require illumination:

- Entrance areas of accommodation buildings and headquarters

- Aprons/forecourts

- Transshipment points

- Utility building/messing facility yards

- Parking areas

- Open storage spaces

- Above-ground fuel storage facilities.

Workplace and hardstand lighting must be switchable manually both on site and by the guards. Parking area lighting will not be switchable on site. The following areas do not require illumination:

- Parade grounds

- Grounds for basic training

- Sports facilities

- Obstacle courses.

Deviations from these requirements are to be justified in detail and will be handled by the FMoD on a case-by-case basis.

Installation defence lighting

Installation defence lighting is part of military security and must be planned in line with the specifications in the “Guard building” room and area logbook.

## General construction requirements

Road infrastructure

The traffic load/traffic load ratio pursuant to the Guidelines for the Standardisation of Pavement Structures for Traffic Areas (Richtlinien für die Standardisierung des Oberbaues von Verkehrsflächen – RStO) determines the load category of road infrastructure (paved areas).

The traffic load ratio (heavy-duty traffic) on roads inside military facilities is between 10 and 60 vehicles/24 h.

Roads, trafficable areas and aprons in the technical support/maintenance area have to withstand heavy loads (heavy vehicles, tracked vehicles, etc.). The requirement for these areas is asphalt or concrete paving with a wear-resistant surface for roads and trafficable areas outside of gas stations.

The following load categories (Belastungsklassen – BK) are applicable in Bundeswehr facilities (see Fig. 9 and 10):

- BK 1.0 (normal case)

- BK 1.8 (heavy loads)

- BK 0.3 (light loads).

Roads outside of military facilities must be suitable for use by military vehicles (incl. heavy vehicles).

Paved areas usually do not have a kerb and are sloped just like the road base.

This applies with the exception of cases where kerbs have a particular purpose, e.g. on bridges, hardstands, parking areas, aprons, traffic islands or sidewalks along the road.

Edges

The kerb between the road and the sidewalk is to be dropped to a maximum height of 3 cm where required (e.g. crossroads and access roads). Sloping kerbs are required for the main entrance and exit area.

Drainage

For the trafficable areas in the accommodation and the technical support/maintenance areas – roads, aprons, hardstands, gas stations/filling areas, storage spaces and other paved areas – drainage measures in line with water rights and environmental protection regulations pursuant to the respective authorised use are required.

Drainage comprises all structural and natural ways of keeping or draining water from traffic infrastructure.

All effects expected to impair the usability and survivability of a road caused by surface water or water beneath ground must be borne in mind during planning.

This includes ensuring that water is absorbed and drained without causing any damage until it enters the drainage ditch.

The following areas can be taken as a rough guideline for the number of road drains:

- Roads: approx. 400 m²/drain

- Parking areas: approx. 500 m²/drain

- Basic training/parade grounds: approx. 750 m²/drain.

As a rule, water permeable traffic areas require a surface runoff. Drainage areas may be larger depending on the water permeability of the surface course.

If possible, storm sewers should not be located on the roadway.

+ Minimum diameter nominal size 250 mm

In technical support/maintenance areas, where waste water polluted with hydrocarbon is generated, light liquid separators must be installed.

National water laws and provisions by responsible water authorities

must be observed.

Seepage of precipitation water

The goal is to minimise surface sealing and to allow the local, large-scale seepage of surface water. In doing so, national and local water rights must be observed.

Lawn and planting

Green areas are to be planned in such a way that roots cannot damage paved areas or underground piping. Twigs, branches or shrubs must not extend into the clearance gauge. When planning green areas, cost-effective maintenance is an important factor to keep in mind. What is more, encroachments on the landscape require ecological compensatory measures.

Other equipment

Guidance equipment (marker posts, side rails, etc.) must only be installed in duly justified cases and if traffic safety so requires.

Should guardrails become necessary, they are to be provided IAW the Guidelines for Passive Roadside Protection Using Vehicle Barriers (Richtlinien für passiven Schutz an Straßen durch Fahrzeug-Rückhaltesysteme).

Traffic and road signs must comply with the German road traffic regulations (Straßenverkehrsordnung - StVO). If public roads are affected, any measures must be coordinated with the responsible traffic authorities. Handicap-accessible facilities (e.g. disabled access/parking/toilet) must be indicated.

## Specific individual construction requirements

Road widths/parking strip and shoulder dimensions

As a general rule, roads are to be designed to allow fire brigade use at any time. Bend radiuses are to be defined based on the vehicle with the largest turning radius (e.g. “Sattelzugmaschine 2” (SaZgM 2) truck tractor with a “Sattelanhänger 2” (SAnh 2) semitrailer). The following table specifies road widths as well as parking strip and shoulder dimensions inside Bundeswehr facilities.

| Bw facilities | Type of road | Roadway widths incl. shoulder (required paved width) |  |  | Parking/stopping strip | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Roadway with 1 lane | Roadway with 2 lanes | Roadway width incl. passing places |  |  |
| Troop housing/camp | Main thoroughfare (from main entrance) |  | 6.5 m |  | 2.3 m (cars) 3 m (trucks) |  |
|  |  |  | 8.5 m |  | 2.3 m (cars) 3 m (trucks) | Tracked vehicles (troop camp) |
|  | Roads in the storage and technical support/maintenance areas | 4 m | 7 m |  | 3 m (trucks) |  |
|  |  | 4.75 m | 8.5 m |  |  | Mainly tracked vehicles and/or heavy vehicles |
|  | Roads in headquarters and accommodation areas and in other areas | 4 m | 6 m |  | 2.3 m (cars) 3 m (trucks) |  |
|  | Brake testing track |  | 7 m |  |  | 2 x 1.5 m sidestrip |
|  | Main entrance | 2 lanes with 3.5 m each |  |  | 10 m x 2.3 m for passenger cars 25 m x 3 m for trucks | Positioning of the stopping strips IAW infrastructure requirement |
|  |  | 2 lanes with 4 m each |  |  | 10 m x 2.3 m for passenger cars 25 m x 3 m for trucks | Mainly tracked vehicles and/or heavy vehicles, positioning of the stopping strips IAW infrastructure requirement |
|  | Secondary entrance |  | 7 m |  |  |  |
|  |  |  | 8.5 m |  |  | Mainly tracked vehicles and/or heavy vehicles |
|  | Emergency exit | 4 m |  |  |  |  |
|  |  | 5 m |  |  |  | Mainly tracked vehicles and/or heavy vehicles |
| Depot installations | Connecting roads |  | 6.5 m |  | 30 m x 3 m (inside) | Positioning of the stopping strips IAW infrastructure requirement |
|  | Camp roads | 4 m |  |  |  |  |
|  | Main entrance |  | 6.5 m |  | 120 m x 3 m for trucks (outside) | Positioning of the stopping strips IAW infrastructure requirement |
|  | Emergency exit | 4 m |  |  |  |  |
| Administrative premises / naval base | Main and connecting road | 4 m | 6.5 m |  | 2.3 m (cars) 3 m (trucks) |  |
| Garrison ammunition storage site | Road inside | 4 m | 6.5 m |  |  |  |
|  | Road in the ECM area | 4 m |  |  |  |  |
|  | Main entrance Emergency exit | 4 m | 6.5 m |  | 150 m x 3 m 200 m x 3 m (outside) | Up to 360 m² ECMs > 360 m² ECMs (ECMs = earth covered magazines) |
| Airfield | Connecting road | 4 m | 6.5 m |  |  |  |
| Training areas | Ring roads |  | 7 m |  |  |  |
|  |  |  | 9.5 m |  |  | Tracked vehicles |
|  | Connecting roads | 4 m |  | 8 m |  |  |
|  |  | 4.75 m |  | 9.5 m |  | Tracked vehicles |
|  | Other roads | 4 m | 6 m |  |  |  |
| Garrison ranges |  | 4 m | 6 m |  |  |  |

Fig. 4 Example: Overview of road widths and parking strip/shoulder dimensions

Roadways that are not confined by kerbs have two lanes and a 0.5 m-wide shoulder on both sides. Bundeswehr traffic areas generally do not have markings on the shoulders. Sidestrips must be stable (e.g. gravel lawn). Their width usually is 1 m. Unstable sidestrips may only be chosen in exceptional cases.

Road structure

The construction of roads is generally based on the Guidelines for the Standardisation of Pavement Structures for Traffic Areas (Richtlinien für die Standardisierung des Oberbaus von Verkehrsflächen – RStO). Typically, the road structure will be dimensioned in accordance with predefined load categories (Belastungsklassen, BK).

Main roads inside facilities, roads in the technical support/maintenance area, aprons/manoeuvring areas and brake testing tracks are subject to the heaviest loads.

The vehicle with the highest specific ground pressure or axle loads in the materiel storage area is the container handler (assumption: Hyster/Reachstacker RS46-33XD/62) with a net weight of 77 tons and a useful load of up to 35 tons. The total weight is therefore approximately 112 tons, the respective load per axle (front) with a useful load of 35 tons is 102 tons (see Fig. 5).

Figure 5, example: Hyster/Reachstacker

For the ammunition storage site, the weight as well as the load per axle of COTS trucks is to be assumed to be ≥15 tons.

Concrete surfaces used by tracked vehicles will not be opened to traffic until at least 90% of the required compressive strength of the concrete has been reached.

Provisions for turning

Provisions for turning (turning areas/circles/loops) are to be made at the end of dead-end-streets. Bend radiuses are to be defined based on the vehicle with the largest turning radius.

Sidewalks

Walkways to entrance areas, hardstands for waste containers, dog kennels, antenna supports, flagpoles, etc. will be built in accordance with this regulation. For dropped kerbs, the “Edges” section (p. 5) applies.

The width of sidewalks usually is 1.6 m (winter maintenance). In general, the vertical clearance above sidewalks is 2.5 m. In case a sidewalk is directly adjacent to a parking area with 90-degree parking, the overhang is not to be included in the sidewalk width.

The transversal gradient of sidewalks should normally be 2% (barrier-free construction

- planning concepts 2013) but not less than 1%. The longitudinal gradient should usually be ≤ 3%.

The construction of sidewalks is based on the Guidelines for the Standardisation of Pavement Structures for Traffic Areas (Richtlinien für die Standardisierung des Oberbaus von Verkehrsflächen – RStO). Concrete paving stones

or interlocking blockstones are possible pavement structures. Edgings are usually required.