## Room layout plan

Sample images:

Figure 1: safety bar grating fence with anti-climbing guard and digging protection

| Zu schützender Bereich (Innen) | Area to be protected (inside) |
| --- | --- |
| Angriffsrichtung | Direction of attack |
| H ≥ 2,50 m | Height ≥ 2.50 m |

Figure 2, digging protection as prefabricated reinforced concrete components

Figure 3, anti-climbing guard

| ID no. | Item |  |
| --- | --- | --- |
| A | Safety fence | Enclosure elements |
| B | Digging protection |  |
| C | Anti-climbing guard |  |

## Structural and technical space requirements

This room and area logbook only describes requirements for the facility enclosure, i.e. the external fence system of the ZAP facility. Requirements regarding the main entrance can be found in the “SOR Wachgebäude mit Haupteinfahrt” (Guard building with main entrance SOR).

The fence system of the facility is intended to prevent unauthorised access and damage and is to be constructed as what is referred to as “security fence with anti-climbing guard and digging protection” (digging with manual digging tools).

There must be clear zones with a width of at least 5 m on both sides of the enclosure.

A trafficable sentry path (single-lane, width 2.5 m, weight ≤ 3.5 tons) shall be provided within a distance of at least 5 m from the enclosure.

The fence system shall be provided as panel mesh fencing or double horizontal wire welded mesh panel.

For panel mesh fencing, the following applies:

- Height (fence element): 3.0 m maximum

- Width (distance between posts):2.50 m maximum

- Horizontal bar (top and bottom): at least 50 mm x 30 mm x 2.5 mm (in case of a paling fence design),

- continuous frame: at least 60 mm x 40 mm x 2 mm (in case of a frame design),

- filling (vertical rods):

  - Round rods, diametre at least 20 mm; clear distance 150 mm maximum or

  - square pipes, at least 30 mm x 20 mm x 2 mm, clear distance 150 mm maximum.

- Fence posts:

  - Solid profile: min. 60 mm x 60 mm or

  - Hollow profile (square tube): min. 100 mm x 100 mm x 4 mm

  - Top end of posts is to be covered (e. g. with post caps made of aluminium or plastic)

- The fence elements are to be fastened by means of U-bolts or through bolts.

For fences made of double horizontal wire welded mesh panel:

- Mesh panels: - Length (height of fence element): max. 2.50 m, - width (distance between posts): max. 2.60 m, - design: at least 8/6/8 vertical wire, diametre ≥ 8 mm, horizontal wire, diametre ≥ 6 mm, - mesh size: W x H = max. 50 mm x 200 mm - Fence posts: - Solid profile: min. 60 mm x 60 mm or - Hollow profile (square tube): min. 60 mm x 40 mm x 2 mm - Top end of posts is to be covered (e. g. with post caps made of aluminium or plastic) - Fence elements are to be fastened by means of U-bolts or through bolts. For the digging protection, the following applies: - Reinforced concrete construction, - firmly connected to the fence system, - depth of digging protection and / or foundation ≥40 cm below ground level, - distance between top edge of digging protection and fence system ≤ 50 mm, - full coverage without gaps, centrally aligned to the fence (centric) For the anti-climbing guard, the following applies: - Overhang systems must always curve outwards. - The height of the overall system (enclosure incl. anti-climbing guard) shall be at least 2.50 m. - Climbing aids (e.g. transformer stations, power distribution boxes, bus stops) in the immediate vicinity must be avoided as far as possible, otherwise additional measures must be taken. - The area must be kept clear from vegetation (e.g. branches hanging over the fence which could be used as climbing aid). - The design of the anti-climbing guard must not favour climbing (e.g. ease of attaching ladders). - When using barbed wire (tape), a minimum of three strands of barbed wire shall be used; the distance between individual strands shall not exceed 150 mm. - A simple strand of barbed wire is not suitable as anti-climbing guard for security fences..

## Furnishings

n/a