## Room layout plan

Sample images:

Figure 1, wire mesh fence with anti-climbing guard

Figure 2, opening for small animals in a wire mesh fence

Figure 3, fence system with anti-digging protection

| OK Gelände | Ground level |
| --- | --- |
| Unterkriechgrenze | Level down to which crawling underneath the fence is impossible |

Figure 4, anti-climbing guard

Figure 5, barrier system with a roadway width of 4 m

Figure 6, hand operated barrier

| Balken (Schrankenarm) | Beam (barrier arm) |
| --- | --- |
| Drehbare Lagerung | Bearings |
| Griff zur Handbetätigung | Handle for manual operation |
| Auflager | Support |
| Gegengewicht | Counterweight |
| Fahrbahn | Roadway |
| Fundament | Foundation |

Figure 7, sentry box

| Einfahrt | Entrance |
| --- | --- |

| ID no. | Item |  |
| --- | --- | --- |
| A | Safety fence | Fence system as enclosure of ammunition storage site |
| B | Digging protection |  |
| C | Anti-climbing guard |  |
| D | Entrance and exit of ammunition storage site with sentry box |  |

## Structural and technical space requirements

The fence system is intended to prevent unauthorised access and damage to the ammunition storage site within the ZAP facility and is to be constructed as wire mesh fence with anti-climbing guard, digging protection (preventing game from entering the premises) and an opening for small animals (internal fence system).

For the wire mesh fence (A), the following applies:

Fence posts

- Hot-galvanised or alloy steel or light metal posts,

- maximum pole spacing 3 m,

- frost-proof foundation,

- top ends of posts to be covered,

- bridges, cross bracings or the like to reinforce fence elements as required.

Fencing wire mesh

- Height at least 2 m,

- nominal mesh size: 50 mm maximum,

- nominal wire diametre: 2.8 mm minimum,

- wire tensile strength: >600 N/mm²,

- corrosion prevention,

tension wires,

- nominal wire diametre: 3 mm minimum,

- galvanised wire tensioners,

- at least four tension wires at equal intervals.

For the digging protection (preventing game from entering the premises) (B), the following applies:

- Burying of steel wire mesh with hexagonal meshes,

- mesh size ≤ 50 mm, regularly twisted,

- wire diametre ≥ 2.0 mm, diametre of outer wires ≥ 2.4 mm,

- vertically buried at least 40 cm below ground level,

- zin alloy.

For the anti-climbing guard (C), the following applies:

- Overhang systems must always curve outwards.

- The height of the overall system (enclosure incl. anti-climbing guard) shall be at least 2.50 m.

- Climbing aids (e.g. transformer stations, power distribution boxes, bus stops) in the immediate vicinity must be avoided as far as possible, otherwise additional measures must be taken.

- The area must be kept clear from vegetation (e.g. branches hanging over the fence which could be used as climbing aid).

- The design of the anti-climbing guard must not favour climbing (e.g. ease of attaching ladders).

- When using barbed wire (tape), a minimum of three strands of barbed wire shall be used; the distance between individual strands shall not exceed 150 mm.

- A simple strand of barbed wire is not suitable as anti-climbing guard for security fences.

For the entry and exit of the ammunition storage site (D), the following applies:

- Single-lane (narrowing), roadway width of 4 m, road surface made of asphalt or concrete pavement, load category 1.0 (see figure 5),

- hand-operated barrier (see figure 6), consisting of:

  - Barrier arm (beam or lifting barrier) to open and close the passage which blocks the road to the required width,

  - support on one or both sides of the roadway to carry the barrier arm and any occurring (static and dynamic) loads,

- lighting of entrance and exit area and forecourts with 100 lx.

For the sentry box (D), the following applies:

- To be installed at the right side of the entrance,

- light structure as a shelter against the weather up to 8 m² maximum with lockable door, door to be opened from outside only with key, fixed window (parapet height 80 cm above finished floor level = height of worktable)

- Worktable as fixed worktop (height: 80 cm, depth: 45 cm),

- a skylight as bottom-hinged sash for ventilation,

- rifle rack for one rifle,

- shelf with coat hooks,

- 1x double socket 230 V / 16 A,

- 1x telephone extension (fixed connection)

## Furnishings:

- Provide chair for sentry box