## Room layout plan

| 950 Liter | 950 l |
| --- | --- |

## Structural and technical space requirements

| A | 25 m² | Refuelling facility | 2x fuel tank, 950 l |
| --- | --- | --- | --- |

The facility serves to refuel fuel-operated materials handling equipment used in the materiel storage site. Accordingly, the floor shall have an anti-slip, oil / acid / alkali resistant, impermeable and non-absorbent surface.

| ID no. | Item | Quantity | Specifications | Remarks |
| --- | --- | --- | --- | --- |
|  | Lighting |  | At least 150 lx | Lighting must be explosionproof. |
|  | Floor |  |  | anti-slip, oil /acid /alkali resistant, impermeable and non-absorbent Load-bearing capacity: ≥ 20 kN/m² |
|  | Walls/ceilings |  | Light structure as a shelter against the weather, open to one side (refuelling side), floor-to-ceiling height: ≥ 4.25 m | Tanks must be protected against precipitation and sunlight. |

## Furnishings:

N/a