Annex 2 to the Procurement Conditions

"Technical Specification"

technical specification

Tents FOR accommodation with supplementary equipment

Public Body Central Project Management Agency is carrying out a public procurement procedure for the procurement of Tents with support equipment for the Armed Forces of Bosnia and Herzegovina (hereinafter referred to as the Beneficiary). The goods offered by the suppliers shall meet the requirements set out in this Technical Specification.

| No. | Feature | Technical specification requirement | Documents confirming compliance with the requirement. |
| --- | --- | --- | --- |
|  | General requirements for the object of procurement |  |  |
| 1.1. | Object of procurement | Tents with rigid frames for accommodation, administrative and storage uses, designed for professional or military use and equipped with ancillary supporting equipment. Tents should be suitable for use during military, rescue, and humanitarian operations. All tents and ancillary support equipment should be manufactured and configured in accordance with the requirements set out below. | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements |
| 1.2. | Delivery location | Sarajevo, Bosnia and Herzegovina *The exact delivery addresses will be communicated to the supplier at the time of execution of the contract. | The supplier declares compliance with this requirement in section 6 of the Tender form. |
| 1.3. | Manufacturing the equipment | The equipment must be new and unused. Used or used and refurbished tents and ancillary support equipment cannot be offered. All tents must be manufactured by the same manufacturer, and should be of the same model and equipped with same equipment. | The supplier declares compliance with this requirement in section 6 of the Tender form. |
| 1.4. | Warranty | Tents and ancillary support equipment must be covered by a warranty of at least 24 months. The warranty service and repair must be provided by the Supplier and/or by the manufacturer and/or its representative in the service location indicated in the territory Bosnia and Herzegovina. Suppliers who cannot/will not be able to provide warranty service in the country of the beneficiary must organize the transportation of the equipment at their own expense from the Beneficiary's places of deployment to a company/institution performing warranty service in another country specified by the Supplier in the proposal. | The supplier declares compliance with this requirement in section 6 of the Tender form and also, the supplier shall submit with the tender a manufacturer's certificate, declaration or other document proving compliance with the requirement for warranty length and location of service centers in Bosnia and Herzegovina or other means of ensuring warranty service, as required by the established requirement. |
| 1.5. | Documentation to be supplied with the equipment. | Each Tent and ancillary support equipment must be supplied with printed maintenance and user manuals in English and/or one of the official languages of the beneficiary. If NATO Stock Number (NSN) is assigned to manufacturer for the object of procurement by designated National Codification Bureau and if such documentation is available for the supplier, this documentation must be supplied to the beneficiary. Documentation must be handed over to the Beneficiary 2 months before delivery of the goods. | The supplier declares compliance with this requirement in section 6 of the Tender form. |
| 2. | General technical requirements applicable for the Tents offered |  |  |
| 2.1. | Functional characteristics of the tents | All offered tents must have the following functional characteristics. - All tents must be free-standing type tents, equipped with rigid frame. - Disassembled tents must be able to be loaded for transport by hand with appropriate measures or mechanized lifting equipment – be equipped with transportation container/s or bag/s, containing all parts of the tent, and equipped with lifting points. - Tents should be able to be assembled in field conditions by hand without the use of lifting equipment. - Tents must be designed for multiple assembly and disassembly cycles. - Tents must be able to connect with Heating, Ventilation and Air conditioning (further in this specification referred as HVAC) equipment from outside the tent through designated inlet and outlet points. - Tents must be equipped with access points for power cables from outside power source. - Tents must be designed to be used with or without rigid floor and equipped with ground sheets. - Tents must withstand the snow load of no less than 40 kg/m2 and be resistant to wind speeds of no less than 25 m/s. - Tents and ancillary support equipment should be suitable for use in temperature limits no less than: from -20° to + 50° C. - Tents must be equipped with equipment for securing the tents during field use (e.g. ground stakes and guy ropes). | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements in paragraphs No.1 through 10. |
| 2.2. | Construction of the tents | All offered tents should be of the following construction: - Main covering of the tent must be manufactured from PVC-coated polyester fabric or equivalent material, weighing no less than 700 g/m². Material must be waterproof, flame resistant, and mildew resistant. - Tents must include removable inner liner manufactured from cotton, polyester, or equivalent material, weighing no less than 120g/ m². Material of inner liner must be washable and flame resistant. - Tents must include ground sheets manufactured from double-sided PVC-coated polyester fabric or equivalent material, weighing not less than 750 g/m². Material must be waterproof, flame resistant, mildew resistant and have non-slip surface. - Tents must have supporting frame, dissasemblable for transportation, manufactured from aluminum elements or light weight steel tubing or other material with not inferior lightness and strength properties. Material of supporting frame must be corrosion resistant (made from corrosion resistant materials or painted, galvanized, or protected from corrosion in other way). - Frame of the tent must be assemblable without tools, final assembly of the tents, attachment to the ground, installation of the guy ropes and other components could be accomplished without tools or with the tool kit, provided by supplier. - Frame of the tent must be attached to the ground at the sides of the non-gable walls of the tent and not have support posts in accommodation, work or storage area of the tent or at the gable ends. - Tents must be able to be connected to each other in any direction using the doors on gable walls and side doors (if equipped with side doors). - All tent doors must be made from the same material as the tent walls and have zipper or other closure solution that ensures easy closure without leaving gaps. - All windows of the tents must be made from PVC or other comparable translucent flexible material. Tents with mesh windows could not be offered. | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements in paragraphs No.1 through 9. For the paragraph No. 5, supplier also must provide information if final assembly of the tents requires tools, and if tools are required – list of tools, provided with the tents. |
| 2.3. | Color of the tents and support equipment | Tents and ancillary support equipment should be of following color: - Main covering must be made from material of Olive green, NATO green or other similar color, corresponding to the range of colors used by the military. - Inner liner must be made from material of white or similar light color. - Ground sheet must be made from material of Black, Olive green, NATO green or other similar color corresponding to the range of colors used by the military. - If supporting frame is visible when tent is assembled for use, visible parts must be non-reflective and painted in matte Black, Olive green, NATO green or other similar color corresponding to the range of colors used by the military. - Ancillary support equipment must be painted or manufactured from materials of Black, Olive green, NATO green or other similar color corresponding to the range of colors used by the military. | The supplier declares compliance with this requirement in section 6 of the Tender form. |
| 3. | Technical requirements for “Accommodation tents with supporting equipment” |  |  |
| 3.1 | Quantity | Minimum quantity: 5 units Maximum quantity: 7 units | The supplier declares compliance with this requirement in section 6 of the Tender form. |
| 3.2 | Dimensions | Accommodation tents should have the following dimensions: - Overall length – not shorter than 10 m. and not longer than 12 m. (outside dimension of the non-gable wall at the ground level). - Overall width – not narrower than 6 m. and not wider than 7 m. (outside dimension of the gable wall at the ground level). - Height from the floor to the roof – not lower than 2 m. and not higher than 2,3 m in the lowest point (height inside of the tent from ground to the roof line at the connection point between the roof and the wall.) - Accommodation capacity expected for these tents is not less than 20 person per tent. | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements in paragraphs No.1 through 4. |
| 3.3 | Construction of the tents | Accommodation tents must have following construction: - Equipped with no less than 2 exits on the opposite sides of the tent with closable doors. - Equipped with no less than 8 windows (4 per long side of the tent) equipped with coverings for blackout conditions. - Equipped with no less than 1 removable partition, capable of dividing space of the tent in 2 equal parts. Partition must be equipped with closable door. Tents must be designed in such way, that use of removable partition would not obstruct heating and ventilation from HVAC equipment for any part of the tent (if partition obstruct heating and ventilation, internal flexible ducting must be included with the tent to transmit heated or air conditioned air throughout the tent) - Equipped with removable floor, comprised from non-flexible rigid interlocking segments, manufactured from aluminum, polymer, composite material, or other material with not inferior rigidity and strength properties. Rigid floor must be non-slippery, manufactured from material of dark color and installable and removable by hand without tools. | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements in paragraphs No.1 through 4. For the paragraph No. 3, supplier also must provide information if tent partitions will obstruct heating and ventilation and if internal ducting will be provided with the tents. |
| 3.4 | Lighting equipment | Each accommodation tent must be equipped with following lighting equipment: - Comprised of no less than 4 LED technology tent lights, equipped with switches (individual switches for each light or single control switch for all lighting equipment in the tent) - Combined power consumption – no more than 100 W/h. - Voltage AC 230 V +/- 10 % 50 hz, connected to outside source through single 1 phase CEE 7/3 (Shuko) connector. - Lights must have environmental protection class not lower than IP54. - Lights must have red light function for blackout conditions. | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements in paragraphs No.1 through 5. |
| 3.5. | HVAC Equipment | Each accommodation tent must be equipped with following HVAC equipment: - Diesel tent heater, capable of being operated outside the tent and equipped with flexible air duct, suitable for connection installed in proposed tents. Tent heater must use diesel fuel for heat generation, and have air flow of no less than 1000 m3/h. - Electric ventilation and air conditioning unit capable of being operated outside the tent and equipped with flexible air ducts, suitable for connection installed in proposed tents. Air conditioning unit must have the air flow of no less than 2000 m3/h. - HVAC equipment must have an environmental protection class not lower than IPX4. - All HVAC equipment must use 1 phase CEE 7/3 (Shuko) or 3 phase CEE (IEC 60309) electrical connection to connect to outside electrical power sources. - HVAC equipment must be of sufficient power to ensure expected living conditions with an internal temperature of +20° (+/- 10%) in the outside temperature range -20° to + 40° C. In the temperature range from +40° to + 50° C HVAC equipment must be of sufficient power to ensure that the internal temperature of the tent will be not less than 20° C lower than the external (outside) temperature. | The supplier declares compliance with this requirement in section 6 of the Tender form. Supplier must also provide manufacturer's technical documentation (catalogs, brochures) and/or product manufacturer's declarations (if the manufacturer's technical documentation does not fully reflect the compliance of the offered product with the requirements of the technical specification) or other equivalent documents proving the compliance of the offered product with the technical requirements in paragraphs No.1 through 5. For the paragraph 5 supplier must provide information with proposed power of the heating and air conditioning units, and calculations, confirming sufficiency of the proposed power of the equipment to ensure required internal temperature. Calculations must include outside temperature range from -20° C to + 50° C. |

NOTES:

Where the technical specification indicates a specific model or source of supply, a specific process specific to the goods or services supplied by a particular supplier, or a trademark, patent, types, specific origin or manufacture, it shall be understood that these characteristics are indicative only and that the suppliers are free to offer an equivalent option. The supplier must demonstrate the equivalence referred to in this paragraph.

In the event that the technical specification specifies values/parameters for a particular technical characteristic, suppliers may offer goods whose values/parameters are not inferior to those specified (goods with no inferior characteristics may be offered).

This is the minimum quantity that the Contracting authority undertakes to purchase.

The Contracting authority does not undertake to purchase the entire quantity. The quantity to be ordered, which cannot be less than the minimum (5 units), will depend on the needs of the CPVA and the Beneficiary, but not exceeding the maximum quantity of 7 units in total.