SERVICES for assessing the Adequacy and flexibility needs of the Lithuanian POWER system for 2028-2035

Technical specification

# Object of purchase

  - Services for assessing the system adequacy and flexibility needs of the Lithuanian power system for 2028, 2030, 2033 and 2035 (hereinafter - Study):

    - System adequacy assessment of Lithuanian power system for 2028, 2030, 2033, 2035 must be done following the methodology for the European resource adequacy assessment (hereinafter - ERAA), approved by the Agency for the Cooperation of Energy Regulators of the European Union (hereinafter – ACER) on 2 October 2020. The study should justify the presence/absence of any adequacy problem of the Lithuanian power system.

  - Flexibility needs assessment at national level for 2028, 2030, 2033, 2035 must follow the provisions of Article 19e of Regulation (EU) 2024/1747 of the European Parliament and of the Council of 13 June 2024 amending Regulations (EU) 2019/942 and (EU) 2019/943 and Draft methodology for the analysis by transmission system operators and distribution system operators of the flexibility needs (hereinafter - FNA) at national level, developed by ENTSO-E and EU DSO Entity, approved by the ACER.

# Scope pf Services:

  - To perform a national resource adequacy assessment (hereinafter - NRAA) of the Lithuanian power system for 2028, 2030, 2033 and 2035 for ERAA 2025 scenario, using detailed PEMMDB data for Lithuania. To compare the obtained results (Loss of Load Expectation (LOLE), [hours/year] and Expected Energy Not Served (EENS), [MWh/year]) with the ERAA 2025 results. To present detailed hourly outcome results (excel), detailed report and a summary.

    - To perform a NRAA of the Lithuanian power system for 2028, 2030, 2033 and 2035 of the 5 relevant sensitivity scenarios with detailed data for Lithuania presented by the Contracting Entity, reflecting National circumstances, such as lower interconnection capacities, low wind and/or solar profiles, early decommissioning of gas generation, etc. To describe in detail the assumptions used, to present detailed hourly outcome results (excel), detailed report and a summary.

    - The model shall include power systems of the Baltic Sea region countries: Lithuania, Latvia, Estonia, Finland, Norway, Sweden, Denmark, Germany and Poland (the rest of the European countries can be modelled in aggregated way) as required by ERAA Art.1 (3) (according to Article 24 (1) of Regulation (EU) 2019/943 of the European Parliament and the Council concerning the internal market in electricity).

    - The consultant shall possess and perform calculations using the ENTSO-E PEMMDB data, or compatible.

    - To calculate and present probabilistic adequacy assessment indicators for 2028, 2030, 2033 and 2035: Loss of Load Expectation (LOLE), [hours/year] and Expected Energy Not Served (EENS), [MWh/year], the duration of the LOLE and ENS and how many times per year such situation occurs.

    - To provide detailed outcome results of economic viability analysis for Lithuanian traditional generation units LE7, LE8, LE9, KTE, ME1, ME2, PE and their contribution to system adequacy in all scenarios.

    - If adequacy problem is identified in any of NRAA scenario (calculated LOLE value is higher than 3 hours/year), to present detail data: hourly demand timeseries, renewable energy sources (hereinafter – RES) generation timeseries, residual load timeseries, storage operation timeseries, LOLE, EENS, RES curtailment extent, occurrence frequency and duration. To calculate how much demand is covered by RES, how much storage devices can reduce LOLE or convert curtailment into energy for demand, how much traditional generation is needed to cover “dunkelflaute” periods.

  - Based on NRAA results and data provided by Contracting Entity on flexibility needs of the distribution network, to perform analysis of system flexibility needs at national level for 2028, 2030, 2033 and 2035 for ERAA 2025 scenario and sensitivity scenarios provided by the Contracting Entity. To present results, insights, conclusions and recommendations for improving flexibility conditions.

  - Deliverables – detailed hourly outcome results (in excel), detailed report (.pdf and .ppt) and a summary (.pdf and .ppt).

# The organisation of the work:

  - Service Provider shall use publicly available data from ENTSO-E, Power exchanges operating in the Baltic region and public authorities’ database to accomplish study objectives. The Service Provider verifies the data of the Lithuanian electric power system with the Contracting Entity.

  - Organisation of the Study shall be done according to a timeline which must be proposed by the Service Provider and approved by the Contracting Entity during the kick-off meeting. The Study must be completed by 01 May 2026 at the latest.

  - The final report shall be drawn up after considering the suggestions and comments received during the presentations of the results.

  - The Service Provider shall be responsible for arrangement of teleconferences via mutually agreed video-conferencing application acceptable to the Contracting Entity and the Service Provider (e.g. MS Teams). The Service Provider shall foresee at least 7 presentations, listed in the table below.

  - The payments of the services shall be done according to the parts of the Study prepared and the results achieved:

|  | Part of the Study | Results | Payment, % from the Contract for the services provided |
| --- | --- | --- | --- |
| Part I | A National adequacy assessment has been performed, results for 2028, 2030, 2033 and 2035 presented. Detailed hourly outcome results submitted | - Initial results of the adequacy assessment were presented (presentation 1). Suggestions and comments received were evaluated. - The results of the adequacy assessment were presented to Ministry of Energy (presentation 2). The suggestions and comments received were evaluated. - The results of the adequacy assessment were presented to stakeholders (presentation 3). Indicative deadline – January 2026. | 40 |
| Part II | A System Flexibility needs assessment has been performed, results for 2028, 2030, 2033 and 2035 presented. A detailed Study Report including detailed hourly outcome results (excel) and a summary has been submitted. | - Initial results of the system FNA were presented (presentation 4). Suggestions and comments received were evaluated. - FNA results were presented to Ministry of Energy (presentation 5). - The results of the FNA were presented to stakeholders (presentation 6). The suggestions and comments received were evaluated. - Study Reports are accepted by the Service Provider. - The results of the Study were publicly presented (presentation 7- optional). Services provided until 1st May 2026. | 60 |

# Required services the Service Provider shall provide under the scope of the Study:

  - According to the previously agreed timeline to prepare and present detailed hourly outcome results (in excel), detailed report (.pdf and .ppt) and a summary (.pdf and .ppt).

  - To prepare “PowerPoint” presentations and present the initial and final results of the Study:

    - To Present an initial result of Part I and Part II.

    - To present the results of Part I and Part II to Ministry of Energy.

    - To present the results of Part I and Part II to National regulatory authority.

    - To present publicly the results of the Study.

  - All deliverables shall be exchanged electronically (e-mail, Clouds etc.) Delivery of hard copies is not accepted if electronic versions are not provided.

  - The Service Provider shall transmit documents to contracting Entity for review and comment and/or approval. The transmittal cover shall identify the document as requiring revision and/or approval and shall identify the date by which the Contracting Entity must respond. The minimum length for review, if not agreed otherwise, shall be 5 working days. The Contracting Entity shall return comments to the Service Provider within the agreed time.

  - At the request of the Contracting Entity, the Service Provider must identify and provide a written explanation to the Contracting Entity on what was taken into account and what changes were made in light of the comments submitted in the Document by the Contracting Entity.

  - Study Report documents provided shall be compatible with standard word processing software.

  - The Study Report will be accepted, and the Study will be considered complete only after the presentation of the results, made by the Service Provider, to stakeholders.

  - Used language for communication and deliverables is English and/or Lithuanian.