| No. | Title of claim | Minimum required characteristics | Nature of the requirement (B - necessary, P - additional). The Supplier must meet the 'essential' requirements. The 'additional' requirements are optional and will be scored (see Annex No. Y of SP') | Compliance with the requirement (to be completed by the Supplier) |
| --- | --- | --- | --- | --- |
| Server equipment requirements: |  |  |  |  |
| 1 | Identify the parameters of the proposed solution | The equipment manufacturer, model name, product and accessory part names, codes (Part Number), versions, licenses, quantities must be identified, together with an unambiguous description of the proposed equipment and its accessories. The Supplier must submit with the Initial Proposal a copy of a valid document (license, permit, certificate, partnership agreement, etc.) issued by the manufacturer of the proposed equipment, authorizing the Supplier to install and support the proposed equipment. | B |  |
| 2 | Description, architecture | - The servers submitted shall provide 4 (four) high availability HCI virtualization clusters (detailed in clause 20). - Two (2) clusters will be dedicated to Critical Infrastructure (CRIT1 and CRIT2) and will need to provide redundancy between the different data centres, as well as data loss protection and automatic migration of virtual resources in case of failure and/or unavailability of one of the data centres. - The third and fourth clusters (CORP1 and CORP2) will be dedicated to corporate infrastructure without duplication between them. - All servers (nodes) in the HCI cluster must be identical on the hardware side and must be identically equipped. - The necessary licences to meet the needs of the Contracting Authority as described in this Technical Specification must be provided. - CRIT1 and CRIT2 clusters will need to replicate synchronously in different data centres, CORP1 and CORP2 clusters do not need to be replicated). - We plan to create a total of 4 reservoirs. CRIT1 and CRIT2 clusters will have to replicate each other via synchronous replication and provide automatic failover of resources from a failed cluster to a running cluster. The supplier shall ensure the correct configuration and operation of both clusters, including the installation and maintenance of a witness host in a third data centre of the contracting organisation. - In addition, 2 (two) additional physical servers with a hypervisor identical to the hypervisor of the 4 (four) clusters to be purchased shall be provided. These additional servers must ensure operational reliability in the event of a failure of one of them and must have at least the same total resource utilization as: - 40 vCPU (pCPU to vCPU ratio not greater than 1:4) - 48GB RAM - 1TB of total usable disk space. - Monitoring of all clusters must be ensured with locally deployed software or a cloud-based service. If additional equipment is required, it must be provided separately. | B |  |
| 3 | Ability to migrate virtual servers from the virtualization platform used by LTG (VMware) with minimal downtime ) | The software of the proposed clusters is compatible with the contracting authority's existing and used VMware vSphere 8 software and ensures that existing virtual resources are migrated to the proposed new clusters with minimal downtime, with the resources being switched from the old cluster to the new one. | P |  |
| 4 | Processors | Applicable to CRIT1, CRIT2 and CORP1 clusters: Each server must have at least two (2) X64 architecture processors supporting: - 64- and 32-bit operating systems and applications; - automatic frequency and voltage control (based on load); - must support automatic system reboot in the event of a processor failure; - must support hardware-assisted virtualization; - must support Hyper-Threading technology or equivalent; - must support AMX, QAT, DSA, DLB technologies or equivalent; - must support SSE4.2, AVX, AVX2, AVX-512 instruction sets or equivalent. - The processor frequency must be at least 2.5 GHz. CRIT1 and CRIT2 clusters shall be identical and each shall have a capacity of at least 1500 virtual processors, with a ratio of pCPUs to vCPUs of at least 1:4. The capacity of the servers in the CORP1 cluster shall be at least 1000 and in CORP2 at least 350 virtual processors, with a ratio of pCPU to vCPU of no more than 1:4. - The processor shall deliver at least (results for a 2 processor system): 644 units on SPECrate2017 int_base test performance; 776 units on SPECrate2017 fp_base test performance. The results must be published on www.spec.org and included in the proposal. Applies to the CORP2 cluster: each server must have at least one (1) X64 architecture processor supporting: 64- and 32-bit operating systems and applications; - − automatic frequency and voltage control (based on load); − must support automatic system reboot in the event of a processor failure; − must support hardware-assisted virtualisation; − must support Hyper-Threading technology or equivalent; - support AMX, QAT, DSA, DLB technology or equivalent; - - support SSE4.2, AVX, AVX2, AVX-512 instruction sets or equivalent. − The processor frequency must be at least 2.9 GHz; The processor shall deliver at least (results for 1 processor system): 511 units on SPECrate2017 int_base test performance; 680 units on SPECrate2017 fp_base test performance. The results must be published on www.spec.org and included in the proposal. | B |  |
| 5 | Amount of RAM in the pool | - The server capacities of the CRIT1 and CRIT2 HCI servers in each cluster shall provide a minimum of 7000 GB of usable RAM, balanced according to the recommendations of the manufacturer of the proposed hardware, but with the possibility of expanding the amount of memory without replacing the existing memory modules with larger ones and with the addition of new memory modules. - The server capacity in corporate HCI clusters shall provide a minimum of 5500 GB of RAM for CORP1 and 2000 GB for CORP2, balanced according to the recommendations of the manufacturer of the proposed hardware, but with the possibility of expanding the amount of memory by adding new modules, without replacing the existing memory modules with higher capacity ones. - RAM must be at least DDR5 5200 MT/s ECC Registered DR or equivalent; Advanced ECC, Online Sparring or equivalent technologies must be supported. | B |  |
| 6 | Disc subsystem | - The HCI cluster data repository data must be automatically distributed to the nodes of the cluster according to the technology and data protection level of the cluster. - All drives provided must be NVMe type. - For the hypervisor to work, the drives must be protected at hardware RAID1 level and have a minimum capacity of 480 GB. - The discs submitted must be tested and qualified by the manufacturer for the proposed solution. - The total usable data capacity of the CRIT1 and CRIT2 HCI pools must be at least 210TB (for all data, using a data block protection mechanism at least as robust as RAID 6 (erasure coding), excluding data compression and degaussing). - The total usable data capacity of the CORP1 HCI pool shall must be at least 150 TB (for all data, using a data block protection mechanism at least as robust as RAID 6 (erasure coding), excluding data compression and degaussing) - The CORP2 HCI cluster must have a total usable data capacity of at least 30 TB (for all data, using a data block protection mechanism at least as robust as RAID 5 (erasure coding), excluding data compression and degaussing). - The performance of the disk subsystem in the CRIT1 and CRIT2, CORP1 and CORP2 HCI clusters shall be at least 110 000 IOPS with a response time of no more than 1 ms for load type OLTP 16K with a random read/write ratio of 70/30. - The drives shall be configured in such a way as to allow an increase of at least 1,5 times the amount of usable space by purchasing additional drives. The capacity of the data disks in the cluster shall be visible as a single aggregate data storage capacity. As new nodes are added to the cluster, the capacity shall be expanded automatically, without interrupting the operation of the cluster and virtual machines. The replacement of disks on nodes and the removal of individual nodes from the cluster or the replacement of old nodes by new nodes shall also be ensured without stopping the cluster . | B |  |
| 7 | Storage functionality | - The pool must be able to provide snapshot and clone copies at the virtual machine level (where only data blocks used by one virtual machine are copied). - All virtual machine read operations shall be performed using the cache memory of the entire pool, provided that the system is in a normal operating state - The pool shall support Storage based policies - the ability to set policies for the storage subsystem (protection level of virtual machines, etc.) - The pool shall provide Quality of Service (QoS) for I/O operations | B |  |
| 8 | Functionality of the storage subsystem | - The storage subsystem must have optimisation functions to ensure data reduction according to algorithms proposed by the manufacturer . If this functionality is licensed, licenses shall be provided for the total data capacity purchased. - It must have the functionality to limit the number of read/write operations (IOPS limit) of virtual machines. - CRIT1, CRIT2 and CORP1 HCI clusters shall use RAID6 and Erasure coding technologies for data protection and distribution to ensure that the cluster will be operational in the event of failure of at least two components. The CORP2 HCI cluster shall use RAID5 and Erasure coding technologies to ensure that the cluster will be operational in the event of failure of at least one component. | B |  |
| 9 | Network ports and adapters | - Each server shall provide at least six (6) network ports of at least 25 Gb and certified to include microcode and software for use in the cluster and for use with HCI virtualisation software. - Must support IEEE 802.1Q, 802.3ad, 802.3x, VMware NetQueue, partial packet processing (TCP/IP Stateless Offloading and Checksum and Segmentation Offloads, VXLAN/Geneve Offload) or equivalent technologies. - Support for large Ethernet transmission units (Jumbo frames up to 9000 bytes) must be available. - 25Gbps SFP28 to SFP28 DAC cable with a length of 3 m shall be provided for each port compatible with Buyer data center network switches. Will be clarified during project implementation. | B |  |
| 10 | Video controller | Must be integrated with a resolution of at least 1600x1200 px. | B |  |
| 11 | Ports | Integrated ports: at least 2 (two) pcs. USB 3.0; at least 1 (one) VGA. | B |  |
| 12 | System management of servers | - An operating system independent management controller (OOB) shall be integrated and communicated with via a dedicated Ethernet RJ-45 port. - The following features must be supported: RedFish, IPMI 2.0, DCMI, virtual OS-independent remote text and graphical console, virtual server power on/off, SSL security, remote server management via WEB browser. - Hardware health monitoring must be available without an operating system. Parameters and components to be monitored: temperature, CPU, RAM, internal drives, fans, network controllers. - Must integrate into a unified management platform provided by the Supplier, providing information on temperature, CPU, RAM, internal drives, fans, network controllers. - It must be independent of the operating system for sending notifications via e-mail. - It must be possible for at least two (2) remote users to log in at the same time and share a console session. - It must support: virtual CD/DVD (ability to use the remote computer's CD/DVD, CD/DVD image and USB storage medium as a server), virtual KVM, user authentication via LDAP service/MS Active Directory. | B |  |
| 13 | Other management functions and high reliability functions | Automatic Server Recovery, Redundant ROM, Remote Flash ROM Integrated Management Log or equivalent. | B |  |
| 14 | Status indication | There must be an LCD or LED fault indication and localisation system on the outside of the case (front panel of the server). | B |  |
| 15 | Power supply | It is designed to be powered from a minimum 230V 50Hz single-phase AC mains supply. Hot-plug redundant power supplies (2 units). The power shall be sufficient to keep the server operational even in the event of a failure of one of the power supplies, even if the memory and disk drive bays are full. | B |  |
| 16 | Cooling | Duplicate cooling modules, at least the manufacturer's reservation, of the 'hot-plug' type. | B |  |
| 17 | Server housing | The height of the server must not exceed 2 U. The server shall be suitable for installation in a standard 19"" rack with sliding, fully retractable brackets and all accessories required for installation (rails, fixings, cable arm, cables). | B |  |
| 18 | Requirements for assembly | The server must be complete, the components (processors, memory, controllers, disks, etc.) must be installed in the server in the manufacturer's factory. All parts and devices of the proposed server must be supplied by a single manufacturer and bear its brand name. | B |  |
| 19 | Virtualisation platform licences | - Licences covering all HCI clusters and not limiting the amount of virtual resources used must be granted for the full duration of the Agreement. - Licences must provide software upgrades and support for the duration of the provision of HCI Services. It must be ensured that, in the event of a failure, the Contracting Authority will have immediate access to technical expert advice by telephone. | B |  |
| 20 | Governance | The management and monitoring of technical resources and virtual machines in HCI clusters must be centralised by providing the Contracting Authority with a management platform, VMware vCenter Server or equivalent, that ensures: - graphical user interface, command line interface, SSH interface; - compatibility checking of the hardware (including firmware of disk controllers, etc.) and software (hypervisor, storage software, management subsystem, etc.) of the cluster ; - automated preparation of the cluster for operation, with wizards assisting with configuration tasks; - Management of updates of the cluster software (hypervisor, storage software, management subsystem, BIOS, disk controllers) without interrupting the operation of the cluster (including uninterrupted operation of virtual machines); A Full Stack Update, including the hypervisor, the cluster software, the management subsystem, the BIOS, the component controller drivers, must be carried out by the solution manufacturer on a regular basis and free of charge throughout the lifetime of the solution. - Management and automation of the inclusion of additional nodes in the cluster. - Access to software the manufacturer or an authorised service centre. The management platforms must be separate and no more than two must be provided: - For the management of the CRIT1 and CRIT2 clusters, a monitoring service station and an additional two-server cluster at a third location. - Designed to manage CORP1 and CORP2 clusters. Management platforms must be capable of providing full licences for the maximum number of servers and clusters to be managed. In HCI clusters , virtual machines and other resources must be distributed automatically and evenly among all nodes in the cluster, depending on the computing power and RAM they use. All licenses required for this purpose shall be made available for the full duration of the Contract. All HCI clusters must support the migration of virtual machines between clusters without downtime or shutdown. All licenses required for this must be provided for the full duration of the Contract. In HCI clusters , the network ports for virtual resources must support configuration from a single point rather than individually for each node in the cluster. | B |  |
| 21 | Reliability | - All resources used must be automatically allocated to the clusters. Data must be distributed simultaneously so that if two nodes in a cluster or two drives in a node are lost stored on multiple nodes (servers), ensuring data preservation in case of loss of at least two nodes (for CRIT1, CRIT2 and CORP1 clusters) and in case of loss of at least one node (for CORP2 cluster). The cluster management software must be active on all nodes and provide uninterrupted cluster management in case of failure of one or more nodes. The nodes of the cluster must be powered from redundant power supplies and shall be connected to the network via redundant interfaces. - The operation of the cluster must not be impaired by the failure of individual nodes or components of nodes. In the event of the failure or loss of one or two servers or components of the cluster , sufficient disk and computing resources of the cluster must be available for the operation of virtual machines. (As described in clauses 4-6) - Virtual machine data must not be lost unless more failures have occurred than the protection level. | B |  |
| 22 | Additional virtualisation services | The Contracting Authority shall receive these additional virtualisation services throughout the duration of the Services: - A resource accounting and reporting tool (vRealize Operations Advanced or equivalent), the requirements for which are specified in Section 3.6 of this Technical Specification. - A tool for capturing, processing and displaying system records (Vrealize log insight or equivalent). | B |  |
| 23 | Compatibility with backup and recovery software | The Supplier must propose a backup and recovery system compatible with the proposed solution, provide its licences (without limiting the amount of data to be copied, the number of virtual machines, the number of containers, and with a minimum of processors smaller than the number of processors proposed in the Supplier's proposal), including. 5-year upgrade. | B |  |
| 24 | Cluster security functionality | The equipment must have: - Data encryption functionality (data-at-rest); - The encryption technology of the on-premises data store must be recognised as FIPS 140-2 Level 1 or equivalent; -Have a data integrity checking mechanism; - Must comply with the Cybersecurity Directive of the Republic of Lithuania (link). | B |  |
| 25 | Warranty, repair time | - From the date of signature of the Acceptance and Handover Deed, the hardware and software of the clusters shall be subject to a period of technical support guarantee for the duration of the Contract, the parameters of which are set out in this Technical Specification. Criteria for when the Acceptance and Handover Deed is signed are described in point 3.10 of Technical Specification. - The HCI Supplier is obliged to eliminate malfunctions in the operation of the equipment free of charge, or to cover all costs related to the restoration of the operation of the equipment. All repair work and spare parts must be included technical support guarantee at no additional cost. - There must be a 24x7x365 line for support cases and remote repair. Minimum 5 year manufacturer's technical support guarantee with on-site service, 7 days a week, 24 hours a day, including holidays, 4 hours on-site response time. - The manufacturer must guarantee the compatibility of all hardware and software elements and the timely update of drivers and versions of all components (including the virtualisation platform) free of charge in the future, throughout the life cycle of the cluster equipment. - There must be free supply of parts and free repairs for replacement of processors, memory, disks, power supplies, cooling modules if there has been a prefailure warning ('prefailure warranty') by the HCI manufacturer or its authorised service centre or a manufacturer-certified supplier who is qualified by the manufacturer to provide warranty service. - SSDs in the premises must be covered by the manufacturer's warranty, which also covers the wear and tear of the drives/modules (the warranty covers the replacement of the drives in the event of wear and tear or malfunction). Replaceable storage media that do not fail on power-down will not be removed from the premises of the Contracting Authority and will not be returned to the HCI manufacturer or its authorised service centre. - Communication with the Supplier will be in English or Lithuanian. - If the problem cannot be solved remotely, an engineer from the HCI manufacturer or its authorised service centre shall visit the installation site to replace the defective components - The contracting authority must obtain a link to the manufacturer's website, the technical documentation containing information on the main characteristics of the proposed warranty and technical support and compliance with the requirements of the Technical Specification. - The contracting authority must obtain a link to the manufacturer's website , which enables the verification of the manufacturer's warranty by means of the product code and serial number. - The Contracting Authority must receive free of charge hardware updates and the latest versions of the software with licences to use them throughout the warranty period. The installation of equipment upgrades must be ensured by the Supplier without interrupting the operation of the clusters. A cluster equipment manufacturer's upgrade service must be included according to a pre-agreed schedule. - The manufacturer's helpdesk shall have the service (at no extra charge) of collecting/receiving equipment performance reports directly automatically over a secure encrypted two-way internet channel, and connecting to the monitored system to resolve the problem. - technical support guarantee has to be provided by manufacturer or manufacturers officially authorized representative ensuring “one window” principle. The manufacturer, or an authorised representative, is fully responsible for providing both hardware and software warranty services in accordance with all requirements of this Technical Specification. | B |  |
| 26 | Single manufacturer responsibility and centralised provision of maintenance and support services | Warranty and maintenance of all components - hardware (servers,etc.) and software (hypervisor, vSphere (or equivalent), virtual storage and network virtualisation, continuous data protection and snapshots, cluster health monitoring and analytics, etc. (e.g. software for storage, storage, data centre, storage management, etc.) failures and malfunctions shall be logged directly with a single HCI manufacturer (dedicated phone number, email, web-based system) and resolved by the same single HCI manufacturer, ensuring a 'one-stop shop' principle. All of the above requirements are guaranteed by a single HCI solution manufacturer and provided centrally. A "single HCI manufacturer" is defined as a solution that has been developed jointly by hardware and software manufacturers and is not "hardware independent". | P |  |
| 27 | Central monitoring of services | Unified software shall be provided and deployed to centrally monitor the performance of the physical and virtual infrastructure of all HCI clusters and to identify failures, ensuring: - An assessment of the current status ("Health"), together with the causes of the problems and links to the knowledge base of possible solutions, carried out at the level of the solution as a whole (i.e. at the level of the solution's constituent clusters), or at the level of a single cluster or a single node, as appropriate; - Hardware (CPU, RAM, networks, storage) utilization, load analysis and anomaly detection; - Monitoring the performance of virtual machines and identifying the source of problems; - AI-based forecasting of future use of hardware resources (e.g. data storage capacity). -The project shall include the integration of the centralised monitoring solution into the ServiceNow system used by the Contracting Authority. "Integration" is understood as the sending of incidents to the ServiceNow system from the centralised monitoring tool of the HCI service. ServiceNow configuration changes (if necessary) will be made by the ServiceNow support team. A licence must be included that covers all HCI clusters and does not limit the number of virtual machines used. A licence shall be provided for the full duration of the HCI Services, ensuring resolution of the incident in accordance with the timeframes for troubleshooting set out in this TS. | B |  |
| Requirements for backup and recovery software and hardware |  |  |  |  |
| 28 | Protection and restoration | The software must have a data protection solution that includes backup, recovery, storage and replication. The solution must support a variety of platforms including virtual machines, physical servers, cloud environments and containers. | B |  |
| 29 | Backup and recovery options | The software must be able to recover data quickly and reliably, including restoring files, applications and the entire system. The software must have the ability to store/replicate backups to at least one of the cloud platforms used by the Contracting Authority: Azure or AWS. The backup and recovery software itself, with all its configuration, backup information and backup schedules, must be recoverable. | B |  |
| 30 | Disaster recovery | The application must be able to plan and test disaster recovery. | B |  |
| 31 | Cloud integration | It must support backup and recovery on at least one of the cloud platforms used by the Contracting Authority: Azure or AWS. The ability to restore systems regardless of where they were backed up from (from cloud to private cloud, etc.) | B |  |
| 32 | Security and compliance | It must be possible to encrypt backups. Must comply with the Directive on Cyber Security Law of the Republic of Lithuania. It must have immutable backup functionality. | B |  |
| 33 | Management and monitoring | Centralised software management with comprehensive reporting and monitoring tools is required. Must have integration with ServiceNow, incident logging and cancellation functionality. Full centralised management must be possible through the most popular current browsers without any additional plug-ins. A software component shall be provided for the centralised monitoring and management of all the components proposed in this lot (backup software, VM replication, monitoring and analysis, search, hardware and virtual backup devices), running in a web browser. | B |  |
| 34 | Backup and recovery software licences | Licences comprising Table 2, row 2. No. 28-33 must be provided for the entire duration of the Contract. Licences must provide for software upgrades and support throughout the term of the Agreement. It must be ensured that in the event of a failure, the Contracting Authority will be able to obtain technical expert advice immediately by telephone. If licenses are required for backup and backup copies storage to Azure, AWS, they must be included in the bid price. | B |  |
| 35 | Technical equipment | The backup and recovery hardware must include storage for the entire infrastructure being purchased and not limit the number of virtual machines. The solution must be easily scalable and support a broad ecosystem of applications, at least: ● The proposed hypervisor for the HCI solution; ● Applications:Microsoft SQL, Oracle, MySQL, MongoDB, PostgreSQL, Cassandra; ● Windows and Linux file systems. Warranty and technical assistance contract. The software suite must be backed by at least 60 months of manufacturer's technical support guarantee. It must be possible for registered users to check the warranty level and expiry date on the equipment manufacturer's website. There must be a 24x7x365 line for fault logging and remote repair. Minimum 5 year manufacturer's technical support guarantee with on-site service, 7 days a week, 24 hours a day, including public holidays, 4 hours on-site response time. | B |  |
| 36 | Protection against ransomware | Backup hardware and software must protect the backed-up resources against ransomware. | B |  |
| 37 | Automatic verification of backups | The backup software must regularly test the integrity of the backup data and generate a report if necessary | B |  |
| 38 | Requirements for software components. | - The solution must support encryption functionality. -The solution must support replication technology that allows copying/moving backups from one device to another, while preserving the directory structure - The solution must support automation via REST API. - The solution must support deduplication functionality. | B |  |
| 39 | Requirements for hardware components | - There has to be at least two storages in remote data centers. - Between storages there has to be an automated data replication. - Hardware has to support data safety level of at least RAID6 or equal. It is treated as equal if hardware does not lose its functionality when two disks are lost. - There has to be an integration with ServiceNow directly or a possibility to send notifications to Zabbix. - System has to notify about its components failures and about predictive failure. - The drives shall be configured in such a way as to allow an increase of at least 1,5 times the amount of usable space by purchasing additional drives. There has to be a possibility to expand without the acquisition of additional licenses. | B |  |
| 40 | Plannned copy frequency | The backup and recovery hardware must include storage for the entire infrastructure being purchased and not limit the number of virtual machines. The size of the single backup storage device used has to be sufficient to be able to deliver backup copies according to provided frequencies: Daily - 7 copies Weekly - 4 copies Monthly - 12 copies Yearly - 5 copies | B |  |
| Requirements for Kubernetes cluster management software |  |  |  |  |
| 41 | Load balancing | Must have load balancing capabilities. | B |  |
| 42 | Cost management | There must be the ability to allocate costs according to the use of resources. | B |  |
| 43 | Container Image registry | There must be a private container registry. | B |  |
| 44 | Monitoring | It must be possible to monitor resource metrics. There must be an integration with ServiceNow directly or a possibility to send notifications to Zabbix. Cluster and container level event logs shall be available. | B |  |
| 45 | Updates | Regular updates must be allowed. It must be possible to update containers and their clusters with zero-downtime. | B |  |
| 46 | Centralised access management | Centralised access management must be in place. | B |  |
| 47 | Security | The Kubernetes management platform must meet and support clustering in accordance with the requirements set out in the NSA/CISA Kubernetes Hardening Guide version 1.2. | B |  |
| 48 | Control panel | It must be possible to create a new Kubernetes cluster via the GUI control panel, selecting the Kubernetes version, the cluster resources and the number of servers. It must be possible to extend an existing cluster by adding additional worker nodes. | B |  |
| 49 | Managing Kubernetes clusters on the HCI management platform | Kubernetes cluster management is integrated into the HCI management platform. | P |  |
| 50 | API | When the cluster is launched, there must be a clear way to expose the API. | B |  |
| 51 | Licences | Licences must be provided for all HCI nodes | B |  |
| Requirements for the resource accounting component |  |  |  |  |
| 52 | Architecture | A solution for analysing the cost of virtual resources, compatible with the proposed virtualisation software and management software. The purpose of the solution is to account for the actual cost of the service when providing virtual resources as a service to internal customers (the Contracting Authority itself (or another related organisation to which the Contracting Authority provides the virtual resources service)). | B |  |
| 53 | Development of cost models and metrics | Flexible cost models for: - Measuring fixed costs (costs per virtual machine, such as data centre space costs, electricity costs, cooling costs, administration costs, etc.), - Measuring allocation-based costs (variable costs per virtual machine, such as allocated RAM, CPU or storage space), - Utilisation-based costs (variable costs depending on the actual utilisation of the resources of a particular virtual machine, e.g. average RAM usage, storage space, CPU usage, network I/O, storage I/O), or a combination of all three. - Option to assign additional fixed costs to the virtual machine (e.g. for maintenance, licences) -Ability to monitor the costs of a selected unit (directorate, department, division, etc.) within the organisational structure of the Procuring Entity (or other related organisation to which the Procuring Entity provides the virtual resource service) | B |  |
| 54 | Estimating the cost of virtual resources | When calculating the cost of virtual resources, various factors must be taken into account, including CPU, memory and storage usage. It must be possible to estimate additional costs (for example, electricity or data centre cooling costs) | B |  |
| 55 | Allocation of costs | It must be possible to assign costs to the elements to be measured (e.g. CPU, RAM, storage I/O, network I/O operations). It must be possible to use multipliers according to the methodology chosen by the Contracting Authority (e.g. assigning active-active resources in an HCI cluster; or virtual resources with a chosen higher SLA would also be more expensive, etc.) | B |  |
| 56 | Software support provided by the software manufacturer | A licence must be included that covers all HCI clusters and does not limit the number of virtual machines, processors or other resources used. A licence must be included to ensure continuous manufacturer upgrade/support throughout the lifetime of the HCI Services, ensuring the provision of support as described in Section 3.4, Clause 25 of the Technical Specification. | B |  |
| Delivery and installation timescales |  |  |  |  |
| 57 | Maximum time for delivery, installation and set-up | The equipment must be delivered, installed and set up within a period of 75 (seventy-five) calendar days from the date of signature of the contract. | B |  |
| 58 | Timeframe for delivery, installation and set-up | The equipment must be delivered, installed and set up no later than 45 (forty-five) calendar days from the date of signature of the contract. | P |  |