Annex 2.2 to the Terms and Conditions of the Procurement

Municipal Enterprise "Susisiekimo paslaugos"

(Tender form)

TRAFFIC LIGHT CONTROL SOFTWARE, INSTALLATION, MAINTENANCE, ADDITIONAL SERVICES AND SUPPLY OF INTERSECTION CONTROLLERS

TENDER

_____________________ 20___

| Name(s) and code(s) of the Tenderer (each partner in the group of suppliers) |  |
| --- | --- |
| Address(es) of the Tenderer (each partner in the group of suppliers) |  |
| Person authorised by the tenderer (partner(s) of the group of suppliers) to sign the tender |  |
| Name, telephone number, e-mail address of the person authorised by the tenderer (partners in the group of suppliers) to communicate with the tenderer on the submitted tender |  |
| Name(s) of the manager(s) of the tenderer (each partner in the group of suppliers) |  |
| Name(s) of the person(s) authorised to draw up and sign the financial records of the tenderer (each partner in the group of suppliers) |  |
| Names of members of the management (supervisory board), supervisory body (board of directors) or any other person who has the power to represent or control the tenderer (each partner of the group of suppliers), to take a decision on his behalf, to enter into a transaction |  |

We confirm that we agree to all the terms and conditions set out in the procurement documents.

Here is a description of the quantitative criteria for the proposed procurement object:

| Ser. No | Quantitative criteria | Values for the proposed criteria indicators |
| --- | --- | --- |
| 1. | First criterion - Price (C) | Mark the proposed size of the storage required for the software and data storage, in terabytes (mark only one box with an 'x'): - From 6 TB and more - From 5 TB to less than 6 TB - From 4 TB to less than 5 TB - From 3 TB to less than 4 TB - From 2 TB to less than 3 TB (Remark 1. If the Supplier does not specify the size of the Storage required for the software and migration and storage of existing Sitraffic Scala 2 TB traffic flow data, or specifies more than one value, the maximum size of the Storage will be considered to be proposed and a maximum cost of EUR 50 000.00 will be added to the Supplier's total price as stated in the Tender Form. Remark 2. If the Supplier specifies a storage size of 6 TB or more for the software and data storage, a maximum cost of EUR 50 000,00 will be added to the Supplier‘s total price indicated in the Tender Form and the Supplier must consider that, during the implementation of the System, the Supplier will be obliged to procure at its own expense and to supply, deliver, install, and hand over to the Customer the additional storage unit(s) in accordance with the requirements provided by the Customer. The amount of EUR 50 000,00 specified by the Customer shall be for the maintenance of the extra large storage capacity of 6 TB or more installed and delivered by the Supplier.) |
| 2. | Second criterion - Warranty period for controllers (T1) | Mark the proposed warranty period for the controllers, in years (mark only one box with an "x"): - 5 years - 6 years - 7 years and over (Remark. If the supplier does not indicate the proposed warranty period for the controllers or indicates more than one value, the proposed minimum warranty period for the controllers (60 months) will be considered as the minimum warranty period for the controllers (5 years) and the criterion will be awarded 0 points.) |
| 3. | Third criterion - Method of integration of new controllers into the System (T2) | Mark the proposed warranty period for the controllers, in months (mark only one box with an "x"): - Manual; - Semi-automatic; - Automatic. (Remark. If the supplier does not specify a method for integrating the new controllers into the System or if more than one value is specified, the Manual method for integrating the new controllers into the System shall be considered as the proposed method of integrating the new controllers into the System, and 0 points shall be awarded for the criterion.) |

The proposed controllers fully comply with the requirements set out in the procurement documents and have the following characteristics:

| Ser. No | Required value | Parameter value of the proposed controllers and supporting technical documentation for the controllers* (specify the title, page number or publicly available references of the technical document to justify the conformity of the product with the required value) *Must provide a detailed description of the conformity of the proposed product with the parameter. The supplier shall complete only those parts of the table where the indication 'to be completed by the supplier' is indicated, together with the manufacturer's technical documentation and/or manufacturer's references or other equivalent evidence to substantiate compliance with the required value. Tenders marked 'Yes/No' or 'Compliant' will be rejected as non-responsive. The supporting documentation for conformity must be provided for each required parameter value, including those parts of the table that state 'The proposed controllers must comply with the requirement'. |
| --- | --- | --- |
| 1. | Manufacturer and model of the controller** ** no need to provide supporting documents, just fill in the column of the table "Parameter value of the proposed controllers and supporting technical documentation for the controllers" | /to be completed by the supplier/ |
| 2. | Cabinet: the controller shall be mounted in a polyester or metal (corrosion protected) reinforced box on the foundation. The enclosure shall be IP54 rated. Lockable mortise lock. The body colour RAL 9004 MATT. | /to be completed by the supplier/ |
| 3. | Supply voltage: 230 V AC – 10/+10%; 50 Hz | /to be completed by the supplier/ |
| 4. | Output voltage: 230 V AC – 10/+10%; 50 Hz | /to be completed by the supplier/ |
| 5. | Standard: LST EN 12675 or equivalent | /to be completed by the supplier/ |
| 6. | Current for each output (for traffic light connection): at least 1 A | /to be completed by the supplier/ |
| 7. | Output circuits: Each output is protected by an individual fuse. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 8. | Number of controlled signal groups: must support up to 32 signal groups or more. | /to be completed by the supplier/ |
| 9. | Additional power socket in the cabinet: shall be protected by a separate pick. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 10. | Liquid crystal display (LCD). | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 11. | Clock: electronic quartz integrated in the control board of the controller, time displayed on the LCD of the controller and an additional GPS module for clock synchronisation. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 12. | Operating temperature: -400C - +600C | /to be completed by the supplier/ |
| 13. | Communications, interfaces, protocols: RS485, TCP/IP, USB, OCIT-O 1.1, OCIT-O 2.0, OCIT-O 3.0 | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 14. | Number of external inputs/outputs: must support up to 16 inputs/outputs or more. | /to be completed by the supplier/ |
| 15. | Sensor types available for use: Inductive loops, video, infrared, radar and wireless magnetic sensors. | /to be completed by the supplier/ |
| 16. | Signalling safety (control of LED optical elements): the traffic light controller shall control all colours of LED optical elements - in the event of a burn-out of any LED optical element of any colour, the controller shall record and send a message to the System about the faulty LED optical element in a particular signal group: Control of red LED optical elements for transport: - Where there are TWO red LED optical elements in a signal group: in the event of fault of ONE red LED optical element, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to dark or yellow flashing mode. - Where there are THREE red LED optical elements in a signal group: in the event of fault of ONE red LED optical element INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault and only send a fault message to the System. In the event of fault of TWO red LED optical elements INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to dark or yellow flashing mode. In the event of fault of at least ONE red LED optical element NOT INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. - Where there are FOUR red LED optical elements in a signal group: in the event of fault of ONE or TWO red LED optical elements INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault and only send a fault message to the System. In the event of fault of the optical elements of THREE red LEDs INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System, and must deactivate the intersection to dark or yellow flashing mode. In the event of fault of at least ONE red LED optical element NOT INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. - Where there are FIVE red LED optical elements in one signal group: in the event of fault of ONE or TWO red LED optical elements INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault and only send a fault message to the System. In the event of fault of the optical elements of THREE or FOUR red LEDs INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System, and must deactivate the intersection to dark or yellow flashing mode. In the event of fault of at least ONE red LED optical element NOT INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. Control of red LED optical elements for pedestrians: - In the event of fault of at least ONE red LED optical element, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. Control of green LED optical elements: - In the event of fault of at least ONE green LED optical element, the controller must detect the fault and only send a fault message to the System. - In the event of fault of the optical elements of ALL green LEDs in the signal group, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. Control of green LED optical elements for pedestrians: - In the event of fault of at least ONE green LED optical element, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode Control of yellow LED optical elements: In the event of fault of at least ONE or ALL yellow LED optical elements, the controller must detect the fault and only send a fault message to the System. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 17. | Memory: The controller stores all events and detector readings in its memory according to the manufacturer's maximum time limits. The memory of the controller shall be expanded to the maximum possible size depending on the controller model. The minimum time limit is 24 hours. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 18. | Uninterruptible power supply system: in the event of a power failure, the controller shall send an alarm message to the System using an internal backup power supply (for example, battery). Minimum and maximum voltage monitoring with memory storage. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 19. | Connection to the system and other controllers: compatible with the System according to OCIT protocol requirements, compatible with the System's adaptive module for real-time optimization of operating modes module and the PTP module. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 20. | Door opening alarm: after opening any door of the controller, the opening of the door must be recorded in the controller, and a message shall be sent to the System indicating the opening of the controller door. | The proposed controllers must comply with the requirement (reference to the technical documentation) |
| 21. | Warranty period: At least 5 years from the date of delivery to the Customer | /to be completed by the supplier/ |

We undertake to supply goods and services which fully meet the requirements of the Technical Specification at the following price:

Table 1. Traffic light controllers by sets

| Ser. No | Name | Units of measurement | Quantity**** | Price per unit in EUR, exclusive of VAT | Total price in EUR, exclusive of VAT (4)*(5) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Traffic light controllers _______________________* 1 set | pcs. | 44 |  |  |
| 2. | Traffic light controllers _______________________* 2 sets | pcs. | 110 |  |  |
| 3. | Traffic light controllers _______________________* 3 sets | pcs. | 45 |  |  |
| 4. | Traffic light controllers _______________________* 4 sets | pcs. | 5 |  |  |
| 5. | Traffic light controllers _______________________* 5 sets | pcs. | 4 |  |  |
| 6. | Traffic light controllers _______________________* 6 sets | pcs. | 1 |  |  |
| 7. | Traffic light controllers _______________________* 7 sets | pcs. | 1 |  |  |
| Total comparative tender price in EUR, exclusive of VAT** |  |  |  |  |  |
| VAT (EUR)*** |  |  |  |  |  |
| Total comparative tender price in EUR, including VAT*** |  |  |  |  |  |

* The manufacturer and model of the controllers shall be indicated.

** The price of the tender must be expressed to no more than two decimal places.

*** In cases where the tenderer is not required to pay VAT under the legislation in force, the tenderer shall indicate the total price of the tender, excluding VAT, and the reasons for not paying VAT. The supplier shall assess whether it will become subject to VAT during the performance of the contract. If the supplier will become subject to VAT during the performance of the contract, the tender shall indicate the price including VAT. The prices of the tenders will be evaluated and compared inclusive of all taxes, including VAT. In the event that the contracting authority itself is required to pay VAT to the State budget for the purchase of the subject of the contract in accordance with the procedure laid down by the laws governing taxation and their implementing legislation, this tax shall be included in the price of the tender (if the supplier did not include it when submitting the tender, it shall be added by the contracting authority itself for the purposes of comparison). The tender price must include all taxes and all other direct and indirect costs and charges incurred and/or likely to be incurred by the Supplier in connection with the work/service to be performed.

**** The quantity indicated is the maximum quantity, the Contracting Authority will purchase on an as-needed basis and is not obliged to purchase the full quantity indicated in Table 1. Traffic light controllers by sets.

Table 2. Traffic light control software with licenses and installation services

| Ser. No | Name | Unit of measurement | Quantity | Price in EUR, exclusive of VAT |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1. | Traffic light control software _______________________* with licenses | set | 1 |  |
| 2. | Installation services (installation and configuration of the Traffic light control software ________________*, connection of existing intersection controllers via OCIT protocol and training) | set | 1 |  |
| Total comparative tender price in EUR, exclusive of VAT** |  |  |  |  |
| VAT (EUR)*** |  |  |  |  |
| Total comparative tender price in EUR, including VAT*** |  |  |  |  |

* The manufacturer of the software shall be indicated.

** The price of the tender must be expressed to no more than two decimal places.

*** In cases where the tenderer is not required to pay VAT under the legislation in force, the tenderer shall indicate the total price of the tender, excluding VAT, and the reasons for not paying VAT. The supplier shall assess whether it will become subject to VAT during the performance of the contract. If the supplier will become subject to VAT during the performance of the contract, the tender shall indicate the price including VAT. The prices of the tenders will be evaluated and compared inclusive of all taxes, including VAT. In the event that the contracting authority itself is required to pay VAT to the State budget for the purchase of the subject of the contract in accordance with the procedure laid down by the laws governing taxation and their implementing legislation, this tax shall be included in the price of the tender (if the supplier did not include it when submitting the tender, it shall be added by the contracting authority itself for the purposes of comparison). The tender price must include all taxes and all other direct and indirect costs and charges incurred and/or likely to be incurred by the Supplier in connection with the work/service to be performed.

Table 3. Traffic light control software maintenance services.

| Eil. No. | Name | Unit of measurement | Quantity | Units price EUR without VAT | Total price EUR without VAT (4)*(5) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Maintenance services for the traffic light control software _______________________* | month | 54 |  |  |
| Total comparative tender price in EUR, exclusive of VAT** |  |  |  |  |  |
| VAT (EUR)*** |  |  |  |  |  |
| Total comparative tender price in EUR, including VAT*** |  |  |  |  |  |

* The manufacturer of the software shall be indicated.

** The price of the tender must be expressed to no more than two decimal places.

*** In cases where the tenderer is not required to pay VAT under the legislation in force, the tenderer shall indicate the total price of the tender, excluding VAT, and the reasons for not paying VAT. The supplier shall assess whether it will become subject to VAT during the performance of the contract. If the supplier will become subject to VAT during the performance of the contract, the tender shall indicate the price including VAT. The prices of the tenders will be evaluated and compared inclusive of all taxes, including VAT. In the event that the contracting authority itself is required to pay VAT to the State budget for the purchase of the subject of the contract in accordance with the procedure laid down by the laws governing taxation and their implementing legislation, this tax shall be included in the price of the tender (if the supplier did not include it when submitting the tender, it shall be added by the contracting authority itself for the purposes of comparison). The tender price must include all taxes and all other direct and indirect costs and charges incurred and/or likely to be incurred by the Supplier in connection with the work/service to be performed.

Table 4. Additional services.

| Eil. No. | Name | Unit of measurement | Quantity**** | Units price EUR without VAT | Total price EUR without VAT (4)*(5) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Traffic light control software _______________________* improvement /development services | hour | 300 |  |  |
| 2. | Integration services of new intersection/ crossing controllers into the traffic light control software _______________________* | pcs. | 50 |  |  |
| 3. | Additional training services | hour | 100 |  |  |
| Total comparative tender price in EUR, exclusive of VAT** |  |  |  |  |  |
| VAT (EUR)*** |  |  |  |  |  |
| Total comparative tender price in EUR, including VAT*** |  |  |  |  |  |

* The manufacturer of the software shall be indicated

** The price of the tender must be expressed to no more than two decimal places.

*** In cases where the tenderer is not required to pay VAT under the legislation in force, the tenderer shall indicate the total price of the tender, excluding VAT, and the reasons for not paying VAT. The supplier shall assess whether it will become subject to VAT during the performance of the contract. If the supplier will become subject to VAT during the performance of the contract, the tender shall indicate the price including VAT. The prices of the tenders will be evaluated and compared inclusive of all taxes, including VAT. In the event that the contracting authority itself is required to pay VAT to the State budget for the purchase of the subject of the contract in accordance with the procedure laid down by the laws governing taxation and their implementing legislation, this tax shall be included in the price of the tender (if the supplier did not include it when submitting the tender, it shall be added by the contracting authority itself for the purposes of comparison). The tender price must include all taxes and all other direct and indirect costs and charges incurred and/or likely to be incurred by the Supplier in connection with the work/service to be performed.

**** The quantity indicated is the maximum quantity, the Contracting Authority will purchase on an as-needed basis and is not obliged to purchase the full quantity indicated in Table 4 Additional Services.

Table 5. Total price of the tender (sum of the prices in Tables 1, 2, 3 and 4).

| Ser. No | Name | Total price in eur, exclusive of VAT | VAT, EUR | Total price, EUR including VAT |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 (3)+(4) |
| 1. | Table 1. Traffic light controllers by sets |  |  |  |
| 2. | Table 2. Traffic light control software with licences and installation services. |  |  |  |
| 3. | Table 3. Traffic light control software maintenance services. |  |  |  |
| 4. | Table 4. Additional services |  |  |  |
| Total comparative tender price EUR incl. VAT** |  |  |  |  |

** The price of the tender must be expressed to no more than two decimal places.

*** In cases where the tenderer is not required to pay VAT under the legislation in force, the tenderer shall indicate the total price of the tender, excluding VAT, and the reasons for not paying VAT. The supplier shall assess whether it will become subject to VAT during the performance of the contract. If the supplier will become subject to VAT during the performance of the contract, the tender shall indicate the price including VAT. The prices of the tenders will be evaluated and compared inclusive of all taxes, including VAT. In the event that the contracting authority itself is required to pay VAT to the State budget for the purchase of the subject of the contract in accordance with the procedure laid down by the laws governing taxation and their implementing legislation, this tax shall be included in the price of the tender (if the supplier did not include it when submitting the tender, it shall be added by the contracting authority itself for the purposes of comparison). The tender price must include all taxes and all other direct and indirect costs and charges incurred and/or likely to be incurred by the Supplier in connection with the work/service to be performed.

Tender price in words _________________________________________________________________

(indicate the price of the tender in words in EUR including VAT)

The price of the tender shall include all fees payable by the Service Provider and all costs incurred by the Service Provider in connection with the preparation of the tender and the performance of the contract, including the submission of electronic invoices.

Information on the value of the part of the contractual obligations to be carried out by each partner in the group of suppliers on its own account (to be completed in the case of a group of suppliers):

| Ser. No | Name of the partner | Anticipated obligations under the contract | Value of the partner's share of the contractual obligations in the tender price |  |
| --- | --- | --- | --- | --- |
|  |  |  | EUR, exclusive of VAT | Percentage |
| 1. |  |  |  |  |
| ... |  |  |  |  |
| Total: |  |  |  |  |

The tenderer must disclose in the tender the other entities whose capacities he relies on, as well as other known subcontractors whose capacities are not relied on:

| Ser. No | Name, code and address of the entity | The part of the contract to be performed | Proportion of the contract price for which entities are intended to be used |  |
| --- | --- | --- | --- | --- |
|  |  |  | EUR, exclusive of VAT | Percentage |
| Entities whose capacities are relied on to demonstrate compliance with the qualification |  |  |  |  |
| 1. |  |  |  |  |
| ... |  |  |  |  |
| Total: |  |  |  |  |
| Known sub-suppliers to be used for the performance of the contract whose capacities are not relied on for the purpose of demonstrating compliance with the qualification |  |  |  |  |
| 1. |  |  |  |  |
| ... |  |  |  |  |
| Total: |  |  |  |  |

The following documents shall be submitted with the tender:

| Ser. No | Document titles |
| --- | --- |
| 1. | ESPD |
| ... |  |

This tender contains confidential information:

| Ser. No | Title of the document submitted | Confidential information contained in the document (specify the part of the document / page containing the confidential information) | Justification for confidential information (explaining on what basis the document or part of the document is confidential) |
| --- | --- | --- | --- |
| 1. |  |  |  |
| ... |  |  |  |

Please note that, in accordance with Article 20(2)(2) of the Republic of Lithuania Law on Public Procurement, the price (including rates) indicated in the Service Provider's tender cannot be considered confidential information. In this respect, if the Supplier indicates that the rates and/or prices are confidential, this information of the Service Provider's tender will be published in the Central Public Procurement Information System without further notice, in accordance with Article 86(9) of the Republic of Lithuania Law on Public Procurement.

We declare that the tenderer (each partner in the group of suppliers), the subcontractor (in cases where his share of the value of the contract is greater than 10 %) and the other entity whose capacities are relied on (in cases where his share of the value of the contract is greater than 10 %) are not:

(a) a Russian citizen, natural or legal person, entity or body established in Russia;

(b) a legal person, entity or body in which more than 50 % of the ownership rights are held, directly or indirectly, by the entity referred to in point (a);

(c) a natural or legal person, entity or body acting on behalf of, or at the direction of, the entity referred to in point (a) or (b).

If there are doubts about the information declared in the supplier's tender, the contracting authority may at any time during the procurement procedure ask the supplier to submit documents confirming compliance with Council Regulation (EU) 2022/576 of April 8, 2022, which amends Regulation (EU) No 833/2014 on restrictive measures in view of Russia's actions destabilizing the situation in Ukraine (Article 5k(1))

If the qualifications for the right to pursue the activity in question have not been checked or have not been checked in full, we undertake that the contract will be performed only by persons who are qualified to do so.

The tender is valid until the end of the time limit specified in the procurement documents.

__________________________	__________		__________________________

Tenderer or the person 		signature                                forename and surname

authorised by him

Indicate the reason if there is no such person(s).

The period of the guarantee must be stated in years, without the number of months or days. The Supplier must indicate the exact number of years; the Contracting Authority does not accept ranges or other imprecisely defined terms.

To be completed if confidential information will be provided. If the tenderer does not complete this table and/or does not indicate 'confidential' in the title of the file/file, the tenderer's submission shall be deemed not to contain confidential information.