Pirkimo sąlygų

1 priedas

Annex 1 to the Procurement conditions

ORO TAIKINIO (OT) PALEIDIMO PASLAUGA

TECHNINĖ SPECIFIKACIJA

AIR TARGET (AT) LAUNCH SERVICE

TECHNICAL SPECIFICATION

| 1. PIRKIMO OBJEKTO PASKIRTIS: Oro taikinio (toliau – OT) paleidimo paslauga (toliau – Paslauga) skirta kovinio šaudymo metu, iš raketinės oro gynybos sistemos RBS–70. | 1. PURPOSE OF THE PURCHASE: Air target (next – AT) launch service (next – service) is intended for use during combat firing from the RBS-70 air defence missile complex. |
| --- | --- |
| 2. BENDRI REIKALAVIMAI: Perkama paslauga skirta kovinio šaudymo pratybų metu, kurios vyks Baltijos jūroje, Lietuvos teritorijoje; 3. TECHNINIAI REIKALAVIMAI: 3.1. OT paleidimas turi įvykti tiksliai nurodytu laiku pagal pratybų vadovo sumanymą; 3.2. Naujo OT paruošimo/pertaisymo paleidimui laikas – ne ilgesnis kaip 3 val.; 3.3. Skrydžio valdymas – skrydžio planas turi būti suderintas su pratybų vadovo nurodyta trajektorija; 3.4. OT techninės charakteristikos pateiktos vadovaujantis leidiniu ,,Šaudymo kursas (Oro gynybos ginklai)“ (A-LK-07.5) kuris patvirtintas Lietuvos kariuomenės vado 2015 m. spalio 26 d. įsakymu Nr. V-1306 ,,Dėl leidinio ,,Šaudymo kursas (Oro gynybos ginklai)“ (A-LK-07.5) tvirtinimo“: 3.4.1. Oro taikinio greitis naikinimo zonoje – ne mažesnis nei 102 m/s (200 mazgų) ir ne didesnis kaip 200 m/s (389 mazgai) su 10 proc. paklaida; 3.4.2. Išskleisto OT ilgis – ne trumpesnis nei 5500 mm ir ne ilgesnis 8500 mm; 3.4.3. OT turi patikimai veikti nuo –10ºC iki +40ºC; 3.4.4. OT priekinis diametras – ne mažesnis nei 500 mm; 3.4.5. OT galinis diametras – ne didesnis nei 960 mm; 3.4.6. Skrydžio laikas ne mažesnis nei 20 min.; 3.4.7. Spalva turi būti raudona, oranžinė arba juoda, kad išsiskirtu iš dangaus ir gamtovaizdžio fono, tam kad būtų galima OT stebėti ir sekti vizualiai; 3.4.8. OT modelis turi turėti ne mažesnį kaip 0,2 m2 radarų sistemoms matomą atspindžio plotą arba papildomą įrenginį, kuris padidina jo atspindžio plotą iki keliamų reikalavimų. 4. GARANTINIAI REIKALAVIMAI: Tiekėjas įsipareigoja atlikti paslaugą kokybiškai ir pagal sutartus reikalavimus. Paslauga laikoma tinkamai suteikta, jei ji atitinka kliento užsakymo sąlygas ir buvo atlikta sutartu laiku bei apimtimi. 5. PAPILDOMA INFORMACIJA: Nėra. | 2. GENERAL REQUIREMENTS: The service to be procured is for a live firing exercise to be held in the Baltic Sea, on the territory of Lithuania; 3. TECHNICAL REQUIREMENTS: 3.1. The AT release shall take place at the exact time specified by the Firing Officer; 3.2. The time to prepare/refurbish a new AT for launch shall not exceed 3 hours; 3.3. Flight control - the flight plan shall be aligned with the trajectory specified by the controller; 3.4. The technical characteristics of the AT are provided in accordance with the publication "Shooting Course (Air Defence Weapons)" (A-LK-07.5) approved by the Order of the Commander of the Lithuanian Armed Forces No V-1306 of 26 October 2015 "On Approval of the Publication 'Shooting Course (Air Defence Weapons)' (A-LK-07.5)": 3.4.1. The air target speed in the kill zone shall be not less than 102 m/s (200 knots) and not more than 200 m/s (389 knots); 3.4.2. The length of the deployed AT shall be not less than 5500 mm and not more than 8500 mm; 3.4.3. The AT shall operate reliably between -10ºC and +40ºC; 3.4.4. The front diameter of the AT shall not be less than 500 mm; 3.4.5. The rear diameter of the AT shall not exceed 960 mm; 3.4.6. A flight time of not less than 20 minutes; 3.4.7. The colour shall be red or black to stand out against the sky and landscape background for visual observation and tracking; 3.4.8. The AT model shall have a minimum reflecting area visible to radar systems of 0,2 m2 or an additional device that increases its reflecting area to the required level. 4. WARRANTY REQUIREMENTS: The Supplier undertakes to carry out the service to the quality and in accordance with the agreed requirements. The service shall be deemed to have been duly provided if it complies with the terms of the customer's order and has been performed within the agreed time and scope. 5. ADDITIONAL INFORMATION: There is no. |