Annex 1.2 to the Terms and Conditions of the Procurement

TECHNICAL SPECIFICATION

OF TRAFFIC LIGHT CONTROL SOFTWARE, INSTALLATION, MAINTENANCE, ADDITIONAL SERVICES AND SUPPLY OF INTERSECTION CONTROLLERS

CONTENTS

I.	MAIN CONCEPTS	2

II.	GLOSSARY OF TERMS	2

III.	GENERAL REQUIREMENTS	5

IV.	SYSTEM REQUIREMENTS	6

a.	System functionality	6

b.	Public transport priority module	14

c.	Cyber security	16

d.	Licences, warranty and copyright	17

V.	INSTALLATION SERVICES	18

a.	System installation	18

b.	Entry of data of intersection controllers	19

c.	Data integration	20

d.	Training	22

VI.	MAINTENANCE SERVICES	23

VII.	ADDITIONAL SERVICES	25

VIII.	SUPPLY OF CONTROLLERS	26

IX.	FINAL PROVISIONS	31

Annex 1 to the Technical Specification	33

Annex 2 to the Technical Specification	46

Annex 3 to the Technical Specification	54

Annex 4 to the Technical Specification	56

Annex 5 to the Technical Specification	58

Annex 6 to the Technical Specification	80

Annex 7 to the Technical Specification	81

- MAIN CONCEPTS

The contracting authority shall mean the municipal enterprise “Susisiekimo paslaugos” (hereinafter - Customer).

Object of the procurement shall mean traffic light control software (hereinafter - the System), installation and maintenance services and the supply of traffic light controllers.

Description of the object of the procurement: the purchased services include:

- The purchase of the System and its licences.

- The purchase of services for the installation, configuration, connection of existing intersection controllers to the System via the OCIT protocol and training (hereinafter - Installation Services).

- The purchase of maintenance services for the System (hereinafter - Maintenance Services).

- The purchase of services for the improvement/development of the System, the integration of new controllers and additional training (hereinafter - Additional Services).

- The supply of 210 new controllers compatible with the System.

Time limit:

- The Installation services must be completed within 6 months from the date of entry into force of the Contract.

- Maintenance Services shall be provided 54 months after completion of the Installation Services and handover to the Customer.

- The delivery of the 210 new controllers is expected to take place within a period of 60 months from the date of entry into force of the Contract.

Place of the Services provision - Laisvės Ave. 10A, in Vilnius city. The Maintenance Services are provided remotely.

- GLOSSARY OF TERMS

2FA – sometimes called two-step verification or two-factor authentication - is a security process where users provide two different authentication factors to verify themselves. It is an access control security method that requires two types of identification to access resources and data.

API (Application Programming Interface) – a method (interface) of communication between systems, which is based on a set of standards and protocols.

ArcGis - geographic information system software.

AVL (Automatic vehicle locating) - automatic vehicle location system for determining the location, speed and stops of vehicles.

Backup – a copy of computer data made and stored elsewhere so that the original can be restored in the event of data loss.

Cycle time – the time interval during which each signal group (intersection direction) receives a green signal.

Synchronization of cycle times – equalization of the cycle time of the signal plans of a given group of intersection controllers occurs.

CVE (Common Vulnerabilities and Exposures) DB – a database containing a list of publicly known information security gaps and vulnerabilities. The purpose of CVE is to identify, define and catalogue publicly disclosed cyber security vulnerabilities.

Operating mode – the signal plan activated in the intersection controller, during the cycle of which the green signal is distributed to all directions of intersections and pedestrian crossings.

Detail information (about the object) window – the system window for viewing detailed information about a particular object (such as an intersection controller), analyzing faults, and managing the object.

DHCP (Dynamic Host Configuration Protocol) – a network management protocol used to dynamically assign an IP address to any network device or node so that it can communicate using IP.

DWH (data warehouse) - electronic data storage of the organisation.

Phase (Stage) – a set of several signal groups that do not conflict with each other. Can be activated once or several times during one cycle time.

GET – a HTTP request method used to retrieve information from a server without modifying it.

GNSS (Global Navigation Satellite System) – a generic term for any constellation of satellites that provides positioning, navigation and timing services on a global or regional scale.

GPS (Global Positioning System) – Global Positioning System. It allows the coordinates of an object to be determined anywhere in the world, in all weather conditions, when signals from four or more satellites are available.

Interactive (operating modes) diagram (diagram) - a real-time graphical interface to the System, showing changes of traffic light signals, sensors, buttons and other equipment data.

Intergreen time shall mean safety time. The safety time is the time interval between the end of the allowable signal of one set of signals and the start of the allowable signal of the next set of signals intersecting with it. During the safety time, the prohibition signals shall be lit simultaneously for road users clearing and about to enter the intersection or a pedestrian crossing controlled by traffic lights. The safety time shall be calculated taking into account the best practices of the Member States of the European Union (e.g. in accordance with the RiLSA Guidelines for Traffic Signals).

IoT (Internet of things) – describes devices with sensors, processing capabilities, software and other technologies that connect and exchange data with other devices and systems over the internet or other communication networks.

Event - a message recorded in the System about a change in the status of an intersection controller or peripheral equipment, about commands sent from the System to the controllers, and other messages automatically generated by the System and necessary for the monitoring of intersection controllers and peripheral equipment.

Travel time (duration) - the total time spent travelling a given distance. Average travel time is the average number of trips on a given link or corridor.

Coordination – the coordinated operation of several or dozens of intersection controllers in a coordinated mode of operation that allows a group of vehicles travelling in the same direction at the speed limit to pass as many intersections as possible through a green traffic signal.

Hotfix or Bugfix – a small piece of code designed to fix a major bug in the System software. A software update that fixes a bug or vulnerability in the System.

Waiting time – the time from the moment a vehicle arrives at a traffic-light-controlled intersection or crossing and joins the traffic queue until it crosses the Stop line at a green traffic light.

LOS (Level of Service) - a qualitative indicator used to evaluate the quality of car traffic services. LOS is used for road and intersection analysis, in categorizing traffic flows, and in assigning traffic quality levels based on indicators such as vehicle speed, density, congestion, etc.

Object – a device integrated into the System (intersection controller, sensor, traffic light, pedestrian button, acoustic device) or a virtual object created in the System (measurement point, coloured line on the map).

Object layer (Layer of objects) - a collection of homogeneous objects (for example, of intersection controllers) that are displayed on a map.

OCIT (Open Communication Interface for Road Traffic Control Systems / Open Interfaces for Traffic Engineering) – open communication interface of road traffic management systems - from the central system of the traffic control centre to the intersection control equipment.

OCPI (Open Content Provider Interface) – a comprehensive interface that allows integrating other systems quickly and cost-effectively. In addition, the interface converters allow maintain communication according to other standards, such as WanCom, TLS, DAV, DATEX II, VDV, etc.

Odoo platform (service desk) – the software installed in the Customer‘s infrastructure that is intended for registering fault in the Customer‘s System, controlling the time limits for fault removal, for providing official questions, answers and comments about the causes of faults and the course of the solution.

Offset time – the time offset, which is the basis for timing the coordination signals. This is the time difference from the reference point in the cycle at the upstream intersection to the same point in the cycle at the downstream intersection. The start of the main street green signal is usually taken as this reference point.

PUSH – a HTTP request method used to upload new information to the server or update the existing information.

Ridango – a system of public transport electronic ticket and of and real-time monitoring of public transport.

Intersection peripheral equipment - the equipment, that is installed at intersections or crossings regulated by traffic lights, which is intended for the management of traffic, pedestrian or cyclist flows, that is, traffic lights, sensors, buttons, acoustic devices, traffic light poles, etc.

Status of intersection peripheral equipment –the status of the sensors, traffic lights, pedestrian buttons, acoustic devices and other traffic control equipment connected to the intersection controller, which informs about the activation, deactivation, fault of the equipment, the fault of the traffic light lamps.

Status of intersection controller – controller status informing about its operating mode: (on, off, broken (warning, alarm, communication fault), runs in local mode, runs according to the System calendar, runs in manual mode, a special plan is activated, runs in flashing mode, etc.).

Intersection controller control window – a graphical interface for enabling the main control functions of the intersection controller.

Intersection controller (or controller) – switching cabinet, which is installed near an intersection or crossing controlled by traffic lights, in which electronic control units (components) intended for the control of traffic lights and other intersection peripheral equipment and data transmission to the System are installed.

Group of intersection controllers - a virtual list of intersections compiled in the System with a user interface, the purpose of which is to control not one, but several intersections at the same time.

Signal group – a group of traffic lights for controlling one direction of an intersection.

Signal plan – the operating mode activated in the intersection controller, during the cycle of which the green signal is distributed to all directions of intersections and pedestrian crossings.

System log files - log files are historical records of everything that happens on the System, including events such as operations, errors, and hacking. This data can be transmitted in a variety of ways and can be structured, semi-structured or unstructured.

System availability (reliability) - property of an object operating in a defined mode and under defined operating and technical service conditions to perform its functions for a specified time, by maintaining the set performance characteristics.

SiTraffic Office – software for planning, configuring and sending the new configuration to the controller for traffic light intersections and crossings.

SiTraffic Scala – software for monitoring and managing traffic light intersections and crossings and other traffic control equipment.

SOAP (Simple Object Access Protocol) – a lightweight XML-based protocol used to exchange information in decentralized, distributed application environments. SOAP messages can be transmitted in any way required by the application if the Customer and server use the same method.

Contract – the contract signed between the Customer and Supplier for the System installation and maintenance and controller supply.

NIS2 - the latest EU directive aimed at strengthening cyber security in critical sectors. The purpose of the Directive: ensure a high overall level of cyber security and improve capabilities to prepare for and respond to cyber incidents in public and private sector organisations which will fall into the list of necessary or important activities.

Control level – functionality that allows executing first the commands that have a higher priority - are at a higher level.

User interface - graphic layout of website or application, which allows users to interact with the System. It is a bridge between the user and the System, allowing users to access and control System functions.

PTP module – a public transport priority module, part of the System for reducing the journey completion times of public transport. The main principle of operation of the PTP module - conditional or unconditional early activation of the green signal or extension of the duration of the green signal when public transport vehicles approach intersections regulated by traffic lights. The PTP module receives information about the location of public transport from the AVL system.

Web application - application software that is accessed through a web browser.

Interactive diagram of green corridors (green corridor diagram or time and distance diagram) - a diagram in which the trajectories of vehicle flow through several intersections or crossings controlled by traffic lights are plotted. The diagram shows how the traffic flow, which started moving at the first intersection of the corridor, will move at a constant speed through the other intersections of the corridor.

- GENERAL REQUIREMENTS

- The Systems requirements are described in Chapter IV:

  - The Supplier must install the System, the functionality of which meets the System functionality requirements described in Chapter IV.a.

  - A public transport priority module, capable of accepting the data provided by the Customer about the movement of public transport vehicles from Ridango AVL system installed in the Customer's infrastructure, must be installed in the System. Detailed requirements for the functionality and data integration of the public transport priority module are presented in Chapter IV.b.

  - The Supplier must install a System that meets the cyber security requirements described in Chapter IV.c.

  - System licensing, warranty conditions and copyright are described in Chapter IV.d.

- The Installation Services are described in Chapter V:

  - The Supplier must install a new System with licences for monitoring and management of traffic light intersections and crossings in Vilnius City, integrating all 290 units of controllers of Vilnius city intersections through the OCIT protocol, according to the System installation requirements described in Chapter V.a.

  - Entry of intersection controller data, and its migration from the existing SiTraffic Scala traffic light regulation software to the new System is described in Chapter V.b.

  - The Supplier must ensure that statistical data on traffic, cyclist and pedestrian flows collected in the newly installed System are transferred to the existing data warehouse of the Customer (hereinafter - DWH). Chapter V.c. Data Integration provides a description and operation of the current data transfer interfaces between the existing SiTraffic Scala, ArcGis and DWH systems.

  - The Supplier must provide the Customer with at least 80 hours of training (with practical tasks), enabling use of the newly installed System and administering of the the System. Detailed training requirements are provided in Chapter V.d. Training.

- After the System installation, the Supplier must ensure continuous maintenance and troubleshooting of the System. The requirements for Maintenance Services are described in the Chapter VI Maintenance Services.

- The Supplier must provide the Customer with the opportunity to purchase additional System improvement/development services at an hourly rate, services of integration of intersection controllers and additional training services (hereinafter - the Additional Services). The list of additional services is provided in Chapter VII.

- The Customer plans to purchase intersection controllers that are compatible with the System and meet the functional requirements described in the Technical Specification. The conditions for the supply of the controllers are set out in Chapter VIII.

- SYSTEM REQUIREMENTS

System functionality

- The purpose of the System is monitoring and management of the traffic light intersections in Vilnius city. The System must have a user-friendly web-based user interface. The main language of the user interface is Lithuanian, the second language is English.

- An up-to-date and constantly updated map of Vilnius city with 290 controller icons of all intersections, as a separate layer of objects, must be displayed in the main window of the System. Information about the status of the intersection controllers and peripheral equipment must be constantly (in real time) updated on the map.

- The icons of the intersection controllers displayed on the map must represent in different colours the different statuses of the intersection controllers or peripheral equipment:

  - Green: The controller is on and is running as per the System calendar, there are no faults of the controller or other peripheral equipment.

  - Yellow: the controller is on and is running locally according to the internal calendar, there are no faults of the controller or other peripheral equipment.

  - Red: the controller has detected a fault of the controller or other peripheral equipment, that caused all traffic lights to go out.

  - Orange: the controller has detected a fault/warning of the controller or other peripheral equipment, traffic lights signals are operating.

  - Purple: the controller has detected a communication fault.

  - Sky blue: the controller has detected malfunction of the local traffic adaptive operation.

  - Dar blue: the controller is running in yellow flashing mode.

  - Other colours: dark green or black represent different control modes – manual plan, special plan, etc.

- The Customer must be granted administration rights to independently configure the colour gamma of the statuses of the intersection controllers or peripheral equipment, and the ranking (priorities) of their representation on the map.

- The Supplier must configure a map to display in real time the LOS parameter for each intersection direction (of each signal group) - a coloured line as a separate layer of objects:

  - The LOS A or B level of any direction of the intersection should be displayed as a green line.

  - The LOS C or D level of any direction of the intersection should be displayed as a yellow line.

  - The LOS E or F level of any direction of the intersection should be displayed as a red line.

- The System must allow the user to switch different object layers by enabling or disabling displaying of the object layer on the map. The user can activate any number of layers simultaneously. The System must have at least five object layers, which can be displayed all at once or one by one on the map:

  - Layer of intersection controllers.

  - Layer of peripheral equipment of intersection.

  - LOS rendering layer.

- The user of the System can interact with any layer of objects by directly clicking on the icon of a certain layer object (for example, intersection controller) shown on the map. When interacting with an object on the map, a detail information window about the object can be called up.

- The System must allow creation for each user individual views of the System windows, including a dashboard with customisable controls bound to the logged-in user, i.e. personalisation of the System. The user can return to the same view from a previous login session.

- The map must be displayed under a free data license.

- The following basic functionality designed for a traffic management specialist must be implemented in the System:

  - Detail information window: detailed status of the intersection controller and peripheral equipment (whether there are no defective traffic light modules, whether the controller connection or voltage supply is has not failed, whether the sensors are running properly, and sensor calculations are provided), operating mode, signal plan, traffic adaptive mode status, history of events, intersection chart (drawing), access to the stream broadcasted by video surveillance cameras, and other information necessary for the control of the intersection.

  - In case of fault of the intersection controller or peripheral equipment, the System must immediately report this in a pop-up window in the user interface or in another interactive way. The System must also send a fault notification by e-mail and SMS. Message templates and the list of contacts must be easily adjusted.

  - The System shall classify errors of controllers connected via the OCIT protocol into several categories based on the specific type of error, the specific controller and its number. The traffic management specialist may configure the error classification.

  - The System user must be allowed to confirm a fault message in the System and add a note or comment to the fault description.

  - The System shall be configurable for various actions when user defined criteria are met in relation to sensor data, e.g. to generate configurable pop-ups with information for the traffic management specialist with customisable recommended actions, or to execute management commands automatically. The system shall also have a configurable hysteresis function for achieving and rejecting the defined criteria. Export of saved configurations in open data format (.json).

  - Link to the intersection controller control window with functions of switching of signal plans, setting and changing the level of traffic adaptive mode, turning on/off the intersection controller, enabling/disabling plans for special services, and with a representation of the operation of the sensors and their calculation must be in the detailed information window.

  - The function for activation/deactivation of the flexible/fixed mode of the intersection (TA - traffic actuation, IT - individual traffic, PT - public transport) shall be implemented from the user interface of the System and shall not only be compatible with the System manufacturer's controllers, but shall also work with the existing Siemens/Yunex controllers that communicate with the System via the OCIT protocol and that are already compatible with this functionality in the SiTraffic Scala system..

  - Functionality of grouping of intersection controllers and control of intersection controller groups must be implemented in the System.

  - Multi-level control for selecting a different operating mode at different control levels of intersections or their groups must be implemented in the System.

  - The System must receive messages from the intersection controllers according to the requirements described in OCIT protocol and store the history of the System events for at least 2 years in the System database. The event log can be printed or exported in files of .pdf and .csv formats.

  - The System must display in real time (no later than within 10 s) the operating mode of the intersection (interactive operating mode diagram) by presenting by means of a graphic diagram rendering of operation of signal groups, sensors, pedestrian buttons, and other peripheral equipment connected to the controller. The diagram resolution is one second. The user can choose which signal group and which sensor to include in the diagram. The user can save the created diagram template and share it with other users.

  - While viewing the diagram, the user can scroll the diagram forward, backward and measure the time elapsed between two events (for example, signal change) on the diagram. The user can enable multiple interactive diagrams of real-time or of historical data at the same time. The data of interactive diagrams can be printed or exported to files of .pdf, .jpg and .csv formats. The user can create search queries/filters based on defined criteria related to the duration of the signal groups or the occupancy or demand data of the sensors displayed in the graph. The system shall store the search criteria.

  - History of at least 2 years about the status of intersection controllers, control commands and an interactive diagram of operating modes must be stored in the System.

  - An interactive diagram of green corridors (time and distance diagram) must be rendered in the System in real time with the possibility to view 2 years old historical data, and to export to files of .pdf and .jpg formats:

    - Numbers of intersections and coordinated signal groups, speed, distances between intersections, speed lanes with one or both directions of coordination must be rendered in the time and distance diagram.

    - The System must allow the user to configure both the number of the intersections and signal groups displayed on the diagram, as well as the speed and distance between the STOP lines of the intersections.

    - The interface of the time and distance diagram shall include an input field where the user can enter the desired speed, and then the coordination behaviour will be simulated for different speed conditions, visualised in an alternative time and distance diagram.

    - The System shall notify the user if the controllers assigned to the coordination corridor are not functioning properly or configured coordination are not in proper operation.

  - The system must carry out a reliability check of the data of transport sensors and report incorrect sensor calculations in the user interface.

  - Data filtering and search functionality, which would allow quick access to a certain window or System object must be installed in the System. Main search and filtering criteria:

    - System objects (intersections, controllers, sensors and other peripheral equipment);

    - users;

    - event history messages;

    - interactive diagram of operating modes;

    - interactive diagram of green corridors;

    - special response plan.

- The Supplier must provide a mobile System management solution for monitoring the status of the System objects from mobile devices (mobile phone or tablet with Android or iOS operating systems).

- The mobile System monitoring solution must provide the user with the following basic information:

  - A map corresponding to the description in Point 7, with icons of intersection controllers, the colour of which corresponds to a certain status of controller described in Point 8.

  - Event history and detailed fault descriptions for intersection controllers (errors, control commands, state changes, etc.)

  - The operating mode that is currently enabled, and history of operating modes, that is, history of switching of signal plans.

  - Interactive diagram of operating modes.

  - Grouping of controllers by status.

  - Display an overview of the states of the controller components.

  - Control of controls: signal plan switching.

  - Manual stage switching or activation of emergency signal plans.

- Statistics module with the following basic functionality must be installed in the System:

  - Export of data (at least 2 years old) about traffic flows in .csv format. Content must match user-defined filters and ranking.

  - Displaying of historical data (window with data graph and export of .pdf, .jpg files). Content must match user-defined filters and ranking.

  - The System must allow creation and saving of statistical data display templates based on user-defined filters.

  - Possibility for the user to create data export templates by selecting different displayed data fields/columns and different data aggregation intervals.

  - The System must display in real time statistical traffic data from sensors or their groups after selecting a particular sensor in the user interface.

  - The System, in using statistical traffic data from the existing sensors, must allow monitoring and analysis of travel times (current, average, maximum, minimum), traffic flow speed (current, average, maximum, minimum), and waiting time data (current, average, maximum, minimum).

  - The system shall be able to classify traffic flow data or to receive classified data from those traffic sensors that are connected directly to the controller network and have an API implemented in them. The main data classes that must be in the System: passenger car, van, truck, public transport vehicle, bicycle, scooter, pedestrian.

- The System shall include data fields (attributes) that data ir data reports can be viewed in the user interface, exported to .csv and other formats agreed with the Customer, and transferred via API to third party systems.

- A quality management module to provide the following reports must be installed in the System:

  - The System shall prepare diagnostic and operational status reports according to the messages stored in the event history and according to the status history of the intersection controllers and peripheral equipment:

    - The fault-free lifetime of the controller and sensor(s) within a user-defined range.

    - Time of different active events between the controller and the sensor(s) (e.g. how long it took for an alarm condition to occur).

    - Duration of connection failures.

    - Values of sensor calculation, data accuracy.

    - Durations of green and red signals of signal groups.

    - Individual transport waiting time.

  - The quality management module must provide the user with reports on the operational reliability of the equipment, the templates of which can be flexibly configured according to user-defined filters.

  - An automatic part of the quality management module, which would perform automatic monitoring of the System parameters and provide the user with a numerical and percentage value must also run in the System:

    - The duration of signals of signal groups and compliance with user-defined ranges.

    - Sensor occupancy and non-occupancy duration.

    - Length of cycle time.

    - Length of waiting time.

- Adaptive module for real-time optimization of operating modes must be installed in the System and work with 210 units of new controllers proposed by the Supplier and with the System Manufacturer's controllers already installed in the city. Its solution can be implemented in two different ways at the System level (the module is integrated into the System or installed in the Customer's infrastructure) or at the controllers (local) level (optimization of the operating modes of intersection controllers takes place in the controllers without interaction with the System. They communicate with each other via OCIT or TCP/IP communication protocols). The Supplier must choose at least one of the solution methods and implement at least one of the solutions provided.

  - If the adaptive module for real-time optimization of operating modes offered by the Supplier is implemented at the System level, then it is mandatory to fulfil the functionality presented in Table 1. The implementation of this point depends on the models of the controllers installed at the intersections and can be implemented after installing controllers that support receiving and executing commands of the adaptive module for real-time optimization of operating modes of the System.

  - If the adaptive module for real-time optimization of operating modes offered by the Supplier is implemented at the level of controllers, then it is mandatory to fulfil the functionality presented in Table 2. The implementation of this point depends on the controllers installed at the intersections and can be implemented with the existing intersection controllers or after the installation of new intersection controllers after the reconstruction of the intersections.

Table 1. Functionality of the adaptive module for real-time optimization of operating modes at the System level.

| Ser. No | Characteristics | Requirement |
| --- | --- | --- |
| 1. | Receipt of real-time traffic flow data from transport sensors needed for generating signal plans, continuous updating and storage in the System | Not less often than every 45 s |
| 2. | Generation of new signal plans with green signal distribution for each stage/signal group according to traffic data received in real time in the System | Not less often than every 5 minutes |
| 3. | Sending the newly generated signal plans to the intersection controllers and enabling them | Immediately after generation, simultaneously on all controllers in one corridor. The maximum delay for sending and activating signal plans is 5 s. Another technology for sending signal plans to the controllers and turning them on can be applied with a special longer delay - for better coordination. |
| 4. | Synchronization of cycle time (if applicable) of newly generated and enabled signal plans | No more than one cycle time |
| 5. | Dynamic adaptation of offset times of green corridors to traffic flow intensity | Mandatory |
| 6. | Duration of green corridor coordination performance | At least 20 min. The synchronisation of the signal plans must not disrupt the coordination of the green corridor. Coordination may not be disrupted more frequently than every 20 minutes. |
| 7. | Algorithm for optimizing the length of traffic queues (congestion) up to the intersection | In generating signal plans, the length of traffic queues to the intersection must be assessed based on data from the existing transport sensors (without installing additional remote sensors) or the output of an adaptive algorithm based on existing traffic sensor calculations |
| 8. | Data reliability | In the event of a disruption in the data providing of transport sensors, historical data for the same time of the day or theoretical data intended for the generation of signal plans must be used. |

Table 2. Functionality of the adaptive module for real-time optimization of operating modes at the controller level

| Ser. No | Characteristics | Requirement |
| --- | --- | --- |
| 1. | Use of real-time data required for generating signal plans | The controller must estimate the amount of traffic in real time and adjust the minimum and maximum ranges of green signal phases/signal groups according to the intensity. It is also necessary to ensure the transmission of real-time traffic flow data to the System at least every 45 seconds. |
| 2. | Generation of new signal plans with green signal distribution for each stage/signal group according to traffic flows from traffic sensors calculated in the controller | During the time of each new cycle, the cycle time and green signal duration correction according to traffic intensity must take place. The controller selects the required cycle time (if necessary) based on the intensity of traffic flows in real time. The controller must incorporate intelligent algorithms (based on AI algorithms and/or traffic flow modelling principles) to select the appropriate green signal duration and/or cycle time. |
| 3. | Sending and enabling newly generated signal plans | During the time of each new cycle. The new signal plan must not disrupt the coordination of the green corridor during each cycle. Coordination may be disrupted no more than every 20 minutes. |
| 4. | Synchronization of cycle time (if applicable) of newly generated and enabled signal plans | Synchronization must not disrupt the coordination of the green corridor during each cycle. Coordination may be disrupted no more than every 20 minutes. |
| 5. | Dynamic adaptation of offset times of green corridors to traffic flow intensity | Mandatory |
| 6. | Duration of green corridor coordination performance | At least 20 min. The synchronisation of the signal plans must not disrupt the coordination of the green corridor. Coordination may not be disrupted more frequently than every 20 minutes. |
| 7. | Algorithm for optimizing the length of traffic queues (congestion) up to the intersection | In generating signal plans, the length of traffic queues to the intersection must be assessed based on data from the existing transport sensors (without installing additional remote sensors) or the output of an adaptive algorithm based on existing traffic sensor calculations |

- A network of coordinated intersections (corridors) must be created in the System based on the transport corridors created in the existing SiTraffic Scala. During the System installation, the Supplier shall select, together with the Customer, the optimal length of transport corridors and shall draw up lists of corridor intersections. The user interface must allow the user to adjust the parameters of the network of intersections.

- Traffic intensity forecasting tools, strategy and scenario development, planning and activation tools, which would allow at the right time, taking into account the changed traffic flows, automatically select for the System the correct operating mode for intersection controllers, must be installed in the System:

  - The System must also detect traffic disruptions based on sensors in the network and initiate the activation of scenario to resolve traffic disruptions. The impact of the scenarios on traffic shall be measurable in terms of average waiting time.

  - The Supplier must prepare all necessary strategies for the morning, evening peak periods, day and nighttime based on the data of the existing SiTraffic Scala system. During the installation of the System, the Customer shall provide the current configuration of strategies and scenarios.

  - The user interface allows the user to create, edit and delete new strategies and scenarios.

  - Triggers for activating strategies should include time of day, travel and waiting time data, speed and intensity of traffic flow.

- The System must have the controller data correction and sending module for new 210 units controllers, for engineers with the following functionality:

  - The System must allow the engineer to correct this basic controller data or enter new data:

  - Intersection number and name;

  - Intersection coordinates;

  - Intersection scheme;

  - Table of physical and logical signal groups;

  - Table of sensors and their parameters;

  - Matrix of conflicts and/or Intergreen times;

  - Stage sequence ;

  - Stage switching logic;

  - Stage transitions;

  - Signal plans;

  - Frame plans and/or stage minimum and maximum times;

  - Work schedule (annual automatic)..

  - The controller data correction and sending module installed in the System must be compatible with the controllers of the System manufacturer:

    - The System must allow the engineer to send controller data corrections (which do not affect traffic safety data/parameters) and run a new controller control program remotely at the intersection.

    - The System must allow the engineer to change remotely the assignments of physical and logical sensor channels.

    - The System must allow adjusting all other parameters related to the traffic adaptive control of the intersection and send the changes remotely.

  - The System must have an intersection planning module that enables the following actions:

  - Intersection topology creation.

  - Input of signal group parameters.

  - Input of detector parameters.

  - Creation of conflict matrices and entry of intergreen times. Planning, editing and creation of stages, stage sequences, stage transitions.

  - Automatic creation of signal plans according to the selected stage sequence.

  - Entry of frame plans or minimum and maximum times for stages.

  - Creation and adjustment of controller logic. Entry of logical conditions and constraints.

  - Creation of daily, yearly calendars.

  - Input and adjustment of parameters of the adaptive module for real-time optimization of operating modes.

Public transport priority module

- The public transport priority (hereinafter - PTP) module intended for prompt response of the System and new 210 pcs traffic light controllers with OCIT protocol version 3.0 and prioritization of public transport vehicles at intersections regulated by traffic lights must be installed in the System.

- The Supplier's installed System shall automatically call the Ridango AVL API and the Supplier shall ensure the integration of the data received from the Ridango AVL System into the PTP module of the System.

  - GPS data from AVL is transmitted using API, PUSH method, at set interval (1-30 s, currently is5 s). If necessary, the Customer will contact Ridango to change the data transmission interval to a minimum of 1 s.

  - Data from AVL can also be received via API, using GET method. Must be able to configure the receive frequency.

  - Data from AVL can also be received via API, using GET method. Must be able to configure the receive frequency.

- The PTP module of the System must support the following basic data of the Ridango AVL system: public vehicle identification number, position (coordinates), date, time, route number.

- The Customer shall provide the data structure for the Ridango AVL API:

Ridango response:

{

"id": 289,

"registrationNumber": "3038",

"internalNumber": "KBBxxx",

"lastUpdateTime": "2025-01-22T13:34:04.406Z",

"incidentOn": false,

"direction": 271.82,

"directionSnapped": 263.6326,

"assetType": null,

"drivingStatus": "IDLING",

"communicationStatus": "NORMAL",

"routeStatus": "ON_ROUTE",

"latitude": 54.680084666666666,

"longitude": 25.417521000000004,

"latitudeSnapped": 54.67998039525881,

"longitudeSnapped": 25.41754112651155,

"routeId": 15647,

"tripId": "1363354213255713978",

"distanceOnRoute": 14448,

"distanceOnRoutePercentage": 0.98619294,

"speed": 0.0,

"gpsSpeed": 0.0,

"routeName": "115",

"routeDestination": "Antakalnis–Rokantiškės–Parko—Linksmoji g.",

"routeVariant": "1",

"routeLenght": 14651.16375853587,

"driver": {

"id": 1031,

"code": "3269",

"fullName": "Last name First name",

"alternativeFullName": null

}

- The Customer is responsible for generating data in the Ridango AVL system every 1 second. The Supplier is responsible for retrieving the data from the Ridango AVL system, using the data and prioritising the PT. PTP module must process the data received from Ridango AVL system and send the priority assignment command to the controller no later than within 1 s.

- An interface for configuring PTP module must be created in the System. The interface must be user friendly. When using the interface, the functionality for configuring the performance criteria of PTP module must be implemented, such as:

  - Virtual grouping of public transport vehicles (Example 1: Express, Trolleybuses, Buses. Example 2: Buses1, Buses2, etc.).

  - System calibration/change of modes depending on the time of day. Entry of different PTP parameters for peak and off-peak times.

  - System calibration/change of modes depending on atypical or non-working days. Entry of different PTP parameters for holidays, weekends or during an event.

  - Conditional priority of a group of public transport vehicles must be implemented:

    - Placing groups to the priority queue (for example, if the "Express" group has a higher priority than "Buses", then at the intersection, the bus from the "Express" group gets priority in the event of a conflict with the public transport vehicle in the "Buses" group).

    - Setting the delay/rush time limits, when the priority would be activated, for a group of public transport vehicles (Example 1: if the time is set to 0, all latecomers would get priority and rushers would not. Example 2: if the time is set to 1 min, vehicles rushing for more than 1 minute would no longer receive priority; Example 3: if the time is set to 3 min, only those who are later would get priority).

    - If there are delayed public transport vehicles of the same group in conflicting directions, then priority is given to the more delayed vehicle.

    - If there are non-delayed public transport vehicles of the same group in conflicting directions, priority is given to the vehicle that arrived first.

    - The possibility to have a priority template for all intersections, and the possibility to configure each intersection separately.

- The Supplier shall configure the virtual detection (check-in and check-out) zones for PTP module on the new System at those intersections where the latest System manufacturer's controllers that are fully compatible with the System and support the PTP commands are installed.

- The principle of operation of the PTP module of the System is presented in Fig. 1.

Fig. 1. Principle of operation of the PTP module

- The PTP module of the System, the statistics or quality control module must provide reports with data of public transport movement and prioritization:

  - Public transport priority call data by intersections and routes.

  - Travel time of public transport vehicles.

  - Waiting time of public transport vehicles.

  - Average, minimum and maximum speed of public transport vehicles.

  - Other manually configurable report templates which allow selecting the analysis of the priority applied to a specific route and make it possible to study the efficiency of the priority.

Cyber security

- The Supplier must install a System that meets modern cyber security standards:

  - At least two-factor authentication (2FA) for administration and user interfaces to ensure that only authorized persons access the System must be implemented in the System.

  - Management of different levels of rights for different users must be ensured in the System.

  - A log, which would record all logins of different users to the System, must be easily accessible in the System.

  - The Customer must be granted the System administration right to configure the number of users, rights, levels, etc.

  - User actions must be reflected in the history of the System intersection management events: the user action having sent each System command to a controller or peripheral equipment must be recorded in the event history.

  - The System log files must be easily accessible in the System.

- The Supplier must ensure that the System solution does not have known cyber security gaps that are marked in the publicly available CVE database.

- The Supplier must transfer not only the passwords required for the use of the System, but also the passwords for the System administration.

- The Customer must be granted the right to administer the detailed information of user accounts and the level of granted rights.

- In providing the Services or the Additional Services, the Supplier must follow the following remote access procedure:

  - Ensure the security of the System data against data leakage, copying, transfer, transmission to third parties when performing remote logins through the access provided by the Customer.

  - The data contained in the System databases are considered confidential information of the Customer. It is prohibited to copy, transfer or transmit this information to third parties using portable media, remote logins or other methods of transfer, except when a separate instruction or permission of the Customer has been received.

The Supplier must inform the Customer in advance by sending an e-mail to  eismo@judu.lt and  eos.sviesoforai@judu.lt about possible logins to the System remotely by specifying the date, time and reason for logging in. The Customer can confirm or reject the date and time of the logins planned by the Supplier.

Licences, warranty and copyright

- The Customer shall, by acquiring the System, also acquire the System Licence(s), which shall be perpetual and shall be transferred to the ownership of the Customer. The Customer shall not require the Supplier to provide the source code(s) for the System.

- The Supplier must foresee and install at least 350 unique licences for intersection controllers during the implementation of the System. The System shall not be limited to a maximum of 350 unique intersection controller licences and may be increased to 500 licences as required.

- The Supplier must foresee an unlimited license (number of licences) for intersection sensors or measuring points. The System shall not limit the maximum number of traffic sensors or measuring points.

- The Supplier must create at least 10 unique Customer user accounts for the management and administration of the System.

- The Supplier must not restrict the integration of third-party software into the System through the use of APIs or other integration methods.

- The Supplier shall provide a warranty of not less than 5 years for the System and the Installation Services provided.

- The Supplier shall provide a warranty of not less than 2 years for the Additional Services provided.

- During the warranty period, the Supplier must remedy, at his own expense and resources, all defects and/or deficiencies in the System, Installation Services and in the Additional Services provided, except for defects and/or deficiencies:

  - caused by the fault of the Customer;

  - that resulted from force majeure circumstances, as defined in Article 6.212 of the Civil Code of the Republic of Lithuania and in the Rules for Exemption from Liability in the Event of Force Majeure, approved by Resolution No 840 of the Government of the Republic of Lithuania of 15 July 1996 on the Approval of the Rules for Exemption from Liability in the Event of Force Majeure.

- During the Warranty Period, the Supplier must, at his own expense and resources, remedy any deficiencies and/or defects in the System, Installation Services or in the Additional Services registered by the Customer on the Odoo Platform within the terms set out in Table 4.

- The Supplier, having provided Additional Services of System improvement/development whereby the functionality of existing System Modules has been enhanced, additional System Integrated Functionality has been developed, or System Integrated Modules with new functionality have been developed, shall not be obliged to unconditionally transfer to the Customer the entire intellectual property in the subject-matter of the enhancements developed.

- The Supplier, having provided Additional Services of System improvement/development during which modules, applications or other objects of the programming result separate from the System have been developed, the Supplier shall unconditionally transfer to the Customer all intellectual property rights in the object developed during the improvement (i.e. without any limitation all the author's property rights provided for in the Law on Copyright and Related Rights of the Republic of Lithuania), including the right to change, modify and rework the object of copyright.

- INSTALLATION SERVICES

System installation

- Before starting the System installation work, the Supplier must provide and agree with Customer the detailed System installation plan and schedule within 20 working days from the date of entry into force of the Contract.

- Before starting the implementation of the System, the Supplier shall provide an architectural scheme of the System with a detailed description of the requirements of the System within 20 working days from the date of entry into force of the Agreement. The Customer shall prepare the server infrastructure resources (virtual machines, number of processor cores, RAM, storage) required for the installation and operation of the System, with the operating systems installed in the virtual machines, based on the Supplier's architectural scheme and requirements.

  - The Supplier shall specify the size of the storage required for the System (including at least 2 years old of Customer's traffic flow data that needs to be migrated from existing Sitraffic Scala software) when submitting the proposal and when assessing the cost-effectiveness criteria. Based on the storage size indicated by the Supplier for the System, the Customer shall include the additional costs (to the Supplier's quotation) it will incur to expand the server infrastructure. The capacity of Customer’s 2 years old traffic flow data in Sitraffic Scala software is 2 TB.

  - If the Supplier specifies an incorrect storage size for the System, or the storage size required for the installation of the System and the storage of the Customer's data is 6 TB or more, the Supplier shall be obliged to procure, deliver, install and deliver to the Customer, at the Supplier's own responsibility, at the Supplier's own cost and resources, additional storage(s) in accordance with the requirements to be provided by the Customer at the time of the System installation.

  - If the Supplier does not specify the size of the storage required for the System in its proposal, the Customer will apply the maximum size of the storage required for the System in accordance with the cost-effectiveness calculation methodology and will include the maximum cost (in addition to the Supplier's quotation price) that it will incur in expanding the server infrastructure.

- The Supplier must install, to the server infrastructure prepared by the Customer, the System software. The Supplier must install the database development and management software necessary to ensure the operation of the System and the storage of data.

- The System must be accessible through a browser window as a web application. Separate client-specific software must not be installed on the Customer‘s computer (on the Customer‘s workstation computer). It is possible to install a separate application dedicated only to the correction and sending of the data of the intersection controllers.

- The Customer shall be responsible for the proper functioning of the operating systems, the timely automatic backup and, if necessary, recovery of operating systems and virtual machines. The System Supplier shall agree with the Customer the procedure for backing up the operating systems and virtual machines after the installation of the System.

- The Customer shall be responsible for the installation of the anti-virus software on the System ready virtual machines and for the automatic update of the anti-virus software.

- During the installation of the System, the Supplier, in cooperation with the Customer, must configure the network parameters of the System and adapt the System to the existing network configuration of the controllers. Only the IP address of the central server and the IP address of the time server may be changed on the controllers if necessary, but the basic network configuration (IP address, host name and mask of the controllers) will not be changed.

- The installation of the new System must not disrupt the operation of the existing SiTraffic Scala system. The Supplier must assess the potential risks in carrying out the network configuration of the new System and must prevent possible duplication of IP addresses and disruption of controller communication.

- The Supplier shall enter the information necessary for the OCIT configuration of the existing controllers (host name and IP address of the controllers) into the installed System. The Customer must provide basic information on the controllers in Annex 1 to the Technical Specification. The Customer shall provide the unique names and IP addresses of the intersection controllers only from the date of entry into force of the Contract, during the System installation.

- During the System installation, the Customer shall provide a time server network configuration to be used by the Customer in configuring the new System.

- The System must allow intersection controllers to receive time synchronisation data. The System shall in turn synchronise the time with the time of the Customer‘s time server.

Entry of data of intersection controllers

- After installing the System, the Supplier must enter the data of 290 intersection controllers. The Supplier must enter the basic data of the existing controllers into the new System:

  - Intersection number and name;

  - Intersection coordinates;

  - Intersection scheme;

  - Table of signal groups;

  - Table of sensors

  - Signal plans;

  - Work schedule (annual automatic).

- The Supplier must harmonize with the Customer the detailed plan for switching the intersection controllers. The Customer, in cooperation with the Supplier, will perform the work of disconnecting the intersection controllers from the existing SiTraffic Scala traffic light regulation software. After disconnecting the intersection controllers from the existing SiTraffic Scala software, the Supplier must connect the controllers to the new System on the same day. The Customer shall provide a preliminary plan for switching intersections in Annex 2 to the Technical Specification.

- The System installed by the Supplier must support the correct version of the OCIT protocol, which must interoperate with OCIT protocol versions 1.1 and 2.0 currently used in the controllers. Existing Siemens, Yunex, Swarco, Dynniq and Cross controllers in Vilnius support OCIT protocol versions 1.1 and 2.0.

- The Supplier shall perform testing of the basic OCIT functionality of the existing controllers (Siemens, Yunex, Swarco, Dynniq and Cross) (switching of signalling programs, display of the interactive diagram, display of detector data, display of errors and warnings and storing of error messages) according to the test protocol provided by the Customer (Annex 3 to the Technical Specification).

- The Supplier must create intersection control groups in the System, that are analogous to the intersection control groups currently available in the existing SiTraffic Scala system. The groups of intersections are presented in Annex 4 to the Technical Specification.

- The Supplier must create the currently valid schedules of the operating modes of the intersection controllers for each day of the week for the individual intersection controllers or groups of intersection controllers. The Customer shall provide the current schedule of the operating modes that is valid in the existing SiTraffic Scala system in Annex 5 to the Technical Specification.

- The Supplier must prepare in the System special schedules of operating modes for holidays, and create special purpose response plans, which are intended to control atypical situations in the city. Information on special response plans is provided by the Customer in Annex 6 to the Technical Specification.

- A database for storing intersection controller data, for their correction, for entering data of new intersection controllers must be created in the System.

Data integration

- The existing SiTraffic Scala traffic light control equipment transmits real-time traffic, cyclist and pedestrian traffic data (hourly averages of intensity, speed, occupancy) from each sensor every 90 s to the Customer's ArcGis and DWH databases using the SOAP protocol (service) through the OCPI interface. The Supplier must adapt the System configuration to the already created and operating SOAP service in the Customer's infrastructure. If this is not possible, the Supplier, in cooperation with the Customer, must create a new data transfer service/interface for data transfer from the new System to the Customer's ArcGis and DWH databases.

- The Customer shall undertake to provide a list of the unique names of the sensors whose data is transferred from SiTraffic Scala to the ArcGis and DWH databases during the implementation of the System. It is important that the names of the sensors are used in the same format when entering controller data into the System. Currently there are about 2900 unique sensor names in the existing SiTraffic Scala.

- The Customer shall provide maximum detail of the current data interface between SiTraffic Scala and ArcGis and DWH in Figure 2 and Figure 3.

Fig. 2. Principle chart of data transfer from SiTraffic Scala to Customer's ArcGis and DWH

Fig. 3. SOAP service syntax

- A basic API functionality (SOAP-based) for data transfer, supporting the following data transfer methods, must be implemented in the System: PUT, PUSH, GET, DELETE. In addition to the basic API functionality for the transfer of data to third party systems, additional API functionalities shall be implemented to allow the integration of additional third-party tools such as city planning systems, smart city platforms, traffic control systems.

- The System must be easily adaptable to new technologies, in particular the integration of Internet of Things (IoT) devices. This will allow for the future connection of new traffic and flow monitoring devices such as advanced sensors or cameras. The System to be deployed must also support V2X technology to monitor, integrate and control V2X devices.

- The intersection planning module of the System must support the export of controller data to files in a format supported by VISSIM software or The System shall be adapted for VISSIM simulation, e.g. by interconnecting the controller traffic logic with VISSIM simulation software. The export of intersection control logic and other adaptive parameters shall be implemented.

Training

- After the installation of the System, the Supplier shall provide at least 80 hours of training for traffic management specialists and engineers. The training course may be divided into phases.

- The training shall be provided in the official national language of Lithuania. The training may be conducted in English but must be translated into Lithuanian.

- Part of the training, up to a maximum of 50%, may be delivered remotely. The Supplier must agree the training plan with the Customer.

- Training for traffic management specialists must cover the following topics:

  - Map navigation.

  - Checking the status of intersection controllers.

  - Event history viewing.

  - Interactive diagram.

  - Evaluation of sensor data.

  - Management of intersection controls, switching of signalling plans.

  - Development and implementation of special reaction/response plans.

  - Development and management of System scenarios and strategies.

  - Working with the statistics module, generating reports, creating new templates for reports.

  - Monitoring of PTP module, generation of reports and creation of report templates.

- Training for engineers must cover the following topics:

  - System administration: changing the order (priority) of displaying the status of intersection controllers, changing the colour gamma of status of intersection’s controllers.

  - Controller data input, import, export. Integrating new controllers into the System

  - Correction of controller data, correction of new signal plans, signal groups, sensors, stages, frame plans, control logic and sending to the controller.

  - Working with the intersection planning module: intersection topology creation, intergreen time calculation, automatic or semi-automatic creation of stages, stage sequences, stage transitions, signal plans.

  - Work with the adaptive module for real-time optimization of operating modes: configuration of signal plan generation parameters, configuration of green corridor coordination parameters, configuration of green signal distribution parameters, etc.

  - Configuration of the PTP module.

- During the training, the Supplier must prepare practical exercises for both traffic management specialists, who perform intersection monitoring and control functions, and engineers, who perform intersection data correction and System configuration functions.

- After having provided the training, the Supplier must provide training materials and instructions for the use and administration of the System in e-format:

  - The Supplier must provide the training materials in Lithuanian and English.

  - The Supplier must provide instructions for the use of the System for traffic management specialists in Lithuanian and English.

  - The Supplier must provide instructions for the administration and configuration of the System for engineers in Lithuanian and English.

- MAINTENANCE SERVICES

- The Customer shall determine the boundaries of the responsibilities for maintenance of the System:

  - The Supplier shall be responsible for the operation, maintenance, upgrading of the System software and for the operation, maintenance, upgrading of any third-party software installed by the Supplier that is necessary for the operation of the System.

  - The Customer shall be responsible for the operation of the server infrastructure (physical/technical server hardware), the creation, maintenance, upgrading of the virtual machines, backups, the operation and upgrading of the operating systems, the configuration of the networks and the network provision between the servers, the virtual machines and the intersection controllers, the operation of the time server, and the upgrading of the anti-virus software.

- The Supplier must ensure that the System availability is at least 99%. The System availability shall be measured using the NINES method, which indicates the percentage of time the System was available and operational (Table 3).

Table 3. System availability indicators

| Operability in percentage | Non-operability in percentage | Non-operability per year | Non-operability per week |
| --- | --- | --- | --- |
| 99 % | 1 % | 3,65 days | 1 hour 41 min. |

- The Supplier must carry out a periodic remote inspection of the System (installed on the Customer's server infrastructure) 2 times a month (in the first and third weeks of the month), consisting of:

  - Verification of the functioning of all modules of the System (user interface and basic functionality of the System, statistics module, quality management module, adaptive module for real-time optimization of operating modes, controller data correction and sending module, intersection planning module, PTP module and other modules not listed here but necessary for the functioning of the System);

  - Review of the System log files, search and analysis of errors and warnings. If an error is found in the System, the Supplier shall immediately initiate correction of the error.

  - The Supplier must verify that the resources allocated to the System on the Customer's server infrastructure are sufficient for the normal operation of the System. The Supplier shall notify the Customer in advance if it sees a need to expand the Customer's server infrastructure.

- The Supplier shall be obliged to provide System updates, critical bug fix packages or completely new versions of the System during the maintenance period. The period for the installation of new versions of the System shall be agreed with the Customer. Critical System error correction packages shall be deployed no later than 24 hours after the occurrence of the error correction package. The deployment of new versions of the System or bug fix packages shall be available on working days from 10 AM to 2 PM or from 8 PM to 12 AM. After upgrading the System, the Supplier shall check that all modules of the System are operational. System updates, critical bug fix packages can be installed automatically on working days from 10 AM to 2 PM, from 8 PM to 6 AM or weekends - 24 hours a day. At other times, automatic updates of the System and installation of critical bug fix packages are prohibited.

- The Supplier must receive and respond to notification (requests) registered by the Customer about System malfunctions on the Customer's service desk Odoo platform.

- The Supplier must respond to the Customer's registered request of the System faults during working hours on working days from 08:00 AM to 5:00 PM Lithuanian time in accordance with the official Lithuanian working day calendar. The Supplier must respond to a received request within 1 working hour.

- Response time - the time to provide a plan for further action to resolve the identified System fault.

- The Customer, by logging System fault requests on Odoo Platform, shall assign a level of fault to each request. The fault resolution time shall be attributed depending on the selected level of fault. The resolution time shall include the response time. The levels of fault, their description and the resolution time are given in Table 4.

Table 4. Levels of System faults and resolution times

| Level of System fault | Fault description | Response time | Resolution time |
| --- | --- | --- | --- |
| Critical | - Cyber security incident. - System user interface is down. - Map display of intersection statuses is stuck, statuses do not change - Intersection management functionality is broken, it is not possible to switch the signal plan or disable intersection traffic adaptive mode quickly. - Multi-level control of intersection controllers is stuck, it is not possible to deactivate/activate commands on certain levels. - Intersection group management functionality is broken. - Adaptive module for real-time optimization of operating modesmodule is broken. The System does not generate signal plans adapted to the traffic volume. Intersections switched to inflexible or local control mode of operation. - Other fault that affects the performance of the direct function of the traffic managemenet specialist. | 1 working hour | No more than 8 working hours |
| Average | - Map navigation has failed. - Detail information window not working - Interactive diagramdisplay has failed. - Intersection controller event history viewer function failed. - The controller data correction and sending module malfunctioned. - The PTP module malfunctioned. - Fault of the forecasting, strategy or scenario development and management tools. 16. Other fault that affects the engineer's direct function. | 1 working hour | No more than 16 working hours |
| Low | - The map does not show the LOS or other object layers. - The intersection controllers' state ordering and state colour gamma configuration functions are not working. - The statistics module or some of its functions have malfunctioned. - Intersection planning module malfunction. - Other faults which do not directly affect the operational control of intersections. | 1 working hour | No more than 40 working hours |

- The Supplier shall, upon receiving a request via the Odoo Platform about a System fault, having analysed the cause of the fault and having determined that the cause of the fault is within the scope of the Customer's responsibility (in accordance with Point 82), immediately inform the Customer thereof.

- In the event of fault of the Customer‘s server infrastructure, the Customer undertakes to immediately restore the hardware and virtual machines using previously created backups. In case of need and disruption of the System, the Supplier shall restore the System after the Customer has restored the hardware (the restoration of the System shall be included in the cost of maintaining the System). In this case, the timeframe for restoring the System shall be determined by mutual agreement between the Supplier and the Customer on the basis of the principle of reasonableness.

- ADDITIONAL SERVICES

- The Supplier must provide the Customer with the possibility to order the Additional Services consisting of:

  - The System enhancement services (maximum 300 hours during the term of the Contract) for the development or improvement of new or existing System functionality or the System modules.

  - Integration (data entry) of new intersection/crossing controllers into the System, testing of connectivity and basic OCIT functionality (maximum 50 traffic light controllers to be integrated) in accordance with the testing protocol (Annex 3 to the Technical Specification).

  - Additional training services (maximum 100 hours during the term of the Contract).

- The provision of the Additional Services is subject to the separate order of the Customer:

  - The Supplier shall, upon receipt of an order from the Customer for the provision of System improvement/development services, assess the possibility of implementation of the functionality requested by the Customer and submit a commercial proposal within no more than 5 working days. The standard timeframe for performance of the System improvement/development services shall be 40 working days. The timeframe for performance of the System improvement/development services may be longer depending on the complexity of the requested services but must be agreed upon with the Customer.

  - The Supplier shall, upon receipt of an order and the necessary data from the Customer for the integration (data input) of new intersection/ crossing controllers into the System, provide integration services in the following order:

    - If the Customer orders the integration of 1-5 controllers into the System in one order, the Supplier shall provide the integration services within 5 working days from the date of the order.

    - If the Customer orders the integration of 6-10 controllers into the System in one order, the Supplier shall provide the integration services within 10 working days from the date of the order.

    - If the Customer orders the integration of 11-15 controllers into the System in one order, the Supplier shall provide the integration services within 15 working days from the date of the order.

    - If the Customer orders the integration of 16-20 controllers into the System in one order, the Supplier shall provide the integration services within 20 working days from the date of the order.

    - If the Customer orders the integration of 21-25 controllers into the System in one order, the Supplier shall provide the integration services within 25 working days from the date of the order.

    - If the Customer orders the integration of 26-30 controllers into the System in one order, the Supplier shall provide the integration services within 30 working days from the date of the order.

    - If the Customer orders the integration of 31-35 controllers into the System in one order, the Supplier shall provide the integration services within 35 working days from the date of the order.

    - If the Customer orders the integration of 36-40 controllers into the System in one order, the Supplier shall provide the integration services within 40 working days from the date of the order.

    - If the Customer orders the integration of 41-45 controllers into the System in one order, the Supplier shall provide the integration services within 45 working days from the date of the order.

    - If the Customer orders the integration of 46-50 controllers into the System in one order, the Supplier shall provide the integration services within 50 working days from the date of the order.

- Payment for Additional Services is based on actual services provided. Additional Services shall be included in the current month's Statement of Services on a separate line. The Supplier shall provide a completed test report in accordance with the Customer's template (Annex 3 to the Technical Specification) after the completion of the controller integration services.

- The Supplier must ensure that the Customer is able to ask questions and receive answers free of charge by electronic means after the training provided, in relation to the operation, recovery or administration and configuration of the System functions and modules.

- SUPPLY OF CONTROLLERS

- The Controllers shall be supplied on a purchase order basis from the entry into force of the Contract until the end of the Maintenance Services. The Customer intends to purchase a minimum of 50 units and a maximum of 210 units of controllers. The Customer is not obliged to purchase a maximum of 210 units of controllers, which shall be purchased on an as needed basis.

- The Customer shall order the supply of controllers in packages during the performance of the Contract. Annex 7 to the Technical Specification sets out the controller line-up and indicative order packages with a maximum quantity of 210 units of controllers.

  - The Customer plan to place orders for up to four controller packages per Contract year. Indicative supply plan for controller packages:

    - First year of the Contract from the entry into force of the Contract - 1-4 controller packages.

    - Second Contract year after the entry into force of the Contract - 5-8 controller packages.

    - Third Contract year after the entry into force of the Contract – 9-11 controller packages.

    - Fourth Contract year after the entry into force of the Contract – 12-13 controller packages.

    - Fifth Contract year after the entry into force of the Contract – 14 controller packages.

  - The schedule for the supply of controller packages is indicative, as it depends on the progress of the design and repair/reconstruction of the intersections (in accordance with the priorities of the Vilnius City Municipal Administration for the maintenance of the city's streets) and on the actual need for the installation of new controllers and is therefore adjustable during the performance of the Contract.

    - The Customer informs that during the progress of the contract, adjustments to the indicative plan for the supply of controller packages may result in the need to order a larger number of controller packages during the first or subsequent years of the contract (e.g. 5-10 controller packages will be ordered in the second year of the contract).

    - The Customer informs that due to the change of priorities of the Vilnius City Municipality Administration for the maintenance of the city's streets, it may be necessary to adjust the ordering of controller packages during the execution of the Contract (e.g. in the first year of the Contract 5-8 controller packages will be ordered).

  - The Customer may, at the time of ordering Controller packages, adjust the Controller package (without changing the maximum quantity of Controllers and their internal components).

  - The supply of controller packages will be ordered in separate orders. The time limit for the completion of an order for one package of controllers (delivery of complete controllers to the Customer) shall be 40 working days from the date of placing the order.  Depending on the Supplier's production capacity for controllers, the deadline for the delivery of a single package of controllers may be extended up to 60 working days from the date of placing the order.

- The Supplier must offer to the Customer the System-compatible controllers to be integrated into the System via the OCIT protocol and to support the receipt and execution of commands from the System's adaptive module for real-time optimization of operating modes and the PTP module.

- The Supplier shall include the cost of developing the programmes for the controllers in the price of the controllers. The Customer shall provide all information necessary for the development of the controller programme at the time of ordering the controllers. For orders for a larger number of controller packages (more than four packages per Contract year), the deadline for the development of the programmes may be extended up to 60 working days from the date of the order.

- The Supplier shall not include the cost of the installation work in the price of the controllers. The installation of the controllers will be carried out during the intersection repair/reconstruction works according to the intersection repair/reconstruction designs/requirements (separate contract).

- The Customer informs that it will not carry out the installation of the controllers itself but will outsource the installation of the controllers to a contractor for the repair/reconstruction of the intersections, who will carry out the repair/reconstruction works under a separate contract. The contractor carrying out the installation of the controller will be responsible for ensuring that the appropriate specialists are in place to carry out the work, that they have received the necessary training and/or that they have obtained certificates from the controller Supplier or directly from the controller manufacturer proving that they are competent to carry out the installation of the controllers.

- The controllers proposed by the supplier shall fulfil all the technical characteristics set out in Table 5 of the controller technical specification. The Supplier shall submit with the tender, the declarations of conformity (confirming compliance with the requirement set out in Table 5, point 4). In the Tender Form (Annex 2 to the Terms and Conditions of the Procurement), the Supplier shall declare with the proposal the values of the parameters of the proposed controllers and shall provide the manufacturer's technical documentation and/or manufacturer's references and/or other equivalent evidence. The Supplier shall provide the Customer a description of the technical specifications of the controllers (technical data) with the operating and usage instructions, the firmware and the instructions for its loading when delivering the controllers.

Table 5. Traffic light controller with a panel of devices

| Ser. No | Characteristics |  | Technical data |
| --- | --- | --- | --- |
| 1. | Cabinet |  | The controller shall be mounted in a polyester or metal (corrosion protected) reinforced box on the foundation. The enclosure shall be IP54 rated. Lockable mortise lock. The body colour RAL 9004 MATT. |
| 2. | Supply voltage |  | 230 V AC – 10/+10%; 50 Hz |
| 3. | Output voltage |  | 230 V AC – 10/+10%; 50 Hz |
| 4. | Standard |  | LST EN 12675 or equivalent |
| 5. | Current for each output (for traffic light connection) |  | At least 1 A |
| 6. | Output circuits |  | Each output is protected by an individual fuse. |
| 7. | Number of controlled signal groups |  | Must support up to 32 signal groups or more. |
| 8. | Additional power socket in the cabinet |  | Mandatory. Shall be protected by a separate pick. |
| 9. | Liquid crystal display (LCD) |  | Mandatory |
| 10. | Clock |  | Mandatory, electronic quartz integrated in the control board of the controller, time displayed on the LCD of the controller and an additional GPS module for clock synchronisation. |
| 11. | Operating temperature |  | -400C - +600C |
| 12. | Communications, interfaces, protocols |  | RS485, TCP/IP, USB, OCIT-O 1.1, OCIT-O 2.0, OCIT-O 3.0 |
| 13. | Number of external inputs/outputs |  | Must support up to 16 inputs/outputs or more. |
| 14. | Sensor types available for use |  | Inductive loops, video, infrared, radar and wireless magnetic sensors |
| 15. | Signalling safety (control of LED optical elements) | The traffic light controller shall control all colours of LED optical elements - in the event of a burn-out of any LED optical element of any colour, the controller shall record and send a message to the System about the faulty LED optical element in a particular signal group: Control of red LED optical elements for transport: - Where there are TWO red LED optical elements in a signal group: in the event of fault of ONE red LED optical element, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to dark or yellow flashing mode. - Where there are THREE red LED optical elements in a signal group: in the event of fault of ONE red LED optical element INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault and only send a fault message to the System. In the event of fault of TWO red LED optical elements INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to dark or yellow flashing mode. In the event of fault of at least ONE red LED optical element NOT INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. - Where there are FOUR red LED optical elements in a signal group: in the event of fault of ONE or TWO red LED optical elements INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault and only send a fault message to the System. In the event of fault of the optical elements of THREE red LEDs INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System, and must deactivate the intersection to dark or yellow flashing mode. In the event of fault of at least ONE red LED optical element NOT INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. - Where there are FIVE red LED optical elements in one signal group: in the event of fault of ONE or TWO red LED optical elements INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault and only send a fault message to the System. In the event of fault of the optical elements of THREE or FOUR red LEDs INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System, and must deactivate the intersection to dark or yellow flashing mode. In the event of fault of at least ONE red LED optical element NOT INSTALLED ABOVE THE CARRIAGEWAY, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. Control of red LED optical elements for pedestrians: - In the event of fault of at least ONE red LED optical element, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. Control of green LED optical elements: - In the event of fault of at least ONE green LED optical element, the controller must detect the fault and only send a fault message to the System. - In the event of fault of the optical elements of ALL green LEDs in the signal group, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode. Control of green LED optical elements for pedestrians: - In the event of fault of at least ONE green LED optical element, the controller must detect the fault, send a fault message to the System and must deactivate the intersection to the dark or yellow flashing mode Control of yellow LED optical elements: - In the event of fault of at least ONE or ALL yellow LED optical elements, the controller must detect the fault and only send a fault message to the System. |  |
| 16. | Memory |  | The controller stores all events and detector readings in its memory according to the manufacturer's maximum time limits. The memory of the controller shall be expanded to the maximum possible size depending on the controller model. The minimum time limit is 24 hours. |
| 17. | Uninterruptible power supply system |  | In the event of a power failure, the controller shall send an alarm message to the System using an internal backup power supply (for example, battery). Minimum and maximum voltage monitoring with memory storage. |
| 18. | Connection to the system and other controllers |  | Compatible with the System according to OCIT protocol requirements, compatible with the System's adaptive module for real-time optimization of operating modes module and the PTP module. |
| 19. | Door opening alarm |  | After opening any door of the controller, the opening of the door must be recorded in the controller, and a message shall be sent to the System indicating the opening of the controller door. |
| 20. | Warranty period |  | At least 5 years from the date of delivery to the Customer |

- The controller proposed by the Supplier must be compatible with the System:

  - The controller must respond to all control commands sent from the System via OCIT protocol and must be compatible with the basic functionality of the System.

  - The controller must fulfil the functionality requirements of adaptive module for real-time optimization of operating modes at the System or controller level:

    - Either a controller or a group of controllers must respond and interact with the System to fulfil the functionality requirements of Table 1.

    - Either a controller or a group of controllers must independently/locally (without necessarily receiving commands from the System) fulfil the functionality requirements of Table 2.

  - The Controller must be compatible with the System controller data correction and the sending module and must accept all commands related to sending new corrected data to the controller in accordance with Point 24.

  - The controller must receive and execute commands received from the PTP module of the System.

- The Supplier shall declare in a formal written confirmation from the manufacturer of the controllers that the manufacture and sale of spare parts and the repair of internal components for the proposed controllers will be available for a period of 10 years from the date of delivery of the controller to the Customer.

- The warranty period of the controllers shall commence on the date of delivery of the controllers to the Customer. The procedure for resolving warranty failures of controllers or their internal components is as follows:

  - In the event of a failure of a controller component that has a direct impact on traffic safety (signal monitoring is not working, traffic light signals are not working, traffic light signals are jammed, the controller does not switch on, traffic light signals of conflicting directions are switched on at the same time), the Customer or the Customer's contractor performing maintenance of traffic light equipment in Vilnius City shall deliver to the Supplier or its representative within the city limits of Vilnius City, a defective controller component. The Supplier, in case the failure of a controller component is determined to be a warranty failure, shall as soon as possible, but not later than 15 working days from the date of receipt of the defective component, provide the Customer with a new or analogous, but not worse quality, working controller component.

  - In the event of a failure of a controller component that does not directly affect traffic safety (I/O board not working, UPS system not working, communication board not working, intersection operating in fixed mode, other software or physical components not working), the Customer or the Customer's contractor performing maintenance of traffic light equipment in Vilnius City shall deliver to the Supplier or its representative within the city limits of Vilnius City, a defective controller component. The Supplier, in case the failure of a controller component is determined to be a warranty failure, shall as soon as possible, but not later than 30 working days from the date of receipt of the defective component, provide the Customer with a new or analogous, but not worse quality, working controller component.

- FINAL PROVISIONS

- The Supplier must ensure that all data transmission and storage infrastructure of the System complies with the highest standards of data security, including GDPR requirements.

- The party responsible for the development of the System shall provide evidence that third party access to information processing facilities is strictly controlled and complies with EN ISO 9001 (or equivalent) and ISO/IEC 27001 (or equivalent). The Supplier shall submit with the tender the EN ISO 9001 (or equivalent) and ISO/IEC 27001 (or equivalent) certificates or other equivalent evidence.

- The Supplier must provide a report on the Maintenance Services provided (together with the Maintenance Services Provided Act, once a month) with a detailed description of: when the periodic check was carried out, how many and what kind of faults were recorded, the progress of the fault rectification, and how many and what kind of Additional Services were provided.

- This procurement is considered a green procurement as it is subject to the application of the Description of the Procedure for the Application of Environmental Criteria in Green Procurement approved by Order No D1-508 of the Minister of the Environment of the Republic of Lithuania of 28 June 2011 (wording of Order No D1-401 of the Minister of the Environment of the Republic of Lithuania of 13 December 2022, (hereinafter - Description):

  - The System, its Installation, Maintenance and Additional services are subject to sub-paragraph 4.4.3 of the Description: only software shall be procured and only services of an intangible nature shall be provided, i.e. intellectual services not related to the creation of a tangible object, the provision of which is not expected to have a significant negative impact on the environment, does not create a source of pollution or generate waste.

  - For controllers, point 4.4.4.4 of the Description applies: the goods (controllers) must be guaranteed for at least 5 years, with the possibility for suppliers to offer a longer guarantee period.

  - Minimum environmental criteria (Annex 2 to the Description), p. 2: Packaging: The packaging of all controllers covered by this Technical Specification (if provided in secondary packaging) shall be considered as recyclable packaging within the meaning of the Law of the Republic of Lithuania on the Environmental Pollution Tax, and/or shall be homogeneous packaging made of a single material type:

Table 6. Packaging and labelling

| Ser. No | Packaging material | Labelling |
| --- | --- | --- |
| 1. | Glass | GL (arba GL nuo 70 iki 79) |
| 2. | Metal | FE (arba FE 40), ALU (arba ALU 41) Nuo 42 iki 49 |
| 3. | Paper or cardboard | PAP (arba PAP nuo 20 iki 39) |
| 4. | Wood or cork | FOR (arba FOR nuo 50 iki 59) |
| 5. | Cotton or jute | TEX (arba TEX nuo 60 iki 69) |
| 6. | Polyethylene terephthalate | PET arba PET 1 |
| 7. | High-density polyethylene | HDPE (arba HDPE 2) |
| 8. | Polyvinyl chloride | PVC (arba PVC 3) |
| 9. | Low-density polyethylene | LDPE (arba LDPE 4) |
| 10. | Polypropylene | PP (arba PP 5) |
| 11. | Polystyrene | PS (arba PS 6) |

    - Evidence of compliance shall be provided at the time of performance of the contract: documentation from the supplier or manufacturer proving that the packaging is homogeneous and/or appropriately labelled, or compliance with standards demonstrating that packaging materials are recyclable e.g. LST EN 13432 "Packaging. Requirements for used packaging intended for composting and biodegradation", Voluntary Standard for Repulping and Recycling Corrugated Fiberboard Treated to Improve Its Performance in the Presence of Water and Water Vapour, RecyClass or other equivalent standard, or the website of the Environmental Protection Agency (https://aaa. lrv.lt/) published on the list of waste handlers entitled to issue documents proving the management of waste products and/or packaging waste, or documents from recyclers or exporters justifying that such packaging which has become waste can be recycled.

Annex 1 to the Technical Specification

Initial information about intersection controllers

| Ser. No. | Intersection No . | Name of intersection | Latitude coordinate, WGS-84 | Longitude coordinate, WGS-84 | Host name | Controller manufacturer | Controller model | IP address | Mask | Gateway |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | K1001 | Rinktinės–Kazliškių g. | 54,6986 | 25,28687 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 2 | K1002 | Žirmūnų g. perėja prie Tuskulėnų rimties p. | 54,69812 | 25,30102 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 3 | K1003 | Olandų–Filaretų g. | 54,68496 | 25,30899 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 4 | K1004 | Subačiaus g. pėsčiųjų perėja | 54,67713 | 25,30297 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 5 | K1005 | Drujos–Manufaktūrų g. | 54,67861 | 25,3111 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 6 | K1006 | Liepkalnio pėsčiųjų perėja | 54,6681 | 25,29669 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 7 | K1007 | Rasų–Sukilėlių–Ribiškių didžioji g. | 54,66666 | 25,3053 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 8 | K1008 | Dunojaus–Liepkalnio–Dzūkų g. | 54,6624 | 25,30104 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 9 | K1009 | Liepkalnio g.–Žirnių g.–Minsko pl. | 54,65427 | 25,30285 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 10 | K101 | Žygimantų–A. Goštauto–Vilniaus g. | 54,69051 | 25,27984 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 11 | K1010 | Šilo–V. Grybo g. | 54,70424 | 25,31598 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 12 | K1011 | Žirmūnų g. 67 (prie PC „LIDL“) | 54,71629 | 25,30527 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 13 | K1013 | Šiaurės–A. Kojelavičiaus | 54,69506 | 25,39683 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 14 | K1014 | Kalvarijų–Didlaukio g. | 54,72756 | 25,28006 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 15 | K1015 | Didlaukio g. ties Baltupio m-kla | 54,72906 | 25,27311 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 16 | K1016 | Geležinio Vilko–Didlaukio g. | 54,72908 | 25,26781 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 17 | K1017 | Gelvonų g. perėja (ties PC „Maxima“) | 54,71808 | 25,25764 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 18 | K1018 | Gelvonų–Paberžės g. | 54,71622 | 25,25776 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 19 | K1019 | Šeškinės–Paberžės g. | 54,71573 | 25,25127 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 20 | K102 | A. Goštauto–A. Vienuolio A. Goštauto–A.Jakšto g. | 54,69088 | 25,27804 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 21 | K1020 | S. Stanevičiaus g. pėsčiųjų perėja (ties „Jovaru“) | 54,72348 | 25,25393 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 22 | K1021 | Buivydiškių g. perėja | 54,71359 | 25,23769 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 23 | K1023 | Pašilaičių g. pėsčiųjų perėja | 54,73166 | 25,23112 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 24 | K1024 | Ukmergės g. perėja (ties Gedvydžių g.) | 54,73437 | 25,2342 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 25 | K1026 | L. Asanavičiūtės g. perėja (ties „Kometa“) | 54,69322 | 25,21482 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 26 | K1027 | L. Asanavičiūtės–V. Maciulevičiaus g. | 54,68811 | 25,20782 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 27 | K1028 | Architektų g. pėsčiųjų perėja | 54,67804 | 25,20221 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 28 | K1029 | Architektų–Erfurto g. | 54,67401 | 25,20146 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 29 | K103 | A. Goštauto g.–Vasario 16-osios g. | 54,6916 | 25,27338 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 30 | K1030 | Metalo–Geologų g. | 54,63758 | 25,26691 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 31 | K1032 | Galvės–Kirtimų g. | 54,63271 | 25,15163 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 32 | K1033 | Lazdynėlių–Šltnamių g. | 54,67007 | 25,19812 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 33 | K1035 | Santariškių pėsčiųjų perėja | 54,75497 | 25,28054 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 34 | K1036 | Metalo g.–Eišiškių pl. | 54,64613 | 25,26541 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 35 | K1038 | Molėtų pl.–Skersinės g. | 54,76484 | 25,27174 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 36 | K1039 | Nemenčinės pl.–Svajonių g. | 54,73435 | 25,33764 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 37 | K104 | A. Goštauto g.–J. Tumo-Vaižganto g. | 54,69226 | 25,27172 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 38 | K1040 | Ukmergės–S. Nėries g. | 54,73752 | 25,23224 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 39 | K1041 | Žemaitės g. (ties PC „LIDL“) | 54,67199 | 25,25653 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 40 | K1042 | Saulėtekio al.–Šatrijos Raganos g. | 54,7239 | 25,33838 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 41 | K1043 | Eišiškių pl. 13 (ties VĮ Oro navigacija) | 54,64961 | 25,26834 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 42 | K1044 | Justiniškių–Virbeliškių g. | 54,72921 | 25,21857 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 43 | K1045 | Ukmergės–M. Lietuvio g. | 54,74501 | 25,22014 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 44 | K1046 | Vakarinis aplink.–Pavilnionių g. | 54,73572 | 25,20845 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 45 | K1047 | S. Stanevičiaus g. 2A pėsčiųjų perėja | 54,72116 | 25,2558 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 46 | K1048 | P. Lukšio–J. Kubiliaus g. | 54,71132 | 25,29864 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 47 | K1049 | Drujos–Rasų g. | 54,67373 | 25,2983 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 48 | K105 | J. Tumo-Vaižganto–Lukiškių g., J. Tumo-Vaižganto-Savickio g. | 54,69121 | 25,27024 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 49 | K1050 | Pilaitės pr. –V. Pociūno g | 54,70151 | 25,20459 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 50 | K1051 | Savanorių pr.–Sausupio g. | 54,64683 | 25,21114 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 51 | K1052 | Rygos - Taikos g. (prie PC „IKI“) | 54,71879 | 25,21414 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 52 | K1053 | Pilaitės pr.–Įsruties g. | 54,71025 | 25,18208 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 53 | K1055 | G. Vilko g.–Mokslininkų g.–Molėtų pl. | 54,7533 | 25,26929 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 54 | K1056 | Mokslininkų–Baltupio g. | 54,75295 | 25,27124 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 55 | K1057 | Mokslininkų g. perėja | 54,75077 | 25,27639 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 56 | K1058 | Panerių g. perėja | 54,6682 | 25,26416 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 57 | K1059 | Naugarduko–Iešmininkų g. | 54,66714 | 25,25371 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 58 | K106 | Gedimino pr.–J. Tumo-Vaižganto g. | 54,689 | 25,26866 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 59 | K1060 | Naugarduko–Gerosios Vilties g. | 54,66657 | 25,25207 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 60 | K1061 | Panerių–Iešmininkų g. | 54,66628 | 25,25459 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 61 | K1062 | Vydūno–Karaliaučiaus g. | 54,70734 | 25,17495 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 62 | K1063 | Molėtų pl.–Santaros g. | 54,75804 | 25,27208 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 63 | K1064 | Santariškių–Santaros g. | 54,75764 | 25,281 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 64 | K1065 | Santarišnių g. 1A perėja | 54,75249 | 25,28007 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 65 | K1066 | Rygos–Taikos g. (prie PC „Maxima“) | 54,71656 | 25,22834 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 66 | K1067 | Gelvonų g. perėja | 54,71473 | 25,25707 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 67 | K1068 | Kalvarijų–J. Kazlausko g. | 54,723 | 25,28262 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 68 | K1069 | Verkių–P. Lukšio g. | 54,71232 | 25,29264 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 69 | K107 | V. Kudirkos–Pamėnkalnio–J. Jasinskio g. | 54,68761 | 25,26786 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 70 | K1070 | Žirmūnų–P. Lukšio g. | 54,7113 | 25,30484 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 71 | K1071 | Drujos–Subačiaus g. | 54,67679 | 25,307 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 72 | K1072 | Parko–Pergalės g. | 54,68847 | 25,41126 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 73 | K1073 | Parko g. perėja | 54,68562 | 25,42234 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 74 | K1074 | Ukmergės–Perkūnkiemio g. | 54,74235 | 25,22322 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 75 | K1075 | Vikingų. (prie Oreivių g.) | 54,65009 | 25,27874 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 76 | K1076 | Kalvarijų g. 294 perėja | 54,73375 | 25,27888 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 77 | K1078 | F. Vaitkaus g.–Vikingų g.–Rodūnios kel. | 54,64304 | 25,27778 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 78 | K1079 | Naugarduko g. pėsčiųjų perėja | 54,66551 | 25,2469 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 79 | K108 | V. Kudirkos–Z. Sierakausko g. | 54,68397 | 25,26486 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 80 | K1080 | Gariūnų–Paneriškių g. | 54,65903 | 25,15995 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 81 | K1081 | Ukmergės-Pašilaičių-J. Baltrušaičio g. | 54,72934 | 25,23655 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 82 | K1082 | P. Žadeikos–S. Nėries g. | 54,7309 | 25,2397 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 83 | K1083 | Ukmergės–Bieliūnų g. (prie PC BIG) | 54,73969 | 25,22699 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 84 | K1084 | Laisvės pr. 80 perėja | 54,722862 | 25,233653 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 85 | K1085 | Ukmergės perėja ties P. Žadeikos g. | 54,730991 | 25,237644 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 86 | K1086 | Ukmergės perėja ties J. Baltrušaičio g. | 54,728544 | 25,239768 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 87 | K1087 | B. Radvilaitės-Pilies g. | 54,6848 | 25,28939 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 88 | K1089 | Pavilnionių g. 55 | 54,73572 | 25,21195 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 89 | K109 | V. Kudirkos–M. K. Čiurlionio g. | 54,6824 | 25,26498 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 90 | K1090 | Galvės–J. Tiškevičiaus g. | 54,62387 | 25,11128 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 91 | K1091 | Pilaitės pr.–Pilkalnio g. | 54,71045 | 25,16486 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 92 | K1092 | Pavilnionių–Perkūnkiemio–Grigalaukio g. | 54,73575 | 25,21659 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 93 | K1093 | Saulėtekio g. perėja | 54,72408 | 25,3402 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 94 | K1094 | Nemenčinės pl. 112 | 54,75528 | 25,36614 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 95 | K1095 | V. Pociūno–Sidaronių g. | 54,70539 | 25,20583 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 96 | K1096 | Nemenčinės pl. 15 | 54,73091 | 25,3323 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 97 | K1097 | L. Asanavičiūtės g. 48 | 54,69423 | 25,21591 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 98 | K1098 | S. Batoro g. pėsčiųjų perėja | 54,68598 | 25,32399 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 99 | K1099 | Vikingų-Oro uosto | 54,64416 | 25,27756 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 100 | K110 | Švitrigailos–J. Basanavičiaus g. | 54,68133 | 25,26535 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 101 | K1100 | Olandų g. perėja ties Polocko st. | 54,68291 | 25,31255 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 102 | K1101 | Pilaitės pr.- Karaliaučiaus g. | 54,710378 | 25,17266 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 103 | K111 | Švitrigailos–A. Vivulskio g. | 54,67923 | 25,26684 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 104 | K112 | Švitrigailos–T. Ševčenkos g. | 54,67753 | 25,26744 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 105 | K113 | Švitrigailos–Naugarduko g. | 54,67438 | 25,26857 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 106 | K114 | Švitrigailos–Kauno g. | 54,67185 | 25,26944 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 107 | K115 | Švitrigailos–Panerių g. | 54,66942 | 25,27044 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 108 | K116 | Dariaus ir Girėno–Pelesos–Prūsų g. | 54,66589 | 25,27288 | xxxxxx | Siemens | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 109 | K117 | Dariaus ir Girėno–Brolių g. | 54,66313 | 25,27337 | xxxxxx | Siemens | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 110 | K118 | Dariaus ir Girėno–Šaltkalvių–Kapsų g. | 54,66146 | 25,27352 | xxxxxx | Siemens | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 111 | K119 | Dariaus ir Girėno g.–Eišiškių pl. | 54,65227 | 25,27151 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 112 | K120 | Dariaus ir Girėno g. perėja | 54,64617 | 25,27341 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 113 | K121 | Dariaus ir Girėno–F. Vaitkaus g. | 54,64304 | 25,274 | xxxxxx | Siemens | C800V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 114 | K151 | J. Jasinskio g. perėja (ties mokykla) | 54,68818 | 25,26379 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 115 | K153 | Žemaitės–Kauno–Naugarduko g. Kauno–Smolensko g. | 54,67013 | 25,26024 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 116 | K154 | Naugarduko g. (ties PC „Maxima“) | 54,66954 | 25,25787 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 117 | K155 | Kauno g. perėja (ties suaugusiųjų m-kla) | 54,67075 | 25,26338 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 118 | K156 | Kauno–Vytenio g. | 54,67131 | 25,26639 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 119 | K201 | Kalvarijų–Upės–Žvejų g. | 54,69207 | 25,28008 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 120 | K202 | Kalvarijų–A. Juozapavičiaus g. | 54,69342 | 25,28102 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 121 | K203 | Kalvarijų g.–Šeimyniškių g.–Konstitucijos pr. | 54,69472 | 25,28178 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 122 | K204 | Kalvarijų–Krokuvos g. | 54,69701 | 25,28187 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 123 | K205 | Kalvarijų g., pėsčiųjų perėja prie turgaus | 54,69978 | 25,28274 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 124 | K206 | Kalvarijų–Žalgirio g. | 54,70377 | 25,28382 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 125 | K207 | Žalgirio–Rinktinės–Tuskulėnų g., Minties–Apkasų g. | 54,70409 | 25,2888 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 126 | K209 | Kalvarijų g. perėja prie m-klos | 54,70688 | 25,28483 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 127 | K210 | Kalvarijų–Apkasų g. | 54,70872 | 25,28564 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 128 | K211 | Kalvarijų–P. Lukšio g. Kalvarijų g. 127 perėja | 54,71343 | 25,2868 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 129 | K213 | Kalvarijų–Kareivių–Ozo g. | 54,71505 | 25,2868 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 130 | K214 | Kalvarijų–V. Gerulaičio g. | 54,71722 | 25,28655 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 131 | K215 | Kareivių–Verkių g. | 54,71719 | 25,29484 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 132 | K216 | Kareivių–Žirmūnų g. | 54,72031 | 25,30246 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 133 | K217 | Kareivių–Valakupių tiltas (perėja) | 54,72382 | 25,30785 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 134 | K218 | Kareivių g., ties degaline | 54,71884 | 25,29982 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 135 | K219 | Žirmūnų g. 87 perėja | 54,71904 | 25,30366 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 136 | K220 | Tuskulėnų g. pėsčiųjų perėja | 54,7025 | 25,28987 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 137 | K251 | Konstitucijos pr. - L. Kačynskio g. | 54,69507 | 25,27936 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 138 | K252 | Konstitucijos pr. (ties PC „Europa“) | 54,69616 | 25,27517 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 139 | K253 | Konstitucijos pr., ties „Quadrum“ | 54,69749 | 25,27123 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 140 | K254 | Konstitucijos pr.–Linkmenų g. | 54,69824 | 25,26767 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 141 | K255 | Geležinio Vilko–Ukmergės g. | 54,70164 | 25,26282 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 142 | K256 | G. Vilko–Žalgirio g. | 54,70414 | 25,26647 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 143 | K257 | Žalgirio–Linkmenų g. | 54,70471 | 25,27076 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 144 | K258 | Žalgirio–Kernavės g. | 54,70429 | 25,27606 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 145 | K259 | Žalgirio–Giedraičių g. | 54,70398 | 25,27991 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 146 | K260 | Kalvarijų–Žvalgų g. | 54,72199 | 25,28443 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 147 | K261 | Apkasų–Ulonų g., Ulonų–S. Žukausko g. | 54,7084 | 25,29084 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 148 | K262 | Kalvarijų–J. Treinio g. | 54,71013 | 25,2861 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 149 | K263 | Lvovo–Gegužės 3-iosios g. | 54,69728 | 25,27653 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 150 | K264 | Lvivo - Kernavės g. | 54,69825 | 25,27376 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 151 | K265 | Žalgirio-Širvintų g. | 54,70418 | 25,27765 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 152 | K301 | Gedimino pr. –Šventaragio g. | 54,68594 | 25,28616 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 153 | K302 | Arsenalo–Žygimantų–T. Vrublevskio g. | 54,68825 | 25,28805 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 154 | K303 | T. Kosciuškos pėsčiųjų perėja | 54,68902 | 25,29365 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 155 | K304 | T. Kosciuškos–Sluškų g. | 54,69078 | 25,29832 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 156 | K306 | Antakalnio g., prie L. Sapiegos g. | 54,69599 | 25,30795 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 157 | K308 | Antakalnio g. perėja ties J. Lelevelio m-kla | 54,70199 | 25,30948 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 158 | K309 | Antakalnio g. 35 perėja | 54,70294 | 25,30982 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 159 | K310 | Antakalnio g. 37 perėja | 54,70385 | 25,31015 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 160 | K311 | Antakalnio–Šilo g. | 54,70503 | 25,31061 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 161 | K312 | Žirmūnų–Minties g. | 54,70572 | 25,30454 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 162 | K313 | Antakalnio - Tramvajų g. | 54,70681 | 25,31175 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 163 | K314 | Antakalnio–Klinikų g. | 54,70885 | 25,31334 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 164 | K315 | Antakalnio g. perėja prie poliklinikos, Antakalnio–Žolyno g. | 54,71175 | 25,3149 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 165 | K317 | Antakalnio–Švyturio g. | 54,71386 | 25,3156 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 166 | K318 | Antakalnio–P. Širvio g. | 54,71498 | 25,31598 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 167 | K319 | Antakalnio–Tverečiaus | 54,71664 | 25,31648 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 168 | K320 | Antakalnio–M. K. Oginskio | 54,71883 | 25,31712 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 169 | K321 | Antakalnio g. perėja ties universitetine ligonine | 54,72029 | 25,3177 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 170 | K351 | Rinktinės–Olimpiečių–Žvejų g. | 54,68958 | 25,28841 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 171 | K352 | Rinktinės–A. Juozapavičiaus g. | 54,69198 | 25,2893 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 172 | K353 | Šeimyniškių–Rinktinės g. | 54,69475 | 25,2883 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 173 | K354 | Šeimyniškių g. pėsčiųjų perėja ties Slucko g. | 54,69438 | 25,28515 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 174 | K355 | Olandų ties Vilniaus technikos kolegija | 54,69079 | 25,30461 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 175 | K356 | Žirmūnų g. pėsčiųjų perėja ties Minties st. | 54,70391 | 25,30436 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 176 | K357 | Nemenčinės pl.–Žirgo g. | 54,72386 | 25,32104 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 177 | K358 | Nemenčinės pl.–Saulėtekio al. | 54,7269 | 25,32613 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 178 | K359 | Rinktinės g.–Ceikinių g. | 54,69358 | 25,28873 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 179 | K401 | J. Basanavičiaus–Vytenio g. | 54,68081 | 25,26288 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 180 | K402 | J. Basanavičiaus–S. Konarskio g. | 54,68009 | 25,26034 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 181 | K403 | Savanorių pr.–Kedrų g. | 54,67739 | 25,25502 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 182 | K404 | Savanorių pr.–Žemaitės g.–V. Pietario g. | 54,67549 | 25,25128 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 183 | K405 | S. Konarskio–V. Pietario g. | 54,67702 | 25,24887 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 184 | K406 | Savanorių pr. pėsčiųjų perėja prie PC „Maxima“ | 54,67376 | 25,24789 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 185 | K407 | Savanorių pr. 43A perėja | 54,67274 | 25,2459 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 186 | K408 | Savanorių pr.–Gerosios Vilties g. | 54,67135 | 25,24319 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 187 | K409 | Geležinio Vilko–Gerosios Vilties g. | 54,67325 | 25,24027 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 188 | K410 | Savanorių pr. 68 perėja | 54,67033 | 25,24126 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 189 | K411 | Savanorių pr.–Vilkpėdės g | 54,66369 | 25,23591 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 190 | K412 | Savanorių pr.– N.Riovonių g. | 54,65918 | 25,23295 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 191 | K413 | Savanorių pr. (prie „Plastos“) | 54,64986 | 25,21651 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 192 | K414 | Savanorių pr.–Vaduvos g. | 54,64416 | 25,20559 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 193 | K415 | Savanorių pr. 6 perėja | 54,67881 | 25,25779 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 194 | K451 | Žemaitės–Eglių g. | 54,67362 | 25,25416 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 195 | K501 | Vasario 16-osios–Šermukšnių g. | 54,69029 | 25,27287 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 196 | K502 | Vasario 16-osios g.–Gedimino pr. | 54,68842 | 25,27215 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 197 | K503 | Gedimino pr.–A. Jakšto g. | 54,68793 | 25,27506 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 198 | K504 | Vasario 16-osios–Pamėnkalnio g. | 54,68713 | 25,27166 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 199 | K505 | Pamėnkalnio–A. Stulginskio g. | 54,68603 | 25,2745 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 200 | K506 | Pylimo–Pamėnkalnio–Jogailos g. | 54,68457 | 25,27713 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 201 | K507 | Mindaugo–K. Kalinausko–Tauro g. | 54,68266 | 25,27331 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 202 | K508 | Pylimo–Palangos–K. Kalinausko | 54,68285 | 25,2773 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 203 | K509 | Pylimo–Klaipėdos g. | 54,68188 | 25,27725 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 204 | K510 | Mindaugo–J. Basanavičiaus g. | 54,68088 | 25,27263 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 205 | K511 | Pylimo–J. Basanavičiaus–Trakų g. | 54,67989 | 25,27784 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 206 | K512 | Mindaugo–T. Ševčenkos g. (perėja) | 54,67843 | 25,27352 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 207 | K513 | Pylimo g., prie PC „IKI“, Pylimo–Naugarduko g. | 54,67816 | 25,27935 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 208 | K515 | Pylimo–Ligoninės g. | 54,67713 | 25,28045 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 209 | K516 | Mindaugo–Naugarduko g. | 54,67639 | 25,2743 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 210 | K517 | Mindaugo–Šaltinių g. | 54,67501 | 25,27478 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 211 | K518 | Pylimo–Šv. Stepono–Rūdninkų g. | 54,67552 | 25,28319 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 212 | K519 | Pylimo–Sodų g. | 54,67427 | 25,28506 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 213 | K520 | Aušros Vartų–Bazilijonų–M. Daukšos g. | 54,67405 | 25,28956 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 214 | K521 | M. Daukšos g. perėja prie Senamiesčio m-klos | 54,67466 | 25,29219 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 215 | K522 | Mindaugo–Kauno g. | 54,67267 | 25,27559 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 216 | K523 | Kauno–Aguonų g. | 54,6728 | 25,27799 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 217 | K524 | Kauno–Šv. Stepono–V. Šopeno g. | 54,67244 | 25,27993 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 218 | K525 | V. Šopeno–Sodų g. | 54,672 | 25,2826 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 219 | K551 | J. Basanavičiaus–Algirdo g. | 54,6817 | 25,26955 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 220 | K552 | Algirdo–T. Ševčenkos g. | 54,67802 | 25,2708 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 221 | K553 | Algirdo–Naugarduko g. | 54,6755 | 25,27178 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 222 | K554 | Kauno–Algirdo g. | 54,67245 | 25,27281 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 223 | K555 | Aušros Vartų–Geležinkelio–Drujos g. | 54,67218 | 25,29029 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 224 | K556 | Panerių–Algirdo g. | 54,67 | 25,27366 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 225 | K601 | T. Narbuto–Saltoniškių g. | 54,69877 | 25,25874 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 226 | K602 | T. Narbuto pėsčiųjų perėja | 54,69941 | 25,25356 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 227 | K603 | T. Narbuto–Sėlių g., T. Narbuto g. perėja, netoli Sėlių g. | 54,69938 | 25,24341 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 228 | K605 | Sėlių g. perėja | 54,69846 | 25,24777 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 229 | K606 | Kęstučio–Latvių g. | 54,69487 | 25,25052 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 230 | K607 | Kęstučio–A. Mickevičiaus g. | 54,69001 | 25,25184 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 231 | K608 | Vytauto–A. Mickevičiaus g. | 54,69041 | 25,25646 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 232 | K609 | Liubarto–Vytauto g. | 54,68904 | 25,25639 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 233 | K610 | Gedimino pr.–A. Goštauto | 54,6904 | 25,25991 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 234 | K701 | A. Goštauto–Gynėjų g. | 54,69506 | 25,26431 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 235 | K702 | Gynėjų–Lukiškių g. | 54,69357 | 25,26486 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 236 | K721 | Sėlių–Saltoniškių g., Vytauto–Latvių–Upės g. | 54,69626 | 25,256 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 237 | K740 | Ozo g., ties Licėjumi (ties PC „Ozas“) | 54,71311 | 25,27813 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 238 | K741 | Ozo–Gelvonų g. | 54,711 | 25,25731 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 239 | K742 | Ozo–Ukmergės g. | 54,71014 | 25,24909 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 240 | K743 | Ozo g. perėja | 54,70982 | 25,2452 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 241 | K744 | Ozo–Buivydiškių g. | 54,7089 | 25,2403 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 242 | K761 | Laisvės pr. –Pilaitės pr.–T. Narbuto g. | 54,69933 | 25,22227 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 243 | K762 | Laisvės pr.–Justiniškių g. | 54,70342 | 461 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 244 | K763 | Laisvės pr.–Viršuliškių g. (prie PC „Mada“) | 54,70727 | 25,22659 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 245 | K764 | Laisvės pr. ties degaline (prie PC „Mada“) | 54,70965 | 25,22622 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 246 | K765 | Laisvės pr.–Viršuliškių g. (prie bažnyčios) | 54,71212 | 25,22749 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 247 | K766 | Laisvės pr.–Rygos g.–Buivydiškių g. | 54,71616 | 25,23081 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 248 | K767 | Laisvės pr.-Čiobiškio g. | 54,71919 | 25,23249 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 249 | K768 | Laisvės pr.–Žemynos g. | 54,72622 | 25,23367 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 250 | K781 | Justiniškių–Viršilų g. | 54,70386 | 25,21887 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 251 | K782 | Justiniškių–Šešuolių g. | 54,7061 | 25,21565 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 252 | K783 | Justiniškių–Lūžių g. | 54,71124 | 25,21714 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 253 | K784 | Justiniškių–Rygos g. | 54,71701 | 25,22145 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 254 | K785 | Justiniškių–Taikos g. | 54,72114 | 25,22274 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 255 | K786 | Taikos–Lūžių g. | 54,71177 | 25,21447 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 256 | K787 | Rygos g. 8 | 54,71666 | 25,22481 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 257 | K801 | Ukmergės–Fabijoniškių g. (ties „Topo centru“) | 54,7211 | 25,24422 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 258 | K802 | Ukmergės g. perėja ties poliklinika | 54,72413 | 25,2425 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 259 | K803 | Ukmergės g., ties PC „Senukai | 54,72279 | 25,24325 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 260 | K821 | Ateities–Fabijoniškių g. | 54,73057 | 25,24529 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 261 | K822 | Ateities g. ties silpnaregių centru | 54,7315 | 25,24798 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 262 | K823 | Ateities–S. Stanevičiaus g. | 54,7336 | 25,25343 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 263 | K824 | Ateities–Didlaukio g. | 54,73527 | 25,25889 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 264 | K825 | Ateities–Baltupio g. | 54,73692 | 25,26588 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 265 | K826 | Ateities-Bitininkų g., Ateities-Maumedžių g. | 54,73807 | 25,26987 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 266 | K828 | Kalvarijų–Ateities g. | 54,73926 | 25,27676 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 267 | K829 | Jeruzalės–Braškių g. | 54,74357 | 25,27739 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 268 | K830 | Jeruzalės–Mokyklos g. | 54,74488 | 25,27795 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 269 | K831 | Ateities (prie PC „LIDL“) | 54,73916 | 25,2739 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 270 | K832 | Ateities–Visorių g. | 54,73566 | 25,2611 | xxxxxx | Dynniq | EC-2 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 271 | K841 | Vydūno–Papilėnų g. | 54,70723 | 25,18659 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 272 | K842 | Vydūno–Įsruties g., Vydūno–Tolminkiemio g. | 54,70729 | 25,18195 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 273 | K861 | Saulėtekio al. prie VGTU, Saulėtekio al., prie VU | 54,7247 | 25,33171 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 274 | K881 | S. Batoro–Dūmų g. | 54,69216 | 25,40793 | xxxxxx | Siemens | C900V | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 275 | K882 | Pramonės–Pergalės g. | 54,693 | 25,41053 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 276 | K883 | Juodasis kel.–Gurių g. | 54,66298 | 25,339 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 277 | K884 | Juodasis kel.–Tolimoji g. | 54,66262 | 25,34136 | xxxxxx | Yunex | sX | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 278 | K1000 | Ukmergės–Gabijos–S. Neries g | 54,73692 | 25,230989 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 279 | K1106 | Ukmergės–Pašilaičių g. | 54,730214 | 25,235767 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 280 | K1105 | Molėtų pl. perėja | 54,769787 | 25,271624 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 281 | K250 | Linkmenų–Lvovo g. | 54,69968 | 25,26856 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 282 | K360 | Šeimyniškių g. perėja | 54,695175 | 25,292526 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 283 | K417 | Konarskio g. perėja | 54,67921 | 25,255346 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 284 | K833 | L. Giros–M. Šleževičiaus g. | 54,734648 | 25,251692 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 285 | K760 | Pilaitės pr.–Spaudos–Sietyno g. | 54,699663 | 25,212215 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 286 | K416 | Savanoriu pr. - Račių g. | 54,654258 | 25,224401 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 287 | K1102 | Vikingų g. ties Žirnių g. viaduku | 54,652821 | 25,282544 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 288 | K1103 | Vikingų g. 1 | 54,651629 | 25,282324 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 289 | K745 | Buivydiškių–Dūkštų g. | 54,712365 | 25,239081 | xxxxxx | Swarco | ITC-3 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |
| 290 | K1104 | Kovo 11 g. (Grigiškės) | 54,669231 | 25,097359 | xxxxxx | Cross | RS5 | xxx.xxx.xxx.xxx | 255.255.255.0 | xxx.xxx.xxx.xxx |

Annex 2 to the Technical Specification

Preliminary plan for switching intersections from the existing SiTraffic Scala traffic light control software to the new System.

| Intersection number | Intersection name | Controller manufacturer | Controller model | Control level | Switching stage |
| --- | --- | --- | --- | --- | --- |
| K1002 | Žirmūnų g. perėja prie Tuskulėnų rimties p. | Dynniq | EC-2 | Local | 1 |
| K1006 | Liepkalnio pėsčiųjų perėja | Siemens | C900V | Control centre | 1 |
| K1011 | Žirmūnų g. 67 (prie PC „LIDL“) | Dynniq | EC-2 | Local | 1 |
| K1013 | Šiaurės–A. Kojelavičiaus | Siemens | C900V | Control centre | 1 |
| K1020 | S. Stanevičiaus g. pėsčiųjų perėja (ties „Jovaru“) | Siemens | C900V | Control centre | 1 |
| K1021 | Buivydiškių g. perėja | Siemens | C900V | Local | 1 |
| K1023 | Pašilaičių g. pėsčiųjų perėja | Siemens | C900V | Control centre | 1 |
| K1024 | Ukmergės g. perėja (ties Gedvydžių g.) | Siemens | C900V | Control centre | 1 |
| K1026 | L. Asanavičiūtės g. perėja (ties „Kometa“) | Siemens | C900V | Local | 1 |
| K1028 | Architektų g. pėsčiųjų perėja | Siemens | C900V | Control centre | 1 |
| K1035 | Santariškių pėsčiųjų perėja | Siemens | C900V | Local | 1 |
| K1047 | S. Stanevičiaus g. 2A pėsčiųjų perėja | Dynniq | EC-2 | Local | 1 |
| K1057 | Mokslininkų g. perėja | Dynniq | EC-2 | Local | 1 |
| K1058 | Panerių g. perėja | Dynniq | EC-2 | Local | 1 |
| K1065 | Santariškių g. 1A perėja | Dynniq | EC-2 | Local | 1 |
| K1067 | Gelvonų g. perėja | Dynniq | EC-2 | Local | 1 |
| K1073 | Parko g. perėja | Dynniq | EC-2 | Local | 1 |
| K1076 | Kalvarijų g. 294 perėja | Dynniq | EC-2 | Local | 1 |
| K1079 | Naugarduko g. pėsčiųjų perėja | Dynniq | EC-2 | Local | 1 |
| K1084 | Laisvės pr. 80 perėja | Dynniq | EC-2 | Local | 1 |
| K1093 | Saulėtekio g. perėja | Yunex | sX | Local | 1 |
| K1098 | S. Batoro g. pėsčiųjų perėja | Yunex | sX | Local | 1 |
| K120 | Dariaus ir Girėno g. perėja | Siemens | C800V | Local | 1 |
| K303 | T. Kosciuškos pėsčiųjų perėja | Siemens | C900V | Local | 1 |
| K304 | T. Kosciuškos–Sluškų g. | Siemens | C900V | Local | 1 |
| K1106 | Ukmergės–Pašilaičių g. | Cross | RS5 | Local | 1 |
| K1105 | Molėtų pl. perėja | Cross | RS5 | Local | 1 |
| K360 | Šeimyniškių g. perėja | Cross | RS5 | Local | 1 |
| K417 | Konarskio g. perėja | Cross | RS5 | Local | 1 |
| K1104 | Kovo 11 g. (Grigiškės) | Cross | RS5 | Local | 1 |
| K1010 | Šilo–V. Grybo g. | Siemens | C900V | Control centre | 2 |
| K1015 | Didlaukio g. ties Baltupio m-kla | Siemens | C900V | Control centre | 2 |
| K1017 | Gelvonų g. perėja (ties PC „Maxima“) | Siemens | C900V | Control centre | 2 |
| K1018 | Gelvonų–Paberžės g. | Siemens | C900V | Control centre | 2 |
| K1019 | Šeškinės–Paberžės g. | Siemens | C900V | Local | 2 |
| K1027 | L. Asanavičiūtės–V. Maciulevičiaus g. | Siemens | C900V | Control centre | 2 |
| K1030 | Metalo–Geologų g. | Siemens | C900V | Control centre | 2 |
| K1032 | Galvės–Kirtimų g. | Siemens | C900V | Local | 2 |
| K1033 | Lazdynėlių–Šltnamių g. | Siemens | C900V | Local | 2 |
| K1036 | Metalo g.–Eišiškių pl. | Dynniq | EC-2 | Local | 2 |
| K1038 | Molėtų pl.–Skersinės g. | Dynniq | EC-2 | Local | 2 |
| K1041 | Žemaitės g. (ties PC „LIDL“) | Dynniq | EC-2 | Local | 2 |
| K1042 | Saulėtekio al.–Šatrijos Raganos g. | Dynniq | EC-2 | Local | 2 |
| K1043 | Eišiškių pl. 13 (ties VĮ Oro navigacija) | Dynniq | EC-2 | Local | 2 |
| K1052 | Rygos - Taikos g. (prie PC „IKI“) | Dynniq | EC-2 | Local | 2 |
| K1062 | Vydūno–Karaliaučiaus g. | Dynniq | EC-2 | Local | 2 |
| K1071 | Drujos–Subačiaus g. | Dynniq | EC-2 | Local | 2 |
| K1072 | Parko–Pergalės g. | Dynniq | EC-2 | Local | 2 |
| K1075 | Vikingų. (prie Oreivių g.) | Dynniq | EC-2 | Local | 2 |
| K1082 | P. Žadeikos–S. Nėries g. | Dynniq | EC-2 | Local | 2 |
| K1085 | Ukmergės perėja ties P. Žadeikos g. | Dynniq | EC-2 | Local | 2 |
| K1086 | Ukmergės perėja ties J. Baltrušaičio g. | Dynniq | EC-2 | Local | 2 |
| K1092 | Pavilnionių–Perkūnkiemio–Grigalaukio g. | Dynniq | EC-2 | Local | 2 |
| K1094 | Nemenčinės pl. 112 | Dynniq | EC-2 | Local | 2 |
| K1096 | Nemenčinės pl. 15 | Dynniq | EC-2 | Manual | 2 |
| K1101 | Pilaitės pr.- Karaliaučiaus g. | Swarco | ITC-3 | Local | 2 |
| K355 | Olandų ties Vilniaus technikos kolegija | Siemens | C900V | Control centre | 2 |
| K356 | Žirmūnų g. pėsčiųjų perėja ties Minties st. | Dynniq | EC-2 | Local | 2 |
| K787 | Rygos g. 8 | Siemens | C900V | Local | 2 |
| K1004 | Subačiaus g. pėsčiųjų perėja | Siemens | C900V | Control centre | 3 |
| K1009 | Liepkalnio g.–Žirnių g.–Minsko pl. | Yunex | sX | Local | 3 |
| K1039 | Nemenčinės pl.–Svajonių g. | Siemens | C900V | Local | 3 |
| K1044 | Justiniškių–Virbeliškių g. | Dynniq | EC-2 | Local | 3 |
| K1045 | Ukmergės–M. Lietuvio g. | Dynniq | EC-2 | Local | 3 |
| K1046 | Vakarinis aplink.–Pavilnionių g. | Dynniq | EC-2 | Local | 3 |
| K1048 | P. Lukšio–J. Kubiliaus g. | Dynniq | EC-2 | Local | 3 |
| K1049 | Drujos–Rasų g. | Dynniq | EC-2 | Local | 3 |
| K1050 | Pilaitės pr. –V. Pociūno g | Yunex | sX | Local | 3 |
| K1051 | Savanorių pr.–Sausupio g. | Dynniq | EC-2 | Local | 3 |
| K1053 | Pilaitės pr.–Įsruties g. | Dynniq | EC-2 | Local | 3 |
| K1055 | G. Vilko g.–Mokslininkų g.–Molėtų pl. | Dynniq | EC-2 | Local | 3 |
| K1056 | Mokslininkų–Baltupio g. | Dynniq | EC-2 | Local | 3 |
| K1059 | Naugarduko–Iešmininkų g. | Dynniq | EC-2 | Local | 3 |
| K1060 | Naugarduko–Gerosios Vilties g. | Dynniq | EC-2 | Local | 3 |
| K1061 | Panerių–Iešmininkų g. | Dynniq | EC-2 | Local | 3 |
| K1063 | Molėtų pl.–Santaros g. | Dynniq | EC-2 | Local | 3 |
| K1064 | Santariškių–Santaros g. | Dynniq | EC-2 | Local | 3 |
| K1066 | Rygos–Taikos g. (prie PC „Maxima“) | Dynniq | EC-2 | Local | 3 |
| K1068 | Kalvarijų–J. Kazlausko g. | Dynniq | EC-2 | Local | 3 |
| K1069 | Verkių–P. Lukšio g. | Dynniq | EC-2 | Local | 3 |
| K1070 | Žirmūnų–P. Lukšio g. | Dynniq | EC-2 | Local | 3 |
| K1074 | Ukmergės–Perkūnkiemio g. | Dynniq | EC-2 | Local | 3 |
| K1078 | F. Vaitkaus g.–Vikingų g.–Rodūnios kel. | Yunex | sX | Local | 3 |
| K1081 | Ukmergės-Pašilaičių-J. Baltrušaičio g. | Dynniq | EC-2 | Local | 3 |
| K1083 | Ukmergės–Bieliūnų g. (prie PC BIG) | Dynniq | EC-2 | Local | 3 |
| K1087 | B. Radvilaitės-Pilies g. | Dynniq | EC-2 | Local | 3 |
| K1090 | Galvės–J. Tiškevičiaus g. | Dynniq | EC-2 | Local | 3 |
| K1091 | Pilaitės pr.–Pilkalnio g. | Dynniq | EC-2 | Local | 3 |
| K1099 | Vikingų-Oro uosto | Yunex | sX | Local | 3 |
| K1001 | Rinktinės–Kazliškių g. | Siemens | C900V | Control centre | 4 |
| K1003 | Olandų–Filaretų g. | Siemens | C900V | Control centre | 4 |
| K1005 | Drujos–Manufaktūrų g. | Yunex | sX | Control centre | 4 |
| K1007 | Rasų–Sukilėlių–Ribiškių didžioji g. | Siemens | C900V | Control centre | 4 |
| K1014 | Kalvarijų–Didlaukio g. | Siemens | C900V | Control centre | 4 |
| K1016 | Geležinio Vilko–Didlaukio g. | Siemens | C900V | Control centre | 4 |
| K1029 | Architektų–Erfurto g. | Siemens | C900V | Control centre | 4 |
| K1080 | Gariūnų–Paneriškių g. | Dynniq | EC-2 | Local, Manual | 4 |
| K1095 | V. Pociūno–Sidaronių g. | Yunex | sX | Local | 4 |
| K1097 | L. Asanavičiūtės g. 48 | Yunex | sX | Local | 4 |
| K119 | Dariaus ir Girėno g.–Eišiškių pl. | Siemens | C800V | Local | 4 |
| K121 | Dariaus ir Girėno–F. Vaitkaus g. | Siemens | C800V | Local | 4 |
| K260 | Kalvarijų–Žvalgų g. | Siemens | C900V | Local | 4 |
| K261 | Apkasų–Ulonų g., Ulonų–S. Žukausko g. | Siemens | C900V | Local | 4 |
| K264 | Lvivo - Kernavės g. | Siemens | C900V | Local | 4 |
| K301 | Gedimino pr. –Šventaragio g. | Siemens | C900V | Local | 4 |
| K357 | Nemenčinės pl.–Žirgo g. | Siemens | C900V | Local | 4 |
| K841 | Vydūno–Papilėnų g. | Siemens | C900V | Local | 4 |
| K842 | Vydūno–Įsruties g., Vydūno–Tolminkiemio g. | Siemens | C900V | Local | 4 |
| K881 | S. Batoro–Dūmų g. | Siemens | C900V | Local | 4 |
| K882 | Pramonės–Pergalės g. | Yunex | sX | Local | 4 |
| K883 | Juodasis kel.–Gurių g. | Yunex | sX | Local | 4 |
| K884 | Juodasis kel.–Tolimoji g. | Yunex | sX | Local | 4 |
| K1000 | Ukmergės–Gabijos–S. Neries g | Swarco | ITC-3 | Local | 4 |
| K250 | Linkmenų–Lvovo g. | Swarco | ITC-3 | Local | 4 |
| K833 | L. Giros–M. Šleževičiaus g. | Swarco | ITC-3 | Local | 4 |
| K760 | Pilaitės pr.–Spaudos–Sietyno g. | Swarco | ITC-3 | Local | 4 |
| K416 | Savanoriu pr. - Račių g. | Swarco | ITC-3 | Local | 4 |
| K1102 | Vikingų g. ties Žirnių g. viaduku | Cross | RS5 | Local | 4 |
| K1103 | Vikingų g. 1 | Cross | RS5 | Local | 4 |
| K745 | Buivydiškių–Dūkštų g. | Swarco | ITC-3 | Local | 4 |
| K1008 | Dunojaus–Liepkalnio–Dzūkų g. | Siemens | C900V | Control centre | 5 |
| K1040 | Ukmergės–S. Nėries g. | Dynniq | EC-2 | Manual | 5 |
| K1089 | Pavilnionių g. 55 | Yunex | sX | Manual | 5 |
| K1100 | Olandų g. perėja ties Polocko st. | Yunex | sX | Manual | 5 |
| K358 | Nemenčinės pl.–Saulėtekio al. | Siemens | C900V | Control centre | 5 |
| K409 | Geležinio Vilko–Gerosios Vilties g. | Siemens | C900V | Manual | 5 |
| K411 | Savanorių pr.–Vilkpėdės g | Siemens | C900V | Control centre | 5 |
| K412 | Savanorių pr.– N.Riovonių g. | Siemens | C900V | Control centre | 5 |
| K413 | Savanorių pr. (prie „Plastos“) | Swarco | ITC-3 | Control centre | 5 |
| K414 | Savanorių pr.–Vaduvos g. | Siemens | C900V | Control centre | 5 |
| K701 | A. Goštauto–Gynėjų g. | Siemens | C900V | Control centre | 5 |
| K702 | Gynėjų–Lukiškių g. | Siemens | C900V | Control centre | 5 |
| K741 | Ozo–Gelvonų g. | Siemens | C900V | Control centre | 5 |
| K742 | Ozo–Ukmergės g. | Siemens | C900V | Control centre | 5 |
| K743 | Ozo g. perėja | Siemens | C900V | Control centre | 5 |
| K744 | Ozo–Buivydiškių g. | Siemens | C900V | Control centre | 5 |
| K761 | Laisvės pr. –Pilaitės pr.–T. Narbuto g. | Siemens | C900V | Control centre | 5 |
| K801 | Ukmergės–Fabijoniškių g. (ties „Topo centru“) | Siemens | C900V | Control centre | 5 |
| K802 | Ukmergės g. perėja ties poliklinika | Siemens | C900V | Control centre | 5 |
| K803 | Ukmergės g., ties PC „Senukai | Siemens | C900V | Control centre | 5 |
| K861 | Saulėtekio al. prie VGTU, Saulėtekio al., prie VU | Siemens | C900V | Control centre | 5 |
| K762 | Laisvės pr.–Justiniškių g. | Siemens | C900V | Control centre | 6 |
| K763 | Laisvės pr.–Viršuliškių g. (prie PC „Mada“) | Siemens | C900V | Control centre | 6 |
| K764 | Laisvės pr. ties degaline (prie PC „Mada“) | Siemens | C900V | Control centre | 6 |
| K765 | Laisvės pr.–Viršuliškių g. (prie bažnyčios) | Siemens | C900V | Control centre | 6 |
| K766 | Laisvės pr.–Rygos g.–Buivydiškių g. | Siemens | C900V | Control centre | 6 |
| K767 | Laisvės pr.-Čiobiškio g. | Siemens | C900V | Control centre | 6 |
| K768 | Laisvės pr.–Žemynos g. | Siemens | C900V | Control centre | 6 |
| K781 | Justiniškių–Viršilų g. | Siemens | C900V | Control centre | 6 |
| K782 | Justiniškių–Šešuolių g. | Siemens | C900V | Control centre | 6 |
| K783 | Justiniškių–Lūžių g. | Siemens | C900V | Control centre | 6 |
| K784 | Justiniškių–Rygos g. | Siemens | C900V | Control centre | 6 |
| K785 | Justiniškių–Taikos g. | Siemens | C900V | Control centre | 6 |
| K786 | Taikos–Lūžių g. | Siemens | C900V | Control centre | 6 |
| K821 | Ateities–Fabijoniškių g. | Siemens | C900V | Control centre | 6 |
| K822 | Ateities g. ties silpnaregių centru | Siemens | C900V | Control centre | 6 |
| K823 | Ateities–S. Stanevičiaus g. | Siemens | C900V | Control centre, manual | 6 |
| K824 | Ateities–Didlaukio g. | Siemens | C900V | Control centre | 6 |
| K825 | Ateities–Baltupio g. | Siemens | C900V | Control centre | 6 |
| K826 | Ateities-Bitininkų g., Ateities-Maumedžių g. | Siemens | C900V | Control centre | 6 |
| K828 | Kalvarijų–Ateities g. | Siemens | C900V | Control centre | 6 |
| K829 | Jeruzalės–Braškių g. | Siemens | C900V | Control centre | 6 |
| K830 | Jeruzalės–Mokyklos g. | Siemens | C900V | Control centre | 6 |
| K831 | Ateities (prie PC „LIDL“) | Siemens | C900V | Control centre | 6 |
| K832 | Ateities–Visorių g. | Dynniq | EC-2 | Control centre | 6 |
| K401 | J. Basanavičiaus–Vytenio g. | Siemens | C900V | Motion | 7 |
| K402 | J. Basanavičiaus–S. Konarskio g. | Siemens | C900V | Motion | 7 |
| K403 | Savanorių pr.–Kedrų g. | Siemens | C900V | Motion | 7 |
| K404 | Savanorių pr.–Žemaitės g.–V. Pietario g. | Siemens | C900V | Motion | 7 |
| K405 | S. Konarskio–V. Pietario g. | Siemens | C900V | Motion | 7 |
| K406 | Savanorių pr. pėsčiųjų perėja prie PC „Maxima“ | Siemens | C900V | Motion | 7 |
| K407 | Savanorių pr. 43A perėja | Siemens | C900V | Motion | 7 |
| K408 | Savanorių pr.–Gerosios Vilties g. | Siemens | C900V | Motion | 7 |
| K410 | Savanorių pr. 68 perėja | Siemens | C900V | Motion | 7 |
| K415 | Savanorių pr. 6 perėja | Siemens | C900V | Motion | 7 |
| K451 | Žemaitės–Eglių g. | Siemens | C900V | Motion | 7 |
| K601 | T. Narbuto–Saltoniškių g. | Siemens | C900V | Motion | 7 |
| K602 | T. Narbuto pėsčiųjų perėja | Siemens | C900V | Motion | 7 |
| K603 | T. Narbuto–Sėlių g., T. Narbuto g. perėja, netoli Sėlių g. | Siemens | C900V | Motion | 7 |
| K605 | Sėlių g. perėja | Siemens | C900V | Motion | 7 |
| K606 | Kęstučio–Latvių g. | Siemens | C900V | Motion | 7 |
| K607 | Kęstučio–A. Mickevičiaus g. | Siemens | C900V | Motion | 7 |
| K608 | Vytauto–A. Mickevičiaus g. | Siemens | C900V | Motion | 7 |
| K609 | Liubarto–Vytauto g. | Siemens | C900V | Motion | 7 |
| K610 | Gedimino pr.–A. Goštauto | Siemens | C900V | Motion | 7 |
| K721 | Sėlių–Saltoniškių g., Vytauto–Latvių–Upės g. | Siemens | C900V | Motion | 7 |
| K501 | Vasario 16-osios–Šermukšnių g. | Siemens | C900V | Motion | 8 |
| K502 | Vasario 16-osios g.–Gedimino pr. | Siemens | C900V | Motion | 8 |
| K503 | Gedimino pr.–A. Jakšto g. | Siemens | C900V | Motion | 8 |
| K504 | Vasario 16-osios–Pamėnkalnio g. | Siemens | C900V | Motion | 8 |
| K505 | Pamėnkalnio–A. Stulginskio g. | Siemens | C900V | Motion | 8 |
| K506 | Pylimo–Pamėnkalnio–Jogailos g. | Siemens | C900V | Motion | 8 |
| K507 | Mindaugo–K. Kalinausko–Tauro g. | Siemens | C900V | Motion | 8 |
| K508 | Pylimo–Palangos–K. Kalinausko | Siemens | C900V | Motion | 8 |
| K509 | Pylimo–Klaipėdos g. | Siemens | C900V | Motion | 8 |
| K510 | Mindaugo–J. Basanavičiaus g. | Siemens | C900V | Motion | 8 |
| K511 | Pylimo–J. Basanavičiaus–Trakų g. | Siemens | C900V | Motion | 8 |
| K512 | Mindaugo–T. Ševčenkos g. (perėja) | Siemens | C900V | Motion | 8 |
| K513 | Pylimo g., prie PC „IKI“, Pylimo–Naugarduko g. | Siemens | C900V | Motion | 8 |
| K515 | Pylimo–Ligoninės g. | Siemens | C900V | Motion | 8 |
| K516 | Mindaugo–Naugarduko g. | Siemens | C900V | Motion | 8 |
| K517 | Mindaugo–Šaltinių g. | Siemens | C900V | Motion | 8 |
| K518 | Pylimo–Šv. Stepono–Rūdninkų g. | Siemens | C900V | Motion | 8 |
| K519 | Pylimo–Sodų g. | Siemens | C900V | Motion | 8 |
| K520 | Aušros Vartų–Bazilijonų–M. Daukšos g. | Siemens | C900V | Motion | 8 |
| K521 | M. Daukšos g. perėja prie Senamiesčio m-klos | Siemens | C900V | Motion | 8 |
| K522 | Mindaugo–Kauno g. | Siemens | C900V | Motion | 8 |
| K523 | Kauno–Aguonų g. | Siemens | C900V | Motion | 8 |
| K524 | Kauno–Šv. Stepono–V. Šopeno g. | Siemens | C900V | Motion | 8 |
| K525 | V. Šopeno–Sodų g. | Siemens | C900V | Motion | 8 |
| K153 | Žemaitės–Kauno–Naugarduko g. Kauno–Smolensko g. | Siemens | C900V | Motion | 9 |
| K154 | Naugarduko g. (ties PC „Maxima“) | Siemens | C900V | Motion | 9 |
| K155 | Kauno g. perėja (ties suaugusiųjų m-kla) | Siemens | C900V | Motion | 9 |
| K156 | Kauno–Vytenio g. | Siemens | C900V | Motion | 9 |
| K220 | Tuskulėnų g. pėsčiųjų perėja | Siemens | C900V | Motion | 9 |
| K251 | Konstitucijos pr. - L. Kačynskio g. | Siemens | C900V | Motion | 9 |
| K252 | Konstitucijos pr. (ties PC „Europa“) | Siemens | C900V | Motion | 9 |
| K253 | Konstitucijos pr., ties „Quadrum“ | Siemens | C900V | Motion | 9 |
| K254 | Konstitucijos pr.–Linkmenų g. | Siemens | C900V | Motion | 9 |
| K255 | Geležinio Vilko–Ukmergės g. | Siemens | C900V | Motion | 9 |
| K256 | G. Vilko–Žalgirio g. | Siemens | C900V | Motion | 9 |
| K257 | Žalgirio–Linkmenų g. | Siemens | C900V | Motion | 9 |
| K258 | Žalgirio–Kernavės g. | Siemens | C900V | Motion | 9 |
| K259 | Žalgirio–Giedraičių g. | Siemens | C900V | Motion | 9 |
| K262 | Kalvarijų–J. Treinio g. | Siemens | C900V | Motion | 9 |
| K263 | Lvovo–Gegužės 3-iosios g. | Siemens | C900V | Motion | 9 |
| K265 | Žalgirio-Širvintų g. | Siemens | C900V | Motion | 9 |
| K551 | J. Basanavičiaus–Algirdo g. | Siemens | C900V | Motion | 9 |
| K552 | Algirdo–T. Ševčenkos g. | Siemens | C900V | Motion | 9 |
| K553 | Algirdo–Naugarduko g. | Siemens | C900V | Motion | 9 |
| K554 | Kauno–Algirdo g. | Siemens | C900V | Motion | 9 |
| K555 | Aušros Vartų–Geležinkelio–Drujos g. | Siemens | C900V | Motion | 9 |
| K556 | Panerių–Algirdo g. | Siemens | C900V | Motion | 9 |
| K302 | Arsenalo–Žygimantų–T. Vrublevskio g. | Siemens | C900V | Motion | 10 |
| K306 | Antakalnio g., prie L. Sapiegos g. | Siemens | C900V | Motion | 10 |
| K308 | Antakalnio g. perėja ties J. Lelevelio m-kla | Siemens | C900V | Motion | 10 |
| K309 | Antakalnio g. 35 perėja | Siemens | C900V | Motion | 10 |
| K310 | Antakalnio g. 37 perėja | Siemens | C900V | Motion | 10 |
| K311 | Antakalnio–Šilo g. | Siemens | C900V | Motion | 10 |
| K312 | Žirmūnų–Minties g. | Dynniq | EC-2 | Motion | 10 |
| K313 | Antakalnio - Tramvajų g. | Siemens | C900V | Motion | 10 |
| K314 | Antakalnio–Klinikų g. | Siemens | C900V | Motion | 10 |
| K315 | Antakalnio g. perėja prie poliklinikos, Antakalnio–Žolyno g. | Siemens | C900V | Motion | 10 |
| K317 | Antakalnio–Švyturio g. | Siemens | C900V | Motion | 10 |
| K318 | Antakalnio–P. Širvio g. | Siemens | C900V | Motion | 10 |
| K319 | Antakalnio–Tverečiaus | Siemens | C900V | Motion | 10 |
| K320 | Antakalnio–M. K. Oginskio | Siemens | C900V | Motion | 10 |
| K321 | Antakalnio g. perėja ties universitetine ligonine | Siemens | C900V | Motion | 10 |
| K351 | Rinktinės–Olimpiečių–Žvejų g. | Siemens | C900V | Motion | 10 |
| K352 | Rinktinės–A. Juozapavičiaus g. | Siemens | C900V | Motion | 10 |
| K353 | Šeimyniškių–Rinktinės g. | Siemens | C900V | Motion | 10 |
| K354 | Šeimyniškių g. pėsčiųjų perėja ties Slucko g. | Siemens | C900V | Motion | 10 |
| K359 | Rinktinės g.–Ceikinių g. | Siemens | C900V | Motion | 10 |
| K202 | Kalvarijų–A. Juozapavičiaus g. | Siemens | C900V | Motion | 11 |
| K203 | Kalvarijų g.–Šeimyniškių g.–Konstitucijos pr. | Siemens | C900V | Motion | 11 |
| K204 | Kalvarijų–Krokuvos g. | Siemens | C900V | Motion | 11 |
| K205 | Kalvarijų g., pėsčiųjų perėja prie turgaus | Siemens | C900V | Motion | 11 |
| K206 | Kalvarijų–Žalgirio g. | Siemens | C900V | Motion | 11 |
| K207 | Žalgirio–Rinktinės–Tuskulėnų g., Minties–Apkasų g. | Siemens | C900V | Motion | 11 |
| K209 | Kalvarijų g. perėja prie m-klos | Siemens | C900V | Motion | 11 |
| K210 | Kalvarijų–Apkasų g. | Siemens | C900V | Motion | 11 |
| K211 | Kalvarijų–P. Lukšio g. Kalvarijų g. 127 perėja | Siemens | C900V | Motion | 11 |
| K213 | Kalvarijų–Kareivių–Ozo g. | Siemens | C900V | Motion | 11 |
| K214 | Kalvarijų–V. Gerulaičio g. | Siemens | C900V | Motion | 11 |
| K215 | Kareivių–Verkių g. | Siemens | C900V | Motion | 11 |
| K216 | Kareivių–Žirmūnų g. | Siemens | C900V | Motion | 11 |
| K217 | Kareivių–Valakupių tiltas (perėja) | Siemens | C900V | Motion | 11 |
| K218 | Kareivių g., ties degaline | Siemens | C900V | Motion | 11 |
| K219 | Žirmūnų g. 87 perėja | Siemens | C900V | Motion | 11 |
| K740 | Ozo g., ties Licėjumi (ties PC „Ozas“) | Siemens | C900V | Motion | 11 |
| K101 | Žygimantų–A. Goštauto–Vilniaus g. | Siemens | C900V | Motion | 12 |
| K102 | A. Goštauto–A. Vienuolio A. Goštauto–A.Jakšto g. | Siemens | C900V | Motion | 12 |
| K103 | A. Goštauto g.–Vasario 16-osios g. | Siemens | C900V | Motion | 12 |
| K104 | A. Goštauto g.–J. Tumo-Vaižganto g. | Siemens | C800V | Motion | 12 |
| K105 | J. Tumo-Vaižganto–Lukiškių g., J. Tumo-Vaižganto-Savickio g. | Siemens | C800V | Motion | 12 |
| K106 | Gedimino pr.–J. Tumo-Vaižganto g. | Siemens | C900V | Motion | 12 |
| K107 | V. Kudirkos–Pamėnkalnio–J. Jasinskio g. | Siemens | C800V | Motion | 12 |
| K108 | V. Kudirkos–Z. Sierakausko g. | Siemens | C900V | Motion | 12 |
| K109 | V. Kudirkos–M. K. Čiurlionio g. | Siemens | C900V | Motion | 12 |
| K110 | Švitrigailos–J. Basanavičiaus g. | Siemens | C800V | Motion | 12 |
| K111 | Švitrigailos–A. Vivulskio g. | Siemens | C900V | Motion | 12 |
| K112 | Švitrigailos–T. Ševčenkos g. | Siemens | C900V | Motion | 12 |
| K113 | Švitrigailos–Naugarduko g. | Siemens | C900V | Motion | 12 |
| K114 | Švitrigailos–Kauno g. | Siemens | C800V | Motion | 12 |
| K115 | Švitrigailos–Panerių g. | Siemens | C800V | Motion | 12 |
| K116 | Dariaus ir Girėno–Pelesos–Prūsų g. | Siemens | C900V | Motion | 12 |
| K117 | Dariaus ir Girėno–Brolių g. | Siemens | C900V | Motion | 12 |
| K118 | Dariaus ir Girėno–Šaltkalvių–Kapsų g. | Siemens | C900V | Motion | 12 |
| K151 | J. Jasinskio g. perėja (ties mokykla) | Siemens | C900V | Motion | 12 |
| K201 | Kalvarijų–Upės–Žvejų g. | Siemens | C900V | Motion | 12 |

Annex 3 to the Technical Specification

Date:

Object:

PROTOCOL FOR TESTING THE ______________CONTROLLER'S INTERFACING WITH THE SYSTEM VIA THE OCIT PROTOCOL

Verification of the control functions of the _______ controller after the controller has been connected to the System.

Tests and results:

| No | Name of the tests | Notes | Result of the tests |
| --- | --- | --- | --- |
| 1. | Switching signal plans from the System user interface. | Successful switching of plans. Existing signal plans can be switched off or on, switched to yellow flashing and switched back to operational mode. The controller understands the commands for switching signal plans from different levels of System Control |  |
| 2. | Controller shutdown - simulation of power supply error | The test shall be carried out with the communication equipment connected to an uninterruptible power supply. |  |
| 3. | Interactive diagram of sensors in the System. | The intersection controller interactive diagram shall show the occupancy of the sensors in real time. |  |
| 4. | Interactive diagram of signal groups in the System. | The intersection interactive diagram shall show the real-time status of the signal groups. |  |
| 5. | Sensor statistics in the System user interface and statistics module | Sensors connected to the controller are visible in the System user interface and in the statistics module |  |
| 6. | Receipt and display of messages/events from controllers | The controller shall report to the System and the System shall display to the user information, warning, error and critical error messages according to the OCIT protocol specification: - Actual stage fault. - Red lamp fault - Detector fault - Detector ok - Signal plan on - Signal plan off - Mains off - Mains on - Communication fault - Communication ok - Door open - Door closed |  |
| 7. | Change of controller status on the map, | Controller change from central to local mode - the colour of the intersection controller icon changes from green to yellow. Controller changes to yellow flashing mode - the colour of the intersection controller icon changes from green (or yellow) to blue. Controller fault (secondary error - detector or lamp) - the colour of the intersection controller icon changes from green (or yellow) to orange. Controller fault (primary error - red lamp) - the colour of the intersection controller icon changes from green (or yellow) to red. Controller fault (communication error, no communication) - the colour of the intersection controller icon changes from green (or yellow) to pink (purple). Other icon colours according to the fault types and priorities configured in the System. |  |
| 8. | Display of changes in signal plans | A detailed list of signal plan changes for the intersection is displayed. |  |
| 9. | Synchronisation of the controller clock | The controller clock is synchronised with the System Server clock. |  |
| 10. | Controller receives IP address from the System DHCP server | The Host name, the IP address of the DHCP server shall be specified in the controller configuration and the controller shall automatically accept the IP address assigned by the System. |  |

Prepared by:

Test has been performed by:

Annex 4 to the Technical Specification

| Ser. No | Name of the group of intersections | Intersection included in the group |
| --- | --- | --- |
| 1 | Main group 1 EVR K121-K116 | K116, K117, K118, K120, K121, K119 |
| 2 | Group 1-a EVR K121-K119 | K119, K120, K121 |
| 3 | Group 1-b EVR K118-K116 | K116, K117, K118 |
| 4 | Main group 2 EVR K115-K106 | K111, K113, K114, K115, K107, K108, K109, K110, K106, K112 |
| 5 | Group 2-a EVR K115-K111 | K111, K113, K114, K115, K112 |
| 6 | Group 2-b EVR K110-K106 | K106, K107, K108, K109, K110 |
| 7 | Main group 3 EVR K511-K402 | K510, K511, K110, K551, K402, K401 |
| 8 | Group 3-a EVR K511-K551 | K510, K511 K551 |
| 9 | Group 3-b EVR K110-K402 | K110, K402, K401 |
| 10 | Main group 4 EVR K415-K410 | K404, K410, K406, K407, K415, K403, K408 |
| 11 | Group 4-a EVR K415-K404 | K404, K415, K403 |
| 12 | Group 4-b EVR K415-K410 | K410, K406, K407, K408 |
| 13 | Cycle road race | K1033, K1028, K1027, K1029 |
| 14 | K1003_K1005_K1007_K1010_ K1013_ K1014_K1016_K1017_K1018_K1027_ K1029_K1030 | K1030, K1017, K1013, K1027, K1029, K1018, K1010, K1016, K1014, K1007, K1005, K1003 |
| 15 | K101-K116_K151-K156_K201 | K112, K156, K111, K113, K114, K115, K151, K153, K154, K155, K107, K108, K109, K116, K110, K105, K102, K103, K101, K106, K104, K201 |
| 16 | K1059-1060-1061 | K1059, K1061, K1060 |
| 17 | K117-K118 | K117, K118 |
| 18 | K202-K216_K218_K262_K251-K254 | K254, K263, K207, K214, K215, K216, K218, K211, K213, K210, K204, K205, K209, K262, K203, K206, K202 |
| 19 | K213-K211 | K211, K213 |
| 20 | K255-K256 | K255, K256 |
| 21 | K255-K257 | K255, K256, K257 |
| 22 | K301-K304_K501-K503 | K101, K301, K302, K303, K304, K351, K502, K503, K608, K610, K721 |
| 23 | K302-K354 | K353, K354, K359, K352, K302, K351 |
| 24 | K303-K304 | K303, K304 |
| 25 | K306-K321 | K320, K321, K310, K311, K313, K314, K315, K317, K318, K319, K306, K308, K309 |
| 26 | K355_K1001_K1004_K1006_K1015_ K1020_K1023_K1028 | K1023, K1015, K1006, K1001, K1004, K1028, K355 |
| 27 | K401-K410_K415_K451 | K404, K410, K406, K407, K405, K451, K402, K401, K415, K403, K408 |
| 28 | K411-K412 | K411, K412 |
| 29 | K413-K414 | K413, K414 |
| 30 | K501-K556 | K555, K511, K518, K515, K513, K509, K508, K521, K519, K520, K505, K506, K522, K516, K512, K556, K510, K525, K523, K524, K501, K517, K507, K554, K553, K552, K551, K504, K503, K502 |
| 31 | K501-K502-K503 | K501, K502, K503 |
| 32 | K502-K503 | K503, K502 |
| 33 | K601-K610-K721 | K602, K603, K601, K605, K609,K606,K607 K721,K608, K610 |
| 34 | K741-K744 | K742, K744, K741, K743 |
| 35 | K762-K767 | K766, K767, K762, K763, K764, K765 |
| 36 | K781-K786 | K784, K785, K786, K782, K783, K781 |
| 37 | K801-K803 | K801, K802, K803 |
| 38 | K821-K828 | K823, K821, K822, K825, K826, K828, K832, K831, K824 |
| 39 | K829-K830 | K829, K830 |
| 40 | Christmas_running | K302, K303, K304 |
| 41 | Christmas_running_2 | K1087, K301 |
| 42 | Kalvariju_202-262 | K207, K210, K204, K205, K209, K262, K203, K206, K202 |
| 43 | KAZIUKAS | K610, K301, K502, K503 |
| 44 | Konstitucijos_251-254 | K264, K263, K254, K201, K202 |
| 45 | Velomarathon | K302, K351, K303, K304, K301, K202, K502, K503, K610, K102, K103, K101, K106, K104, K701, K201, K721, K608 |
| 46 | Vilnius_marathon-1 | K105, K1087, K505, K506, K102, K103, K101, K106, K104, K701, K201, K721, K608, K504, K503, K202, K610, K501, K302, K351, K303, K304, K301,K502 |
| 47 | Vilnius_ Half Marathon | K352, K506, K505, K105, K104 ,K701, K721, K608 K610, K102, K103, K201, K501, K302, K351, K202, K101, K303, K304, K502, K503 |
| 48 | Vilnius marathon-2 | K605, K609, K606, K607, K107, K108, K109 |
| 49 | We Run marathon | K105, K1087, K102, K103, K101, K106, K104, K701 K501, K303, K304, K202, K201, K302, K351, K610, K301, K502, K503 |

Annex 5 to the Technical Specification

| Monday-Thursday |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Switching on time |  | Type |  | Name of the object | Action |
| 22:00:00 |  | Single intersection |  | K202 | SP4 |
| 22:00:00 |  | Single intersection |  | K259 | SP4 |
| 00:00:00 |  | Single intersection |  | K358 | SP2 |
| 06:00:00 |  | Single intersection |  | K358 | SP1 |
| 15:00:00 |  | Single intersection |  | K358 | SP3 |
| 20:00:00 |  | Single intersection |  | K358 | SP2 |
| 00:00:00 |  | Single intersection |  | K411 | SP2 |
| 06:00:00 |  | Single intersection |  | K411 | SP16 |
| 07:00:00 |  | Single intersection |  | K411 | SP20 |
| 08:00:00 |  | Single intersection |  | K411 | SP16 |
| 13:00:00 |  | Single intersection |  | K411 | SP15 |
| 21:00:00 |  | Single intersection |  | K411 | SP2 |
| 00:00:00 |  | Single intersection |  | K412 | SP2 |
| 06:00:00 |  | Single intersection |  | K412 | SP15 |
| 07:00:00 |  | Single intersection |  | K412 | SP20 |
| 08:00:00 |  | Single intersection |  | K412 | SP15 |
| 15:00:00 |  | Single intersection |  | K412 | SP25 |
| 19:00:00 |  | Single intersection |  | K412 | SP15 |
| 21:00:00 |  | Single intersection |  | K412 | SP2 |
| 00:00:00 |  | Single intersection |  | K702 | SP4 |
| 06:30:00 |  | Single intersection |  | K702 | SP5 |
| 10:00:00 |  | Single intersection |  | K702 | SP6 |
| 20:30:00 |  | Single intersection |  | K702 | SP2 |
| 22:00:00 |  | Single intersection |  | K702 | SP4 |
| 00:00:00 |  | Single intersection |  | K761 | SP4 |
| 06:00:00 |  | Single intersection |  | K761 | SP1 |
| 06:30:00 |  | Single intersection |  | K761 | SP5 |
| 07:08:00 |  | Single intersection |  | K761 | SP6 |
| 07:40:00 |  | Single intersection |  | K761 | SP1 |
| 08:10:00 |  | Single intersection |  | K761 | SP5 |
| 08:20:00 |  | Single intersection |  | K761 | SP1 |
| 10:00:00 |  | Single intersection |  | K761 | SP2 |
| 15:00:00 |  | Single intersection |  | K761 | SP3 |
| 20:00:00 |  | Single intersection |  | K761 | SP2 |
| 22:00:00 |  | Single intersection |  | K761 | SP4 |
| 00:00:00 |  | Single intersection |  | K768 | SP4 |
| 06:00:00 |  | Single intersection |  | K768 | SP10 |
| 06:50:00 |  | Single intersection |  | K768 | SP10 |
| 09:00:00 |  | Single intersection |  | K768 | SP13 |
| 16:00:00 |  | Single intersection |  | K768 | SP13 |
| 19:00:00 |  | Single intersection |  | K768 | SP13 |
| 22:00:00 |  | Single intersection |  | K768 | SP4 |
| 00:00:00 |  | Single intersection |  | K861 | SP4 |
| 06:00:00 |  | Single intersection |  | K861 | SP2 |
| 20:00:00 |  | Single intersection |  | K861 | SP4 |
| 00:00:00 |  | Single intersection |  | K1008 | SP2 |
| 06:00:00 |  | Single intersection |  | K1008 | SP1 |
| 21:00:00 |  | Single intersection |  | K1008 | SP2 |
| 00:00:00 |  | Single intersection |  | K1020 | SP6 |
| 07:00:00 |  | Single intersection |  | K1020 | SP5 |
| 08:20:00 |  | Single intersection |  | K1020 | SP6 |
| 16:15:00 |  | Single intersection |  | K1020 | SP5 |
| 18:45:00 |  | Single intersection |  | K1020 | SP6 |
| 00:00:00 |  | Single intersection |  | K1024 | SP4 |
| 06:00:00 |  | Single intersection |  | K1024 | SP1 |
| 09:30:00 |  | Single intersection |  | K1024 | SP2 |
| 16:00:00 |  | Single intersection |  | K1024 | SP3 |
| 20:00:00 |  | Single intersection |  | K1024 | SP2 |
| 22:00:00 |  | Single intersection |  | K1024 | SP4 |
| 00:00:00 |  | Single intersection |  | K1035 | SP4 |
| 00:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 06:15:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP1 |
| 09:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 15:30:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP3 |
| 19:30:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 22:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 00:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 06:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP1 |
| 10:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP3 |
| 20:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 00:00:00 |  | Group |  | K302-K354 | SP2 |
| 06:45:00 |  | Group |  | K302-K354 | SP1 |
| 09:30:00 |  | Group |  | K302-K354 | SP3 |
| 15:00:00 |  | Group |  | K302-K354 | SP3 |
| 20:00:00 |  | Group |  | K302-K354 | SP2 |
| 00:00:00 |  | Group |  | K306-K321 | SP2 |
| 06:00:00 |  | Group |  | K306-K321 | SP1 |
| 10:00:00 |  | Group |  | K306-K321 | SP3 |
| 20:00:00 |  | Group |  | K306-K321 | SP2 |
| 00:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 06:30:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP2 |
| 09:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 14:30:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP2 |
| 19:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 00:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 06:00:00 |  | Group |  | K401-K410_K415_K451 | SP1 |
| 10:00:00 |  | Group |  | K401-K410_K415_K451 | SP3 |
| 20:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 00:00:00 |  | Group |  | K413-K414 | SP8 |
| 06:00:00 |  | Group |  | K413-K414 | SP1 |
| 09:30:00 |  | Group |  | K413-K414 | SP6 |
| 15:30:00 |  | Group |  | K413-K414 | SP1 |
| 19:30:00 |  | Group |  | K413-K414 | SP6 |
| 22:00:00 |  | Group |  | K413-K414 | SP8 |
| 00:00:00 |  | Group |  | K501-K556 | SP2 |
| 06:00:00 |  | Group |  | K501-K556 | SP1 |
| 10:00:00 |  | Group |  | K501-K556 | SP3 |
| 20:00:00 |  | Group |  | K501-K556 | SP2 |
| 00:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 06:00:00 |  | Group |  | K601-K610-K721 | SP1 |
| 10:00:00 |  | Group |  | K601-K610-K721 | SP3 |
| 20:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 00:00:00 |  | Group |  | K701 | SP4 |
| 06:30:00 |  | Group |  | K701 | SP1 |
| 10:00:00 |  | Group |  | K701 | SP3 |
| 20:30:00 |  | Group |  | K701 | SP2 |
| 22:00:00 |  | Group |  | K701 | SP4 |
| 00:00:00 |  | Group |  | K741-K744 | SP2 |
| 06:00:00 |  | Group |  | K741-K744 | SP1 |
| 11:00:00 |  | Group |  | K741-K744 | SP3 |
| 21:00:00 |  | Group |  | K741-K744 | SP2 |
| 00:00:00 |  | Group |  | K762-K767 | SP2 |
| 06:00:00 |  | Group |  | K762-K767 | SP1 |
| 06:53:00 |  | Group |  | K762-K767 | SP10 |
| 09:00:00 |  | Group |  | K762-K767 | SP1 |
| 10:00:00 |  | Group |  | K762-K767 | SP3 |
| 16:00:00 |  | Group |  | K762-K767 | SP13 |
| 18:30:00 |  | Group |  | K762-K767 | SP3 |
| 22:00:00 |  | Group |  | K762-K767 | SP2 |
| 00:00:00 |  | Group |  | K781-K786 | SP2 |
| 06:30:00 |  | Group |  | K781-K786 | SP10 |
| 09:00:00 |  | Group |  | K781-K786 | SP3 |
| 16:00:00 |  | Group |  | K781-K786 | SP13 |
| 18:30:00 |  | Group |  | K781-K786 | SP3 |
| 22:00:00 |  | Group |  | K781-K786 | SP2 |
| 00:00:00 |  | Group |  | K801-K803 | SP2 |
| 06:00:00 |  | Group |  | K801-K803 | SP11 |
| 11:00:00 |  | Group |  | K801-K803 | SP8 |
| 15:00:00 |  | Group |  | K801-K803 | SP10 |
| 21:00:00 |  | Group |  | K801-K803 | SP3 |
| 22:00:00 |  | Group |  | K801-K803 | SP2 |
| 00:00:00 |  | Group |  | K821-K828 | SP2 |
| 06:00:00 |  | Group |  | K821-K828 | SP3 |
| 06:45:00 |  | Group |  | K821-K828 | SP16 |
| 08:45:00 |  | Group |  | K821-K828 | SP3 |
| 16:00:00 |  | Group |  | K821-K828 | SP16 |
| 19:00:00 |  | Group |  | K821-K828 | SP3 |
| 22:00:00 |  | Group |  | K821-K828 | SP2 |
| 00:00:00 |  | Group |  | K829-K830 | SP2 |
| 06:00:00 |  | Group |  | K829-K830 | SP1 |
| 14:00:00 |  | Group |  | K829-K830 | SP3 |
| 22:00:00 |  | Group |  | K829-K830 | SP2 |
| Friday |  |  |  |  |  |
| Switching on time |  | Type |  | Name of the object | Action |
| 00:00:00 |  | Single intersection |  | K358 | SP2 |
| 06:00:00 |  | Single intersection |  | K358 | SP1 |
| 15:00:00 |  | Single intersection |  | K358 | SP3 |
| 20:00:00 |  | Single intersection |  | K358 | SP2 |
| 00:00:00 |  | Single intersection |  | K411 | SP2 |
| 06:00:00 |  | Single intersection |  | K411 | SP16 |
| 07:00:00 |  | Single intersection |  | K411 | SP20 |
| 08:00:00 |  | Single intersection |  | K411 | SP16 |
| 13:00:00 |  | Single intersection |  | K411 | SP15 |
| 21:00:00 |  | Single intersection |  | K411 | SP2 |
| 00:00:00 |  | Single intersection |  | K412 | SP2 |
| 06:00:00 |  | Single intersection |  | K412 | SP15 |
| 07:00:00 |  | Single intersection |  | K412 | SP20 |
| 08:00:00 |  | Single intersection |  | K412 | SP15 |
| 14:00:00 |  | Single intersection |  | K412 | SP25 |
| 19:00:00 |  | Single intersection |  | K412 | SP15 |
| 21:00:00 |  | Single intersection |  | K412 | SP2 |
| 00:00:00 |  | Single intersection |  | K702 | SP4 |
| 06:30:00 |  | Single intersection |  | K702 | SP5 |
| 10:00:00 |  | Single intersection |  | K702 | SP6 |
| 20:30:00 |  | Single intersection |  | K702 | SP2 |
| 22:00:00 |  | Single intersection |  | K702 | SP4 |
| 00:00:00 |  | Single intersection |  | K761 | SP4 |
| 06:00:00 |  | Single intersection |  | K761 | SP1 |
| 06:30:00 |  | Single intersection |  | K761 | SP5 |
| 07:08:00 |  | Single intersection |  | K761 | SP6 |
| 07:40:00 |  | Single intersection |  | K761 | SP1 |
| 08:10:00 |  | Single intersection |  | K761 | SP5 |
| 08:20:00 |  | Single intersection |  | K761 | SP1 |
| 10:00:00 |  | Single intersection |  | K761 | SP2 |
| 14:00:00 |  | Single intersection |  | K761 | SP3 |
| 20:00:00 |  | Single intersection |  | K761 | SP2 |
| 22:00:00 |  | Single intersection |  | K761 | SP4 |
| 00:00:00 |  | Single intersection |  | K768 | SP4 |
| 06:00:00 |  | Single intersection |  | K768 | SP10 |
| 06:50:00 |  | Single intersection |  | K768 | SP10 |
| 09:00:00 |  | Single intersection |  | K768 | SP13 |
| 15:00:00 |  | Single intersection |  | K768 | SP13 |
| 19:00:00 |  | Single intersection |  | K768 | SP13 |
| 22:00:00 |  | Single intersection |  | K768 | SP4 |
| 00:00:00 |  | Single intersection |  | K861 | SP4 |
| 06:00:00 |  | Single intersection |  | K861 | SP2 |
| 20:00:00 |  | Single intersection |  | K861 | SP4 |
| 00:00:00 |  | Single intersection |  | K1008 | SP2 |
| 06:00:00 |  | Single intersection |  | K1008 | SP1 |
| 21:00:00 |  | Single intersection |  | K1008 | SP2 |
| 00:00:00 |  | Single intersection |  | K1020 | SP6 |
| 07:00:00 |  | Single intersection |  | K1020 | SP5 |
| 08:20:00 |  | Single intersection |  | K1020 | SP6 |
| 16:00:00 |  | Single intersection |  | K1020 | SP5 |
| 18:45:00 |  | Single intersection |  | K1020 | SP6 |
| 00:00:00 |  | Single intersection |  | K1024 | SP4 |
| 06:00:00 |  | Single intersection |  | K1024 | SP1 |
| 09:30:00 |  | Single intersection |  | K1024 | SP2 |
| 15:00:00 |  | Single intersection |  | K1024 | SP3 |
| 20:00:00 |  | Single intersection |  | K1024 | SP2 |
| 22:00:00 |  | Single intersection |  | K1024 | SP4 |
| 00:00:00 |  | Single intersection |  | K1035 | SP4 |
| 00:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 06:15:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP1 |
| 09:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 14:30:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP3 |
| 19:30:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 22:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 00:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 06:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP1 |
| 10:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP3 |
| 20:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 00:00:00 |  | Group |  | K302-K354 | SP2 |
| 06:45:00 |  | Group |  | K302-K354 | SP1 |
| 09:30:00 |  | Group |  | K302-K354 | SP3 |
| 15:00:00 |  | Group |  | K302-K354 | SP3 |
| 20:00:00 |  | Group |  | K302-K354 | SP2 |
| 00:00:00 |  | Group |  | K306-K321 | SP2 |
| 06:00:00 |  | Group |  | K306-K321 | SP1 |
| 10:00:00 |  | Group |  | K306-K321 | SP3 |
| 20:00:00 |  | Group |  | K306-K321 | SP2 |
| 00:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 06:30:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP2 |
| 09:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 14:30:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP2 |
| 19:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 00:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 06:00:00 |  | Group |  | K401-K410_K415_K451 | SP1 |
| 10:00:00 |  | Group |  | K401-K410_K415_K451 | SP3 |
| 20:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 00:00:00 |  | Group |  | K413-K414 | SP8 |
| 06:00:00 |  | Group |  | K413-K414 | SP1 |
| 09:30:00 |  | Group |  | K413-K414 | SP6 |
| 14:30:00 |  | Group |  | K413-K414 | SP1 |
| 19:30:00 |  | Group |  | K413-K414 | SP6 |
| 22:00:00 |  | Group |  | K413-K414 | SP8 |
| 00:00:00 |  | Group |  | K501-K556 | SP2 |
| 06:00:00 |  | Group |  | K501-K556 | SP1 |
| 10:00:00 |  | Group |  | K501-K556 | SP3 |
| 20:00:00 |  | Group |  | K501-K556 | SP2 |
| 00:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 06:00:00 |  | Group |  | K601-K610-K721 | SP1 |
| 10:00:00 |  | Group |  | K601-K610-K721 | SP3 |
| 20:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 00:00:00 |  | Group |  | K701 | SP4 |
| 06:30:00 |  | Group |  | K701 | SP1 |
| 10:00:00 |  | Group |  | K701 | SP3 |
| 20:30:00 |  | Group |  | K701 | SP2 |
| 22:00:00 |  | Group |  | K701 | SP4 |
| 00:00:00 |  | Group |  | K741-K744 | SP2 |
| 06:00:00 |  | Group |  | K741-K744 | SP1 |
| 11:00:00 |  | Group |  | K741-K744 | SP3 |
| 21:00:00 |  | Group |  | K741-K744 | SP2 |
| 00:00:00 |  | Group |  | K762-K767 | SP2 |
| 06:00:00 |  | Group |  | K762-K767 | SP1 |
| 06:53:00 |  | Group |  | K762-K767 | SP10 |
| 09:00:00 |  | Group |  | K762-K767 | SP1 |
| 10:00:00 |  | Group |  | K762-K767 | SP3 |
| 15:00:00 |  | Group |  | K762-K767 | SP13 |
| 18:30:00 |  | Group |  | K762-K767 | SP3 |
| 22:00:00 |  | Group |  | K762-K767 | SP2 |
| 00:00:00 |  | Group |  | K781-K786 | SP2 |
| 06:30:00 |  | Group |  | K781-K786 | SP10 |
| 09:00:00 |  | Group |  | K781-K786 | SP3 |
| 16:00:00 |  | Group |  | K781-K786 | SP13 |
| 18:30:00 |  | Group |  | K781-K786 | SP3 |
| 22:00:00 |  | Group |  | K781-K786 | SP2 |
| 00:00:00 |  | Group |  | K801-K803 | SP2 |
| 06:00:00 |  | Group |  | K801-K803 | SP11 |
| 11:00:00 |  | Group |  | K801-K803 | SP8 |
| 14:30:00 |  | Group |  | K801-K803 | SP10 |
| 21:00:00 |  | Group |  | K801-K803 | SP3 |
| 22:00:00 |  | Group |  | K801-K803 | SP2 |
| 00:00:00 |  | Group |  | K821-K828 | SP2 |
| 06:00:00 |  | Group |  | K821-K828 | SP3 |
| 06:45:00 |  | Group |  | K821-K828 | SP16 |
| 08:45:00 |  | Group |  | K821-K828 | SP3 |
| 15:00:00 |  | Group |  | K821-K828 | SP16 |
| 19:00:00 |  | Group |  | K821-K828 | SP3 |
| 22:00:00 |  | Group |  | K821-K828 | SP2 |
| 00:00:00 |  | Group |  | K829-K830 | SP2 |
| 06:00:00 |  | Group |  | K829-K830 | SP1 |
| 14:00:00 |  | Group |  | K829-K830 | SP3 |
| 22:00:00 |  | Group |  | K829-K830 | SP2 |
| Saturday |  |  |  |  |  |
| Switching on time | Type |  | Name of the object |  | Action |
| 00:00:00 | Single intersection |  | K358 |  | SP2 |
| 00:00:00 | Single intersection |  | K411 |  | SP2 |
| 06:00:00 | Single intersection |  | K411 |  | SP1 |
| 21:00:00 | Single intersection |  | K411 |  | SP2 |
| 00:00:00 | Single intersection |  | K412 |  | SP2 |
| 06:00:00 | Single intersection |  | K412 |  | SP1 |
| 21:00:00 | Single intersection |  | K412 |  | SP2 |
| 00:00:00 | Single intersection |  | K702 |  | SP4 |
| 07:00:00 | Single intersection |  | K702 |  | SP2 |
| 22:00:00 | Single intersection |  | K702 |  | SP4 |
| 00:00:00 | Single intersection |  | K761 |  | SP4 |
| 06:00:00 | Single intersection |  | K761 |  | SP2 |
| 13:00:00 | Single intersection |  | K761 |  | SP10 |
| 16:00:00 | Single intersection |  | K761 |  | SP2 |
| 22:00:00 | Single intersection |  | K761 |  | SP4 |
| 00:00:00 | Single intersection |  | K768 |  | SP4 |
| 06:00:00 | Single intersection |  | K768 |  | SP10 |
| 22:00:00 | Single intersection |  | K768 |  | SP4 |
| 00:00:00 | Single intersection |  | K861 |  | SP4 |
| 00:00:00 | Single intersection |  | K1008 |  | SP2 |
| 06:00:00 | Single intersection |  | K1008 |  | SP1 |
| 21:00:00 | Single intersection |  | K1008 |  | SP2 |
| 00:00:00 | Single intersection |  | K1020 |  | SP6 |
| 00:00:00 | Single intersection |  | K1024 |  | SP4 |
| 09:30:00 | Single intersection |  | K1024 |  | SP2 |
| 18:00:00 | Single intersection |  | K1024 |  | SP4 |
| 00:00:00 | Single intersection |  | K1035 |  | SP4 |
| 00:00:00 | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 |  | SP4 |
| 06:00:00 | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 |  | SP2 |
| 22:00:00 | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 |  | SP4 |
| 00:00:00 | Group |  | K101-K116_K151-K156_K201 |  | SP2 |
| 00:00:00 | Group |  | K302-K354 |  | SP2 |
| 00:00:00 | Group |  | K306-K321 |  | SP2 |
| 00:00:00 | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 |  | SP4 |
| 00:00:00 | Group |  | K401-K410_K415_K451 |  | SP2 |
| 00:00:00 | Group |  | K413-K414 |  | SP8 |
| 07:00:00 | Group |  | K413-K414 |  | SP9 |
| 10:00:00 | Group |  | K413-K414 |  | SP6 |
| 18:00:00 | Group |  | K413-K414 |  | SP9 |
| 22:00:00 | Group |  | K413-K414 |  | SP8 |
| 00:00:00 | Group |  | K501-K556 |  | SP2 |
| 00:00:00 | Group |  | K601-K610-K721 |  | SP2 |
| 00:00:00 | Group |  | K701 |  | SP4 |
| 07:00:00 | Group |  | K701 |  | SP2 |
| 22:00:00 | Group |  | K701 |  | SP4 |
| 00:00:00 | Group |  | K741-K744 |  | SP2 |
| 00:00:00 | Group |  | K762-K767 |  | SP2 |
| 06:00:00 | Group |  | K762-K767 |  | SP1 |
| 10:00:00 | Group |  | K762-K767 |  | SP3 |
| 22:00:00 | Group |  | K762-K767 |  | SP2 |
| 00:00:00 | Group |  | K781-K786 |  | SP2 |
| 06:00:00 | Group |  | K781-K786 |  | SP3 |
| 22:00:00 | Group |  | K781-K786 |  | SP2 |
| 00:00:00 | Group |  | K801-K803 |  | SP2 |
| 06:00:00 | Group |  | K801-K803 |  | SP1 |
| 10:00:00 | Group |  | K801-K803 |  | SP3 |
| 21:00:00 | Group |  | K801-K803 |  | SP2 |
| 00:00:00 | Group |  | K821-K828 |  | SP2 |
| 07:00:00 | Group |  | K821-K828 |  | SP3 |
| 21:00:00 | Group |  | K821-K828 |  | SP2 |
| 00:00:00 | Group |  | K829-K830 |  | SP2 |
| Sunday |  |  |  |  |  |
| Switching on time |  | Type |  | Name of the object | Action |
| 00:00:00 |  | Single intersection |  | K358 | SP2 |
| 00:00:00 |  | Single intersection |  | K411 | SP2 |
| 06:00:00 |  | Single intersection |  | K411 | SP1 |
| 21:00:00 |  | Single intersection |  | K411 | SP2 |
| 00:00:00 |  | Single intersection |  | K412 | SP2 |
| 06:00:00 |  | Single intersection |  | K412 | SP1 |
| 21:00:00 |  | Single intersection |  | K412 | SP2 |
| 00:00:00 |  | Single intersection |  | K702 | SP4 |
| 07:00:00 |  | Single intersection |  | K702 | SP2 |
| 22:00:00 |  | Single intersection |  | K702 | SP4 |
| 00:00:00 |  | Single intersection |  | K761 | SP4 |
| 06:00:00 |  | Single intersection |  | K761 | SP2 |
| 22:00:00 |  | Single intersection |  | K761 | SP4 |
| 00:00:00 |  | Single intersection |  | K768 | SP4 |
| 06:00:00 |  | Single intersection |  | K768 | SP10 |
| 22:00:00 |  | Single intersection |  | K768 | SP4 |
| 00:00:00 |  | Single intersection |  | K861 | SP4 |
| 00:00:00 |  | Single intersection |  | K1008 | SP2 |
| 06:00:00 |  | Single intersection |  | K1008 | SP1 |
| 21:00:00 |  | Single intersection |  | K1008 | SP2 |
| 00:00:00 |  | Single intersection |  | K1020 | SP6 |
| 00:00:00 |  | Single intersection |  | K1024 | SP4 |
| 15:00:00 |  | Single intersection |  | K1024 | SP1 |
| 21:00:00 |  | Single intersection |  | K1024 | SP4 |
| 00:00:00 |  | Single intersection |  | K1035 | SP4 |
| 00:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 06:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 22:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 00:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 00:00:00 |  | Group |  | K302-K354 | SP2 |
| 00:00:00 |  | Group |  | K306-K321 | SP2 |
| 00:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 00:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 00:00:00 |  | Group |  | K413-K414 | SP8 |
| 07:00:00 |  | Group |  | K413-K414 | SP9 |
| 18:00:00 |  | Group |  | K413-K414 | SP1 |
| 21:00:00 |  | Group |  | K413-K414 | SP9 |
| 22:00:00 |  | Group |  | K413-K414 | SP8 |
| 00:00:00 |  | Group |  | K501-K556 | SP2 |
| 00:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 00:00:00 |  | Group |  | K701 | SP4 |
| 07:00:00 |  | Group |  | K701 | SP2 |
| 22:00:00 |  | Group |  | K701 | SP4 |
| 00:00:00 |  | Group |  | K741-K744 | SP2 |
| 00:00:00 |  | Group |  | K762-K767 | SP2 |
| 06:00:00 |  | Group |  | K762-K767 | SP1 |
| 10:00:00 |  | Group |  | K762-K767 | SP3 |
| 22:00:00 |  | Group |  | K762-K767 | SP2 |
| 00:00:00 |  | Group |  | K781-K786 | SP2 |
| 06:00:00 |  | Group |  | K781-K786 | SP3 |
| 22:00:00 |  | Group |  | K781-K786 | SP2 |
| 00:00:00 |  | Group |  | K801-K803 | SP2 |
| 06:00:00 |  | Group |  | K801-K803 | SP1 |
| 10:00:00 |  | Group |  | K801-K803 | SP3 |
| 21:00:00 |  | Group |  | K801-K803 | SP2 |
| 00:00:00 |  | Group |  | K821-K828 | SP2 |
| 07:00:00 |  | Group |  | K821-K828 | SP3 |
| 21:00:00 |  | Group |  | K821-K828 | SP2 |
| 00:00:00 |  | Group |  | K829-K830 | SP2 |
| Pre-holiday (similar to Friday) |  |  |  |  |  |
| Switching on time |  | Type |  | Name of the object | Action |
| 00:00:00 |  | Single intersection |  | K1080 | SP9 |
| 06:30:00 |  | Single intersection |  | K1080 | SP10 |
| 14:30:00 |  | Single intersection |  | K1080 | SP12 |
| 18:00:00 |  | Single intersection |  | K1080 | SP10 |
| 21:00:00 |  | Single intersection |  | K1080 | SP9 |
| 00:00:00 |  | Single intersection |  | K1083 | SP2 |
| 06:00:00 |  | Single intersection |  | K1083 | SP1 |
| 09:30:00 |  | Single intersection |  | K1083 | SP4 |
| 15:00:00 |  | Single intersection |  | K1083 | SP3 |
| 20:00:00 |  | Single intersection |  | K1083 | SP4 |
| 22:00:00 |  | Single intersection |  | K1083 | SP2 |
| 00:00:00 |  | Single intersection |  | K1084 | SP1 |
| 06:00:00 |  | Single intersection |  | K1084 | SP2 |
| 06:50:00 |  | Single intersection |  | K1084 | SP4 |
| 09:00:00 |  | Single intersection |  | K1084 | SP2 |
| 10:00:00 |  | Single intersection |  | K1084 | SP3 |
| 15:00:00 |  | Single intersection |  | K1084 | SP5 |
| 18:30:00 |  | Single intersection |  | K1084 | SP3 |
| 22:00:00 |  | Single intersection |  | K1084 | SP1 |
| 00:00:00 |  | Single intersection |  | K1085 | SP2 |
| 06:00:00 |  | Single intersection |  | K1085 | SP3 |
| 09:30:00 |  | Single intersection |  | K1085 | SP1 |
| 15:00:00 |  | Single intersection |  | K1085 | SP4 |
| 20:00:00 |  | Single intersection |  | K1085 | SP1 |
| 22:00:00 |  | Single intersection |  | K1085 | SP2 |
| 00:00:00 |  | Single intersection |  | K1086 | SP2 |
| 06:00:00 |  | Single intersection |  | K1086 | SP3 |
| 09:30:00 |  | Single intersection |  | K1086 | SP1 |
| 15:00:00 |  | Single intersection |  | K1086 | SP4 |
| 20:00:00 |  | Single intersection |  | K1086 | SP1 |
| 22:00:00 |  | Single intersection |  | K1086 | SP2 |
| 00:00:00 |  | Single intersection |  | K301 | SP15 |
| 03:55:00 |  | Single intersection |  | K301 | SP16 |
| 19:05:00 |  | Single intersection |  | K301 | SP15 |
| 00:00:00 |  | Single intersection |  | K358 | SP2 |
| 06:00:00 |  | Single intersection |  | K358 | SP1 |
| 15:00:00 |  | Single intersection |  | K358 | SP3 |
| 20:00:00 |  | Single intersection |  | K358 | SP2 |
| 00:00:00 |  | Single intersection |  | K409 | SP1 |
| 06:30:00 |  | Single intersection |  | K409 | SP2 |
| 07:00:00 |  | Single intersection |  | K409 | SP3 |
| 09:00:00 |  | Single intersection |  | K409 | SP2 |
| 15:45:00 |  | Single intersection |  | K409 | SP3 |
| 18:00:00 |  | Single intersection |  | K409 | SP2 |
| 20:00:00 |  | Single intersection |  | K409 | SP1 |
| 00:00:00 |  | Single intersection |  | K411 | SP2 |
| 06:00:00 |  | Single intersection |  | K411 | SP15 |
| 21:00:00 |  | Single intersection |  | K411 | SP2 |
| 00:00:00 |  | Single intersection |  | K412 | SP2 |
| 06:00:00 |  | Single intersection |  | K412 | SP15 |
| 14:00:00 |  | Single intersection |  | K412 | SP25 |
| 19:00:00 |  | Single intersection |  | K412 | SP15 |
| 21:00:00 |  | Single intersection |  | K412 | SP2 |
| 00:00:00 |  | Single intersection |  | K702 | SP2 |
| 06:30:00 |  | Single intersection |  | K702 | SP5 |
| 10:00:00 |  | Single intersection |  | K702 | SP6 |
| 20:30:00 |  | Single intersection |  | K702 | SP2 |
| 00:00:00 |  | Single intersection |  | K761 | SP4 |
| 06:00:00 |  | Single intersection |  | K761 | SP1 |
| 06:30:00 |  | Single intersection |  | K761 | SP5 |
| 07:08:00 |  | Single intersection |  | K761 | SP6 |
| 07:40:00 |  | Single intersection |  | K761 | SP1 |
| 08:10:00 |  | Single intersection |  | K761 | SP5 |
| 08:20:00 |  | Single intersection |  | K761 | SP1 |
| 10:00:00 |  | Single intersection |  | K761 | SP2 |
| 14:00:00 |  | Single intersection |  | K761 | SP3 |
| 20:00:00 |  | Single intersection |  | K761 | SP2 |
| 22:00:00 |  | Single intersection |  | K761 | SP4 |
| 00:00:00 |  | Single intersection |  | K768 | SP4 |
| 06:00:00 |  | Single intersection |  | K768 | SP10 |
| 06:50:00 |  | Single intersection |  | K768 | SP10 |
| 09:00:00 |  | Single intersection |  | K768 | SP13 |
| 15:00:00 |  | Single intersection |  | K768 | SP13 |
| 19:00:00 |  | Single intersection |  | K768 | SP13 |
| 22:00:00 |  | Single intersection |  | K768 | SP4 |
| 00:00:00 |  | Single intersection |  | K861 | SP4 |
| 06:00:00 |  | Single intersection |  | K861 | SP2 |
| 20:00:00 |  | Single intersection |  | K861 | SP4 |
| 00:00:00 |  | Single intersection |  | K1008 | SP2 |
| 06:00:00 |  | Single intersection |  | K1008 | SP1 |
| 21:00:00 |  | Single intersection |  | K1008 | SP2 |
| 00:00:00 |  | Single intersection |  | K1009 | SP4 |
| 06:00:00 |  | Single intersection |  | K1009 | SP5 |
| 06:45:00 |  | Single intersection |  | K1009 | SP3 |
| 08:20:00 |  | Single intersection |  | K1009 | SP7 |
| 08:50:00 |  | Single intersection |  | K1009 | SP11 |
| 10:00:00 |  | Single intersection |  | K1009 | SP13 |
| 15:00:00 |  | Single intersection |  | K1009 | SP11 |
| 20:00:00 |  | Single intersection |  | K1009 | SP5 |
| 22:00:00 |  | Single intersection |  | K1009 | SP4 |
| 00:00:00 |  | Single intersection |  | K1024 | SP4 |
| 06:00:00 |  | Single intersection |  | K1024 | SP1 |
| 09:30:00 |  | Single intersection |  | K1024 | SP2 |
| 15:00:00 |  | Single intersection |  | K1024 | SP3 |
| 20:00:00 |  | Single intersection |  | K1024 | SP2 |
| 22:00:00 |  | Single intersection |  | K1024 | SP4 |
| 00:00:00 |  | Single intersection |  | K1026 | SP13 |
| 06:30:00 |  | Single intersection |  | K1026 | SP12 |
| 07:00:00 |  | Single intersection |  | K1026 | SP14 |
| 08:30:00 |  | Single intersection |  | K1026 | SP13 |
| 15:30:00 |  | Single intersection |  | K1026 | SP14 |
| 19:00:00 |  | Single intersection |  | K1026 | SP13 |
| 00:00:00 |  | Single intersection |  | K1035 | SP4 |
| 00:00:00 |  | Single intersection |  | K1045 | SP9 |
| 06:00:00 |  | Single intersection |  | K1045 | SP11 |
| 09:30:00 |  | Single intersection |  | K1045 | SP10 |
| 15:00:00 |  | Single intersection |  | K1045 | SP12 |
| 20:00:00 |  | Single intersection |  | K1045 | SP10 |
| 22:00:00 |  | Single intersection |  | K1045 | SP9 |
| 00:00:00 |  | Single intersection |  | K1051 | SP5 |
| 06:00:00 |  | Single intersection |  | K1051 | SP2 |
| 09:30:00 |  | Single intersection |  | K1051 | SP3 |
| 14:30:00 |  | Single intersection |  | K1051 | SP4 |
| 19:30:00 |  | Single intersection |  | K1051 | SP3 |
| 22:00:00 |  | Single intersection |  | K1051 | SP5 |
| 00:00:00 |  | Single intersection |  | K1055 | SP11 |
| 06:00:00 |  | Single intersection |  | K1055 | SP12 |
| 06:30:00 |  | Single intersection |  | K1055 | SP5 |
| 09:20:00 |  | Single intersection |  | K1055 | SP12 |
| 13:30:00 |  | Single intersection |  | K1055 | SP14 |
| 20:00:00 |  | Single intersection |  | K1055 | SP11 |
| 22:00:00 |  | Single intersection |  | K1055 | SP11 |
| 00:00:00 |  | Single intersection |  | K1056 | SP2 |
| 06:00:00 |  | Single intersection |  | K1056 | SP6 |
| 06:30:00 |  | Single intersection |  | K1056 | SP5 |
| 09:20:00 |  | Single intersection |  | K1056 | SP6 |
| 13:30:00 |  | Single intersection |  | K1056 | SP8 |
| 20:00:00 |  | Single intersection |  | K1056 | SP2 |
| 22:00:00 |  | Single intersection |  | K1056 | SP2 |
| 00:00:00 |  | Single intersection |  | K1063 | SP2 |
| 06:00:00 |  | Single intersection |  | K1063 | SP16 |
| 06:30:00 |  | Single intersection |  | K1063 | SP12 |
| 09:20:00 |  | Single intersection |  | K1063 | SP16 |
| 13:30:00 |  | Single intersection |  | K1063 | SP13 |
| 20:00:00 |  | Single intersection |  | K1063 | SP2 |
| 22:00:00 |  | Single intersection |  | K1063 | SP2 |
| 00:00:00 |  | Single intersection |  | K1066 | SP1 |
| 15:00:00 |  | Single intersection |  | K1066 | SP3 |
| 18:30:00 |  | Single intersection |  | K1066 | SP1 |
| 00:00:00 |  | Single intersection |  | K1074 | SP2 |
| 06:00:00 |  | Single intersection |  | K1074 | SP4 |
| 09:30:00 |  | Single intersection |  | K1074 | SP3 |
| 15:00:00 |  | Single intersection |  | K1074 | SP5 |
| 18:00:00 |  | Single intersection |  | K1074 | SP2 |
| 20:00:00 |  | Single intersection |  | K1074 | SP3 |
| 00:00:00 |  | Single intersection |  | K1097 | SP5 |
| 06:30:00 |  | Single intersection |  | K1097 | SP2 |
| 07:00:00 |  | Single intersection |  | K1097 | SP4 |
| 08:30:00 |  | Single intersection |  | K1097 | SP3 |
| 15:30:00 |  | Single intersection |  | K1097 | SP9 |
| 19:00:00 |  | Single intersection |  | K1097 | SP3 |
| 20:00:00 |  | Single intersection |  | K1097 | SP2 |
| 00:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 06:15:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP1 |
| 09:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 14:30:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP3 |
| 19:30:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 22:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 00:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 06:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP1 |
| 10:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP3 |
| 20:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 00:00:00 |  | Group |  | K1059-1060-1061 | SP5 |
| 06:00:00 |  | Group |  | K1059-1060-1061 | SP7 |
| 09:00:00 |  | Group |  | K1059-1060-1061 | SP6 |
| 15:00:00 |  | Group |  | K1059-1060-1061 | SP8 |
| 19:00:00 |  | Group |  | K1059-1060-1061 | SP6 |
| 22:00:00 |  | Group |  | K1059-1060-1061 | SP5 |
| 00:00:00 |  | Group |  | K302-K354 | SP2 |
| 06:45:00 |  | Group |  | K302-K354 | SP1 |
| 09:30:00 |  | Group |  | K302-K354 | SP3 |
| 15:00:00 |  | Group |  | K302-K354 | SP3 |
| 20:00:00 |  | Group |  | K302-K354 | SP2 |
| 00:00:00 |  | Group |  | K306-K321 | SP2 |
| 06:00:00 |  | Group |  | K306-K321 | SP1 |
| 10:00:00 |  | Group |  | K306-K321 | SP3 |
| 20:00:00 |  | Group |  | K306-K321 | SP2 |
| 00:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 06:30:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP2 |
| 09:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 14:30:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP2 |
| 19:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 00:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 06:00:00 |  | Group |  | K401-K410_K415_K451 | SP1 |
| 10:00:00 |  | Group |  | K401-K410_K415_K451 | SP3 |
| 20:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 00:00:00 |  | Group |  | K413-K414 | SP8 |
| 06:00:00 |  | Group |  | K413-K414 | SP1 |
| 09:30:00 |  | Group |  | K413-K414 | SP6 |
| 14:30:00 |  | Group |  | K413-K414 | SP1 |
| 19:30:00 |  | Group |  | K413-K414 | SP6 |
| 22:00:00 |  | Group |  | K413-K414 | SP8 |
| 00:00:00 |  | Group |  | K501-K556 | SP2 |
| 06:00:00 |  | Group |  | K501-K556 | SP1 |
| 10:00:00 |  | Group |  | K501-K556 | SP3 |
| 20:00:00 |  | Group |  | K501-K556 | SP2 |
| 00:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 06:00:00 |  | Group |  | K601-K610-K721 | SP1 |
| 10:00:00 |  | Group |  | K601-K610-K721 | SP3 |
| 20:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 00:00:00 |  | Group |  | K701 | SP2 |
| 06:30:00 |  | Group |  | K701 | SP1 |
| 10:00:00 |  | Group |  | K701 | SP3 |
| 20:00:00 |  | Group |  | K701 | SP2 |
| 00:00:00 |  | Group |  | K741-K744 | SP2 |
| 06:00:00 |  | Group |  | K741-K744 | SP1 |
| 11:00:00 |  | Group |  | K741-K744 | SP3 |
| 21:00:00 |  | Group |  | K741-K744 | SP2 |
| 00:00:00 |  | Group |  | K762-K767 | SP2 |
| 06:00:00 |  | Group |  | K762-K767 | SP1 |
| 06:53:00 |  | Group |  | K762-K767 | SP10 |
| 09:00:00 |  | Group |  | K762-K767 | SP1 |
| 10:00:00 |  | Group |  | K762-K767 | SP3 |
| 15:00:00 |  | Group |  | K762-K767 | SP13 |
| 18:30:00 |  | Group |  | K762-K767 | SP3 |
| 22:00:00 |  | Group |  | K762-K767 | SP2 |
| 00:00:00 |  | Group |  | K781-K786 | SP2 |
| 06:30:00 |  | Group |  | K781-K786 | SP10 |
| 09:00:00 |  | Group |  | K781-K786 | SP3 |
| 16:00:00 |  | Group |  | K781-K786 | SP13 |
| 18:30:00 |  | Group |  | K781-K786 | SP3 |
| 22:00:00 |  | Group |  | K781-K786 | SP2 |
| 00:00:00 |  | Group |  | K801-K803 | SP2 |
| 06:00:00 |  | Group |  | K801-K803 | SP11 |
| 11:00:00 |  | Group |  | K801-K803 | SP8 |
| 14:30:00 |  | Group |  | K801-K803 | SP10 |
| 21:00:00 |  | Group |  | K801-K803 | SP3 |
| 22:00:00 |  | Group |  | K801-K803 | SP2 |
| 00:00:00 |  | Group |  | K821-K828 | SP2 |
| 06:00:00 |  | Group |  | K821-K828 | SP3 |
| 06:45:00 |  | Group |  | K821-K828 | SP16 |
| 08:45:00 |  | Group |  | K821-K828 | SP3 |
| 15:00:00 |  | Group |  | K821-K828 | SP16 |
| 19:00:00 |  | Group |  | K821-K828 | SP3 |
| 22:00:00 |  | Group |  | K821-K828 | SP2 |
| 00:00:00 |  | Group |  | K829-K830 | SP4 |
| 06:00:00 |  | Group |  | K829-K830 | SP2 |
| 06:45:00 |  | Group |  | K829-K830 | SP15 |
| 08:45:00 |  | Group |  | K829-K830 | SP2 |
| 15:00:00 |  | Group |  | K829-K830 | SP16 |
| 19:00:00 |  | Group |  | K829-K830 | SP2 |
| 22:00:00 |  | Group |  | K829-K830 | SP4 |
| 00:00:00 |  | Single intersection |  | K1038 | SP11 |
| 06:00:00 |  | Single intersection |  | K1038 | SP13 |
| 06:30:00 |  | Single intersection |  | K1038 | SP8 |
| 08:50:00 |  | Single intersection |  | K1038 | SP12 |
| 15:30:00 |  | Single intersection |  | K1038 | SP8 |
| 19:30:00 |  | Single intersection |  | K1038 | SP12 |
| 22:00:00 |  | Single intersection |  | K1038 | SP11 |
| Public holiday (similar to Saturday) |  |  |  |  |  |
| Switching on time |  | Type |  | Name of the object | Action |
| 00:00:00 |  | Single intersection |  | K1080 | SP9 |
| 06:30:00 |  | Single intersection |  | K1080 | SP10 |
| 21:00:00 |  | Single intersection |  | K1080 | SP9 |
| 00:00:00 |  | Single intersection |  | K1083 | SP2 |
| 09:30:00 |  | Single intersection |  | K1083 | SP4 |
| 18:00:00 |  | Single intersection |  | K1083 | SP2 |
| 00:00:00 |  | Single intersection |  | K1084 | SP1 |
| 06:00:00 |  | Single intersection |  | K1084 | SP2 |
| 10:30:00 |  | Single intersection |  | K1084 | SP3 |
| 22:00:00 |  | Single intersection |  | K1084 | SP1 |
| 00:00:00 |  | Single intersection |  | K1085 | SP2 |
| 09:30:00 |  | Single intersection |  | K1085 | SP1 |
| 18:00:00 |  | Single intersection |  | K1085 | SP2 |
| 00:00:00 |  | Single intersection |  | K1086 | SP2 |
| 09:30:00 |  | Single intersection |  | K1086 | SP1 |
| 18:00:00 |  | Single intersection |  | K1086 | SP2 |
| 00:00:00 |  | Single intersection |  | K1087 | SP6 |
| 08:00:00 |  | Single intersection |  | K1087 | SP7 |
| 20:00:00 |  | Single intersection |  | K1087 | SP6 |
| 00:00:00 |  | Single intersection |  | K1094 | SP14 |
| 00:00:00 |  | Single intersection |  | K1095 | SP30 |
| 00:00:00 |  | Single intersection |  | K301 | SP15 |
| 00:00:00 |  | Single intersection |  | K358 | SP2 |
| 00:00:00 |  | Single intersection |  | K411 | SP2 |
| 06:00:00 |  | Single intersection |  | K411 | SP1 |
| 21:00:00 |  | Single intersection |  | K411 | SP2 |
| 00:00:00 |  | Single intersection |  | K412 | SP2 |
| 06:00:00 |  | Single intersection |  | K412 | SP1 |
| 21:00:00 |  | Single intersection |  | K412 | SP2 |
| 00:00:00 |  | Single intersection |  | K702 | SP2 |
| 00:00:00 |  | Single intersection |  | K761 | SP4 |
| 06:00:00 |  | Single intersection |  | K761 | SP2 |
| 13:00:00 |  | Single intersection |  | K761 | SP10 |
| 16:00:00 |  | Single intersection |  | K761 | SP2 |
| 22:00:00 |  | Single intersection |  | K761 | SP4 |
| 00:00:00 |  | Single intersection |  | K768 | SP4 |
| 06:00:00 |  | Single intersection |  | K768 | SP10 |
| 22:00:00 |  | Single intersection |  | K768 | SP4 |
| 00:00:00 |  | Single intersection |  | K861 | SP4 |
| 00:00:00 |  | Single intersection |  | K883 | SP4 |
| 06:00:00 |  | Single intersection |  | K883 | SP2 |
| 21:00:00 |  | Single intersection |  | K883 | SP4 |
| 00:00:00 |  | Single intersection |  | K884 | SP4 |
| 06:00:00 |  | Single intersection |  | K884 | SP16 |
| 21:00:00 |  | Single intersection |  | K884 | SP4 |
| 00:00:00 |  | Single intersection |  | K1008 | SP2 |
| 06:00:00 |  | Single intersection |  | K1008 | SP1 |
| 21:00:00 |  | Single intersection |  | K1008 | SP2 |
| 00:00:00 |  | Single intersection |  | K1009 | SP4 |
| 06:00:00 |  | Single intersection |  | K1009 | SP5 |
| 10:00:00 |  | Single intersection |  | K1009 | SP11 |
| 18:00:00 |  | Single intersection |  | K1009 | SP5 |
| 21:00:00 |  | Single intersection |  | K1009 | SP4 |
| 00:00:00 |  | Single intersection |  | K1024 | SP4 |
| 09:30:00 |  | Single intersection |  | K1024 | SP2 |
| 18:00:00 |  | Single intersection |  | K1024 | SP4 |
| 00:00:00 |  | Single intersection |  | K1026 | SP13 |
| 07:00:00 |  | Single intersection |  | K1026 | SP13 |
| 19:00:00 |  | Single intersection |  | K1026 | SP13 |
| 00:00:00 |  | Single intersection |  | K1035 | SP4 |
| 00:00:00 |  | Single intersection |  | K1045 | SP9 |
| 00:00:00 |  | Single intersection |  | K1051 | SP5 |
| 07:00:00 |  | Single intersection |  | K1051 | SP6 |
| 10:00:00 |  | Single intersection |  | K1051 | SP3 |
| 18:00:00 |  | Single intersection |  | K1051 | SP6 |
| 22:00:00 |  | Single intersection |  | K1051 | SP5 |
| 00:00:00 |  | Single intersection |  | K1055 | SP11 |
| 07:00:00 |  | Single intersection |  | K1055 | SP11 |
| 10:00:00 |  | Single intersection |  | K1055 | SP12 |
| 20:00:00 |  | Single intersection |  | K1055 | SP11 |
| 22:00:00 |  | Single intersection |  | K1055 | SP11 |
| 00:00:00 |  | Single intersection |  | K1056 | SP2 |
| 07:00:00 |  | Single intersection |  | K1056 | SP2 |
| 10:00:00 |  | Single intersection |  | K1056 | SP6 |
| 20:00:00 |  | Single intersection |  | K1056 | SP2 |
| 22:00:00 |  | Single intersection |  | K1056 | SP2 |
| 00:00:00 |  | Single intersection |  | K1063 | SP2 |
| 07:00:00 |  | Single intersection |  | K1063 | SP2 |
| 10:00:00 |  | Single intersection |  | K1063 | SP16 |
| 20:00:00 |  | Single intersection |  | K1063 | SP2 |
| 22:00:00 |  | Single intersection |  | K1063 | SP2 |
| 00:00:00 |  | Single intersection |  | K1066 | SP1 |
| 00:00:00 |  | Single intersection |  | K1074 | SP2 |
| 09:30:00 |  | Single intersection |  | K1074 | SP3 |
| 18:00:00 |  | Single intersection |  | K1074 | SP2 |
| 00:00:00 |  | Single intersection |  | K1097 | SP5 |
| 07:00:00 |  | Single intersection |  | K1097 | SP3 |
| 19:00:00 |  | Single intersection |  | K1097 | SP5 |
| 00:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 06:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 22:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 00:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 00:00:00 |  | Group |  | K1059-1060-1061 | SP5 |
| 06:00:00 |  | Group |  | K1059-1060-1061 | SP6 |
| 22:00:00 |  | Group |  | K1059-1060-1061 | SP5 |
| 00:00:00 |  | Group |  | K302-K354 | SP2 |
| 00:00:00 |  | Group |  | K306-K321 | SP2 |
| 00:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 00:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 00:00:00 |  | Group |  | K413-K414 | SP8 |
| 07:00:00 |  | Group |  | K413-K414 | SP9 |
| 10:00:00 |  | Group |  | K413-K414 | SP6 |
| 18:00:00 |  | Group |  | K413-K414 | SP9 |
| 22:00:00 |  | Group |  | K413-K414 | SP8 |
| 00:00:00 |  | Group |  | K501-K556 | SP2 |
| 00:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 00:00:00 |  | Group |  | K701 | SP2 |
| 00:00:00 |  | Group |  | K741-K744 | SP2 |
| 00:00:00 |  | Group |  | K762-K767 | SP2 |
| 06:00:00 |  | Group |  | K762-K767 | SP1 |
| 10:00:00 |  | Group |  | K762-K767 | SP3 |
| 22:00:00 |  | Group |  | K762-K767 | SP2 |
| 00:00:00 |  | Group |  | K781-K786 | SP2 |
| 06:00:00 |  | Group |  | K781-K786 | SP3 |
| 22:00:00 |  | Group |  | K781-K786 | SP2 |
| 00:00:00 |  | Group |  | K801-K803 | SP2 |
| 06:00:00 |  | Group |  | K801-K803 | SP1 |
| 10:00:00 |  | Group |  | K801-K803 | SP3 |
| 21:00:00 |  | Group |  | K801-K803 | SP2 |
| 00:00:00 |  | Group |  | K821-K828 | SP2 |
| 00:00:00 |  | Group |  | K829-K830 | SP2 |
| 00:00:00 |  | Single intersection |  | K1038 | SP11 |
| 07:00:00 |  | Single intersection |  | K1038 | SP10 |
| 10:00:00 |  | Single intersection |  | K1038 | SP12 |
| 20:00:00 |  | Single intersection |  | K1038 | SP10 |
| 22:00:00 |  | Single intersection |  | K1038 | SP11 |
| Public holiday, return to the city (similar to Sunday) |  |  |  |  |  |
| Switching on time |  | Type |  | Name of the object | Action |
| 00:00:00 |  | Single intersection |  | K1080 | SP9 |
| 06:30:00 |  | Single intersection |  | K1080 | SP10 |
| 21:00:00 |  | Single intersection |  | K1080 | SP9 |
| 00:00:00 |  | Single intersection |  | K1083 | SP2 |
| 15:00:00 |  | Single intersection |  | K1083 | SP1 |
| 21:00:00 |  | Single intersection |  | K1083 | SP2 |
| 00:00:00 |  | Single intersection |  | K1084 | SP1 |
| 06:00:00 |  | Single intersection |  | K1084 | SP2 |
| 10:30:00 |  | Single intersection |  | K1084 | SP3 |
| 22:00:00 |  | Single intersection |  | K1084 | SP1 |
| 00:00:00 |  | Single intersection |  | K1085 | SP2 |
| 15:00:00 |  | Single intersection |  | K1085 | SP3 |
| 21:00:00 |  | Single intersection |  | K1085 | SP2 |
| 00:00:00 |  | Single intersection |  | K1086 | SP2 |
| 15:00:00 |  | Single intersection |  | K1086 | SP3 |
| 21:00:00 |  | Single intersection |  | K1086 | SP2 |
| 00:00:00 |  | Single intersection |  | K1087 | SP6 |
| 08:00:00 |  | Single intersection |  | K1087 | SP7 |
| 20:00:00 |  | Single intersection |  | K1087 | SP6 |
| 00:00:00 |  | Single intersection |  | K1094 | SP14 |
| 00:00:00 |  | Single intersection |  | K1095 | SP30 |
| 00:00:00 |  | Single intersection |  | K301 | SP15 |
| 00:00:00 |  | Single intersection |  | K358 | SP2 |
| 00:00:00 |  | Single intersection |  | K409 | SP1 |
| 14:00:00 |  | Single intersection |  | K409 | SP2 |
| 20:00:00 |  | Single intersection |  | K409 | SP1 |
| 00:00:00 |  | Single intersection |  | K411 | SP2 |
| 06:00:00 |  | Single intersection |  | K411 | SP1 |
| 21:00:00 |  | Single intersection |  | K411 | SP2 |
| 00:00:00 |  | Single intersection |  | K412 | SP2 |
| 06:00:00 |  | Single intersection |  | K412 | SP1 |
| 21:00:00 |  | Single intersection |  | K412 | SP2 |
| 00:00:00 |  | Single intersection |  | K702 | SP2 |
| 00:00:00 |  | Single intersection |  | K761 | SP4 |
| 06:00:00 |  | Single intersection |  | K761 | SP2 |
| 22:00:00 |  | Single intersection |  | K761 | SP4 |
| 00:00:00 |  | Single intersection |  | K768 | SP4 |
| 06:00:00 |  | Single intersection |  | K768 | SP10 |
| 22:00:00 |  | Single intersection |  | K768 | SP4 |
| 00:00:00 |  | Single intersection |  | K861 | SP4 |
| 00:00:00 |  | Single intersection |  | K883 | SP4 |
| 06:00:00 |  | Single intersection |  | K883 | SP2 |
| 21:00:00 |  | Single intersection |  | K883 | SP4 |
| 00:00:00 |  | Single intersection |  | K884 | SP4 |
| 06:00:00 |  | Single intersection |  | K884 | SP16 |
| 21:00:00 |  | Single intersection |  | K884 | SP4 |
| 00:00:00 |  | Single intersection |  | K1008 | SP2 |
| 06:00:00 |  | Single intersection |  | K1008 | SP1 |
| 21:00:00 |  | Single intersection |  | K1008 | SP2 |
| 00:00:00 |  | Single intersection |  | K1009 | SP4 |
| 06:00:00 |  | Single intersection |  | K1009 | SP5 |
| 10:00:00 |  | Single intersection |  | K1009 | SP11 |
| 15:00:00 |  | Single intersection |  | K1009 | SP11 |
| 19:00:00 |  | Single intersection |  | K1009 | SP5 |
| 21:00:00 |  | Single intersection |  | K1009 | SP4 |
| 00:00:00 |  | Single intersection |  | K1024 | SP4 |
| 15:00:00 |  | Single intersection |  | K1024 | SP1 |
| 21:00:00 |  | Single intersection |  | K1024 | SP4 |
| 00:00:00 |  | Single intersection |  | K1026 | SP13 |
| 00:00:00 |  | Single intersection |  | K1035 | SP4 |
| 00:00:00 |  | Single intersection |  | K1045 | SP9 |
| 15:00:00 |  | Single intersection |  | K1045 | SP11 |
| 21:00:00 |  | Single intersection |  | K1045 | SP9 |
| 00:00:00 |  | Single intersection |  | K1051 | SP5 |
| 07:00:00 |  | Single intersection |  | K1051 | SP6 |
| 18:00:00 |  | Single intersection |  | K1051 | SP2 |
| 21:00:00 |  | Single intersection |  | K1051 | SP6 |
| 22:00:00 |  | Single intersection |  | K1051 | SP5 |
| 00:00:00 |  | Single intersection |  | K1055 | SP11 |
| 07:00:00 |  | Single intersection |  | K1055 | SP11 |
| 10:00:00 |  | Single intersection |  | K1055 | SP12 |
| 16:00:00 |  | Single intersection |  | K1055 | SP12 |
| 18:00:00 |  | Single intersection |  | K1055 | SP5 |
| 21:00:00 |  | Single intersection |  | K1055 | SP11 |
| 22:00:00 |  | Single intersection |  | K1055 | SP11 |
| 00:00:00 |  | Single intersection |  | K1056 | SP2 |
| 07:00:00 |  | Single intersection |  | K1056 | SP2 |
| 10:00:00 |  | Single intersection |  | K1056 | SP6 |
| 16:00:00 |  | Single intersection |  | K1056 | SP6 |
| 18:00:00 |  | Single intersection |  | K1056 | SP5 |
| 21:00:00 |  | Single intersection |  | K1056 | SP2 |
| 22:00:00 |  | Single intersection |  | K1056 | SP2 |
| 00:00:00 |  | Single intersection |  | K1063 | SP2 |
| 07:00:00 |  | Single intersection |  | K1063 | SP2 |
| 10:00:00 |  | Single intersection |  | K1063 | SP16 |
| 16:00:00 |  | Single intersection |  | K1063 | SP16 |
| 18:00:00 |  | Single intersection |  | K1063 | SP12 |
| 21:00:00 |  | Single intersection |  | K1063 | SP2 |
| 22:00:00 |  | Single intersection |  | K1063 | SP2 |
| 00:00:00 |  | Single intersection |  | K1066 | SP1 |
| 00:00:00 |  | Single intersection |  | K1074 | SP2 |
| 15:00:00 |  | Single intersection |  | K1074 | SP4 |
| 19:00:00 |  | Single intersection |  | K1074 | SP2 |
| 00:00:00 |  | Single intersection |  | K1097 | SP5 |
| 08:00:00 |  | Single intersection |  | K1097 | SP3 |
| 19:00:00 |  | Single intersection |  | K1097 | SP5 |
| 00:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 06:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP2 |
| 22:00:00 |  | Group |  | K1003_K1005_K1007_K1010_ K1013_ K1014_ K1016_K1017_K1018_K1027_K1029_K1030 | SP4 |
| 00:00:00 |  | Group |  | K101-K116_K151-K156_K201 | SP2 |
| 00:00:00 |  | Group |  | K1059-1060-1061 | SP5 |
| 06:00:00 |  | Group |  | K1059-1060-1061 | SP6 |
| 22:00:00 |  | Group |  | K1059-1060-1061 | SP5 |
| 00:00:00 |  | Group |  | K302-K354 | SP2 |
| 00:00:00 |  | Group |  | K306-K321 | SP2 |
| 00:00:00 |  | Group |  | K355_K1001_K1004_K1006_K1015_K1020_K1023_K1028 | SP4 |
| 00:00:00 |  | Group |  | K401-K410_K415_K451 | SP2 |
| 00:00:00 |  | Group |  | K413-K414 | SP8 |
| 07:00:00 |  | Group |  | K413-K414 | SP9 |
| 18:00:00 |  | Group |  | K413-K414 | SP1 |
| 21:00:00 |  | Group |  | K413-K414 | SP9 |
| 22:00:00 |  | Group |  | K413-K414 | SP8 |
| 00:00:00 |  | Group |  | K501-K556 | SP2 |
| 00:00:00 |  | Group |  | K601-K610-K721 | SP2 |
| 00:00:00 |  | Group |  | K701 | SP2 |
| 00:00:00 |  | Group |  | K741-K744 | SP2 |
| 00:00:00 |  | Group |  | K762-K767 | SP2 |
| 06:00:00 |  | Group |  | K762-K767 | SP1 |
| 10:00:00 |  | Group |  | K762-K767 | SP3 |
| 22:00:00 |  | Group |  | K762-K767 | SP2 |
| 00:00:00 |  | Group |  | K781-K786 | SP2 |
| 06:00:00 |  | Group |  | K781-K786 | SP3 |
| 22:00:00 |  | Group |  | K781-K786 | SP2 |
| 00:00:00 |  | Group |  | K801-K803 | SP2 |
| 06:00:00 |  | Group |  | K801-K803 | SP1 |
| 10:00:00 |  | Group |  | K801-K803 | SP3 |
| 21:00:00 |  | Group |  | K801-K803 | SP2 |
| 00:00:00 |  | Group |  | K821-K828 | SP2 |
| 00:00:00 |  | Group |  | K829-K830 | SP2 |
| 00:00:00 |  | Single intersection |  | K1038 | SP11 |
| 07:00:00 |  | Single intersection |  | K1038 | SP10 |
| 10:00:00 |  | Single intersection |  | K1038 | SP12 |
| 18:00:00 |  | Single intersection |  | K1038 | SP8 |
| 21:00:00 |  | Single intersection |  | K1038 | SP12 |
| 22:00:00 |  | Single intersection |  | K1038 | SP11 |

Annex 6 to the Technical Specification

| Category | Name of the special response plan (Response plan) | Start time | End time |
| --- | --- | --- | --- |
| 1. Special plans | Evening: K1030 SP3 | 16:00:00 |  |
| 1. Special plans | Evening: K1080 SP12 | 14:30:00 |  |
| 1. Special plans | Evening: K721 VAKARAS NUO16 IKI 19 | 16:00:00 |  |
| 1. Special plans | Evening: K742 | 16:30:00 |  |
| 1. Special plans | K1094 |  |  |
| 1. Special plans | K409: SP3 | 14:45:00 |  |
| 1. Special plans | k512 full adaptive | 22:00:00 |  |
| 1. Special plans | k823 sp5 Full_TA Evening peak | 15:00:00 |  |
| 1. Special plans | k823 sp6 Full_TA Morning peak | 06:45:00 |  |
| 1. Special plans | Morning: K603 SP7 | 06:30:00 |  |
| 1. Special plans | Morning: K721 SP1 | 07:30:00 |  |
| 1. Special plans | Morning: K828 SP15 | 06:45:00 |  |
| 1. Special plans | Night: K502, K503,SP5 | 22:00:00 |  |
| 1. Special plans | Weekend: K741 SP1 | 09:00:00 |  |

Annex 7 to the Technical Specification

Complete set of traffic light controllers and the maximum quantity per set

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 | No | Yes | 24V/10A | 44 |
| 2 | 16 | 16 | 230 | No | Yes | 24V/10A | 110 |
| 3 | 16 | 32 | 230 | No | Yes | 24V/10A | 45 |
| 4 | 24 | 16 | 230 | No | Yes | 24V/10A | 5 |
| 5 | 24 | 32 | 230 | No | Yes | 24V/10A | 4 |
| 6 | 32 | 16 | 230 | No | Yes | 24V/10A | 1 |
| 7 | 32 | 32 | 230 | No | Yes | 24V/10A | 1 |

Traffic light controller packages

Package 1

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 2 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 10 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 5 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 1 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 2

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 3 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 5 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 7 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 1 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 3

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 3 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 6 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 7 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 1 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 1 |

Package 4

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 8 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 9 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 5

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 2 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 1 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 2 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 6

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 4 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 8 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 3 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 7

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 1 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 17 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 1 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 8

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 4 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 8 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 3 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 9

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 6 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 1 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 1 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 10

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 1 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 7 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 3 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 2 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 2 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 11

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 2 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 10 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 2 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 12

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 5 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 9 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 2 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 2 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 13

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 8 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 6 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 3 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

Package 14

| Set of traffic light controller | Number of signal groups | Number of I/O (inputs and outputs) | Voltage of traffic light modules | Dimming function | Uninterruptible power supply for the controller's CPU board and communication equipment | Additional power supply for transport sensors | Number of controllers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 16 | 230 V | No | Yes | 24V/10A | 1 |
| 2 | 16 | 16 | 230 V | No | Yes | 24V/10A | 8 |
| 3 | 16 | 32 | 230 V | No | Yes | 24V/10A | 6 |
| 4 | 24 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 5 | 24 | 32 | 230 V | No | Yes | 24V/10A | 0 |
| 6 | 32 | 16 | 230 V | No | Yes | 24V/10A | 0 |
| 7 | 32 | 32 | 230 V | No | Yes | 24V/10A | 0 |

The Supplier may implement open, fully documented and testable interfaces for sending/receiving commands to controllers from the adaptive module for real-time optimization of operating modes when implementing the adaptive module for real-time optimization of operating modes.

The adaptive module for real-time optimization of operating modes must generate new signal plans based on traffic flows with the selection of stage sequence, calculation of green signal durations and offset evaluation. The selection and activation of existing signal plans already present (preselected) in the System is inappropriate as it is not sufficient for a more efficient intersection management.

If at the time of installation of the System in Vilnius City there are already System manufacturer's controllers installed (under previous city contracts), the Supplier shall configure, run and test the PTP module on the System and on those controllers. The configuration of the PTP module will be adjusted during the installation of the System.

The creation and fulfilment of the criteria and conditions of the PTP module depends on the type of data transmitted from the Ridango AVL system. If the absence of a particular data type in the Ridango AVL API (e.g. if no PT delay or other PT schedules data is transmitted via the Ridango AVL API) does not oblige the Supplier to fulfil the requirements for the PTP module in relation to the prioritisation of a delayed PT , but it shall remain possible to fulfil all the requirements of the PTP module in the event of additional data types appearing in the Ridango AVL API interface.