Sub-annex 1 of Annex 6 to the Procurement Conditions

COMPLIANCE TABLE TO THE TECHNICAL SPECIFICATION OF SUPPLIER`S PROPOSED UNMANNED AERIAL VEHICLE (UAV) OMNIDIRECTIONAL BLOCKING MEASURE

(II-nd procurement lot)

| No | Requirements | Compliance with the requirements (to be completed by the Supplier, indicating a specific value (if requested) or whether the offered product meets or does not meet a specific requirement) | Document confirming compliance (filled in by the Supplier, indicating the names of the documents and/or links to their points, proving the compliance of the offered goods with the technical requirements)* |
| --- | --- | --- | --- |
| I. | Introduction |  |  |
|  | Purchase object – Unmanned aircraft vehicle blocking measure (toliau – UAVBM). UAVBM is designed to protect objects such as fortifications, trenches and vehicles. The equipment blocks control, video and navigation signals in all directions and provides protection against swarms of unmanned aerial vehicles (hereinafter referred to as UAV). The UAVBM will be used to block the Unmanned Aerial Vehicle's frequencies or a selected range of radio frequencies (jamming, electronic attacks). It will have to operate continuously (24/7) in times of crisis and war, in all seasons of the year, day and night, as well as in limited visibility conditions, in challenging weather (cold/hot weather, dust, mud, humidity, snow and ice) and electromagnetic environment | - | - |
| II. | General Requirements for the Equipment |  |  |
|  | The purchased equipment must be new and unused and come in the manufacturer’s original packaging. The equipment supplied cannot be upgraded by the manufacturer | Yes/No |  |
|  | The Seller must ensure that the manufacturer has not declared discontinuation of production or development of the proposed equipment (e.g. “End of life time” or “Discontinued”) | Yes/No |  |
|  | Technical/functional requirements |  |  |
|  | be capable of suppressing the electromagnetic wave ranges required for the use of UAV within a radius of at least 300 m (360); | indicate the exact value proposed, (distance in metres and radius in degrees):________ |  |
|  | be capable of operating against both single and groups of UAV using navigation, video and control signals at different frequencies | Yes/No |  |
|  | shall be capable of suppressing electromagnetic wave ranges when neutralising UAV | Yes/No |  |
|  | UAV location detection L1 GNSS navigation signals | Yes/No |  |
|  | UAV video transmission and control signals: 900MHz, 2x2400 MHz and 2x5800 MHz | specify the exact value proposed, (signals in MHz):________ |  |
|  | the radiated power (over the range) of the suppression components at the output of the radio module shall be at least: 900MHz - 20 W; 2400MHz - 2x10 W; 5800MHz - 2x10 W; L1 GNSS - 10 W | specify the exact value proposed, (power in MHz - W):________ |  |
|  | the gain of the omnidirectional antennas installed in the equipment shall be at least: 900MHz - 5dBi; 2x2400 MHz - 4dBi; 2x5800 MHz - 7dBi; L1 GNSS - 5dBi | specify the exact value proposed, (gain in MHz - dBi): ________ |  |
|  | provision shall be made for separate switching on/off of suppressed frequencies | Yes/No |  |
|  | the equipment shall have an indicator light that turns on when the equipment is activated | Yes/No |  |
|  | the equipment shall be powered by built-in rechargeable batteries | Yes/No |  |
|  | the battery shall be equipped with a charge level indicator | Yes/No |  |
|  | the equipment shall have a charger to recharge the built-in batteries | Yes/No |  |
|  | a 230V AC power supply shall be provided to ensure constant operation of the unit in radiant mode | Yes/No |  |
|  | all modules and antennas shall be mounted on ABS plastic or lightweight aluminium alloy body | specify the exact meaning proposed, ( material):________ |  |
|  | shall be activated at the touch of a button | Yes/No |  |
|  | have an environmental protection rating of at least IP 65 or equivalent | specify the exact value proposed, (IP level): _______ |  |
|  | fully charged batteries shall operate for at least 5 hours in activated mode | specify the exact value proposed, (duration, hours):________ |  |
|  | weight not more than 12 kg | specify the exact value proposed, (weight, kg):________ |  |
|  | be capable of being mounted on the roof of the vehicle | Yes/No |  |
|  | the equipment shall use passive soundproof cooling | Yes/No |  |
|  | shall include a User Manual in Ukrainian Lithuanian or English. | specify the exact meaning proposed, (language of the user guide):________ |  |
| III. | Support and Maintenance |  |  |
|  | The equipment shall be guaranteed for a period of at least 24 months from the date of signing of the handover and acceptance certificate between the Buyer and the Seller | indicate the exact value proposed (in months):________ |  |
|  | The equipment must be supplied with an IP67 hard case for safe transport and storage. | Yes/No |  |
| IV. | Other requirement |  |  |
|  | The Supplier of the goods, its sub-supplier, the entity whose capacities are relied upon or the manufacturer and the person controlling them cannot be registered in a country (if the manufacturer or the person controlling it is a natural person - permanently residing in that country or having the nationality of that country), which is included in the list of countries approved by the Government of the Republic of Lithuania by the Governmental Decree of 30 March 2022, No. 280, “Regarding the Implementation of the Provisions of Article 92, paragraphs (13), (14), and (15), of Public Procurement Law of the Republic of Lithuania”. | The proposal shall be accompanied by a completed Annex 5 to the Procurement Documents |  |

* Documents that the Supplier must submit together with the Price Proposal in accordance with Clause 9.10.3 of the Procurement Conditions. This section is mandatory.

*Notes:

1. The Supplier, when specifying a particular item or confirming product compliance, is required to clearly indicate, to the procuring organization, the precise location within the Supplier's CPP IS where the claim confirmation and proving data is contained. This indication should include the specific folder or file, the document name, and the page number.

2. Each technical requirement item must be substantiated by technical documents and/or manufacturer-approved declarations (if not otherwise specified). These supporting documents are to be included with the offer submission.

3. The phrase "or equivalent" is appended to all specific materials and trade names. In the event a Supplier proposes a product with equivalent properties, it is imperative that they take necessary measures during the offer submission to validate that the proposed product is indeed equivalent and fully compliant with the requirements outlined in the technical specifications.