Annex 2

to the Procurement Conditions

TECHNICAL SPECIFICATION FOR COUNTER UNMANNED AERIAL SYSTEMS

INTRODUCTION

The Counter Unmanned Aerial System (CUAS) (stationary) will be used to detect and block (suppress, launch electronic attacks) unmanned aerial vehicles (UAV) on frequencies or on a selected range of radio frequencies. The CUAS System will be installed permanently at a site specified by the Buyer.

- GENERAL SYSTEM REQUIREMENTS

- The system shall be installed in accordance with the requirements of INTERNATIONAL AMMUNITION TECHNICAL GUIDELINES (IATG) 05.60 “Hazards of electromagnetic radiation” or equivalent standard.

The number of system modules shall ensure the detection and suppression of UAVs over and in the surrounding area of the protected facility.

The system consists of:

- Automatic Targeting Module

- Optical module (daylight and thermal imaging cameras)

- Passive Radio Sensor Module

- Directional Blocking Module

- Omnidirectional Prevention Module

- Centralised control module with software

- Stem

- Requirements for the Automatic Targeting Module:

  - Azimuth range not less than 360;

  - Angle of inclination from the horizontal of at least -30 to +80 throughout the operating space;

  - Maintained rotation speed: at least 20/s vertical and 40/s horizontal;

  - Environmental protection: at least IP 65 or equivalent;

  - Temperature range requirement - minimum -30C to +55C.

- Requirements for the optical module (daylight and thermal imaging cameras):

  - Daytime camera:

    - Sensor size at least 8.23 mm(H) x 6.68 mm(H);

    - Resolution - at least 1920x1080;

    - Optical focal length of at least 300mm;

    - Temperature range requirement - at least -30C to +55C;

    - Environmental protection - at least IP 65 or equivalent.

  - Thermal imaging camera:

    - The camera shall be cooled to an increased detection sensitivity of at least 25 mK at a temperature of +25ºC for extracting small objects from the background;

    - The infrared sensor shall have a pixel size of at least 10m;

    - Optical focal length of at least 300 mm;

    - Continuous optical zoom to detect targets more than 1 km away - at least x20;

    - Additional digital zoom - at least x4;

    - A field of view range for target detection of at least 2.4 to 44.9;

    - Resolution - at least 1280x1024;

    - The transparency level of the lens shall be at least 90%;

    - Temperature range requirement - at least -30C to +55C;

    - Environmental protection: at least IP 65 or equivalent.

- Requirements for the Passive Radio Sensor Module:

  - The module shall operate passively without emitting radio waves;

  - The module must operate in a frequency range no narrower than 400 MHz to 6 GHz. The module must be able to analyse received signals and identify technologies and protocols used by the UAV;

  - If a normal WiFi connection is used to control the UAV or to transmit video, the module shall be able to detect the signals sent by the UAV based on the list of UAV manufacturers’ network addresses stored in the database;

  - Determination of the location of the UAV and the UAV operator must be determined by the principle of triangulation, using at least two frequency detectors installed in different places of the protected object;

  - The module shall provide at least 360 coverage;

  - Sensors must have no moving parts - passive cooling;

  - Detection distance: The operating distance of the system must be at least 10 km (in an open area);

  - The module shall have the option of triangulation mode - combining multiple modules;

  - The module shall support Remote Drone ID function;

  - The module shall have an integrated GPS;

  - Situation information must be transmitted to a centralised control system;

  - It must be possible to work without using GPS;

  - Temperature range requirement - minimum -30°C to +55°C;

  - Environmental protection: at least IP 65 or equivalent.

- Requirements for a Directional Blocking Module:

  - Operating range - at least 3 km;

  - The module must operate in a frequency range no narrower than 400 MHz to 6 GHz;

  - It must be possible to block only those frequencies where UAV is detected in order to minimize the effect on others;

  - It must be possible to install additional frequencies;

  - There shall be no moving parts in the body - passive cooling;

  - The module must be activated via a centralised control system;

  - Temperature range requirement - at least -30°C to +55°C;

  - Environmental protection: at least IP 65 or equivalent.

- Requirements for an Omnidirectional Prevention Module;

  - The radius of attenuation shall be at least 360°;

  - Operating range - at least 500m;

  - The module must operate in a frequency range no narrower than 400 MHz to 6 GHz  It must be possible to install additional frequencies;

  - It must be possible to block only those frequencies where UAV is detected in order to minimize the effect on others

  - There shall be no moving parts in the body - passive cooling;

  - The module must be activated via a centralised control system;

  - Temperature range requirement - minimum -30°C to +55°C;

  - Environmental protection: at least IP 65 or equivalent.

- Requirements for the centralised control module and software:

  - The system shall integrate data from the radar, RF sensor and optical modules, with the ability to be customised to any module;

  - There shall be an attenuator control and an additional frequency control;

  - A history of use must be kept: direction, frequency, time, location;

  - It must be possible for the user to upload their own maps;

  - It shall be possible to record the image captured by the cameras;

  - The user interface must be designed for single operator operation;

  - It must be possible to view both daylight and thermal imaging cameras at the same time;

  - Controlling the cameras: zoom, focus, direction;

  - The field of view of the cameras shall be displayed on the map;

  - Radar and RF module information shall be displayed on the map;

  - It must be possible to control the map (rotation, drawing different zones);

  - It shall be possible for the cameras and silencer to be automatically reversed according to selected map points or targets;

  - It shall have the capability to synchronise the zooms of the day and thermal imaging cameras;

  - There shall be automatic target tracking with directional suppression using coordinates from any sensor;

  - There shall be optical target tracking;

  - There shall be an audible signal when a target is detected in the tracking area;

  - It shall be possible to operate the system by joystick or keypad;

  - Must be able to connect to another operator on the network (observer mode);

  - The user software must be installed on a computer with an operating system designed to run the software;

  - The computer platform must be Intel vPro or equivalent;

  - Memory - at least 16 GB DDR5;

  - 2x23“ LED monitors with brackets: resolution not less than 1920x1080, refresh rate not less than 60Hz, response time not more than 8 ms, IPS technology, brightness not less than 250 cd/m ;

  - Must have a keyboard and a professional video surveillance joystick.

- Requirements for stems:

  - Height - the site location is flat and wooded, and the radar, RF, optical and firewall modules must have line of sight (LOS) conditions.

  - It shall be able to accommodate radar, RF, optical or jamming modules.

  - The type of supports supplied must be openwork.

SUPPORT AND MAINTENANCE

The seller must, at his own expense, train the users, the technical staff and instructors (up to 5 soldiers) who will perform maintenance and repair of the equipment, within 3 months from the day of the acceptance of the equipment - the day of signing the transfer deed.

The guaranteed period of use of the equipment must be at least 36 months from the date of signature of the handover and acceptance certificate between the Buyer and the Seller.

The price of the procurement object must include the technical and license support of the software required for the functioning of the procurement object's systems for at least 36 months.

The price of the purchase object must include timely updates of the database of drones detected by the system, thus enabling the system to recognize new UAV and their data transmission protocols that have appeared on the market. Updates must be guaranteed for at least 36 months.

OTHER REQUIREMENTS

The Supplier of the goods, its sub-supplier, the entity whose capacities are relied upon, or the manufacturer and the person controlling them, must not be registered in a country (if the manufacturer or the person controlling it is a natural person - permanently residing in, or possessing the citizenship of, that country) which the Government of the Republic of Lithuania, by its Resolution No. 280 “On the implementation of the provisions of Article 92(13), (14) and (15) of the Public Procurement Law of the Republic of Lithuania” of 30 March 2022, has included in the list of countries approved.

Within the scope of the project, at the expense of the Supplier, all permits and licenses and certificates required to import the provided equipment into Lithuania or to operate it, but not limited to them, must be obtained and presented to the Buyer

System equipment must be new, never used before, fit for purpose. It is not possible to offer used or refurbished equipment.

The supplier of goods, his sub-supplier, the economic entity whose capabilities are relied on must:

Provide all software, configuration files and backups;

Submit a detailed executive project of the procurement object, reflecting the principles of installation of the system being developed and presenting the equipment connection diagram, by e-mail. the power scheme and explaining the essential technical solutions, the configuration parameters of the system being developed and the characteristics of the equipment;

Provide relevant system user instructions in Lithuanian;

Provide relevant system administrator instructions in Lithuanian;

The scope of the project must include all installation, installation and configuration of the supplied equipment, including, but not limited to, the assembly, installation, connection of the equipment and all other necessary work and materials and tools required for this. The scope of delivery must include all necessary connection cables and other equipment and materials;

In the scope of the project, e-mail must be installed. power supply (with uninterruptible power sources ensuring system operation for at least 4 hours) equipment to connect all equipment supplied and installed during the Project to the existing connection points;

All equipment installed outdoors must have proper grounding, surge protection and installed lightning protection solutions;

The place for the equipment must be chosen in such a way that, after its installation, maximum coverage of the monitored area and its accesses is ensured.

The UAV detection system must be able to be integrated into the existing security system by transmitting "Alarm" and system failure signals, but not limited to them;

Systems must be able to transmit information about UAV detections in an open standard format and/or API to third-party systems;

The scope of the project must include training, during which employees appointed by the Contracting Entity and other users of the system must be trained to properly use the system, perform system administration work;

Provide relevant system user instructions in Lithuanian;

Provide relevant system administrator instructions in Lithuanian;

It is necessary to include testing of the purchased system before signing the contract, in order to make sure that the system conforms to the specified/declared parameters.

___________________________________________________