DEPARTURE CLEARANCE SERVICE

TECHNICAL SPECIFICATION

# Introduction

  - This Technical Specification sets the requirements for Departure Clearance message exchange service (hereinafter - DCL), which the public limited liability company "Oro navigacija" (hereinafter - the Buyer) intents to use over ACARS for direct ATC to pilot data-link communication based on the EUROCAE ED-85A and ARINC 623 standards.

  - As the DCL message exchange source, the Buyer will use ATC System InNOVA (hereinafter – InNOVA System) with DCL functionality.

  - The Buyer will select a data communication service provider (hereinafter referred to as the CSP) in accordance with the established procurement procedure, which having appropriate air-to-ground (A/G) data transmission means will provide DCL message exchange service between InNOVA System and aircraft intending to depart from Vilnius airport in accordance with the established requirements.

  - The provision of DCL service shall begin on June 1, 2026. The date of beginning of the service provision may be earlier or later, the Buyer will inform the Supplier about this by e-mail no later than 1 month in advance.

# Scope of service

  - The DCL message exchange service shall be provided by CSP for all ACARS equipped aircraft through datalink network.

  - The ACARS equipped aircraft departing from Vilnius airport shall be able to transmit and receive the DCL messages.

  - An InNOVA System shall be able to exchange the DCL messages with all properly equipped aircraft via datalink, regardless which data CSP that aircraft is cooperating with, SITA or ARINC, or other, interworking via ACARS.

# Service Descriptions

  - DCL shall be provided within the Vilnius airport area for the aircraft on the ground.

  - The DCL message types and contents exchanged via datalink between the aircraft and InNOVA System, will be set in accordance with the standard ED-85A, and comprise the following:

    - Departure Clearance Request (RCD), Downlink – RCD,

    - Departure Clearance Uplink (CLD), Uplink – CLD,

    - Departure Clearance Readback (CDA), Downlink – CDA,

    - Flight System Message (FSM), Uplink – FSM.

  - The CSP shall transmit DCL uplink messages which sends the InNOVA System.

  - The CSP shall transmit DCL downlink messages which sends the properly equipped aircraft.

  - Upon receipt of DCL downlink message, the InNOVA System will behave as ATS Domain is defined in EUROCAE ED-85A.

  - Upon receipt of DCL uplink message, the aircraft system will behave as Airborne Domain is defined in EUROCAE ED-85A.

  - CSP shall behave as Communication Domain is defined in EUROCAE ED-85A.

# Regulatory requirement

  - The CSP shall provide evidence of its compliance with Article 11.6 of Regulation (EU) 2024/2803 of the European Parliament and of the Council of 23 October 2024 on the implementation of the Single European Sky.

# Operational Environment

The considered operational environment shall be as follows:

  - Traffic Density: 150 aircraft per hour.

  - Peak Number of departure clearances per hour per control position: 112.

  - Maximum instantaneous DCL requests per ATS ground system: 50.

  - Service Coverage: Vilnius airport area, for the aircraft on the ground.

This operational environment is the maximum environment assumed in this document.

# Performance requirements

  - The end-to-end availability of the DCL data-link system from aircrew standpoint shall be more than 99,30 %.

  - The end-to-end availability of the DCL data-link system from air traffic controller standpoint shall be more than 99,40 %.

  - The corrupted message ratio for the total volume of delivered messages shall be less than 1x10⁻².

  - The lost message ratio for the total volume of messages shall be less than 5x10⁻².

  - The maximum value of t0 for 95 % of the total volume of delivered messages shall be less than 65 sec. t0 is the elapsed time between the sending by the aircrew of the departure clearance request (RCD message) and the notification to them of the FSM logical acknowledgement arrival, whenever this FSM is implemented.

  - The maximum value of t2 for 95 % of the total volume of delivered messages shall be less than 65 sec. t2 is the elapsed time between the sending by the aircrew of the CDA read-back and the notification to them of the FSM to the CDA.

  - The end-to-end delivery time for 95 % of the total volume of delivered messages shall be less than 32,5 sec.

  - Communication link requirements:

    - Delay for 95 % of the total volume of messages shall be less than 25 sec.

    - The availability for character-oriented communications shall be more than 99,50 %.

    - The corrupted message ratio for the total volume of messages shall be less than 1x10⁻².

    - The lost message ratio for the total volume of messages shall be less than 5x10⁻².

# Additional conditions

  - The CSP should refer and comply to the EUROCAE ED-85A Communication domain corresponding additional requirements not referred in this Specification.

  - The CSP should refer to the paragraph 5.3.1 of EUROCAE ED-85-A for the Airborne domain requirements.

  - The CSP should refer to the paragraph 5.3.3 of EUROCAE ED-85-A for the ATS domain requirements.

  - The CSP shall during the implementation design phase agree with the Buyer and setup the communication parameters and configuration not described in this Specification.

# Data transmission network requirements

  - The Buyer's InNOVA System will be accessible via the NewPENS access point in Vilnius, in the Buyer's technical room.

  - For that reason, the Buyer will request ATN service from NewPENS service provider with purpose to establish connection between the Buyer's and CSP and to encourage contact the NewPENS service provider and CSP to make appropriate configurations on the interconnection gateway between the NewPENS and the CSP for the datalink service.

  - The performance of the CSP network shall comply with EUROCAE ED-85A.

  - The CSP provided DCL service shall comply with AEEC specifications 622 and 623.

# Requirements for procurement documents

The supplier shall submit together with the offer:

  - Service description,

  - Service delivery diagram,

  - The necessary infrastructure and data for InNOVA System of the Buyer to install and provide the DCL service.

  - Assessment of service vulnerability testing from a cyber security point of view.

# Obligations of the Buyer

  - To provide the CSP the connection to the Buyer's InNOVA System via NewPENS and to encourage the NewPENS service provider to contact the CSP for ACARS DCL establishment and proper configuration.

  - To configure the InNOVA System in conformance to ED-85A to send responses to aircraft inquiries through the DCS provider’s infrastructure.

# Obligations of the CSP

  - To provide the Buyer with information on connecting the CSP's data communication infrastructure to the Buyer's InNOVA system.

  - The entire flow of DCL messages shall be registered by the means of CSP and shall be provided to the Buyer in a monthly report.

  - The Provider shall appoint its own responsible person who can be contacted in the event of a failure or disruption in the provision of the service and other issues related to the provision of the service during the business hours of the working days.

  - The Provider shall have the necessary means to protect the infrastructure providing the datalink communication service from any unauthorized access to this infrastructure, unauthorized connection, disconnection of communication, data modification, forgery or theft.

  - The CSP shall assure the Buyer that the necessary protective measures will be applied continuously throughout the period of service provision.

  - The Provider shall provide an opportunity for the Buyer and the competent authority supervising the  Buyer (Transport Competence Agency) to check the provision of Services in accordance with the Agreement, as set out in Regulation (EU) 373/2017 ATM/ANS.OR.B.015 "Activities carried out under the Agreement" rule and upon request by Buyer and the competent authority supervising it, provide information and documents related to the provision of Services.

# Reliability and availability

  - The service shall be provided without interruption.

  - Service availability shall be at least 99%.

  - The infrastructure of the CSP shall be resilient enough to ensure the high reliability.

  - The infrastructure shall have automatic continuous performance monitoring and control functions.

# DCL service implementation

  - The DCL service shall be implemented within the terms specified in the public procurement documents with this technical specification attached.

# Support service

  - Throughout the provision period of the DCL, the CSP shall provide support service, which include notifications of DCL disruptions or failures, informing the Buyer about planned maintenance work with the termination of the DCL provision.

  - Support service shall be provided by e-mail or telephone during business hours of working days.

  - The CSP shall respond to inquiries sent by the Buyer and eliminate DCL malfunctions or degradation on the same day as soon as it is possible.

# Documents

# The service provider shall submit documents in electronic form in Lithuanian or English:

  - Description of the service, including features, data flows, flowchart, interfaces, etc.

  - Description of service monitoring and control,

  - Assessment of service vulnerability testing in relation to cyber security.

  - Other documents confirm the service's compliance with requirements, availability, uplink success rate and uplink delivery time.

# References

  - ICAO Annex 11 Air Traffic Services, 15th Edition, July 2018.

  - ICAO Doc 4444, PANS-ATM, 16th Edition, 2016.

  - EUROCAE ED-85A Data-Link Application System Document (DLASD) for “Departure Clearance” Data-Link Service.

  - AEEC specification 622 - ATS Datalink Applications over ACARS Air-Ground Network.

  - AEEC specification 623 - Character-Oriented Air Traffic Service (ATS) Applications.

  - RFC 2351 - Mapping of Airline Reservation, Ticketing, and Messaging Traffic over IP.

# Abbreviations

| ACARS | Aircraft Communications Addressing and Reporting System |
| --- | --- |
| AEEC | Airlines Electronic Engineering Committee |
| ATN | Aeronautical Telecommunication Network |
| ATS | Air Traffic Services |
| CSP | Communication Service Provider |
| DCL | Departure Clearance Message Exchange Service |
| EUROCAE | European Organisation for Civil Aviation Equipment |
| ICAO | International Civil Aviation Organization |
| IP | Internet protocols |
| NewPENS | New pan-European Network Service |