|  | ## UAB Geoconsulting tel.: 0-612-84305, email: info@geoconsulting.lt, www.geoconsulting.lt |
| --- | --- |

Registration number with the Lithuanian Geological Survey: 50432 -2024

| Client: | Ministry of National Defence of the Republic of Lithuania |
| --- | --- |
| Facility: | Exploratory engineering geological and geotechnical investigations of land plots, with cadastral No. No. 5552/0001:557 and 5552/0001:37, Kairiai village, Klaipėda district municipality |

ENGINEERING GEOLOGICAL RESEARCH REPORT

Survey stage:	Preliminary surveys

Geotechnical category:	Second

Report issuance date: September 2024

| Contractor: UAB Geoconsulting /signature/ |  |  |
| --- | --- | --- |
| Director/Head of investigation | / Stamp / | T. Skara |
| Engineering geologist |  | A. Bičkauskienė |

Klaipėda, 2024

TABLE OF CONTENTS

Survey report summary	3

Explanatory note

1.	Introduction	4

2.	Work methodology	5

3.	General information on the construction site	6

4.	Geological structure	7

5.	Hydrogeological conditions	8

6.	Soil composition and engineering geological layers 	8

7.	Physical and mechanical properties of soil	11

8.	Geological processes and phenomena	12

9.	Findings and recommendations	12

10.	List of references	13

Text annexes

1.	Subsurface Exploration Licence No 1404841	15

2.	Technical task for engineering geological surveys	16

3.	List of coordinates and elevations of survey points	17

4.	Metrological verification of geotechnical testing (CPT) equipment		18

5.	Laboratory test results	21

6.	Abbreviations, symbols, and measurement units used in the report	24

Graphical appendices

1.	Survey site layout	 1 sheet

2.	Plan with the points of surveys	3 pages

3.	Borehole logs with geotechnical test (CPT) curves	16 pages

4.	Engineering geological sections	 3 sheets

## Survey report summary

UAB Geoconsulting conducted exploratory engineering geological surveys on the plots with cadastral No.  5552/0001:557 and 5552/0001:37, located in Kairiai village, Klaipėda district.

During the survey, boreholes were drilled by auger drilling at 16 locations, with geotechnical probing performed alongside. (CPT – TE1). The geological structure of the site up to a depth of 11.3 m consists of: vegetation layer (pdIV), Holocene eolian (vIV), biogenic (blIV) deposits and lacustrine (lIV) sediments, as well as Upper Pleistocene Baltic Ice Lake sediments, (lglIIB), Upper Pleistocene Upper Nemunas Formation marginal glacial formations, (gtIIInm3) and limnoglacial sediments. (lgIIInm3). During the survey, the groundwater table was observed at depths ranging from 0.2 to 4.3 m below ground level.

Following the interpretation of field and laboratory survey data, 30 engineering geological layers were identified (IGS) The key properties of the layers are presented in Table 1.

Table 1. Key properties of the layers.

| IGS No. | Type of soil | Stratigraphic index | Soil name | qc, MPa | % kN/m3 | φ, º | Eo, MPa |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Hu | pd IV | Hu | 0.8 | - |  |  |
| 2 3 4 5 | Sa | v IV | SaP | 1.5 | * | 30 | 2 |
|  |  |  | SaP | 3.7 | - | 33 | 11 |
|  |  |  | SaP | 6.5 | - | 36 | 50 |
|  |  |  | SaP | 12.1 | - | 37 | 93 |
| 6 7 8 9 10 11 12 13 14 15 16 | O Sa | b IV | 0: (Pt) | 0.4 | - |  |  |
|  |  | IIV | 0: (Dy) | 0.6 |  |  |  |
|  |  |  | oSaP | 1.8 |  | 30 | 3 |
|  |  |  | oSaP | 3.7 |  | 33 | 10 |
|  |  |  | SaP | 8.1 |  | 36 | 34 |
|  |  |  | SaP | 11.2 |  | 37 | 42 |
|  |  |  | SaFG | 3.1 |  | 32 | 9 |
|  |  | Ig III B | SaFP | 1.6 |  | 30 | 2 |
|  |  |  | SaFP | 3.8 |  | 33 | 11 |
|  |  |  | SaFP | 7.4 |  | 36 | 32 |
|  |  |  | SaFP | 15.2 |  | 33 | 9 |
| 17 |  |  | SaFP | 27.8 |  | 41 | 82 |
| 18 |  |  | SaP | 35.9 |  | 42 | 98 |
| 19 | a | gt III n m. | saClL | 0.4 |  |  | 4 |
| 20 |  |  | saCIL | 0.8 | 20.6 |  | 8 |
| 21 |  |  | saClL | 1.5 | - |  | 15 |
| 22 |  |  | saCIL | 3.8 | 22.1 |  | 34 |
| 23 |  |  | saCIL | 5.3 | - |  | 44 |
| 24 | Sa |  | siSa | 6.5 | • | 32 | 28 |
| 25 |  | Ig III n m3 | siSa | 7.8 | - | 33 | 33 |
| 26 |  |  | siSa | 15.2 | - | 35 | 53 |
| 27 |  |  | siSa | 22.1 | - | 37 | 70 |
| 28 | Si |  | saSiL | 3.0 | - | - | 14 |
| 29 |  |  | saSiL | 13.7 | 21.4 | - | 63 |
| 30 | d-Si |  | OL-SiL | 12.7 | 21.0 | - | 61 |

## Explanatory note

1.	INTRODUCTION

UAB Geoconsulting conducted exploratory engineering geological surveys on the plots with cadastral No.  Nr. 5552/0001:557 and 5552/0001:37, located in Kairiai village, Klaipėda district.

The purpose of the survey is to determine the geological conditions of the site, i.e., to obtain initial (preliminary) information about the geological structure of the site where the client intends to carry out construction, distinguishing engineering geological layers (IGS) and determining their geotechnical probing values and statistical indicators.

Client of the survey: State Border Guard Service at the Ministry of Interior of the Republic of Lithuania

Survey area boundary coordinates:

| Number | X | Y |
| --- | --- | --- |
| 1 | 6169820 | 323653 |
| 2 | 6169924 | 323839 |
| 3 | 6169274 | 324255 |
| 4 | 6169359 | 324608 |
| 5 | 6169221 | 324657 |
| 6 | 6169058 | 324397 |
| 7 | 6169161 | 324329 |
| 8 | 6169004 | 324035 |
| 9 | 6169082 | 323987 |
| 10 | 6168821 | 323531 |
| 11 | 6169188 | 323353 |

Permit number for subsurface surveys: 1404841. Date 01/07/2020

Fieldwork carried out on 30 July this year. Work performers:

•	Engineer geologist Tomas Skara – Fieldwork;

•	Driller Sigitas Linkis – Fieldwork;

•	Geologist Toma Dagytė – Laboratory soil testing;

•	Engineering geologist A. Bičkauskienė – interpretation of survey data and report preparation.

During the survey, boreholes were drilled using an auger drilling method at 16 locations, 11 soil samples were collected, and geotechnical probing was performed nearby. (CPT – TE1).

The soils identified during the fieldwork were described in accordance with the LST EN ISO 14688-1:2018 [2] standard and classified according to the LST EN ISO 14688-2:2018 [3] standard and the Classification of Soils for Engineering Geological and Geotechnical Surveys, approved by Order No. 1-175 of 13 June 2019 of the Director of the Lithuanian Geological Survey under the Ministry of Environment “On the Approval of the Classification of Soils for Engineering Geological and Geotechnical Surveys” and its amendment [7], [8] .

The description and methodology of the work are provided in Section 2.

2.	WORK METHODOLOGY

The methods for exploratory surveys were selected in order to:

1.	Identify and determine the boundaries of soil layers of different compositions and physical states, as well as measure the depths of their tops and bases;

2.	Measure the depth of the groundwater level.

Cone Penetration Test (CPT) (CPT, TE1)

The CPT probe is pressed into the soil using a “stand-alone” type penetrometer (compression force of 100 kN, extraction force of 120 kN, stroke length of 1200 mm, penetration speed during CPT testing of 20 ± 5 mm/s), which is anchored to the ground surface using soil anchors.

During the CPT test, the following parameters are directly measured and recorded in a computer at 1 cm intervals: cone resistance, sleeve lateral friction resistance, pore water pressure (u2 type, only for TE2 tests), probe inclination angle, penetration speed, and probing depth. The measurements are conducted using the Geomil system, which consists of:

a)	CPTU “subtraction” type probe S10CFIIP.S241192 Cone base area 10 cm2, Cone angle 600, Cone diameter 35.7 mm, Sleeve friction area 150 cm2, Maximum cone load 100 kN, Maximum sleeve friction load 15 kN, Maximum pore water pressure load (20 bar) Allowable overload for all sensors 150%), Metrological verification provided in Text Annex 3

b)	Probing rods (diameter 32 mm, length 1 m)

c)	Data logger Depth gauge Data interface GME500, Probing cable (30 m) Panasonic field computer CF-19);

d)	Software (CPTest).

The tests were carried out in accordance with the requirements of LST EN ISO 22476-1 [4].

Drilling work, initial soil classification, and sampling principles

The boreholes were drilled using an auger drilling method with a VTX 800 drilling rig (90 mm diameter) and an interval-based closed-type soil sampler. Drilling was performed in 1–2 m intervals. The drill core was visually inspected on-site, and an initial soil identification was conducted to determine the primary fraction and describe the secondary fractions [2]. In this way, the soil is assigned to one of six types that most commonly describe its main geotechnical properties: boulders, gravel, sand, silt, and clay. If the soil consists of organic materials, it is classified as organic soil.

Laboratory tests

The laboratory tests of soil samples were conducted by the UAB Geoconsulting laboratory. A summary table of the test results is provided in Text Annex 5. Based on the soil type determined during the initial identification, appropriate testing methods were selected for precise classification into categories:

•	Grading (gravel, sand, silt, and clay). For very coarse soils, testing is not performed.

•	Natural density (fine soils), particle density (fine and coarse soils),

•	Natural, liquidity, (fine and coarse soils), and plasticity moisture content (fine soils).

Preparation of the report

The survey report was prepared in accordance with regulatory documents [1-5] and recommendations [6]. Software used GME CPTask v1.20, Cpet-it v.1.6.0.43, Microsoft Office (Word, Exel), Autocad2011LT. The methodology for constructing the geological model is described below.

The soils identified during the fieldwork were described in accordance with the LST EN ISO 14688-1:2018 [2] standard and classified and named according to the LST EN ISO 14688-2:2018 [3] standard and the requirements of the “Classification of Soils for Engineering Geological and Geotechnical Surveys” [7, 8].

As mentioned in the introduction, one of the main objectives of the engineering geological (IG) surveys is to develop a foundation calculation scheme by distinguishing engineering geological layers (IGS). The differentiation of these layers is based not only on geological characteristics but also on uniformity in composition, physical state, and properties. Composition uniformity is determined by correlating the initial soil classification with laboratory test results. The correlation outcome is the final soil classification according to [3] and [7].

After the IGS classification by composition, the geological model is further detailed. Based on the cone resistance values, the soils are divided into layers of different physical states (sands by density, silts, and clays by strength) [6], and layer boundaries (top and base) are refined using mathematical statistical methods. Model verification is carried out by integrating the test data from all survey points using the Cpet-it software, where statistical parameters (mean and extreme values) are calculated for each IGS within the obtained dataset.

1.	The specific gravity γ is calculated as follows:

γ=ρ*g [kN/m3];

g = free-fall gravitational acceleration. [m/s2]

2.	The effective internal friction angle φ' is determined for gravel and sand soils according to the methodology specified in Eurocode 7 (Annex D, Table D1).

3.	The modulus of deformation Eo is calculated according to the following dependencies: [6]:

For loose uncompacted and organic soil	Eo = qc ;

For very loose sand and gravel	Eo = 1.5 qc ;

For loose sand and gravel	Eo = 3.0 qc ;

For medium-dense and dense sand.	Eo = 3.0 qc 0.71;

For moraine fine-grained soils (sandy clayey silt or sandy silty clay):

when qc < 2,5 MPa,	Eo = 10.0 ф;

when qc > 2,5 MPa,

For non-moraine silty clay, sandy silty clay	Eo = 7.0 qc

For moraine clayey or silty sand (plastic soil)

and non-moraine silt	Eo = 5.0 qc

For pure clay (Cl)	Eo = 8.2 qc -3.1;

Remark: the minimum cone resistance value is used in the formulas. qcmin.

3.	GENERAL INFORMATION ABOUT THE CONSTRUCTION SITE

Natural conditions

The survey site is located in the Klaipėda district municipality, in the village of Kairiai. Geomorphologically, the area belongs to the Holocene and late glacial period as well as the last glaciation, falling within the geomorphological regions of the Samogitian–Courland and Baltic Sea basins, specifically in the micro-regions of the Western Samogitian Plain and the Curonian Lagoon Basin, including the Rimkai moraine ridge fragment and the Dreverna marine plain.

The absolute elevations at the survey sites range from 4.3 to 10.8 m.

The survey area includes several genetic types of terrain. The ground surface slope does not exceed 10°. There are no erosional, thermokarst, suffosional, or other negative landforms on the site. The distance to unstable slopes and eroding banks is more than 100 m.

The standard depth of seasonal frost penetration is up to 1.5 m for clayey soils and 1.2 m for sandy soils.

4.	GEOLOGICAL STRUCTURE

The geological structure of the site up to a depth of 11.3 m consists of: vegetation layer (pdIV), Holocene eolian (vIV), biogenic (bIV) deposits and lacustrine (lIV) sediments as well as Upper Pleistocene Baltic Ice Lake sediments, (lg III B), Upper Pleistocene Upper Nemunas Formation marginal glacial formations, (gtIIInm3) and limnoglacial sediments. (lgIIInm3).

The topsoil layer (pdIV) consists of Soil (Hu), peaty, sandy soil in some areas. The complex is separated at all monitoring points. Its thickness ranges from 0.1 to 0.5 m.

Holocene eolian deposits (vIV) consist of: Well-sorted sand (SaP), grey, brown gray, dark grey, light grey, brown, yellowish, light brown, dark brown, with admixture of organic matter, locally with soil inclusions, moist – water-bearing. This complex was identified at all survey points except No. 4 and 16. Its thickness ranges from 0.5 to 4.4 m.

Holocene biogenic deposits (bIV) consist of organic soil (O): peat (Pt), dark brown, black, locally with sand lenses, sandy, water-saturated, poorly to well-decomposed. This complex was identified at all survey points except No. 5, 9, and 11–16. Its thickness ranges from 0.2 to 3.0 m.

Holocene lacustrine sediments (IIV) consist of:

Organic soil (O): Ooze (Dy), greenish-grey, locally sandy, with peat inclusions, water-saturated;

Well-sorted sand with a small admixture of organic matter (oSaP), grey, brown gray, light brown, brown, dark brown, dark grey, moist – water-bearing;

Well-sorted sand (SaP), grey, water-bearing.

Lowly dusty – clayey sand of varying sorting (SaFG), grey, with organic admixture -watery.

This complex was identified at survey points No. 1–9. Its thickness ranges from 0.5 to 3.3 m.

Upper Pleistocene Baltic Sea glacial lake sediments (lgIIIB) consist of:

Well-sorted, low-dust – clayey sand (SaFP), light grey, grey, brown, dark brown, brown gray, light brown, with gravel admixture, locally with sandy low-plasticity moraine clay inclusions, water-bearing.

Well-sorted sand (SaP), light grey, -watery.

This complex was identified at survey points No. 1, 3–6, and 9–14. Its thickness ranges from 0.5 to 8.7 m.

Upper Pleistocene Upper Nemunas formation marginal glacial deposits (gtIIInm3) consist of:

Sandy low-plasticity moraine clay (saClL), grey, brown gray, brown, with gravel and pebbles up to 5%;

Silty sand (siSa), grey, water-bearing.

This complex was identified at all survey points except No. 10–14. Its thickness ranges from 0.5 to 7.4 m.

Upper Pleistocene Upper Nemunas formation marginal glacial deposits (gtIIInm3) consist of:

Dusty sand (siSa), grey, water-bearing.

Sandy low-plasticity silt (saSiL), greyish-brown;

Low-plasticity clay and silt (ClL-SiL), gray-brown, brown gray, with sandy, low-plasticity silt inclusions.

This complex was identified at survey points No. 4, 8, and 15–16. Its thickness ranges from 0.3 to 7.4 m.

In summary of the survey results, it can be concluded that the subsurface soil consists of eolian, biogenic deposits, and marginal glacial formations. A total of 11 lithological soil types were identified. No pre-Quaternary rocks were found. Relatively weak layers include soil, peat, silt, very loose and loose sand, very weak and weak sandy low-plasticity clay moraine layers, which are found throughout the survey area at depths ranging from 1.8 to 9.6 m. The cross-section features subhorizontal, continuous, and discontinuous layers. No buried palaeorelief forms were identified.

5.	HYDROGEOLOGICAL CONDITIONS

Within the survey area, during the study, the groundwater horizon was found at depths of 0.2 to 4.3 m from the ground surface (2.3 to 8.9 m absolute elevation). Subsurface water has accumulated in layers of well-sorted, low-silt clayey sand, well-sorted sand, variably sorted low-silt clayey sand, and silty sand. Depending on seasonal variations, the groundwater level may fluctuate by 0.5–1.0 m, rising during the wet season and dropping during the dry season. During the surveys, water accumulation was observed in some areas on the ground surface. It is likely that groundwater drains towards the Curonian Lagoon, discharging along the way into the King Wilhelm Canal and small water ponds.

Laboratory tests determined the filtration coefficients of coarse soil samples collected during fieldwork, with values as follows: silty sand – 0.8 m/d, well-sorted sand – 17.4–18.7 m/d, well-sorted low-silt clayey sand – 16.0 m/d, variably sorted low-silt clayey sand – 3.2 m/d, and well-sorted sand with a low organic content – 16.9 m/d (Table 3, Graphic Annex 5).

During construction, surface runoff and groundwater will accumulate in excavations and boreholes.

6.	SOIL COMPOSITION AND ENGINEERING GEOLOGICAL STRATA

Analysis of field survey data identified 30 engineering geological layers. (IGS) Layer descriptions are provided in Table 2.

Table 2. IGS Geological Description

| IGS No. | Layer Geological Description (according to [2]) |
| --- | --- |
| IGS 1 | Soil (Hu), peaty in places, sandy. The layer identified in all survey points. Its thickness ranges from 0.1 to 0.5 m. |
| IGS 2 | Well-sorted sand (SaP), grey, brown gray, dark grey, light grey, brown, light brown, dark brown, with admixture of organic matter, locally with soil inclusions, moist – water-bearing, very loose. The layer is identified in survey points. Nr. 1, 2, 3, 7, 8, 11, 15. Its thickness ranges from 0.3 to 1.1 m. |
| IGS 3 | Well-sorted sand (SaP), brown, light brown, brown, brown gray, light grey, yellowish, locally with soil/organic admixture, moist, friable. The layer is identified in survey points Nr. 1.5 – 7.9 – 14. Its thickness ranges from 0.3 to 2.3 m. |
| IGS 4 | Well-sorted sand (SaP), light brown, brown, yellowish, locally with organic admixture, moist – water-bearing, medium density. The layer is identified in survey points. Nr. 1, 5, 7 – 10. Its thickness ranges from 0.3 to 2.1 m. |
| IGS 5 | Well-sorted sand (SaP), brown, light brown, yellowish, moist – water-bearing, dense. The layer is identified in survey points. Nr. 5 and Nr. 8, 9. Thickness of this layer ranges from 0.4 to 0.8 m. |
| IGS 6 | Organic soil (O): peat (Pt), dark brown, black, locally with sand lenses, sandy, water-saturated, poorly to well-decomposed. The layer is identified in all survey points except No. 5, 9, 11–16. Layer thickness – 0.2–3.0 m. |
| IGS 7 | Organic soil (O): Ooze (Dy), greenish-grey, locally sandy, with peat inclusions, water-saturated. The layer is identified in survey points. Nr. 1-4, 7 – 8. Its thickness ranges from 0.2 to 1.6 m. |
| IGS 8 | Well-sorted sand with a small admixture of organic matter (oSaP), grey, brown gray, light brown, brown, dark brown, dark grey, moist – water-bearing, very loose. This layer identified in survey points. Nr. 2 – 3, 5 – 6, 8 – 9. Its thickness ranges from 0.4 to 1.0 m. |
| IGS 9 | Well-sorted sand with a small admixture of organic matter (oSaP), grey, dark brown, moist – water-bearing, friable. The layer is identified in survey points. Nr. 2, 6. Layer thickness – 0,5 – 0,9 m. |
| IGS 10 | Well-sorted sand (SaP), grey, watered, medium density. The layer is identified in the survey point. No 7 Its thickness is 1.0 m. |
| IGS 11 | Well-sorted sand (SaP), grey, watered, dense. The layer is identified in the survey point. No 7 Its thickness is 0.8 m. |
| IGS 12 | Lowly dusty – clayey sand of varying sorting (SaFG), grey, with organic admixture watered, friable. The layer is identified in the survey point. No 1 Its thickness is 0.4 m. |
| IGS 13 | Well-sorted, low-dust – clayey sand (SaFP), grey, with gravel admixture in places, watered, very loose. The layer is identified in survey points. Nr. 1, 4. Its thickness ranges from 0.4 to 0.5 m. |
| IGS 14 | Well-sorted, low-dust – clayey sand (SaFP), grey, light grey, brown, light brown, brown gray, with gravel admixture, moist – water-bearing, friable. The layer is identified in survey points. Nr. 3, 6, 10, 14. Layer thickness – 0.4 – 1.1 m. |
| IGS 15 | Well-sorted, low-dust – clayey sand (SaFP), grey, light grey, brown, dark brown, light brown, brown gray, with gravel admixture in places, watered, medium density. The layer is identified in survey points. Nr. 1, 3, 5, 6, 9, 10, 11, 13, 14. Layer thickness – 0.4 – 1.4 m. |
| IGS 16 | Well-sorted, low-dust – clayey sand (SaFP), grey, light grey, brown, brown gray, light brown, with gravel admixture in places, locally with sandy low-plasticity moraine clay inclusions, watered, dense. The layer is identified in survey points. Nr. 5, 6, 9 – 13. Layer thickness – 0.3 – 2.2 m. |
| IGS 17 | Well-sorted, low-dust – clayey sand (SaFP), light grey, grey, brown, dark brown, brown gray, light brown, with gravel admixture, locally with sandy low-plasticity moraine clay inclusions, watered, very dense. The layer is identified in survey points. Nr. 5, 6, 9 – 14. Layer thickness – 0.4 – 5.0 m. |
| IGS 18 | Well-sorted sand (SaP), light grey, watered, very dense. The layer is identified in survey points. Nr. 13, 14. Layer thickness – 0.6 – 5.9 m. |
| IGS 19 | Sandy low-plasticity moraine clay (saClL), brown, brown gray, grey, with gravel and cobble up to 5% very weak. The layer is identified in survey points. Nr. 4, 8, 16. Layer thickness – 0.5 – 1.0 m. |
| IGS 20 | Sandy low-plasticity moraine clay (saClL), grey, brown, with gravel and cobble up to 5% weak. The layer is identified in survey points. Nr. 1 – 3, 8, 15. Layer thickness – 0.3–1.7 m. |
| IGS 21 | Sandy low-plasticity moraine clay (saClL), grey, brown gray, brown, with gravel and cobble up to 5% medium strength. The layer is identified in survey points. Nr. 1 – 3, 6, 7, 9, 15, 16. Layer thickness – 0.4 – 2.2 m. |
| IGS 22 | Sandy low-plasticity moraine clay (saClL), grey, brown gray, with gravel and cobble up to 5% strong. The layer is identified in survey points. Nr. 5, 6, 8. Its thickness ranges from 0.3 to 1.0 m. |
| IGS 23 | Sandy low-plasticity moraine clay (saClL), grey, with gravel and cobble up to 5% very strong. The layer is identified in the survey point. No 16 Its thickness is 0.5 m. |
| IGS 24 | Dusty sand (siSa), grey, watered, medium density. The layer is identified in the survey point. No 2 Layer thickness – 0.3 m. |
| IGS 25 | Dusty sand (siSa), grey, watered, medium density. The layer is identified in the survey point. No 4 Its thickness is 4.0 m. |
| IGS 26 | Dusty sand (siSa), grey, watered, dense. The layer is identified in survey points. No 4, 15, 16. Layer thickness – 0.6 – 3.9 m. |
| IGS 27 | Dusty sand (siSa), grey, watered, very dense. The layer is identified in the survey point. No 16 Layer thickness 0.3 m. |
| IGS 28 | Sandy low-plasticity silt (saSiL), gray-brown, strong. The layer is identified at No. 4. Layer thickness – 0.5 m. |
| IGS 29 | Sandy low-plasticity silt (saSiL), gray-brown, very strong. The layer is identified in the survey point. No 4, 15. Its thickness ranges from 0.5 to 1.1 m. |
| IGS 30 | Low-plasticity clay and silt (ClL-SiL), gray-brown, brown gray, with sandy low-plasticity silt inclusions, very strong. The layer is identified in survey points. No 8, 15. Its thickness ranges from 0.3 to 2.8 m. |

7. PHYSICAL AND MECHANICAL PROPERTIES OF SOILS

The geotechnical probing values of the identified engineering geological layers (IGS), key statistical indicators, and summary values of physical and mechanical properties are presented in Table 3.

Table 3. Summary table of geotechnical probing values, key statistical indicators, and physical and mechanical properties of soils.

|  | IGS No. | Type of soil | Stratigraphic index | Soil name | qc, MPa | n | S | qcmin MPa | % kN/mJ | ρ1, Mg/m3 | ρ2, Mg/m3 | k, m/d | w, % | w, % | w, % | Ip, % | Ic, vnt.d. | φ, º | Eo, MPa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Hu | pd IV | Hu | 0.8 | 306 | 0.34 | 0.7 | - |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  |  | SaP | 1.5 | 598 | 0.70 | 1.5 | - | - | - | - | - | - | - | - | - | 30 | 2 |
|  | 3 | Sa | v IV | SaP | 3.7 | 1710 | 1.07 | 3.7 | - | - | 2.65** | 18.5** | 5.8** | - | - | - | - | 33 | 11 |
|  | 4 |  |  | SaP | 6.5 | 609 | 1.32 | 6.4 | - | - | - | - | - |  | - | - | - | 36 | 50 |
|  | 5 |  |  | SaP | 12.1 | 183 | 1.69 | 11.9 | - | - | - | - | - |  | - | - | - | 37 | 93 |
|  | 6 7 | o | b IV | 0: (Pt) 0: (Dy) | 0.4 0.6 | 1119 629 | 0.39 0.74 | 0.4 0.5 |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  | oSaP | 1.8 | 434 | 0.67 | 1.7 | - | - | - | - | - |  | - | - | • | 30 | 3 |
|  | 9 |  | IIV | oSaP | 3.7 | 142 | 1.29 | 3.5 | - | - | 2.66* | 16.9* | 10.4* |  | - | - | - | 33 | 10 |
|  | 10 |  |  | SaP | 8.1 | 101 | 1.41 | 7.9 | - | - | - | - | - |  | - | - | - | 36 | 34 |
|  | 11 |  |  | SaP | 11.2 | 81 | 2.64 | 10.7 | - | - | - | - | - |  | - | • | - | 37 | 42 |
|  | 12 |  |  | SaFG | 3.1 | 41 | 0.88 | 2.9 | - | - | 2.65* | 3.2’ | 21.0* |  | - | - | - | 32 | 9 |
|  | 13 | Sa |  | SaFP | 1.6 | 82 | 0.48 | 1.5 | - | - | - | - | - |  | - | - | - | 30 | 2 |
|  | 14 |  |  | SaFP | 3.8 | 274 | 1.22 | 3.6 | - | - | - | - | - |  | - | - | - | 33 | 11 |
|  | 15 |  | Ig III B | SaFP | 7.4 | 957 | 1.70 | 7.3 | - | - | - | - | - |  | - | * | - | 36 | 32 |
|  | 16 |  |  | SaFP | 15.2 | 1554 | 15.07 | 3.1 | - | - | 2.65* | 16.0* | 15.3* |  | - | - | - | 33 | 9 |
|  | 17 |  |  | SaFP | 27.8 | 1782 | 7.06 | 27.5 | - | - | - | - | - |  | - | - | - | 41 | 82 |
|  | 18 |  |  | SaP | 35.9 | 782 | 9.77 | 35.3 | - | - | 2.65* | 17.4* | 17.4* |  | - | - | - | 42 | 98 |
|  | 19 |  |  | saClL | 0.4 | 177 | 0.12 | 0.4 | - | - | - | - | - |  | - | - | - | - | 4 |
|  | 20 |  |  | saClL | 0.8 | 508 | 0.16 | 0.8 | 20.6 | 2.10* | 2.69* | - | 16.1* | 25.8* | 15.1* | 10.7* | 0.10* | - | 8 |
|  | 21 | a | gtIInm3, | saClL | 1.5 | 1796 | 0.68 | 1.5 | - | - | - | - | - |  | - | - | - | - | 15 |
|  | 22 |  |  | saClL | 3.8 | 107 | 1.26 | 3.6 | 22.1 | 2.25’ | 2.69* | - | 11.8* | 25.4* | 11.6* | 13.8* | 0.02* | - | 34 |
|  | 23 |  |  | saOL | 5.3 | 41 | 0.79 | 5.1 | - | - | - | - | - |  | - | - | - | - | 44 |
|  | 24 |  |  | siSa | 6.5 | 62 | 1.64 | 6.1 | - | - | - | - | - |  | - | - | - | 32 | 28 |
|  | 25 |  |  | siSa | 7.8 | 402 | 1.61 | 7.7 | - | - | - | - | - |  | - | - | - | 33 | 33 |
|  | 26 | ьа |  | siSa | 15.2 | 553 | 3.06 | 15.0 | - | - | 2.66* | 0.8* | 18.1* |  | - | - | - | 35 | 53 |
|  | 27 |  | Ig IIInnm3 | siSa | 22.1 | 102 | 1.49 | 21.8 | - | - | - | - | - |  | - | * | - | 37 | 70 |
|  | 28 | Si |  | saSiL | 3.0 | 64 | 1.26 | 2.8 | - | - | - | - | - |  | • | - | - | - | 14 |
|  | 29 |  |  | saSiL | 13.7 | 159 | 8.05 | 12.7 | 21.4 | 2.18* | 2.67* | - | 18.7* | 19.8* | 17.1* | 2.7* | 0.58* | - | 63 |
|  | 30 | a-si |  | OL-SiL | 12.7 | 304 | 6.65 | 12.1 | 21.0 | 2.14* | 2.68* | - | 19.1* | 21.4* | 17.2* | 4.2* | 0.45* | - | 61 |

* – Laboratory test results are presented.

** – The arithmetic mean of laboratory test results is provided.

8.	GEOLOGICAL PROCESSES AND PHENOMENA

No modern physical and geological processes that could negatively impact the construction, reconstruction, and operation of buildings in the area were identified. However, a relatively high groundwater level may have a negative impact on construction, reconstruction, and operation of buildings.

Based on yj karst suffosion risk, the area is classified as non-hazardous.

9.	CONCLUSIONS AND RECOMMENDATIONS

3.	The survey site is located in the Klaipėda district municipality, in the village of Kairiai. Geomorphologically, the area belongs to the Holocene and late glacial period as well as the last glaciation, falling within the geomorphological regions of the Samogitian–Courland and Baltic Sea basins, specifically in the micro-regions of the Western Samogitian Plain and the Curonian Lagoon Basin, including the Rimkai moraine ridge fragment and the Dreverna marine plain.

4.	The absolute elevations at the survey sites range from 4.3 to 10.8 m.

5.	The geological structure of the site up to a depth of 11.3 m consists of: vegetation layer (pdIV), Holocene eolian (vIV), biogenic (bIV) deposits and lacustrine (lIV) sediments also the Upper Pleistocene Baltic Ice Lake sediments (lgIIIB), Upper Pleistocene Upper Nemunas Formation marginal glacial formations, (gtIIInm3) and limnoglacial sediments. (lgIIInm3).

6.	No modern physical and geological processes that could negatively impact the construction, reconstruction, and operation of buildings in the area were identified. However, a relatively high groundwater level may have a negative impact on construction, reconstruction, and operation of buildings.

7.	Within the survey area, during the survey, the groundwater aquifer was found at a depth of 0.2–4.3 m from the ground surface (2.3–8.9 m a.s.l.).

8.	The geological model of the site identified a total of 30 engineering geological layers (IGS). Layer deposition conditions are shown in borehole lithological columns (Graphic Annex 3) and engineering geological sections (graphical annex 4).

9.	Calculated values of the physical and mechanical properties of IGS soils are provided in Section 7 of the report (Table 4).

10.	The soil layers identified in the study area, IGS 1 – IGS 3, IGS 6 – IGS 9, IGS 12 – IGS 14, and IGS 19 – IGS 20, consist of very weak and weak soils, found at depths ranging from 1.8 to 9.6 m below the existing ground surface.

11.	Silty soils (IGS 24 – IGS 29) may exhibit thixotropic properties, meaning they can be sensitive to dynamic loads and, under specific conditions, may lose their defined physical and mechanical properties (liquefying under vibration).

12.	If, during construction, the presented geological model is found to be inconsistent with the actual conditions, the contractor must be informed immediately.

Engineering geologist							A. Bičkauskienė

/signature/

10.	BIBLIOGRAPHY

1.	STR. 1.04.02:2011 “Engineering Geological and Geotechnical Surveys.” Official Gazette, 2012-01-07, No. 5-144.

2.	LST EN ISO 14688-1: 2018. Geotechnical investigation and testing. Identification and classification of soil. Part 1. Identification and description.

3.	LST EN ISO 14688-2: 2018. Geotechnical investigation and testing. Identification and classification of soil. Part 2. Principles for classification.

4.	LST EN ISO 22476-1. Geotechnical investigation and testing. Field testing. Part 1. Press test using an electric and piezoelectric cone.

5.	LST EN 1997-2. Eurocode 7. Geotechnical design. Part 2. Ground investigations and testing.

6.	Recommendations for project-related engineering geological and geotechnical surveys. TAR, 201511-16, Nr. 18162.

7.	Classification of soils for engineering geological and geotechnical surveys, approved by Order No. 1-175 of 13 June 2019 of the Director of the Lithuanian Geological Survey under the Ministry of Environment “On the Approval of the Classification of Soils for Engineering Geological and Geotechnical Surveys”

8.	Order No. 1-216 of 13 May 2024 of the Director of the Lithuanian Geological Survey under the Ministry of Environment “On Amendments to Order No. 1-175 of 13 June 2019 of the Director of the Lithuanian Geological Survey under the Ministry of Environment “On the Approval of the Classification of Soils for Engineering Geological and Geotechnical Surveys'“

### TEXT APPENDICES

The document was electronically signed by

GIEDRIUS,GIPARAS

Date: 2020-07-01 11:13:57

APPROVED BY

Order No. I-207 of 11 June 2020 of the Director of the Lithuanian Geological Survey under the Ministry of Environment

LITHUANIAN GEOLOGICAL SURVEY UNDER THE MINISTRY OF THE ENVIRONMENT

PERMIT FOR EXPLORATION OF THE UNDERGROUND

No 1404841 of 2020-07-01

Vilnius

#### UAB Geoconsulting

(The legal entity's data is collected and stored in the Register of Legal Entities, code 141884781, address, Klaipėda City Municipality, Klaipeda city, Žolynu g. 29-1)

Authorised activities:

Exploration and prospecting of non-metallic mineral resources.

Exploration and prospecting of valuable minerals.

Engineering geological (geotechnical) surveys.

Environmental geological surveys.

Environmental geological mapping.

Geological mapping.

Geochemical mapping.

Engineering geological mapping.

Mapping of mineral resource reserves.

| Director |  | Giedrius Giparas |  |
| --- | --- | --- | --- |
| (job title) | L.S. | (signature) | (name and surname) |

Text Annex 2

Ministry of National Defence of the Republic of Lithuania

Name of the compiler

TERMS OF REFERENCE

2024-07-19	07/19-1

Document date 				Document registration number

Stage of IGG surveys (underline): exploratory, project-related, additional, control.

Name of the subject of the surveys: Exploratory engineering geological and geotechnical surveys. Address of the survey site (municipality, eldership, settlement, street, building number): land plots, cadastral Nr. 5552/0001:557 and 5552/0001:37, Kairiai village, Klaipėda district municipality Client details: Ministry of National Defence of the Republic of Lithuania. Totorių g., 25, LT-01121 Vilnius; tel.: +370 5 2735 570: e-mail: kam@kam.lt

Designer details: To be selected later

Type of construction (underline): new construction, reconstruction, major renovation, other (the surveys are conducted to determine the geological conditions of the plots)

Purpose of the construction works. To be specified during the construction project preparation.

Structure category (underline): To be specified during the preparation of the construction project

Code of the Register of Immovable Cultural Properties (if applicable): none

Geotechnical category (for project-related surveys): (underline): first, second, third.

Data on structure parameters (length, width, height, depth, area):

Transferred foundation loads and their intensity:

| Survey area boundary coordinates: |  |  |
| --- | --- | --- |
| Number | X | Y |
|  | 6169820 | 323653 |
| S | 6169924 | 323839 |
| 3 | 6169274 | 324255 |
| 4 | 6169359 | 324608 |
| 5 | 6169221 | 324657 |
| 6 | 6169058 | 324397 |
| 7 | 6169161 | 324329 |
| 8 | 6169004 | 32.4035 |
| 9 | 6169082 | 323987 |
| 10 | 6168821 | 323531 |
| 11 | 6169188 | 323353 |

Additional geotechnical parameters and soil requirements determined:

1. 16 boreholes drilled to a depth of 10 m. Static probing tests conducted adjacent to them. Measured the stabilised groundwater level and filtration properties of coarse soils. Determined the physical properties of the soils.

List of normative documents governing the surveys:

I. STR 1.04.02:2011. “Engineering Geological and Geotechnical Surveys.”

Previous geological surveys conducted on the site: no data

Client: Ministry of National Defence of the Republic of Lithuania. Ernestas [illegible] 2024-07-12 /signature/

full name, signature, date

Project manager: To be selected later

full name, signature, date

Survey manager (assignment received) UAB Geconsulting Tomas Skara /signature/ 2024-07-23

full name, signature, date

Text Annex 3

List of coordinates and elevations of survey points

| Survey point and its number | Coordinates (LKS’94) |  | Altitude, m |
| --- | --- | --- | --- |
|  | X | Y | Z |
| 1 | 6169751 | 323800 | 5,1 |
| 2 | 6169519 | 323889 | 4,5 |
| 3 | 6169400 | 323953 | 4,7 |
| 4 | 6169082 | 324142 | 4,3 |
| 5 | 6169655 | 323659 | 6,5 |
| 6 | 6169480 | 323782 | 5,8 |
| 7 | 6169298 | 323876 | 5,7 |
| 8 | 6169139 | 323970 | 5,6 |
| 9 | 6169376 | 323601 | 5,0 |
| 10 | 6169215 | 323695 | 5,8 |
| 11 | 6169037 | 323812 | 5,3 |
| 12 | 6169269 | 323409 | 4,6 |
| 13 | 6169104 | 323521 | 5,6 |
| 14 | 6168929 | 323634 | 5,3 |
| 15 | 6169179 | 324418 | 7,6 |
| 16 | 6169305 | 324578 | 10,8 |

Coordinate system – state coordinate system (LKS’94)

Elevation system – LAS'07.

4.1 Text Annex

4.2Text Annex

4.3 Text Annex

#### 5.1 Text Annex

Summary form of laboratory tests on soil physical properties

| Facility: | Exploratory engineering geological and geotechnical surveys on land plots, cadastral Nr. 5552/0001:557 and 5552/0001:37 Kairiai village, Klaipėda district municipality |
| --- | --- |
| Date | 14-08-2024 / |
| Performed by: | Engineering geologist. T. Dagyte |

Summary Table of Soil Physical Properties

| Sample No. | Sampling depth m | Granulometric composition (soil retained on sieve) % |  |  |  |  |  |  |  |  |  |  | Density, Mg/nv |  |  | Moisture content, % | Filtration coefficient rrVd | Aterberg nbos, % |  |  |  | Soil name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sieve mesh size, mm |  |  |  |  |  |  |  |  |  | Dust/clay content % |  |  |  |  |  |  |  |  |  |  |
|  |  | 31,5 | 15 | 8 | 4 | 2 | 1 | 0.5 | 0.25 | 0,125 | 0,063 |  | ρ | ρ1 | ρ2 | w | k | wL | wp | lp | lL |  |
| 1/1 | 0.9-1.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 2.7 | 61.3 | 33,0 | 1.2 | 1.30.4 |  | - | 2,65 | 5.0 | 18.2 | - | - | - | - | SaP |
| 1/2 | 4 8-5.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.8 | 5.1 | 12.3 | 51.8 | 19,0 | 0.9 | 980.3 |  |  | 2.65 | 21.0 | 3.2 |  | - |  |  | SaFG with org adm. 1.07% |
| 1/3 | 70-7.2 | 0.0 | 0.0 | 1.9 | 0.5 | 1.5 | 1.5 | 1.8 | 5,0 | 13.7 | 12,5 | 45.8/15.7 | 2.10 | 181 | 2.69 | 16.1 | - | 25.8 | 15.1 | 10.7 | 0,10 | saClL |
| 6/1 | 2.0-2.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.6 | 50.5 | 41.6 | 1.4 | 1.6/0.2 | - | - | 2.66 | 10.4 | 16.9 | - | - |  | - | OSaP with org adm. 2.78% |
| 8/1 | 7.1-7.3 | 0,0 | 0.0 | 0.0 | 1.1 | 1.3 | 2,3 | 2.7 | 5,2 | 12.6 | 15.0 | 46.8/13.0 | 2.25 | 201 | 2.89 | 11.8 | • | 25.4 | 11.6 | 13.8 | 0.02 | saCIL |
| 82 | 7.7-7.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.5 | 1,3 | 9.1 | 8.6 | 63.9/16.6 | 2.14 | 1.80 | 2.68 | 19.1 | - | 21,4 | 172 | 42 | 0.45 | CIL-SIL |
| 12/1 | 1.0-1.3 | 0.0 | 0.0 | 0.0 | 0.0 | 2.0 | 5.1 | 11.5 | 56.5 | 19.4 | 1.2 | 3.9Ю.4 | - | -• | 2.65 | 6.5 | 18.7 | - | - |  | - | SaP |
| 12/2 | 2.8-3.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 7.3 | 21.2 | 50.6 | 12.5 | 2,1 | 6.2/0.3 | - | - | 2.65 | 15.3 | 16.0 | - | - | - | - | SaFP |
| 12/3 | 95-9.8 | 0.0 | 0.0 | 0.0 | 0.0 | 1.4 | 4.0 | 14.7 | 50.4 | 23.8 | 1.6 | 3 5/0 5 | - | - | 2.65 | 17.4 | 17.4 | - | - |  | - | SaP |
| 15/1 | 7.0-7.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.8 | 0,8 | 31,8 | 50.5 | 15.9/0.2 | - | - | 2.66 | 18.1 | 0.8 | - | - | - | - | siSa |
| 15/2 | 90-9.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0,0 | 5.3 | 35.1 | 55.5/5.1 | 2.18 | 1 84 | 2.67 | 18.7 | - | 19.8 | 17.1 | 2.7 | 0.58 | saSiL |

#### 5.2 Text Annex

Cumulative soil fractions

Cumulative mass fraction %

Diameter, mm

Žvyras= Gravel

Smėlis= Sand

#### Dulkės+Molis=Dust+Clay

#### 5.3 Text Annex

Cumulative soil fractions

Cumulative mass fraction, %

Diameter, mm

Žvyras= Gravel

Smėlis= Sand

#### Dulkės+Molis=Dust+Clay

Text Annex 6

Abbreviations, symbols, and measurement units used in the report

γ – specific gravity, kN/m3

γw – specific gravity of water kN/m3

ρ – natural (mass) density Mg/m3

ρs – solid particle (mass) density Mg/m3

e – porosity coefficient, pcs.d.

w – natural moisture content%,

Wl – flowable moisture content, %

Wp –plasticity moisture content, %

Ip – plasticity index %

Il – flow index, pcs.d.

Id – density index, pcs.d.

k – filtration coefficient m/d

g – acceleration due to free fall m/s2

Eo – deformation modulus (total deformation modulus) MPa

ф' – effective internal friction angle, degrees

qc – conical strength MPa

f – lateral friction strength kPa

Rf – lateral friction strength to cone strength ratio %

n – sample

x – sample mean

S – standard deviation

Gr. – borehole

IGS – engineering geological layer

x, y –Coordinates (LKS 94), m

Abs.a. – absolute elevation, m

GVG – groundwater table depth m

GVL – groundwater level, m abs.a.

PVL – piezometric level altitude m

CPT- cone penetrometer test

GRAPHICAL APPENDICES

Graphic Annex 1

Location scheme of the survey site

http://www.maps.lt/map/