Annex 2: Specifications for Buildings, Structures, and Territory, Document 18.

Technical Specifications for Weapon Pyramid

LITHUANIAN ARMED FORCES

TECHNICAL SPECIFICATIONS FOR WEAPON PYRAMID

- General Part:

  - The weapon pyramid (hereinafter referred to as “the pyramid”) must be new and unused.

- Descriptive Part:

  - The dimensions of the pyramid (H x W x D): 2000 x 1400 x 300 mm.

  - The pyramid is designed to hold 10 units of G-36, AK-type automatic firearms, or other types of rifles, one (light or medium-weight) portable machine gun, and one sniper rifle. These weapons are secured with a specially designed frame that prevents their removal from the stand when locked.

  - The pyramid consists of an open stand for the aforementioned weapons, a closed and lockable box for storing additional accessories, and a closed and lockable magazine box for storing ammunition. Stability-supporting legs are welded to the bottom of the stand.

  - The pyramid is made of sheet steel and rectangular profile tubes.

  - The lower support for the weapons consists of a 25 mm thick glued plywood shelf with adjustable height, depending on the type of weapon, with 15 mm deep notches for securing the weapon butts.

  - Notch dimensions: 45 x 170 mm.

  - The lower movable support is divided into two sections: one for holding 10 automatic firearms, and the other for holding a (light or medium-weight) portable machine gun and a sniper rifle.

  - Adjustment interval for the lower movable support: 250 mm.

  - The front of the lower movable support must be slightly elevated to prevent weapons from falling out of the pyramid when the weapon-holding frame is lifted. The lifting angle is determined based on the weapons provided by the Lithuanian Armed Forces to the manufacturer.

  - The storage location for the first weapon (from the left side of the pyramid) must be positioned so that the mowing weapon-pressing frame does not interfere with the first weapon.

  - The upper support for the weapons is a shelf with notches made of 10 mm thick glued plywood.

  - The upper support for the weapons is height-adjustable Adjustment interval for the upper movable support: 400 mm.

  - Frame: 25 x 25 x 2 mm tube; stand and middle support: 25 x 25 x 2 mm and 40 x 25 x 2 mm tube; weapon-pressing holder: 25 x 25 x 2 mm and 40 x 25 x 2 mm tube.

  - The additional accessories storage box is made of 1.2 mm thick cold-rolled metal sheet with vertical partitions dividing it into 10 equal sections of 100 mm width and 2 equal sections of 200 mm width.

  - The additional accessories storage box is intended for storing cleaning supplies and other accessories and is attached (welded) to the top of the weapon pyramid.

  - The sides of the additional accessories storage box must have metal cover holders (chains) and loops for securing them.

  - The front part of the additional accessories storage box must have ventilation holes, with air intake from the bottom side.

  - The magazine box is made of 1.2 mm thick cold-rolled metal sheet with vertical partitions dividing it into 10 equal sections of 100 mm width and 2 equal sections of 200 mm width.

  - The magazine box is intended for storing ammunition magazines and is attached to the upper part of the pyramid (on the additional accessories storage box).

  - The magazine box is secured to the additional accessories storage box with screws.

  - The magazine box must be removable from the weapon pyramid.

  - The upper part of the magazine box must have handles for carrying.

  - The sides of the magazine box must have metal cover holders (metal rods) and loops for securing them.

  - The front part of the magazine box must have ventilation holes, with air intake from the bottom side.

  - Sealing with plasticine devices is required for the weapon-pressing holder, the cover of the additional accessories storage box, and the cover of the magazine box.

  - The screws for attaching the magazine box and the additional accessories storage box must be placed in the second storage sections for magazines and additional accessories from the edges of the pyramid.

  - The supporting parts of the pyramid must be protected to prevent floor scratching.

  - The permissible tolerance for the specified dimensions is ± 10 mm.

Additional accessories storage box

Ventilation holes

Magazine box

Interior of the additional accessories storage box

Interior of the magazine box

Weapon pressing holder

Height-adjustable support for automatic firearms, machine guns, and sniper rifles

Weapon stand frame

Ventilation holes

Axonometric View

Stand for weapons

Magazine box