Annex 2: Specifications for Buildings, Structures, and Territory, Document 19.

Technical Specification for Soldier's Wardrobe

TECHNICAL SPECIFICATION FOR SOLDIER'S WARDROBE

- General Part:

  - The soldier's wardrobe (hereinafter referred to as “the wardrobe”) must be new and unused.

- Descriptive Part:

  - Dimensions of the wardrobe (H x W x D): 1900 x 800 x 550 mm.

  - It must have a base section with a height of at least 100 mm, a width of 800 mm, and a depth of 500 mm.

  - The body and base section are made of 1 mm bent sheet steel, while the bottom is made of 2 mm sheet steel, with height-adjustable legs attached.

  - Door height: 1798 mm.

  - The wardrobe space is divided into two sectios by a vertical partition: a section for hanging clothes (440 mm wide) and a shelving unit (300 mm wide), which must have two drawers with a metal sliding structure at the bottom.

    - In the hanging clothes section, at a height of 950 mm, there must be a shelf dividing the section into two parts.

    - The hanger rod in the hanging clothes section must be positioned 60 mm from the top of the wardrobe and 270 mm from the back wall.

    - The shelving unit consists of four shelves (at the top of the unit) and two drawers (at the bottom). The bottom drawer height: 360 mm, top drawer height: 160 mm. The distances between the three lower shelves (starting from the lowest shelf above the top drawer) are 260 mm, 380 mm, and 380 mm.

  - The wardrobe must have a door-locking handle and upper and lower ventilation holes.

  - Door-locking handle height: 1000 mm from the floor.

  - The doors must be lockable with a padlock.

  - A loop for a padlock must be positioned 180 mm above the door-locking handle.

  - In the upper left corner of the door, 180 mm from the edge and 105 mm from the top, there must be a holder for the soldier's wardrobe label, with dimensions (H x W): 50 x 100 mm.

  - Width of the left door: 470 mm.

  - On the inner side of the left door, three metal hooks are attached.

    - Each hook length: 285 mm.

    - Each hook must be positioned 90 mm from the side of the door.

    - Distance between hooks: 140 mm.

    - The upper hook must be positioned 300 mm from the top of the wardrobe.

  - Width of the right door: 330 mm.

  - On the inner side of the right door, four small drawers are attached.

    - Width of each small drawers: 200 mm.

    - Each small drawer must be positioned 65 mm from the side of the door.

    - The bottom of the top small drawer must be positioned 220 mm from the top of the wardrobe.

    - Distance between the top and second small drawer: 280 mm.

    - Distance between the second and third small drawer: 280 mm.

    - Distance between the third and bottom small drawer: 380 mm.

  - The supporting parts of the wardrobe must be protected to prevent floor scratching.

  - The opening angle of the wardrobe doors is 900.

  - The colour of the metal parts is close to RAL 7038.

  - The permissible tolerance for the specified dimensions is ± 10 mm.

Left side door

Left side door

Right side door

Right side door

Ventilation holes of 15 mm diameter

Ventilation holes of 15 mm diameter

View without doors

View without doors