Annex 2: Specifications for Buildings, Structures, and Territory, Document 20.

Technical Specification of the Soldier's Table

TECHNICAL SPECIFICATION OF THE SOLDIER'S TABLE

- General Part:

  - The soldier's table (hereinafter referred to as “the table”) must be new and unused.

- Descriptive Part:

  - Table dimensions (H x W x D): 750 x 1400 x 600 mm

  - The table is designed for two persons.

  - The table is made of laminated chipboard, with the edges of the tabletop covered with a rounded protective PVC strip.

  - The colour of the laminated chipboard is intermediate beech (the final colour will be coordinated with the winner of the tender).

  - The tabletop must be at least 24 mm laminated chipboard, edged with at least 2 mm PVC edge.

  - At the front of the table, across its entire width, there must be a vertical leg cover (hereinafter referred to as “the cover”), with a height of no less than 300 (+50/-10) mm.

  - The cover must be at least 18 mm laminated chipboard, edged with at least 0.8 mm PVC strip.

  - The metal frame must be made of rectangular profile pipes no thinner than 2 mm.

  - The supporting parts of the table must be protected to prevent floor scratching.

  - The permissible deviation for the specified dimensions is ± 10 mm.