Annex 2: Specifications for Buildings, Structures, and Territory, Document 21.
Technical Specification for Bunk Beds

TECHNICAL SPECIFICATION FOR BUNK BEDS

1. General Part:

1.1. The bunk bed (hereinafter referred to as “the bed”) must be new and unused.

2. Descriptive Part:

2.1. Bed dimensions (H × W × D): 2230 × 2050 × 800 mm.
2.2. The bed, including the ladder, is made of: headboards constructed from steel rectangular tubes (25 × 40 × 2.5 mm), headboard support rods made from square tubes (40 × 40 × 2.5 mm),

frame composed of equal-sided angle bars (25 × 25 × 4 mm) and rectangular tubes (25 × 40 × 2.5 mm).
2.3. The bed base is made of 10 mm thick plywood with ventilation holes for mattress airflow.
2.4. The frame is reinforced with steel strips no thinner than 4 mm.
2.5. The plywood base is secured to the steel strips with countersunk screws.
2.6. All steel components (tubes, angles, and strips) are welded with continuous seams.
2.7. The ladder is made of steel rectangular tubes.
2.8. The bed must be easily convertible from a bunk bed into two single beds and vice versa.
2.9. The ladder must be easily repositionable to any desired location on the bed.
2.10. The bed's supporting parts must be equipped with protective elements to prevent floor scratches.
2.11. The colour of the metal parts is similar to RAL 7038.
2.12. The allowable tolerance for the dimensions specified in the diagram is ±10 mm.

Details for beds’ junction

Details for beds’ junction