Annex 2: Specifications for Buildings, Structures, and Territory, Document 22.
Technical Specification for the Weapons Cleaning Table

TECHNICAL SPECIFICATION FOR THE WEAPONS CLEANING TABLE

1. General Part:

1.1. The weapons cleaning table (hereinafter referred to as “the table”) is designed for weapon maintenance and cleaning.

2. Descriptive Part:

2.1. Table dimensions (H × W × D): 890 × 3000 × 700 mm.
2.2. The tabletop is made of marine plywood, at least 12 mm thick, with edges covered by at least a 2 mm PVC trim.
2.3. Plywood colour: intermediate beech (final colour to be coordinated with the competition winner).
2.4. The colour of the table legs is similar to RAL 7038.
2.5. The legs are foldable and can be securely locked in both folded and fully extended positions.
2.6. The table is designed for simultaneous use by 10 people, with 5 on each side.
2.7. Weapon slots must be cut in every second section to ensure that two weapons are not positioned directly opposite each other.
2.8. Weapon slots are cut with a diameter of 50 mm.

2.9. The table’s supporting parts must be equipped with protective elements to prevent floor scratches.
2.10. The allowable tolerance for the external dimensions specified in the drawings is ±10 mm.

Table leg folding and stability lock mechanism

Table leg folding and stability lock mechanism

Slot A-A

Slot A-A