Annex 2: Specifications for Buildings, Structures, and Territory, Document 23.
Technical Specification for the Ammunition Safe

TECHNICAL SPECIFICATION FOR THE AMMUNITION SAFE

1. General Part:

1.1. The ammunition safe (hereinafter referred to as “the safe”) must be new and unused.

2. Descriptive Part:

2.1. External dimensions of the safe (H × W × D): 480 × 1250 × 520 mm.
2.2. The safe is made of 1.2 mm thick cold-rolled metal sheet.
2.3. Edges are reinforced with 30 × 30 × 2 mm rectangular profile tubes.
2.4. Internal dimensions of the safe (H × W × D): 470 × 1200 × 500 mm.
2.5. The bottom is reinforced with additional crossbars.
2.6. The sides of the safe must have four foldable metal handles (two on each side).
2.7. The front of the safe must have a loop for securing a lock.
2.8. The loop must not extend beyond 200 mm.
2.9. A sealing device for plasticine must be installed near the lock loop.
2.10. The loop and sealing device must be designed so that they do not protrude and do not interfere with transportation.
2.11. A locking hook must be installed on the lid of the safe.
2.12. A metal lid holder (chain) must be installed.
2.13. The safe must withstand a weight of at least 150 kg.
2.14. The colour of the safe is similar to RAL 8000.
2.15. The supporting parts of the safe must be protected to prevent floor scratches.
2.16. The permissible tolerance for the dimensions indicated in the diagram is ±20 mm.