Supplement 2.1 Document 30.

Health centre furniture technical specifications

| Item No. | Name and technical characteristics | REMARKS | Unit of measurement | Quantity |
| --- | --- | --- | --- | --- |
| I. OUTPATIENT DEPARTMENT |  |  |  |  |
| 1. | Medical cabinet Technical details: - 1655/1755*x700x320 mm; - lockable; - maximum load for metal shelf – 30 kg, for glass shelf – 10 kg; | Or analogous | pcs | 9 |
| 2. | Procedure couch Technical details: - 560x1930x690 mm; - Frame – square tube profile 25×25 * 1,5 mm; - The furniture consists of two parts – the couch and the headrest; - adjustable headrest; - additional accessory – disposable towel holder; - covering – artificial leather, resistant to abrasion and disinfectants; - maximum load capacity – 180 kg. | Or analogous | pcs | 9 |
| 3. | File cabinet Technical details - 1830x915x370 mm; - Lockable; - shelves with evenly distributed load – 60 kg. | Or analogous | pcs | 6. |
| 4. | Equipment table Technical details: - 450×466 mm, H860 - Made of steel, body thickness 0.6 mm, door thickness 0.7 mm; - Telescopic drawer rails (full extension); - drawer load capacity – 15 kg. | Or analogous | pcs | 9 |
| 5. | Doctor’s chair Technical details: - High backrest – comfortable for people of various heights; - ergonomically shaped seat and backrest; - synchronous mechanism – allows locking in 5 working backrest positions; - seat depth adjustment; - pneumatic height adjustment; - adjustable polyurethane armrests. | Or analogous | pcs | 5 |
| 6. | Patient chair Technical details: - Purpose – visitor chair; - covered with ECO leather; - sturdy steel frame | Or analogous | pcs | 5 |
| 7. | Procedure table Technical details: - 7647×508 mm, H 890 – 1250 mm; - Made of steel, body thickness 0.6 mm, door thickness 0.7 mm; - Rotating tabletop made of white chipboard, thickness 16 mm, edge 2 mm; - rotating tabletop size 534×354 mm; - table rotation angle 360º; - table load capacity -15 kg, drawer 7 kg | Or analogous | pcs | 6 |
| 8. | IV stand with wheels Technical details: - 500×500 mm, H=2300; - Number of hooks: 4 (can be used for two patients simultaneously); - Nominal load up to 5 kg. | Or analogous | pcs | 4 |
| 9. | Refrigerator – showcase for medical use Technical details: - Professional laboratory refrigerator. - Total capacity: 360 l or more. Useful capacity: 344 l or more - External dimensions A x P x G: 1840±50 x 600±50 x 615±50 mm. - Internal dimensions A x P x G: 1635±50 x 440±50 x 435±50 mm. - Dynamic cooling system. - Electronic button control. - Temperature: from +2±1°C to +16±1°C. Digital temperature display. - Free contact for warning signal connection to a remote monitoring system. RS485 connection. - Optical and audible warning signal. - Power failure warning signal. - 5 shelves – grills (plastic-coated), of which 5 are adjustable to the desired height. Useful shelf area P x G : 440±20 x 420±20 mm. - Maximum shelf load: 45±5 kg. - LED interior lighting, turned on with a separate button. - White-coloured body. - Glass doors with a white frame and lock. - Ergonomic bar-shaped handle. - Interior: Special plastic designed for professional equipment. - The direction of the door opening can be changed. - Automatic defrosting. - Connection data for the electrical network: 50 Hz / 220-240V - Adjustable feet. - RS485/RS232 adapter + software. - Lock set (up to 10 locks, all lockable with one key) – available upon additional request. | Or analogous | pcs | 1 |
| 10. | Mobile medical wall Technical details: - 1850x740x510 mm | Or analogous | pcs | 1 |
| 11. | Chair with wheels Technical details: - Made of stainless steel, withstands up to 100 kg; - Height adjustment from 45 to 60 cm; - Large seat with a diameter 29 cm. | Or analogous | pcs | 2 |
| 12. | Laboratory chair Technical details: - Dimensions: length 182 cm, width 62 cm, Safe working load 135 kg; - 3 parts, fixed height 60 cm, steel base covered with epoxy coating, heat-resistant up to 250°C; | Or analogous | pcs | 1 |
| 13. | Bactericidal lamp Technical details: - UV lamp lifespan: 9000 hours or more. - 2-lamp configuration. - Power used: to 66 W - Operating type: Constant. - UV-C lamp radiation intensity at 1 meter distance: 2.0 W/m2 or more. - Protection class: I. - With a working time counter. - Suitable for: Healthcare institutions | Or analogous | pcs | 2 |
| 14. | Hand table Technical details: - 50 x 50 x 70-110 (a) cm; | Or analogous | pcs | 1 |
| 15. | Laboratory table Technical details: - dimensions 1600-2,100mm x 600 – 800mm x 750-900mm; - With drawers; | Or analogous | pcs | 2 |
| II. SUPPORTING PREMISES |  |  |  |  |
| 16. | Laundry cabinet Technical details: - 500x800xs1800 mm | Or analogous | pcs | 6 |
| III. PHYSIOTHERAPIST |  |  |  |  |
| 17. | Couch Technical details: - 3-part massage table, electric height adjustment 47 – 89 cm, steel base covered with epoxy coating, heat-resistant up to 250°C; - Adjustable back section -10° / +75°, Leg section -20° / +70°; - Dimensions: length – 198 cm, width – 75 – 80 cm Safe working load – 175 kg; - Soft part 5 cm thick, covered with artificial leather, with antibacterial protection, heat-resistant, M2 class; - Integrated retractable / liftable wheels for table relocation, NOT for patient transport; | Or analogous | pcs | 1 |