| Supplement 2. Document 33. Repair workshop equipment technical specification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item No. | Name and technical characteristics | REMARKS | Unit of measurement | Quantity |
| I. TRANSPORT VEHICLE ELECTRICAL EQUIPMENT REPAIR ROOM |  |  |  |  |
| 1. | Stainless steel workbench with drawer unit Technical details: - dimensions (approx.) 2000x800x890 mm; - table load capacity no less than 300 kg. | Or analogous | Pcs | 3 |
| 2. | Metal rack Technical details - dimensions (approx.) 1000x550x2000 mm; - Adjustable height shelves; - shelf load not less than 300 kg. | Or analogous | Pcs | 3 |
| II. ARMOR VEHICLE COMMUNICATION SYSTEMS REPAIR ROOM |  |  |  |  |
| 3. | Stainless steel workbench with drawer block Technical details: - dimensions (approx.) 2000x800x890 mm; - table load capacity no less than 300 kg. | Or analogous | Pcs | 3 |
| 4. | Metal rack Technical details: - Dimensions (approximately) 1000x550x2000 mm. - adjustable height shelves; - shelf load not less than 300 kg. | Or analogous | Pcs | 3 |
| III. ARMOR VEHICLE OPTICS REPAIR ROOM |  |  |  |  |
| 5. | Stainless steel workbench with drawer block Technical details: - dimensions (approx.) 2000x800x890 mm; - table load capacity no less than 300 kg. | Or analogous | pcs | 3 |
| 6. | Metal rack Technical details: - dimensions (approx.) 1000x550x2000 mm; - Adjustable height shelves, shelf load of at least 300 kg. | Or analogous | pcs | 3 |
| IV. SPARE PARTS WAREHOUSE |  |  |  |  |
| 7. | Metal rack Technical details: - Dimensions (approx.) 1200x600x2000 mm. - Adjustable height shelves, - shelf load not less than 500 kg. | Or analogous | pcs | 18 |
| V. TOOL ROOM |  |  |  |  |
| 8. | Metal rack Technical details: - Dimensions (approximately) 1200x600x2000 mm; - adjustable height shelves; - shelf load not less than 120 kg. | Or analogous | Pcs | 18 |
| VI. OIL AND SPECIAL FLUIDS WAREHOUSE |  |  |  |  |
| 9. | 5:1 Wall-mounted oil pump 22750 with solenoid valve 48059 and oil level sensor 23191. Mounting kit for wall-mounted oil pump | Alentec Orion 22750 + 48059 + 23191 (Sweden) or equivalent | set | 9 |
| 10. | Used oils/fluids meter 23154 | Alentec Orion 23154 (Sweden) or equivalent | pcs | 2 |
| 11. | Container monitoring module 23430 | Alentec Orion 23430 (Sweden) or equivalent | pcs | 3 |
| 12. | Metal rack Technical details: - Dimensions (approximately) 1400x500x2000 mm; - adjustable height shelves; - shelf load not less than 120 kg. | or equivalent | pcs | 2 |
| 13. | ATEX Safety relay | Or analogous | Pcs | 1 |
| VII. TIRE REPAIR AND MAINTENANCE ROOM |  |  |  |  |
| 14. | Electro-hydraulic truck and agricultural machinery wheel mounting machine SICAM JUMBO TCS 52 ANW Technical details: - For truck and agricultural machinery rims; - Hydraulic mounting tool movement, automatic trolley; - Vertical mounting tool movement – manual; - Wired remote control stand-panel; - Disc diameter fixing from inside – 14”-42” (up to 56” with non-assembled adapters); - Maximum wheel diameter – 2,300 mm; - Maximum wheel width – 1,300 mm; - Minimum engagement diameter – 105 mm; - Maximum wheel weight – 1,400 kg; - Number of clamps – 4 pcs.; - Two rotation speeds – 4,5/9 aps./min; - Installed power – 3,68 kW; - Electrical supply – 400 V, 50 Hz; - mass – 947 kg. | Sicam TCS 52 ANW (Italy) Or analogous | pcs | 1 |
| 15. | Truck wheel balancing stand Technical details: - rim diameter – 6”-40”; - rim width – 1”-24”; - Maximum wheel diameter – 1,200 mm; - Maximum wheel width – 565 mm; - maximum wheel weight – 200 kg; - balancing speed – <100 aps/min; - weight hiding program for hidden rim spokes - (light-duty); - tire – rim position optimization program - (light-duty); - basic wheel mounting kit; - wheel cover; - ability to balance passenger car wheels; - pneumatic lift for the wheel; Wheel balancing machine configuration: - Mounting for heavy vehicle wheels: - Steel high-resistance tightening nut (without quick fastening mechanism) – 1 pcs; - 5 – Support fastening star – 1 pcs; - 4 – Support fastening star – 1 pcs; - 4-centreing bolt set – 1 pcs; - 22.5” rim adapter – 1 pcs; - Distance ring – 1 unit. - mounting cone set for passenger cars: - cone 42-64,5 mm – 1vnt.; - cone 54-79,5 mm – 1 pcs.; - cone 74-111.5 mm – 1 pcs.; - compression cover for ALU rims – 1 unit. - mounting cone set for minivans: - cone 122-174 mm – 1 pcs.; - cone 168,5-194,5 mm – 1 pcs. - calibration weight – 1 pcs; - weight removal pliers – 1 unit. - mass – 183 kg. | Sicam SBM 855 (Italy) or equivalent | pcs | 1 |
| 16. | Automatic wheel mounting machine for working with rims up to 28” Technical details: - engaged rim size – 10-24″ (outer); - engaged rim size – 12-28″ (inner); - maximum wheel diameter – 1,200 mm; - maximum rim width – 12,5″; - tire bead press stroke – from 70 to 397 mm; - working compressed air pressure – 8-10 bar; - mounting machine rotation speeds – 7 and 15 aps/min; - motor – 400 V. Configuration: - wheel mounting machine – 1 pcs; - low-profile tire pressing device Technoroller NG 1 pcs.; - compressed air inflation pressure gauge – 1 pcs; - cover set for wet rims – 2 pcs; - compressed air preparation unit (regulator-oiler) – 1 pcs; - Brush for wheel mounting paste – 1 unit. - mass – 354 kg. | Sicam EVO 624S V1 (Italy) Or analogous | Pcs | 1 |
| 17. | Wheel balancing machine with LED screen, automatic Distance to wheel ruler, internal wheel lighting, and Laser Technical details: - balancing speed (50-60 Hz) – 208-250 aps./min.; - Maximum wheel weight – 70 kg; - maximum rim diameter – 1,200 mm; - maximum rim width – 500 mm; - set rim diameter – 6-40”; - measured rim diameter – 10-27”; - set rim width – 1-24”; - measured rim width – 1-20”; - Power – 0,7 kW; - power supply – 230V/50-60 Hz; - dimensions – 1325x1365x1915 mm. - mass – 223 kg. Configuration: - wheel balancing machine – 1 pcs; - pliers for weights – 1 pcs; - 60 g calibration weight; - manual width gauge; - 3-centreing cone set – 1 pcs; - ProGrip quick tightening handle 1 pc - aluminium rim pressing ring – 1 pcs; - steel rim pressing ring – 1 unit. - mass – 134 kg. | Sicam EVO 624S V1 (Italy) Or analogous | Pcs | 1 |
| 18. | protective wheel inflation cage Technical details: - external dimensions – 1600x800x1550 mm; - internal dimensions – 1500x700x1500 mm; - Maximum wheel diameter – 1,500 mm; - certified in EU; - Inflation station; - mass – 166 kg. | Sicam SSB 1500 (Italy) Or analogous | Pcs | 1 |
| 19. | Armoured vehicle Run-Flat tire mounting Machine Technical details: - dismounting/mounting system consisting of 2 grips, hydraulic cylinder, and pulling-pushing mechanism; - ring dismounting system – independent system designed for the dismounting/mounting of RFT tire metal rings; - control block – low-voltage control circuit (24 V); - ensures operator safety; - lifting system – designed for the transfer of wheels up to 500 kg; - hydraulic block – 100 L volume, working pressure – 250 bar; - power supply – 380 V, 50-60 Hz. - mass – 1,800 kg. | GM GMR-023 (Turkey) Or analogous | Pcs | 1 |
| 20. | Stainless steel workbench with drawer unit Technical details: - dimensions (approx.) 2000x800x890 mm; - table load capacity no less than 300 kg. | Or analogous | Pcs | 1 |
| VIII. TENT REPAIR ROOM |  |  |  |  |
| 21. | Workbench with wooden working surface Technical details: - Expanded frame dimensions: 1150x1050x900 mm. - Workbench size selected according to need. - Adjustable height up to 900 mm. - Adjustable length, width, and height wooden workbench. | Or analogous | pcs | 1 |
| 22. | Metal rack Technical details: - Dimensions (approximately) 1800x600x2000 mm; - Adjustable height shelves, shelf load of at least 120 kg. | Or analogous | Pcs | 1 |
| IX. WELDING, METAL CUTTING WORKROOM |  |  |  |  |
| 23. | Welding table GPPH 1500x1000X650 mm with With wheels Technical details: - legs 60x60 mm; - thickness 8 mm; - holes d-16 or 28; - three hole grids on 4 sides; - engraved scale on the grid 100x100 mm; - manufactured according to standard ISO 2768-1; - load 300 kg per leg; | GPPH, Poland or equivalent | pcs | 1 |
| 24. | Mobile welding smoke extraction system with 360°C GYS UNI 2.0 Technical details: - UNI 2 for: MMA, TIG, MIG/MAG suvirinimas; - foot made of reinforced steel and anodized aluminium alloy; - Filtration in three stages: 1. Metal filter; 2. Secondary filter; 3. Rigid Hepa filter; 4. Carbon filter (optional); - 400V, 32A, 1.1 kW; - current 4.85 A; - airflow 1500 m3/h; - sleeve length 3 m. | GYS, France or equivalent | pcs | 1 |
| 25. | Hydraulic cutting machine/guillotine INANLAR AHGM 3006 Technical details: - Cutting length 3,050 mm; - Cutting power(42 kg/mm2) – 6 mm; - Cutting power(70 kg/mm2) – 4 mm; - Cutting angle – 0,7-2,4°; - max cutting force 17 t.; - max pressure 23 bar.; - Power 11 kW; - back gauge stroke 600 mm; - oil tank capacity 150 l.; - knife gap adjustment 0,05 – 0,6 mm; - back gauge speed 110 mm/s; - number of panel supports – 2 pcs; - hole depth 0 – 300 mm(optional); - table height 900 mm; - machine width 2,350 mm; - transport width 1,700 mm; - dimensions: 3700x1850x1800 mm; - weight 6,500 kg. | INANLAR, Turkey or analogue | Pcs | 1 |
| 26. | Metal cutting machine ARG 500 plus S.A.F. Technical details: - power output: 4 kW, 400 V; - belt speed 20-90m/min.; - working clamp height 870 mm; - hydraulic system oil ~25 l.; - cooling capacity ~30l.; - max. Dimensions 2000x2600x2150 mm; - weight ~1530 kg; | Pilous Czech Republic or equivalent | Pcs | 1 |
| 27. | Welding semi-automatic Badek V-MIG285MMA Technical details: - power output: 7.5 kW, 400V; - welding current 50-280A; - welding method MMA; - wire feed 4 rollers. | Badek, Poland or equivalent | Pcs | 1 |
| 28. | Welding automatTMIG285P Technical details: - wire spool size 15 kg; - safety level IP 21S; - dimensions: 900x320x720 mm; - power: 7.5 kW, 400V; - aluminium wire diameter 0.8-1.2 mm. - welding wire diameter 0.8 mm. - self-shielding wire diameter 0.8-1.2 mm. - steel wire diameter 0.8-1.2 mm. - stainless steel wire diameter 0.8-1.2 mm. - welding current regulation range 10-280 A. - welding electrode diameter 1.6-4.0 mm. - spot welding time control. - post-welding burn-off time and voltage adjustment. - post-welding gas flow time adjustment. | Badek, Poland or equivalent | Pcs | 1 |
| 29. | Electrode welding device TELWIN 817014 Technical details: - dimensions 390x690x480 mm; - Power 9 kW, 400V; - welding electrode diameter 1,6-5; - welding current regulation range 45-250. | TELWIN Poland or equivalent | pcs | 1 |
| 30. | Welding work table with exhaust ventilation ERGO-STW-R Technical details: - table size: 1500x900x880 mm; - lockable tool box; - rotating small parts welding table d-40 cm. - grids for downward pulling 19,5x49 cm; - fan: Power 0.5 kW, 400V; - Capacity at 300Pa pressure 1000m3/hr; - security class IP 54; - safety class IP 54; - elbow: pipe d-160 mm, length 2 m; | KLIMAVENT Poland or analogue | Pcs | 1 |
| 31. | 100-ton electrohydraulic press OMA 666 installed indoors Technical details: - pressure manometer; - V-type supports – included; - Two-speed electrohydraulic pump; - manual control; - manual lifting/lowering valve control; - Power – 100 t; - welded, reliable frame construction; - distance between pressing supports – max 1000 mm; - width between supports (internal) – 1030 mm; Hydraulic - piston stroke – 320 mm; - pump power – 1,5 kW. - voltage - 230-400V +/- 5%; 50 Hz. - mass – 980 kg. | OMA 666 or analogue | Pcs | 1 |
| 32. | Cantilever rack 4 tiers (single-sided) Technical details: - stand for metal rods, load-bearing: 7 t; - shelf load about 1,500kg; - dimensions: I=6000 mm, A=3000 mm, P=660 mm; - not shorter than 6 m. | SCHAFER Germany or analogue | Pcs | 1 |
| 33. | Stand for storing metal sheets Technical details: - dimensions: I=6000 mm, A=3000 mm, P=660 mm; - stand for metal sheets, load-bearing 7 t; - sheets stored in a vertical position; - not shorter than 6m. | SCHAFER Germany or analogue | Pcs | 1 |
| 34. | Pipe profile bending device PRM35F Technical details: - dimensions: 1000x750x1400 mm; - Power 400V/50Hz.; 0,75 kW; - speed 4,5 m/min.; - weight 400 kg. | Or analogous | Pcs | 1 |
| 35. | 1 t x 4,000 mm single-gear, overhead gantry crane STAHL CraneSystems, Germany Technical details: - lifting capacity 1t (1.25 t during technical inspection); - crane structure – I-beam; - lifting width 4,000 mm (to be specified during the working project); - Lifting height: 3 m (to be specified during contract execution). - classification U4/Q2. - lifting class HD1 according to DIN EN 13001-2. - Operating mode. (FEM/ISO) M5. - crane weight 683 kg. - operating environment temperature from -20°C to +40°C. - protection class IP55. - hoist type chain. - hoist lifting capacity 1 t (verified during technical inspection 1.25 t); - monorail hoist lifting height 3 m. Hoist lifting speed 1,0/ 4,0 m/min, two speeds; - Hoist electric motor power 0.07 / 0,32 kW. - monorail hoist trolley approach speed 5.0 / 20.0 m/min, two speeds; - monorail hoist trolley electric motor power 0.07 / 0,32 kW. - crane movement speed 8 / 32 m/min, two speeds; - Crane travel electric motor power 0.07 / 0,32 kW; - Radio wave remote control and emergency wired control; - lifting height limiter. - Cross-type end switches on the crane travel and on the monorail hoist travel: - when the first limit point is reached, the first speed is engaged, - when the second limit point is reached, the crane is stopped. - overload system for weight measurement; - Satisfies C3 corrosion category; - epoxy painting system; - type C-profile with trolleys, onto which flexible cables are attached; - cables along the crane bridge. Length 4,000 mm. Cable material PVC. Fixing to the upper plane of the crane bridge; - electrical supply system via a closed-type trolley along the crane runway; - length 21 m, fastening to the overhead crane rail beam. - minimum power for electrical installation 1,76 kW, 400 V; | or equivalent | pcs | 1 |
| 36. | 1 t x 4,000 mm single girder, supporting overhead crane under-crane path. Technical details: - total length of the crane track 21 m. - I-beams HEA 180, steel S355; - Painted with an epoxy coating system meeting the C3 corrosion category; | Or analogous | Pcs | 1 |
| X. TURNING-MILLING WORKSHOP |  |  |  |  |
| 37. | Disc metal cutting machine. Model MACC NEW350 EDV. Technical details: - rotating head 45° to the right and 90° to the left; - reducing oil bath; - double hinge pin with eccentric insert; - double quick-lock clamp; - motor driven by belt cooling; - cooling fluid filter. - clamp set with scratching device; - current separation system for blade lubrication; - support beam; - cork stopper; - service tools; - 24-volt low-voltage installation with bracket; - dimensions: 1475x1000x1200 mm; - motor 1,8-2,4 kW, speed 40-80 RPM; - clamp width 120 mm, disc diameter 350 mm; | MACC, Italy or equivalent | Pcs | 1 |
| 38. | Milling machine with partial head set FH-150 Technical details: - motor 7,5 kW; - max. Milling diameter 50 mm; - Rotational speed 40-1800/min. - feed rate on X/Y axis 18-627 mm/min; - number of gears on X/Y axis 9; - feed rate on Z axis 48-1670 mm/min.; - number of gears on Z axis 2; - spindle distance to table 150-650 mm; - spindle distance to column 520; - tilt angle of the rotating head +/-30°; - bushing travel 105 mm; - T-type slot 3x18x100 mm; - table dimensions: 1400x400 mm; - table travel on Y axis 400 mm; - table travel on X axis 800 mm; - table travel on Z axis 500 mm; - dimensions: 2290x1770x2120 mm; - weight 3,860 kg. Configuration: - cooling (pump); - workplace lighting; - digital indication; - central lubrication system; - partial head. | PROMA, Czech Republic or analogue | pcs | 1 |
| 39. | Lathe machine Proma SPF-1000PH Technical details: - voltage 400V; 5.5 kW; - turning diameter over support 460 mm; - turning diameter over V-shaped gap 690 mm; - turning diameter over support 275 mm; - turning length 1,000 mm; - V-shaped gap length 200 mm; - spindle travel 80 mm; - pinole travel 170 mm; - range of number of revolutions 25-1800/min.; - number of revolutions range: 12 steps; - with gearbox; - dimensions: 2310x1000x1255 mm; - metric thread range 0,5-20/min; - number of metric thread steps 24; - imperial thread range 1,5/8-72/1“; - number of imperial thread steps 61; - modular thread range 0.25-10; - number of modular thread steps 20; - diametric thread range 3, ¼-96; - number of diametric thread steps 45; - maximum feed of the tool support 135 mm; - maximum cross feed 900 mm; - One division of the Vernier scale: - longitudinal feed 0.1 mm; - cross feed 0.02 mm; - tool support feed 0.05 mm; - horsehead pinole 0.05 mm. - longitudinal automatic feed 0,04-2,456 (122 degrees); - cross automatic feed 0,016-0,982 (122 degrees); - adjustable spindle gap; - weight 2,650 kg. | PROMA Czech Republic or analogue | pcs | 1 |
| 40. | Grinding machine with disc and stand BKS-2500 Technical details: - number of revolutions 2950/min.; - power output: 1,1 kW/400V; - disc diameter 250 mm; - Disc thickness 25 mm. | PROMA Czech Republic or analogue | Pcs | 1 |
| 41. | Tool sharpening machine UWS320Holzmann Technical details: - various-purpose tool grinding machines; - Stand and tool cabinet; - The grinding head can be tilted vertically from +40° to –40° and horizontally +50° to –50°; - max. grinding diameter 125mm, max. grinding length 320mm; - Possible sharpening of rotary drills from 5 to 32 mm; - included: - stand, rear support on the right and left, - spring stop, - drill grinding device, - drive block, - conical point 60°, - half-conical point 60°, - cover washer disc; - diamond cover washer disc (125x35x32 mm)1 simple grinding disc, - 1 flat plate disc (125x15x32 mm); - holder, grinding wheel clamp 16, 22, 32 mm; - tools. | Or analogous | pcs | 1 |
| 42. | Bench drilling machine PROMA E-1720F/400 Technical details: - power output: 1.1 kW, 400 V; - drilling diameter 5 – 20 mm; - max. drilling hole 25 mm; - spindle cone III Mk; - distance between column and spindle 215 mm; - spindle travel 80 mm; - max. distance between spindle and table 640 mm; - max. distance between spindle and base 1,150 mm; - table dimensions 350 x 355 mm; - base dimensions 500 x 290 mm; - column diameter 80 mm. - number of speeds: 16; - revolution range: 180 /240/ 290/ 350/ 410/ 480/ 530/ 590/ 660/ 720/ 1190/ 1350/ 1510/ 1790/ 2100/ 3000 aps/min.; - Table “T” slot 14; - Overall height 1,635 mm. Net/gross weight 85/90 kg. | PROMA Czech Republic or analogue | Pcs | 1 |
| 43. | Stainless steel workbench with drawer unit Technical details: - Dimensions (approximately) 1500x600x890 mm; - table load capacity no less than 300 kg. | Or analogous | pcs | 1 |
| XI. BATTERY |  |  |  |  |
| 44. | Stainless steel workbench Technical details: - Dimensions (approximately) 1300x600x890 mm; - table load capacity no less than 300 kg. | Or analogous | Pcs | 1 |
| 45. | Cabinet for battery charging Technical details: - electrical network: 5.5 kW, single-phase 230 V with separate grounding wire; - automatic fuse 16 A; - connection – single-phase socket with grounding wire; - protection class IP 22; - no less than 2.5 mm² copper wire from the socket; - work area (ceiling height, etc.): - Cabinet dimensions 1023x555x2150mm; - work area according to work safety +500 mm from the front and both sides; - air extraction: 230 m3/h performance integrated fan, from the top of the cabinet; - mass – 220 kg. | Magrini MG269 (Italy) Or analogous | Pcs | 1 |
| 46. | Universal automatic battery charger Technical details: - suitable for servicing all types of lead-acid batteries; - eight-stage smart battery charging system; - special function for deep discharged battery regeneration; - the hoist can be connected to the network and left unattended; - maintaining the required battery charge level. - external temperature sensor, which adjusts the charging current; - “Auto Restart” function – when the power supply is lost and restored, the charger resumes operation according to the last selected parameters; - protection against short circuits, sparking, reverse polarity, incorrect voltage selection; - dust protection; - 12 V (15-300 Ah) and 24 V (15-240 Ah) battery charging; - charging current in 12 V mode- 7 / 15 / 20 A; - charging current in 24 V mode- 7 / 15 A; - Power supply 220 V AC, 1 phаsе; - Power 480 W; - protection class IP31; - charging cable length 1.9 m; - dimensions: 190 x 190 x 73 mm; - weight 1.85 kg. | GYS GYSFLASH 20.12/24 PL (France) or equivalent | pcs | 15 (in cabinet distribution according to requirement, max. 10 in the cabinet) |
| 47. | Automatic battery charger Technical details: - Special function for deep discharged battery regeneration; - the hoist can be connected to the network and left unattended; - maintaining the required battery charge level. - protection against short circuits, sparking, reverse polarity, incorrect voltage selection; - 6V, 12 V and 24 V (35-225 Ah) battery charging; - charging current – 7 / 10 / 15 A; - Power supply 220 V AC, 1 phаsе; - Power 450 W; - dimensions: 210 x 360 x 140 mm; - weight 8.5 kg. | GYS Batium 15.24. (France) or equivalent | pcs | 4 |
| XII. ELECTROLYTE |  |  |  |  |
| 48. | Metal rack Technical details: - dimensions (approx.) 1000x550x2000 mm; - adjustable height shelves, shelf load no less than 300 kg; | Or analogous | pcs | 2 |
| 49. | Work table Technical details: - Dimensions (approximately) 1700x600x900 mm; - adjustable leg height; - the work surface must be resistant to acids and other aggressive materials. | Or analogous | Pcs | 1 |
| 50. | Stainless steel cover Technical details: - dimensions 705x705x500 mm; - diameter: 150 mm; - extracted air 150m³ /h; - resistant to acid vapours. | WORKY Italy or equivalent | Pcs | 1 |
| XIII. REPAIR WORKSHOP |  |  |  |  |
| 51. | Mobile stainless steel workbench Technical details: - dimensions (approx.) 2000x800x890 mm; - table load capacity no less than 300 kg. | Or analogous | Pcs | 1 |
| 52. | Centralized fluid dispensing, collection, and control System Centralized fluid dispensing and control system, which consists of: - 4 fluid dispensing towers, each equipped with oil and coolant reels and dispensing guns; - each tower is equipped with a keyboard for entering data to dispense the desired fluid; - 8 types of oils, 1 type of antifreeze, and 1 used oil container and 1 used fluid pipe; - oil storage tanks, pneumatic oil and coolant pumps, and used oil containers; - Orilink fluid control system, with which fluid dispensing accounting is carried out; - data is entered during each dispensing operation: - which operator; - which vehicle; - which fluid; - what volume was requested/dispensed. - modules and sensors monitor fluid levels in containers, all - data is transmitted to the computer, with the option to link to accounting software (additional integration); - mass– 1,000 kg; | Alentec Orion Tower (Sweden) or equivalent | set | 1 |
| 53. | Dual-coil exhaust gas extraction system workstation for repair of passenger cars and jeeps Technical details: - coil with motorized hose rewind – 450 W, 1 ph; - crush-resistant hose up to 200°C; - hose diameter – 125 mm; - hose length – 12.5 m; - rubber nozzle with clamp for the muffler; - Central extraction fan – 3 kW, 3 ph; - central control panel with automatic duct closure. - mass – 163 kg. Configuration: - motorized reel with hose – 2 pcs; - automatic duct valve – 2 pcs; - rubber nozzle – 2 pcs; - fan – 1 pcs; - control panel – 1 unit; - installation set – 1 set | Worky (Italy) or equivalent | set | 2 |
| 54. | Dual-coil exhaust gas extraction system workstation for repair of wheeled armoured vehicles Technical details: - coil with motorized hose rewind – 450 W, 1 ph; - Hose with steel spiral, resistant to up to 500 °C; - hose diameter – 150 mm; - hose length – 15 m; - stainless steel nozzle with a fastener to the silencer; - Central extraction fan – 3 kW, 3 ph; - Central control panel with automatic duct closure; - mass – 183 kg. Configuration: - motorized reel – 2 pcs; - extraction hose – 2 pcs; - automatic duct valve – 2 pcs; - steel nozzle – 2 pcs; - fan – 1 pcs; - control panel – 1 unit; - installation set – 1 set - mass – 193 kg. | Worky (Italy) Or analogous | set | 3 |
| 55. | Two-reel exhaust gas extraction system for truck and tracked armoured vehicle repair workstation Technical details: - coil with motorized hose rewind – 450 W, 1 ph; - Hose with steel spiral, resistant to up to 500 °C; - hose diameter – 150 mm; - hose length – 15 m; - hose Y-connector for double muffler; - quick coupling for nozzle and branch replacement; - stainless steel nozzle with a fastener to the silencer; - Central extraction fan – 3 kW, 3 ph; - Central control panel with automatic duct closure; Configuration: - Motorized reel – 2 pcs; - Extraction hose– 2 pcs; - Hose Y-connector – 1 pcs; - Automatic duct valve – 2 pcs; - Steel nozzle – 4 pcs; - Fan – 1 pcs; - Control panel – 1 pcs; - Installation kit – 1 set - Weight – 193 kg. | Worky (Italy) Or analogous | set | 3 |
| 56. | Special 1500 x 600 mm nozzle with trolley for military vehicles Technical details: - mobile nozzle for tracked vehicles; - nozzle length – 1,500 mm; - nozzle width – 600 mm; - nozzle height – according to the customer's repairable vehicle; | Worky (Italy) or equivalent | set | 2 |
| 57. | Two-column 5.5 t lift Technical details: - lifting capacity – 5,500 kg; - lifting height – 1,965 mm; - minimum height – 125 mm; - height to the joint – 4,930 mm; - lift height – 5,100 mm; - distance between columns – 3,000 mm; - clearance width – 2,956 mm; - lift width – 3,500 mm; - lifting support length – 900-1825 mm; - lifting/lowering duration – 35 s.; - lift weight – 1,080 kg; - electrical power supply – 3 kW, 400 V, 50 Hz. - symmetric lift with upper connection; - independent column synchronization cable; - ideal for minibuses, jeeps, and long wheelbase vehicles; - protective overload valve; - low-voltage control circuit; - Weight – 1,080 kg; | ArWerther 208I/55.4 (Italy) or equivalent | pcs | 4 |
| 58. | 40 t scissor lift Technical details: - electrohydraulic recessed lift; - electronic platform synchronization; - automatic safety locks with pneumatic control. - technical specifications: - lifting capacity – 40,000 kg; - platform length – 9,000 mm; - lifting height – 2,110 mm; - minimum height – 360 mm; - mass– 6,170 kg; | Ravaglioli RAV835.9I SPECIAL 40t (Italy) Or analogous | Pcs | 4 |
| 59. | Mobile lifting columns Blitz Hydrolift S3 4-7,5 t Technical details: - lifting capacity per column – 7.5t.; - lifting height (cylinder stroke) – 1,753 mm; - communication cable length – 15 m; - power supply cable length – 10 m; - motor power– 3 kW; - grip wheel diameter: 570-1,300 mm; - lifting time – 75 s.; - lowering time – 54 s.; - lifter connection – wired; - lifting principle – electrohydraulic; - column weight – 620 kg; - lifting column synchronization tolerance: 20 mm; - column height – 2,590 mm; - column width – 1,107 mm. - weight – single column weight 630 kg, set – 2,520 kg. | Blitz Hydrolift S3 4-7,5 (Germany) or equivalent | Pcs | 4 |
| 60. | Pit-mounted lift Blitz X 20/15 Basic with 20 T lifting traverser ATS-20 Technical details: - lifting capacity 20000 / 15000 kg; - cylinder stroke 1,200 mm; - maximum lifting height 2,175 mm; - maximum lifting duration 150 s; - maximum lowering duration 58 s; - trolley width: 845 mm; - trolley length: 1100 mm; - cylinder rod diameter: 80 mm; - cylinder rod bore diameter: 55 mm; - total hoist height: 975 mm; - operating pressure: 298 bar; - compressed air pressure: 10 – 15 bar; - compressed air requirements: 200 l/min; - Lift weight: 280 kg; - Energy node – hydro-pneumatic; - control – foot switch; - construction – moving along the pit floor; - rapid deployment – available; - self-lowering – present; - the surface is varnished; - anti-corrosion coating – Ultra High Solid Primer (UHS); - mass – 280 kg. | Blitz X 20/15 Basic (Germany) or equivalent | pcs | 3 |
| 61. | Laser-mechanical heavy transport chassis geometry (wheels/axles/turning angles) measurement stand AXIS500 Technical details: - all angles are measured in the driving position, no axle lifting required; - no rim distortion compensation procedure required; - electronic inclinometer with digital display immediately indicates vertical angles (camber, Caster and KPI); - magnetic grips immediately attach to the steel rim; - for mounting on cast light alloy rims, additional side tire grips are used; - laser head rotates on the axis at 360° angle, - rechargeable head battery; - measured angles: individual alignment, total - alignment, maximum turning angles, camber, caster, kpi, axis perpendicularity, axis displacement, axle parallelism, steering column centre position; - all horizontal angles are measured relative to the frame; - mobile box for stand storage and transport x 1; - electronic inclinometer x 1; - front turning plates x 2; - wheel grips x 2; - laser heads x 2; - magnetic frame gauges x 2; - measuring rulers x 2; - turning angle measurement scales x 2; - small accessory set x 1. | Haweka AXIS500 (Germany) Or analogous | Pcs | 1 |
| 62. | 10 t x 11,000 mm single girder bridge crane STAHL CraneSystems, Germany Technical details: - lifting capacity 10 t (during technical inspection) 12,5 t). - crane structure – I-beam. Clear width 11,000 mm; - Lifting height 7 m; - classification U4/Q2; - lifting class HD1 acc. to DIN EN 13001-2; - Operating mode. (FEM/ISO) M5; - crane weight 3,184 kg; - operating environment temperature from -20°C to +40°C. - protection class IP55; - monorail hoist model: SH 5025-20 4/1 L3; - monorail hoist type: rope; - hoist lifting capacity 10 t (verified during technical inspection 12.5 t); - monorail hoist lifting height 7 m. Hoist lifting speed 0.8/ 5.0 m/min, two speeds; - Hoist electric motor power 1.40 / 9.00 kW. - monorail hoist trolley approach speed 5.0 / 20.0 m/min, two speeds; - monorail hoist trolley electric motor power 0.13 / 0.55 kW. Bridge - crane movement speed 8 / 32 m/min, two speeds; - Crane travel electric motor power 0.09 / 0.37 kW; - Radio wave remote control and emergency wired control; - lifting height limiter; - cross-type end switches on the crane travel and on the monorail hoist travel: - when the first limit point is reached, the first speed is engaged; - when the second limit point is reached, the crane is stopped. - overload system. - C3 corrosion category-compliant epoxy coating system; - type C-profile with trolleys, onto which flexible cables are attached; - cables along the crane bridge; - length 11,000 mm; - The cable material is PVC; - fixing to the upper plane of the crane bridge; - electrical supply system via a closed-type trolley along the crane runway; - length 54 m, fastening to the overhead crane rail beam. - minimum power for electrical installation 10.65 kW, 400 V. | Or analogous | pcs | 1 |
| 63. | 16 t x 5,500 mm single girder bridge crane STAHL CraneSystems, Germany Technical details: - lifting capacity 16 t (20 t during technical inspection); - crane structure – I-beam. Clear width 5,500 mm; - Lifting height: 8 m; - Classification U4/Q2. - Lifting class HD1 acc. to DIN EN 13001-2; - Operating mode. (FEM/ISO) M6; - crane weight 3,105 kg; - operating environment temperature from -20°C to +40°C; - protection class IP55; - monorail hoist model: SHR 6040-12 4/1 L2. - monorail hoist type: rope; - hoist lifting capacity 16 t (verified during technical inspection 20 t); - monorail hoist lifting height 8 m (max. 8.5 m); - monorail hoist lifting speed 0.8 / 5 m/min, two speeds; - Hoist electric motor power 2,5 / 15,00 kW; - hoist trolley travel speed 5.0 / 20.0 m/min, two speeds. - monorail hoist trolley electric motor power 0.32 / 1.25 kW; - crane movement speed 8 / 32 m/min, two speeds; - Crane travel electric motor power 0.13 / 0.55 kW; - Radio wave remote control and emergency wired control; - lifting height limiter; - Cross-type end switches on the crane travel and on the monorail hoist travel: - when the first limit point is reached, the first speed is engaged, - when the second limit point is reached, the crane is stopped. Weight - overload system. C3 corrosion category-compliant epoxy coating system; - type C-profile with trolleys, onto which flexible cables are attached along the crane bridge. - length 5,500 mm; - cable material PVC. Fixing to the upper plane of the crane bridge; - electrical supply system via a closed-type trolley along the crane runway; - length 54 m, fastening to the overhead crane rail beam. - minimum power for electrical installation 11.35 kW, 400 V; | Or analogous | pcs | 1 |
| 64. | 16 t x 11,000 mm single girder bridge Crane STAHL CraneSystems, Germany Technical details: - lifting capacity 16 t (20 t during technical inspection); - crane structure – I-beam; - lifting width 11,000 mm; - Lifting height 8 m; - (to be clarified during contract negotiations). Classification U4/Q2. Lifting - class HD1 according to DIN EN 13001-2; - Operating mode. (FEM/ISO) M6; - crane weight 4,786 kg. - operating environment temperature from -20°C to +40°C; - protection class IP55; - monorail hoist model: SHR 6040-12 4/1 L2; - monorail hoist type: rope; - Hoist trolley lifting capacity 16 t (verified during technical inspection 20 t); - monorail hoist lifting height 8 m (max. 8.5 m); - monorail hoist lifting speed 0.8 / 5 m/min, two speeds; - Hoist electric motor power 2,5 / 15.00 kW; - hoist trolley travel speed 5.0 / 20.0 m/min, two speeds. - monorail hoist trolley electric motor power 0.32 / 1.25 kW; - crane movement speed 8 / 32 m/min, two speeds; - Crane travel electric motor power 0.32 / 1.25 kW; - Radio wave remote control and emergency wired control; - lifting height limiter. - cross-type end switches on the crane travel and on the monorail hoist travel: - when the first limit point is reached, the first speed is engaged; - when the second limit point is reached, the crane is stopped. - overload system for weight measurement; - C3 corrosion category-compliant epoxy coating system; - type C-profile with trolleys, onto which flexible cables are attached along the crane bridge. Length 11,000 mm. - The cable material is PVC; - fixing to the upper plane of the crane bridge; - electrical supply system via a closed-type trolley along the crane runway; - length 54 m, fastening to the overhead crane rail beam. - minimum power for electrical installation 12.75 kW, 400 V; | Or analogous | pcs | 2 |
| 65. | 10 t x 11,000 mm single girder, supporting overhead crane under crane path. Technical details: - total length of the crane track 54 m; - I-beams HEA 400, steel S355; - rectangular rail 50x30, steel S355; - Painted with an epoxy coating system meeting the C3 corrosion category; | Or analogous | Pcs | 1 |
| 66. | 16 t x 5,500 mm single girder, supporting overhead crane under crane path. Technical details: - total length of the crane track 54 m; - I-beams HEA 450, steel S355; - rectangular rail 60x40, steel S355; - Painted with an epoxy coating system meeting the C3 corrosion category; | Or analogous | Pcs | 1 |
| 67. | 16 t x 11,000 mm single girder, supporting bridge cranes under-crane tracks. Technical details: - total length of the crane track 54 m; - I-beams HEA 450, steel S355; - rectangular rail 60x40, steel S355; - Painted with an epoxy coating system meeting the C3 corrosion category; | Or analogous | pcs | 2 |
| XIV. PREPARATION FOR PAINTING ROOM |  |  |  |  |
| 68. | Stainless steel workbench with drawer unit Technical details: - dimensions (approx.) 2000x800x890 mm; - table load capacity no less than 300 kg. | Or analogous | pcs | 1 |
| 69. | Metal rack Technical details: - dimensions (approx.) 1000x550x2000 mm; - adjustable height shelves, shelf load no less than 120 kg; | Or analogous | Pcs | 1 |
| XV. DIAGNOSTICS AND TECHNICAL INSPECTION ROOM |  |  |  |  |
| 70. | Universal suspension testing stand AHS PRUFTECHNIK Technical details: - two testing plates, mounted across the pit; - wireless control panel with spotlight; - cabinet with integrated control system; - hydraulic unit; - cable holder; - maximum axle load – 16,000 kg; - maximum wheel load – 8,000 kg; - maximum pushing force – 20 kN; - motion stroke – 100 mm; - testing plate dimensions: 1050x850x125 mm; - electrical power – 3.3 kW - working hydraulic pressure – 160 bar; - power supply 400/20V/A, 3f. - the stand is equipped with 2 installation frames, - 8 pcs. 5 m long hydraulic hoses; | GST 1602-D (Germany) or equivalent | Pcs | 1 |
| 71. | Pit-mounted lift Technical details: - lifting capacity: 20000 / 15000 kg; - cylinder stroke: 1,200 mm; - maximum lifting height: 2,175 mm; - maximum lifting duration: 150 s; - maximum lowering duration: 58 s; - trolley width: 845 mm; - trolley length: 1,100 mm; - cylinder rod diameter: 80 mm; - cylinder rod bore diameter: 55 m² - total hoist height: 975 mm; - operating pressure: 298 bar; - compressed air pressure: 10 – 15 bar; - compressed air requirements: 200 l/min; - Lift weight: 280 kg; - Energy node – hydro-pneumatic; - control – foot switch; - construction – moving along the pit floor; - rapid deployment – available; - self-lowering – present; - the surface is varnished; - anti-corrosion coating – Ultra High Solid Primer (UHS). | Blitz X 20/15 Basic (Germany) or equivalent | Pcs | 2 |
| 72. | brake testing stand Technical details: - maximum tested axle weight: 16 t.; - testing speed: 2,3 / 4,6 km/h; - measurement system: DMS (driving motor speed); - brake stand roller diameter: 280 mm; - brake stand roller length: 1,100 mm; - brake stand roller rise: 50 mm; - brake stand roller coefficient dry/wet: 0,8/0,7; - tested wheel diameter for brake stand min-max: 450 – 1200 mm; - vehicle track width min-max: 1000 – 3,200 mm; - single module dimensions (length x width x height): 1151 x 1300 x 666 mm; - weight: 710 kg; - distance between roller axles: 438 mm; - tested rim diameter min-max: 13 – 40; - resistance class: IP54; - upper edge height difference between brake stand and floor: 15 m² - Roller surface: - perforated metal plate DIN 791; - mesh length – 62 mm; - mesh width – 21 mm; - mesh height – 2 mm; - covered with plastic and corundum coating. - motor power: 2 x 12,5 kW. | ArBeissbarth BD 6460 (Germany) Or analogous | Pcs | 1 |
| 73. | Channel exhaust system For truck diagnostic workstation Technical details: - 16 m channel system with sliding trolley; - fan – 1 kW, 3 ph; - crush-resistant hose up to 200°C; - hose diameter – 150 mm; - hose length – 7.5 m; - hose Y-connector for double muffler; - quick coupling for nozzle and branch replacement; - rubber nozzle with fixer to muffler - control panel. - equipment: - 16 m channel system – 1 pcs; - extraction hose – 1 pcs; - T-joint for hose – 1 pcs; - rubber nozzle – 3 pcs; - fan – 1 pcs; - control panel – 1 unit; - installation set – 1 set | Worky (Italy) Or analogous | pcs | 1 |
| 74. | Digital-network headlight adjustment stand Beissbarth MLD 815 Technical details: - headlight adjustment for all types – Incandescent, halogen, xenon and LED; - measurement with CMOS camera; - saved photos of measured and adjusted light beam angles; - 5,7“ touch-sensitive LCD screen; - suitable for adjusting headlights of all vehicle categories (passenger cars, motorcycles, commercial vehicles, and trucks); - digital light intensity measurement (result shown on LCD screen) measuring unit – Lx; - laser (horizontal and vertical) positioning system; - high-quality, glass, scratch-resistant measurement lens; - high mechanical stability ensures precise measurements and adjustments; - digital guide for measurement and adjustment procedures – the device automatically specifies how and how much to adjust the respective headlight; - acoustic signal when the correct adjustment position is reached; - autonomous (rechargeable batteries) or wired power supply; - shock, dust, and scratch-resistant housing; - USB and RS232 connection for data transfer; - platform with 4 wheels; - aluminium alloy centring support; - the stand can be connected to a common inspection device network; ( Bosch or Beissbarth); - 3 m track set; - mass– 40 kg; | Beissbarth MLD 815 (Germany) Or analogous | Pcs | 1 |
| 75. | Mobile gasoline engine exhaust gas analyser TEM 899 and mobile diesel engine smoke meter; Beissbarth TEM 499, OBD scaner TEM299 Analyser TEM 899: - special software for vehicle testing; - Bluetooth wireless interface with PC; - complies with OIML R99 requirements; - complies with the European Measuring Instruments Directive 2004/22/EC; - CO measurement (0-9,99%, resolution 0,01%); - CO2 measurement (0-9,9%, resolution 0,1%); - HC measurement (0-10000 ppm, resolution 1 ppm); - O2 measurement (0-21,4%, resolution 0,1%); - NOx measurement (0-5000 ppm, accuracy 1 ppm); - Lambda coefficient measurement (0,5-5,0, resolution 0,001); - Battery pack ensures autonomous operation of the analyser for up to 8 hours; - maximum electrical power used 80 W; - operating environment temperature +5°C – +40°C; - automatic condensate drainage; - electronic automatic device zeroing; - automatic calibration via a special reference gas - cylinder; - response time < 10 s; - maximum heating time 60 s; - dimensions: 460x200x480 mm (with a trolley); - weight 9 kg (with a trolley). Smoke meter TEM 499: - special software for vehicle testing; - Bluetooth wireless interface with PC; - complies with OIML R99 requirements; - Complies with the European Measuring Instruments Directive - 2004/22/EC; - K coefficient measurement (0-9,99 m-1, resolution 0,01); - Absorption coefficient N measurement (0-99,9 %,accuracy - 0.1%); - Battery pack ensures autonomous operation of the analyser - for up to 8 hours; - Maximum electrical power 80 W; - Atmospheric pressure 850-1025 hPa; - Operating environment temperature +5°C – +40°C; - Effective smoke chamber length 200 mm; - Smoke chamber temperature 75℃; - Automatic condensate drainage; - Electronic automatic device zeroing; - Automatic calibration; - Maximum heating time 5 minutes; - Dimensions: 460x200x480 mm (with a trolley); - Weight 9 kg (with a trolley). OBD skaneris TEM299: - device designed for engine RPM and temperature reading and transmission to PC; - used together with the gas analyser and smoke meter; - Bluetooth interface with PC; - comes with a piezoelectric sensor and inductive clamps for - measuring engine RPM on vehicles older than EUR3; - can read engine RPM via the OBD interface; - the device can perform a vehicle OBD test; - supported protocols: ISO 9141, KW2000, PWM, CAN BUS and WWH-OBD. | Beissbarth MLD 815 (Germany) Or analogous | Pcs | 1 |
| 76. | spherical industrial mirror - Diameter: 600 mm; - recommended distance: 12 m; - colour: red; - Material: acrylic; - Shape: round; - weight: 6.6 kg | Beissbarth (Germany) Or analogous | Pcs | 4 |
| XVI. WASHING ROOM |  |  |  |  |
| 77. | Mobile washing equipment Technical details: - Voltage (Ph/V/Hz) 3 / 400 / 50; - Water flow (l/h) 600 – 1300; - operating pressure (bar/MPa) 30 – 200 / 3 – 20 - Water temperature (when supplied water temp. 12°C) (°C) 80 – 155; - Diesel or gas consumption under full load (kg/h) 8,3. - Fuel consumption in eco!efficiency mode (kg/h) 6,6; - Power (kW) 9,3; - Power cable (m) 5; - Fuel tank (l) 25; - dimensions (I x P x A) (mm) 1330 x 750 x 1060 mm; - LED display; | Kaercher HDS 13/20-4S (Germany) Or analogous | Pcs | 4 |
| XVII. PAINTING ROOM |  |  |  |  |
| 78. | Cargo transport painting – drying chamber with gas burner Technical details: - Width internal / external 5000 mm / 5150 mm; - Length internal / external – 12000 mm / 12150 mm. - Gates and doors: - Entrance doors on both sides – folding roll-up doors above - the chamber; - Gate dimensions- 4000 mm x 4000 mm; - Side service doors 700 mm x 2000 mm, 2 pcs. - Walls: - Chamber walls made of interlocking 75 mm thick steel - panels; - Wall thermal insulation layer made of expanded - polystyrene (EPS); - Walls are flame-resistant. - Floor: - Chamber floor – two rows of grids with a filtration system; - Grid segment width- 750 mm; - Foundation pit depth – at least 800 mm. - Air filtration system: - Air filtration system – 3 stages; - Efficiency of the pre-filters of the air intake system > 83%; - Efficiency of the ceiling air filtration system > 97%; - Efficiency of the floor air filtration system (made of glass - fibre) > 70 %. - Lighting: - At the wall/ceiling junction, 14 horizontal LED - light fixtures on each side; - On the walls below, 14 vertical LED light fixtures - on each side; - Each light fixture contains 4 pcs. 16 W LED - lamps, total number 112 pcs. - total lamp electrical power – 1792 W. - Ventilation units for air supply, heating, extraction and - filtration: - Air supply system: 2 electric motors of 7.5 kW each and 2 - pcs. ventilators; - Total air supply system performance – 50000 m³/h; - Air extraction system – 2 electric motors of 7.5 kW each and - 2 ventilators; - Air extraction system performance – 50000 m³/h; - Ventilator drive – direct - Power supply 400 V AC, 3 f.; - Heat exchanger material – heat-resistant stainless - steel; - Natural gas burner RIELLO FS20 (Italy), 200000 - kCal/h; - Automatic hot air recirculation during the drying cycle; - Maximum drying temperature – 60℃; - Internal barometer for measuring pressure inside the chamber; - Automatic extraction damper control by servomotor - for activation of painting/drying cycles; - Processes controlled on the control panel: - on/off, painting/ - drying, lighting, timer, - temperature regulation. - Air flow recuperators: - Cross-flow air flow recuperators – 2 pcs; - Plates and profiles made of aluminium; | Heatex Sweden Or analogous | Pcs | 140 |
| XVIII. PAINT PREPARATION ROOM |  |  |  |  |
| 79. | Paint mixing equipment set Set includes: - Paint storage/mixing cabinet/frame – 1 pcs; - Paint mixer – 1 pcs; - Paint scales – 1 pcs; - Cabinet dimensions – 1200x400x2250 mm. | Disolac (Spain) Or analogous | set | 1 |
| 80. | Work table made of stainless steel Technical details: - Dimensions (approximately) 1300x600x900 mm; - adjustable leg height; | Or analogous | Pcs | 1 |
| 81. | Metal rack Technical details - Dimensions (approximately) 1800x550x2000 mm; - adjustable height shelves, shelf load at least: 120 kg; | Or analogous | Pcs | 1 |
| XIX. OIL STORAGE |  |  |  |  |
| 82. | Tower 1 |  |  |  |
|  | Liquid dispensing-collection tower |  | Pcs | 1 |
|  | Oil dispensing reel |  | Pcs | 8 |
|  | Linear oil/antifreeze meter |  | Set | 9 |
|  | Oil/antifreeze dispensing gun |  | Pcs | 9 |
|  | Drip collection tray |  | Pcs | 9 |
|  | Antifreeze dispensing reel |  | Pcs | 1 |
|  | 5:1 used oil pump |  | Pcs | 1 |
|  | 1:1 used antifreeze pump |  | Pcs | 1 |
|  | Installation kit for used oil/antifreeze pump |  | Set | 2 |
|  | Electromagnetic valve for pump control |  | Pcs | 2 |
|  | Connection adapters |  | Set | 1 |
|  | Coil control/monitoring module |  | Pcs | 3 |
|  | Control keyboard |  | Pcs | 1 |
|  | LED screen for fluid volume |  | Pcs | 1 |
| 83. | Tower 2 |  |  |  |
|  | Liquid dispensing-collection tower |  | Pcs | 1 |
|  | Oil dispensing reel |  | Pcs | 8 |
|  | Linear oil/antifreeze meter |  | Set | 9 |
|  | Oil/antifreeze dispensing gun |  | Pcs | 9 |
|  | Drip collection tray |  | Pcs | 9 |
|  | Antifreeze dispensing reel |  | Pcs | 1 |
|  | 5:1 used oil pump |  | Pcs | 1 |
|  | 1:1 used antifreeze pump |  | Pcs | 1 |
|  | Installation kit for used oil/antifreeze pump |  | Set | 2 |
|  | Electromagnetic valve for pump control |  | Pcs | 2 |
|  | Connection adapters |  | Set | 1 |
|  | Coil control/monitoring module |  | Pcs | 3 |
|  | Control keyboard |  | Pcs | 1 |
|  | LED screen for fluid volume |  | Pcs | 1 |
| 84. | Tower 3 |  |  |  |
|  | Liquid dispensing-collection tower |  | Pcs | 1 |
|  | Oil dispensing reel |  | Pcs | 8 |
|  | Linear oil/antifreeze meter |  | Set | 9 |
|  | Oil/antifreeze dispensing gun |  | Pcs | 9 |
|  | Drip collection tray |  | Pcs | 9 |
|  | Antifreeze dispensing reel |  | Pcs | 1 |
|  | 5:1 used oil pump |  | Pcs | 1 |
|  | 1:1 used antifreeze pump |  | Pcs | 1 |
|  | Installation kit for used oil/antifreeze pump |  | Set | 2 |
|  | Electromagnetic valve for pump control |  | Pcs | 2 |
|  | Connection adapters |  | Set | 1 |
|  | Coil control/monitoring module |  | Pcs | 3 |
|  | Control keyboard |  | Pcs | 1 |
|  | LED screen for fluid volume |  | Pcs | 1 |
| 85. | Tower 4 |  |  |  |
|  | Liquid dispensing-collection tower |  | Pcs | 1 |
|  | Oil dispensing reel |  | Pcs | 8 |
|  | Linear oil/antifreeze meter |  | Set | 9 |
|  | Oil/antifreeze dispensing gun |  | Pcs | 9 |
|  | Drip collection tray |  | Pcs | 9 |
|  | Antifreeze dispensing reel |  | Pcs | 1 |
|  | 5:1 used oil pump |  | Pcs | 1 |
|  | 1:1 used antifreeze pump |  | Pcs | 1 |
|  | Installation kit for used oil/antifreeze pump |  | Set | 2 |
|  | Electromagnetic valve for pump control |  | Pcs | 2 |
|  | Connection adapters |  | Set | 1 |
|  | Coil control/monitoring module |  | Pcs | 3 |
|  | Control keyboard |  | Pcs | 1 |
|  | LED screen for fluid volume |  | Pcs | 1 |