Supplement 2.1. Document 34.

BIM specification.

CUSTOMER REQUIREMENTS FOR BUILDING INFORMATION MODEL (BIM) DEVELOPMENT (for design proposals, technical working project, construction, and completion stages)

CUSTOMER REQUIREMENTS FOR BUILDING INFORMATION MODEL (BIM) DEVELOPMENT (technical working project, construction, and completion stages)

- GENERAL PROVISIONS

- The customer requirements for the development of the building information model (hereinafter – the Customer Requirements) set out the requirements of the customer for the preparation of the construction project, implemented using building information modelling, taking into account the provisions of construction regulations, the customer's needs, and the specificities of the building.

- These Customer Requirements are prepared in accordance with the document “Customer’s Information Requirements” No.  D1-57 approved by the Minister of Environment of the Republic of Lithuania on 24 February 2022. Considering that these Customer Requirements contain all necessary requirements for building information modelling, the Customer does not require the Supplier to fill out the preliminary implementation plan of the Building Information Modelling project. (PIP).

- Building project information:

1. Customer

Ministry of National Defence of the Republic of Lithuania (company code 188602751)

2. Customer

3. Project name

Creation of military settlement infrastructure in Klaipėda district, Kairiai

4. Address:

Kairiai, Klaipėda district

- Project stages:

| Construction project stages | Construction project phases | Remarks |
| --- | --- | --- |
| Planning | Feasibility study/ S0 | Not applicable for procurement |
|  | Project programme/ S1 | Not applicable for procurement |
| Design | Design proposals/ S2 | Applicable for procurement |
|  | Technical working project / S3 | Applicable for procurement |
| Construction | Construction/ S4 | Applicable for procurement |
|  | Construction completion/ S5 | Applicable for procurement |
| Maintenance and use | Building maintenance and uses/ S6 | Not applicable for procurement |

INFORMATION ABOUT THE BUILDING INFORMATION MODELLING PROJECT

| 1. Stages, phases, and goals of the building information modelling project. BIM goals directly describe the Customer's expectations and areas that will be given the highest priority during the implementation control of EIR. When creating the BIM implementation plan (hereinafter – BEP), it is mandatory to clearly and in detail describe the strategy and processes for achieving these goals. |  |  |  |
| --- | --- | --- | --- |
| No. | Building life cycle stage | Objectives of the building information modelling project | REMARKS |
| 1 | 2 | 3 | 4 |
| 1. | Design proposals/ S2 | - Improve the quality and speed of decision-making. - Quality project solutions. - Accurate quantity take-offs. - Collection and use of information. | - To ensure accurate quantity take-offs and avoid additional costs due to unaccounted project solutions or improperly executed quantity take-offs, project take-offs are formed from the BIM model. - To ensure quality project solutions, avoid inaccurate or incorrect design solutions that would need to be changed and corrected, geometric, logical, and visual verification and control of BIM models is ensured. - To collect and use information in the further stages of the building life cycle, BIM models are transferred with the relevant information for use in other project development stages. A common data and collaboration environment (CDE) is used, where all relevant project information is stored. |
| 2. | Technical working project / S3 |  |  |
| 3. | Construction/ S4 |  |  |
| 4. | Construction completion/ S5 |  |  |

| - Building information modelling project implementation programme |  |  |
| --- | --- | --- |
| No. | Building life cycle stage | Results of the building information modelling project (properly implemented SGC stages) |
| 1 | 2 | 3 |
| 1. | Design proposals/ S2 | - With the Customer-approved BIM execution plan ( BEP). It must be fixed until the actual start of the design work. - With the Customer-agreed and used shared data environment ( CDE). Project documentation and BIM models are placed in the CDE. - BIM models prepared according to the S2 stage requirements (BIM application cases, BIM level of detail, etc.). |
| 2. | Technical working project / S3 | - With the Customer-approved BIM execution plan ( BEP). It must be fixed until the actual start of the design work. - With the Customer-agreed and used shared data environment ( CDE). Project documentation and BIM models are placed in the CDE. - BIM models prepared according to the S3 stage requirements (BIM application cases, BIM level of detail, etc.). |
| 3. | Construction/ S4 | - With the Customer-approved BIM execution plan ( BEP). To be approved before the actual construction work begins. - With the Customer-agreed and used shared data environment ( CDE). Project documentation and BIM models are placed in the CDE. - BIM models prepared according to the S4 stage requirements (BIM application cases, BIM level of detail, etc.). |
| 4. | Construction completion/ S5 | - With the Customer-agreed and used shared data environment ( CDE). Project documentation and BIM models are placed in the CDE. - BIM models prepared according to the S5 stage requirements (BIM application cases, BIM level of detail, etc.). |

| 3. Building information modelling application cases, aligned with the building information modelling project implementation programme, their connection to the building life cycle stages and phases. The Supplier-appointed BIM coordinator in the BIM execution plan (BEP) specifies measures, goals, performance indicators, possible deviations, and aligns them with the Customer-appointed Information Manager (BIM manager). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Building information modelling application cases | Stage S2 | Stage S3 | Stage S4 | Stage S5 |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Modelling of existing conditions |  |  |  |  |
| 2. | Economic quantity calculations |  |  |  |  |
| 3. | Planning of building information modelling project stages |  |  |  |  |
| 4. | Functional, volumetric, and plan evaluation |  |  |  |  |
| 5. | Visualization and review of building information modelling project |  |  |  |  |
| 6. | Design / modelling |  |  |  |  |
| 7. | Assessment of compliance of the building information model and the project |  |  |  |  |
| 8. | 3D coordination / clash detection |  |  |  |  |
| 9. | Execution modelling |  |  |  |  |
| - Modelling of existing conditions. This is the process during which a 3D model of the existing site, including surrounding buildings, the plot, and its dependencies, along with other information, is prepared for the building project. The model can be prepared in various cases, depending on the information required for the smooth development of the project. The existing conditions model can be created for the plot or the building to be used / reconstructed, in order to obtain spatial information and information about building elements. The plot (3D model of the plot surface), geodesy/geology, and modelling of existing buildings are based on measurements, information about existing buildings, and research conducted on the site. This information can be supplemented with drawings and other documents. BIM application at the relevant SGC stage - Stage S2: 3D model of the plot surface, underground utilities, access roads, and existing buildings. - In stages S3 and S4: 3D graphic and informational model of the designed plot surface, existing and designed utilities, access roads, and existing buildings and the designed building. - Economic quantity calculations. This is the process during which the BIM model is used to determine the technical-economic indicators of the building, assess the scope of work (quantities and required resources), and based on this, create estimates for all stages of the project life cycle. This allows the costs of changes made during the design stage to be monitored in a timely manner, helping control costs that exceed the budget. This process allows for the assessment of the impact of changes on the cost at all stages of the project development. BIM application at the relevant SGC stage - Stage S2: Assessment of the project's economic indicators: Preliminary quantities (e.g., building area, volume). - Stage S3: Generated quantities from the 3D model, with refined material quantities and engineering system solutions. - In stages S4, S5: Generated quantities from the 3D model, with refined material quantities and engineering system solutions. - Planning of building information modelling project stages. This is the process during which the BIM model information is used to show and manage the project's implementation duration schedule, perform design, construction, manufacturing, and building usage planning, visualize the sequence of design and construction processes, plan logistics, assess space specifics on the construction site, plan and manage building usage processes: - Stage S2: Preliminary determination of project implementation duration. - In stage S3: Detailed construction/manufacturing work schedule. - Stage S4: Refined construction work schedule. - Functional, volumetric, and plan evaluation. This is the process where 3D modelling software is used to accurately assess the project's characteristics in terms of functional, volumetric, and planning requirements. According to the requirements model, the spatial BIM model of the building (and its location on the plot) allows for the analysis of functional, volumetric, and planning solutions and the assessment of the complexity and compatibility of these solutions. Functional, volumetric, and plan solutions can be evaluated in different cases, for example, to avoid the intersection of functional zones in the building, divide the building into fire compartments, define the operating zones of engineering systems, assess safety and technological requirements, and more. After the evaluation, different simulation and analysis models (energy, indoor climate, life cycle (LCA) and life cycle cost (LCC) analysis, etc.) are prepared. During the functional, volumetric, and plan evaluation, using 3D modelling and visualization tools, the customer's needs are discussed, different options are analysed, and the best solution is obtained. BIM application at the relevant SGC stage - Stage S2: Volume model of the plot, internal volume model of the building, selected visualization variant. - In stage S3: Detailed volume model of the plot, internal volume model of the building, visualization. - Visualization and review of the building information modelling project. This is the process where the 3D model is used to visualize the project, conduct project reviews, analyze and compare project solutions, assess the scope of project solutions, and evaluate set criteria (aesthetics, lighting, compliance with requirements for people with disabilities, safety, ergonomics, etc.). Visualizations can be created using photo-realistic visualization and technical illustration technologies. Visualizations and technical illustrations are used as a collaboration tool to ensure smooth information management among project participants. Project changes in real time are possible based on feedback after the review of project solutions from end-users and the customer. The evaluation of designed Facilities and space can be facilitated using virtual reality or augmented reality simulation tools. BIM application at the relevant SGC stage - Stage S2: 3D visualization of the plot and building elements. - In stage S3: Detailed 3D visualization of the plot and building elements. - Design / modelling: This is the process where 3D software is used to prepare the BIM model. The main BIM design tools are divided into two groups: modelling tools and checking/analysis tools. Using the project modelling tools, architectural, structural, and engineering system elements of specific geometries are modeled, assigning the necessary attribute information to them and linking them with external databases. Once the building information model is created, 2D and 3D project documentation is obtained. A properly prepared model is then used for simulations/analyses, applying checking/analysis tools. BIM application at the relevant SGC stage - Stage S2: Drawings, reports, quantity schedules, technical specifications, analysis reports. - In stage S3: Drawings, reports, quantity schedules, technical specifications, analysis reports. - Building information modelling and project compliance assessment: This is the process during which the BIM tools are used to evaluate the compliance of the developed building project with the standards and requirements provided in construction technical regulations and other legal documents. One of the most important compliance evaluations is whether the project's 2D drawings have been exported from the BIM model and whether the 2D drawings comply with the BIM models: - Stage S2: Evaluation of compliance with design proposals is performed, after which 2D drawings are exported. - In stage S3: Evaluation of compliance with the working project is performed, after which 2D drawings are exported. - Stage S4: During the construction work, if there are changes to the working project, an evaluation of compliance with the working project is performed, after which 2D drawings are exported. - Stage S5: After the completion of construction work, a compliance assessment of the BIM model, physically completed works, and 2D drawings is performed, after which the 2D drawings are exported. - 3D coordination / clash detection: This is the process in which clash detection and localization software is used for the 3D coordination of the project, aiming to identify and eliminate potential clashes between different disciplines (e.g., structural and mechanical parts) of the project models. This is a key collaboration tool among the project participants. 3D coordination is also performed to identify potential discrepancies between the building and its systems and the existing conditions on the construction site. The purpose of 3D coordination and clash detection is to identify and resolve clashes in the virtual space before the start of construction, installation, or manufacturing. BIM application at the relevant SGC stage - Stage S2: Verification of the existing conditions model and the design models of different disciplines, elimination of clashes. - In stage S3: Project model collision checks, removal of intersections. - Stage S4: Project model collision checks, removal of intersections. - As-built modelling: This is the process in which 3D modelling software is used to prepare the as-built model of the building based on project, construction, and manufacturing models, i.e., after construction work or during the works, solutions in BIM models are adjusted (if there were changes during construction). The customer plans to use the as-built model later during the building's operational phase, for repair, renovation, or reconstruction projects. - Stage S5: The as-built model of the building and its elements, as well as the engineering systems model, is prepared after the completion of the construction works. Possible discrepancies between the elements depicted in the BIM model and the actual constructed building: ±0,5%, |  |  |  |  |  |

| 4. Training needs |  |  |  |
| --- | --- | --- | --- |
| No. | Training objective | Duration of training | REMARKS |
| 1 | 2 | 3 | 4 |
| 1. | The supplier must provide training and continuous technical consultations for all project participants on working with the selected and agreed CDE environment. | The training should not exceed 2–4 hours. | In case of technical or security incompatibility of the selected CDE environment with the customer's IT infrastructure, BIM models must be viewable using the customer's free BIM Vision viewing software. |

| 5. Project information structure |  |  |  |
| --- | --- | --- | --- |
| No. | Project information model type. | Purpose of the project information model | REMARKS |
| 1 | 2 | 3 | 4 |
| 1. | The BIM coordinator appointed by the supplier, in the BEP document after the contract signing, must detail the project information structure and agree with the customer-appointed Information Manager (BIM manager). | Different disciplines, zones, and so on, where the work will be performed (e.g., model division into modelling zones, so that model creation can be assigned to different teams, allowing work to take place in the same environment simultaneously), are clearly defined. |  |
| 2. | Ensuring the novelty of the information created by the project team, using CDE (angl. common data environment). The BIM coordinator appointed by the supplier in the BEP document must detail the diagram that describes the processes for ensuring the novelty of PIM. | Ensuring the accessibility, clarity, and dissemination of current documentation to the Project team based on their role in the project, access rights, etc. |  |

| 6. Data separation and linking principles of the project information model |  |  |  |
| --- | --- | --- | --- |
| No. | Data separation and linking principles of the project information model |  | REMARKS |
| 1 | 2 |  | 3 |
| 1. | Models are divided by project disciplines. Separation and linking principles are detailed with the customer-appointed Information Manager (BIM manager) in the BEP document. Prepared models of different parts of the project are linked into a single combined model in IFC format. |  |  |
| 2. | The creation and administration of comments, suggestions, and tasks are carried out in the CDE environment, linking them to a specific document or BIM model element(s). |  |  |
| 3. | A unified measurement unit system is implemented across all project stages to ensure coordinate consistency and eliminate the possibility of differing scales. |  |  |
| 4. | BIM models are prepared separately for each part of the project: - Site plan: SP - Building architecture: SA - Building structure: SK - Heating: Š - Ventilation: V - Air conditioning: OK - Electrical engineering: E - Electronic communications: ER - Heat production: ŠG - Heat supply: ŠT | - External water supply and drainage: LVN - External electrical engineering: LE - External electronic communications: LER - Street lighting: GA - Security alarm system: AS - Fire detection and alarm system: GAS - Fire safety: GS - Process control and automation: PVA - Solar power installation: SE - etc. |  |

| 7. Classification system |  |  |
| --- | --- | --- |
| No. | Classification system | REMARKS |
| 1 | 2 | 3 |
| 1. | The National Construction Information Classifier (NSIK), regulated by the Minister of Environment's Order No. 28 October 2024 No. D1-364 "On the Approval of the National Construction Information Classifier," must be used in the project. |  |
| 2. | The minimum NSIK information required is provided in the table below: NSIK detail. <C> COMPLEXES; <E> BUILDINGS; <B> SPACES; <L> ELEMENTS; <H> SGC stages; PROCESSES; <U> Construction INFORMATION; PRODUCTS. Functional systems; Technical systems; Components; <F> Construction WORK; <U>A Project PARTS; <U>B Project PARTS; <U>C Facility CATEGORIES; <U>D Construction DOCUMENTS; <U>E Building TYPES; <P> Building MATERIALS. Level 1 classes; Codes; □; □; □; □; □; □; □. Terms; □; □; □; □; □; □; □. Level 2 classes; Codes; □; □; □; □; □; □; □; □; □; □; □. Terms; □; □; □; □; □; □; □; □; □; □; □. Level 3 classes; Codes; □; □; □; □; □; □. Terms; □; □; □; □; □; □. Types; Codes; □; □; □. Terms; □; □; □. Subtypes; Codes; □. Terms; □. User types; Codes; □; □; □; □; □; □; □; □; □; □; □; □; □. Terms; □; □; □; □; □; □; □; □; □; □; □; □; □. |  |
| 3. | An additional classification system may be provided upon the Supplier's request, with the choice agreed with the Customer-appointed BIM Manager in the BEP document. |  |
| 4. | The selected classification system, along with the codes for all classes used in the project, must be included in the BEP document. | - |
| 5. | The choice of classification system is agreed upon with the Customer-appointed Information Manager (BIM Manager) in the BEP document. The selected classification system (or its elements) must be further used to structure the system and element hierarchy in the information submission plan and to assign fields for information attributes, parameters, types, or other information groups. | - |

| 8. Ensuring the integrity and quality of the project information model. The goal of model coordination and clash detection is to establish rules for information coordination and clash checking, as well as error control guidelines, in order to reduce the number of clashes and corrections in the model during project implementation (across all stages and phases of the project's life cycle). Possible quantity discrepancies between project schedules and quantities generated in the BIM model: ±5%. Integrity check of the model (TAPA) Consistency / Integrity check) is carried out in the unified project model and individual project model parts environments, conducted by the Supplier-appointed BIM coordinator. These checks/searches are integrated into the model control mechanism, which should ensure fewer errors and help avoid unnecessary and excessive amounts of model information. This is a key tool to coordinate the work of different project model parts (disciplines) and various project participants. Similarly, the BIM project change rules (strategy) are agreed upon. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Review | Objective is review | Responsible person | Data formats | REMARKS |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Visual inspection | Check whether there are any inappropriate BIM model elements and whether the project solutions are correctly represented. | Supplier's BIM coordinator, Other project participants | Models are reviewed in IFC format | The software chosen by the Supplier is specified in the BEP document. Inspection is carried out no less than 1 once a month. The final check is carried out by the Customer-appointed Information Manager (BIM Manager). |
| 2. | Clash detection | Perform geometric and logical BIM model clash checks (collisions) in the relevant project section and between different project parts BIM models, manage the clash correction process. | Supplier's BIM coordinator | Reports are provided in BCF or another format agreed upon in the BEP document. |  |
| 3. | Model integrity check | Check whether the combined BIM model meets the integrity requirements (missing, duplicated elements, etc.) specified in the EIR and BEP. |  |  |  |
| 4. | Information check | Check whether all BIM model elements contain the correct information (parametric data). |  |  |  |
| 5. | Model integrity verification | Check for any undescribed, incorrectly defined, or duplicated elements. |  |  |  |
| 7. | Standards and requirements check | Ensure compliance with BIM and CAD principles, standards, and requirements specified in the EIR and BEP documents. | Supplier's BIM coordinator | Reports are provided in the CDE environment or another format agreed upon in the BEP document. | Inspection is carried out no less than 1 once a month. |
| REMARKS: - The Supplier must assess that, if any justified BIM model discrepancies arise during the project or if it becomes clear that a faulty design solution requires correction or replacement, the Supplier commits to adjusting the BIM model and submitting it to the Customer-appointed Information Manager (BIM Manager). - Final BIM models submitted to the Customer must not have unacceptable clashes between elements. The acceptable and unacceptable element clashes are determined by the project preparer – the Project Manager, considering the Construction Law, technical construction regulations, and other legal acts governing the construction process. The Project Manager specifies the unacceptable and acceptable element clashes in the BIM implementation plan (BEP). The Customer-appointed Information Manager (BIM Manager), if necessary, specifies and provides comments on the unacceptable and acceptable element clashes as outlined in the BEP. - Building information models must be properly divided according to spaces, systems, elements, etc. - Possible quantity discrepancies between project schedules and quantities generated in the BIM model: - those that are not geometrically represented in the model: ±5%, - those that are geometrically represented in the model, but the measurement unit in the schedule is not a unit (pcs.), but, for example, m2, m3, t, etc. (e.g., walls, floor slabs, etc.): ±0,5%, - those that are geometrically represented in the model and the measurement unit in the schedule is a unit (pcs.) (e.g., windows, doors, columns): 0%. |  |  |  |  |  |

| 9. Collaboration processes and procedures |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Meeting objective | Building information modelling project stage | Frequency | Participants | Place |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | The project team meetings are organised by the Supplier-appointed BIM coordinator with the project team. During each meeting, BIM model comments, progress results, and other relevant information must be presented. | At all stages of the building life cycle | At least every 3 weeks | - Supplier's BIM coordinator, - Designers - Customer-designated Information Manager (BIM manager) | Preferably remotely |
| 2. | The Supplier-appointed BIM coordinator must ensure the update and submission of the BIM model schedule to the Customer-appointed Information Manager (BIM Manager). The report format is agreed upon with the Customer-appointed Information Manager (BIM Manager) during the preparation of the BIM implementation plan (BEP). | At all stages of the building life cycle | At least once a month. | - Supplier's BIM coordinator, - Customer-designated Information Manager (BIM manager) | By email or during a remote meeting |

| 10. Data naming rules, requirements, standards |  |  |
| --- | --- | --- |
| No. | Data naming rules, requirements, standards | REMARKS |
| 1 | 2 | 3 |
| 1. | The files must have one immutable name to ensure smooth interfacing. For example, the architectural technical work project IFC model would be named 111-TDP-SA.ifc, where: • 111 – Project number. • TDP – Technical work project stage. • SA – Project part, which would be different for each case (according to the part). • .ifc – File format. |  |
| 2. | The markings are based on the recommendations of the Lithuanian Design Companies Association R14-2011 “Abbreviations and Letter Markings in Construction Project Documentation.” |  |

| - Information requirement level and information readiness level. At the start of each project stage, for each applicable method (or several methods), the Supplier-appointed BIM coordinator must prepare and coordinate with the project team and the Customer-appointed Information Manager (BIM Manager) a BIM information delivery plan, which must detail the requirements for each BIM model system and element regarding graphical, geometry (level of geometry – LOG / level of development LOD), and attribute (parametric) information (level of information – LOI) development levels (hereafter referred to collectively as the level of detail LoD). The Supplier-appointed BIM coordinator coordinates the model development levels LOG/LOD and LOI in the BEP document, but the development level of elements in the relevant project parts must not be lower than: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Project part: | Information requirement level and information readiness level |  |  |  | Minimum attribute (parametric) information level (LOI) |
|  |  | SGC S2 stage | SGC S3 stage | SGC S4 stage | SGC S5 stage |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1. | BD | BIM model is not prepared | BIM model is not prepared | BIM model is not prepared | BIM model is not prepared | - Identification parameters (Name, Type, Brand, Material, Colour, Finish, Energy class, Power). - Classifier information (Functional system type, Technical system type, Element/component type). - Reference to drawing or Technical specification number. - System dimensions (Height, Length, Width, Thickness, Weight, etc.). - Fire part (Fire resistance class, Combustibility class, Environmental aggressiveness class, Sound class) (if applicable). |
| 2. | SP | LOD 200 | LOD 200 | LOD 200 | LOD 200 |  |
| 3. | SA | LOD 300 | LOD 350 | LOD 350 | LOD 350 |  |
| 4. | SK | BIM model is not prepared | LOD 350 | LOD 350 | LOD 350 |  |
| 5. | HVAC | BIM model is not prepared | LOD 300 | LOD 300 | LOD 300 |  |
| 6. | E | BIM model is not prepared | LOD 300 | LOD 300 | LOD 300 |  |
| 7. | SO | BIM model is not prepared | BIM model is not prepared | BIM model is not prepared | BIM model is not prepared |  |
| 8. | Kt. | LOD 200 | LOD 200 | LOD 200 | LOD 200 |  |
| 9. | Inventory | LOD 200 | LOD 200 | LOD 200 | LOD 200 |  |
| Project part detailing: - BD – general part. - SP – Site development (site plan); transportation section; outdoor engineering networks (outdoor water supply and sewage, outdoor electronic communications, outdoor electrical networks, relocation of outdoor electrical networks, outdoor electronic communications, outdoor gas pipeline networks, subscriber outdoor electrical networks, street lighting networks, outdoor heat networks, outdoor heat networks, etc., including other infrastructure serving the building outside the site boundaries (if designed)). In cases where a project part is developed by a third party (e.g., ESO), it is necessary to check and ensure that the third party's project part is coordinated within the overall BIM model (a 2D drawing interface (link) may also be used). If modeled outdoor engineering networks intersect with existing non-reconstructed and unchangeable outdoor engineering networks, then the existing outdoor engineering networks are represented at the intersection point (at least 1 meter in length). Including, but not limited to, the representation of pavement, rainwater drainage, location of the treatment plant, small architectural elements, etc. Represented in separate layers: existing, designed, and intersecting plants. If technically possible, earthworks are calculated using the data from the current topographic survey and the modeled proposed ground surface. - SA – building architecture; Interior (if designed). It is important to represent shared elements (e.g., suspended ceilings, frames, etc.). When modelling the interior part, there must be an option to export specific finishes quantities (painting, plastering, window frame trimming, etc.) – to model in separate IFC model layers, colourfully represent the modelled elements (floor coverings, wall finishes, window frames, sills, baseboards, furniture, etc.) as closely as possible to the actual colours. - SK – building structure. - HVAC – Water supply and sewage removal; Heating, ventilation (including mechanical smoke removal, if required), and air conditioning; Heat production and transformation (heat point, renewable energy sources, if designed); Heat production and supply (heat point, renewable energy sources). Represented are volumetric Facilities at the endpoints (sinks, toilets, radiators) in shapes close to those of the actual Facilities. - E – Electrical engineering (including lightning protection); Electronic communications (telecommunications); Fire detection and alarm systems (including fire warning and evacuation control systems); Security alarms; Fire safety; Process control and automation. Represented are volumetric Facilities at the endpoints (light fixtures, switches, sensors, ladders, troughs, etc.) in shapes close to those of the actual Facilities. Electrical cables are not modelled. - SO – Construction preparation and construction work organization section. Temporary access roads, temporary construction site fencing, storage location for construction materials, tower (automobile) crane, and other mechanisms necessary for preparing this project section must be represented. - Other. – the project sections mentioned above, but designed or installed according to the design task (project task), including but not limited to – furniture, technological section. The relevant topographic survey (two-dimensional – 2D, in dwg format) must be placed in the CDE environment with the ability to activate/deactivate the topographic image across the entire design area. When creating the Project quantities schedules and/or Construction cost estimation (KS) section, quantities of elements must be generated from the BIM model. - Inventory – when carrying out the reconstruction, repair, etc., of an existing building or demolition works, all existing building structures must be represented. Including structures where no design/modelling or future construction works are performed. Applicable to the SA, VN, HVAC, E project sections, as specified in the corresponding points 3, 5, and 6 above. When demolition works are carried out and no existing structure is left, there is no requirement to model the demolished building. Remarks: - When creating the list of attribute information in the BEP document, it is necessary to list all model elements and specify what exact attribute information will be provided for each model element. - The attribute information names for the elements must match (be the same) in all BIM models of the project sections. It is aimed that uniform attribute information should be in the same column when exporting quantities schedules from BIM models of different project sections. - Project nodes (principal project nodes developed within the scope of the Technical project), such as floor node, facade node, etc., must be integrated into the corresponding project's building information model as attribute information for the respective element. - LOG, LOI, and LOD development levels are described in more detail and visually presented in the international free BIM Forum publication. Level of development (LOD) specification: https://bimforum.org/resource/level-of-development-specification/. Generally, the LOD values: - LOD200: A model element is graphically presented in the model as a system, Facility, or set with approximate quantities, size, shape, location, and orientation. Non-graphical information may also be added to the model element. - LOD300: A model element is graphically depicted in the model as a specific system, Facility, or set, which can be expressed by quantity, size, shape, location, and orientation. Non-graphical information may also be added to the model element. - LOD350: Sets of model elements are graphically presented in the model as a specific system, Facility, or set by quantity, size, shape, location, orientation, and connections to other construction systems. Non-geometrical information may also be added to the model element. - LOD400: A model element is graphically presented in the model as a specific system, Facility, or set, considering its size, shape, location, quantity, and orientation, with detailed information intended for production, assembly, and installation at the construction site. Non-graphical information may also be added to the model element. - Main drawings, plans, sections, schedules, and other documentation necessary for the project must be generated from the corresponding project section's BIM model and inseparable from it. If changes are made in the BIM model, there must be an option to automatically regenerate the drawings, i.e., ensuring that the BIM model will not have discrepancies with the paper project version. |  |  |  |  |  |  |

| 12. Two-dimensional computer-aided design visualization standards |  |  |
| --- | --- | --- |
| No. | Two-dimensional computer-aided design visualization standards | REMARKS |
| 1 | 2 | 3 |
| 1. | Lithuanian standard LST 1516:2015 "Building Project. General design requirements" and all valid amendments. | It must be coordinated with the Customer when preparing the detailed building information modelling execution plan (BEP). |
| 2. | A direct connection between the BIM model and the design 2D documentation must be ensured. In general, the project drawings are formed: - From the digital information model – BIM model (except in cases where this is not technically possible and this is separately described and coordinated in the BEP document); - Final drawings are formed only when the confirmation from the Customer's designated BIM manager is received, stating that the project is of high quality and suitable for the creation of drawings. |  |

| 13. Types of project information models and data formats |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Project information model type. | Brief description of the project information model | Data submission and/or creation formats | Data exchange formats | Data storage formats |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Models | 3D models of project sections | The Supplier-appointed BIM coordinator details the BEP document | .ifc; .landXML etc. | .ifc; .landXML etc. |
| 2. | 2D project drawings | Project drawings generated from the model | .dwg; .pdf etc. | .pdf etc. | .pdf, .adoc etc. |
| 3. | Textual project section | Explanatory project section, text | .docx and kt. | .docx; .pdf etc. | .pdf, .adoc etc. |
| 4. | Graphs, tables | Various project calculators, project implementation schedule | .xlsx etc. | .xlsx etc. | .pdf etc. |
| 5. | Clash report | Clash detection analysis document, describing and identifying problem areas and proposing a solution approach. | The Supplier-appointed BIM coordinator details the BEP document | .xlsx, .pdf, .bcf etc. | .xlsx, .pdf, .bcf etc. |

| 14. Project information model's coordinate system and geospatial position. Modelling rules are developed by the Supplier's designated BIM coordinator, using the modelling recommendations provided by the main software packages used in the project. The BIM coordinator may specify databases for storing already created model elements and for use in creating the building models, or suggest specific modelling recommendations and methodologies. It is also necessary to specify the measurement system (SI system). |  |  |
| --- | --- | --- |
| No. | Project information model's coordinate system and geospatial position | REMARKS |
| 1 | 2 | 3 |
| 1. | The BIM model can be modeled in the working environment in a coordinate system chosen by the project team. |  |
| 2. | For BIM model coordination, the BIM model must be provided in the global coordinate system, considering the model's orientation towards the directions of the world countries and assessing the actual altitude. For general model coordination in supervision programs, the coordinate of the intersection point of the model axes must be indicated, for example, the intersection of axes A and 1 and its longitude and latitude according to global coordinates and the LKS 94 system and LAS 07 height system. |  |
| 3. | The project BIM coordinator records the coordinates, and all project participants must adhere to them. |  |

| 15. Project information model settings |  |  |
| --- | --- | --- |
| No. | Project information model settings | REMARKS |
| 1 | 2 | 3 |
| 1. | In exported .ifc, landXML, or other format models, stored in the Customer's CDE, the default measurement unit must be set to meters. |  |
| 2. | Ensure different color markings for elements in all BIM models. The color of each specific element must be agreed upon during BEP preparation. |  |

| 16. Software |  |  |  |
| --- | --- | --- | --- |
| No. | Software purpose | Software requirements | REMARKS |
| 1 | 2 | 3 | 4 |
| 1. | The list of software used and its version must be filled in by the Supplier's designated BIM coordinator in the BIM implementation plan (BEP) | Only legal software must be used. The Contractor must provide documentation confirming the acquisition or authorised use of the software planned for the Project. Software that meets OpenBIM criteria must be used. | Software that meets OpenBIM criteria is listed on the international BuildingSmart organization's website: https://www.buildingsmart.org/compliance/software-certification/certified-software/ |

| 17. Data security. The CDE environment administered by the Supplier's designated BIM coordinator must ensure the highest data security requirements. |  |  |
| --- | --- | --- |
| No. | Data security requirements | REMARKS |
| 1 | 2 | 3 |
| 1. | The possibility of ensuring that the CDE environment meets the highest data security requirements regulated by: The Law on State and Official Secrets of the Republic of Lithuania, the Law on Legal Protection of Personal Data of the Republic of Lithuania, the Cybersecurity Law of the Republic of Lithuania, and the related legal acts, the EU General Data Protection Regulation (GDPR), and any other national or EU legal acts governing the principles of information security and privacy. Ensure that any other, not previously mentioned requirements for CDE security, as defined in the Resolution of the Government of the Republic of Lithuania of 13 August 2018 No. 818 "On the Approval of the National Cybersecurity Strategy," are met as needed. |  |
| 2. | Ensure that each person handling data and connecting to the CDE with user rights is: uniquely identifiable. Connects to the system using a password. |  |
| 3. | When preparing the BIM implementation plan and creating the BIM data exchange and project team communication infrastructure (CDE), the project team must plan for the implementation of data protection measures for the model data. The goal of data security is to assign administrative rights to the project participant, i.e., to set limits for the use of the relevant information. The administrative rights, with agreed scope and detail, for each specific project participant are specified in the BIM implementation plan. |  |

| 18. General data environment. It is the responsibility of the Supplier's designated BIM coordinator to organize, administer, and ensure smooth data exchange within the general data environment. (TAPA) Common Data Environment (CDE)). The general data environment (CDE) is a central repository (cloud-based), where information about the construction project is stored – project documentation, graphical models, and non-graphical descriptions (texts, descriptions, protocols, cost estimates, etc.). The aim is to improve collaboration among project team members (Contractor, Customer, Designer) by using a single source of information, which helps reduce errors and avoid information duplication. |  |  |
| --- | --- | --- |
| No. | Requirements for the project’s general data environment | REMARKS |
| 1 | 3 | 4 |
| 1. | To ensure effective collaboration and communication between different project participants, the Supplier chooses an appropriate CDE at their discretion and coordinates it with the Customer’s designated Information Manager (BIM manager). The selected CDE must ensure the following minimum functionalities (requirements): - Security and control. The ability to restrict user rights, record participant actions. User access management at the file level, - Database. The ability to store documents, create a directory structure; - Versioning. Creation of document versions, avoiding excessive numbers of documents; - IFC reading online. The ability to open the IFC file format in the CDE online environment and perform commenting/note writing functions and link/unlink different BIM models from different project parts. - Access via browser. The ability to connect remotely via a tablet, mobile phone, or computer through an internet browser without installing specialized software on the Customer's computers. - The CDE environment must provide the ability to upload the current topographic image (2D, dwg format) with the option to activate/deactivate the topographic image. | Considering the Customer's security restrictions, it is necessary to provide at least three possible alternative CDE environments. The final CDE to be used in the project is selected based on the minimum requirements listed in column 3 and the actual possibility of using the CDE on the Customer's computers. |
| 2. | The Supplier undertakes to provide up to 10 licenses (if paid) free of charge to the Customer's team members for access to the model geometry, attribute information, and documentation through the agreed CDE environment, throughout all project phases. | The Customer must be provided with the ability to review and monitor the entire building information model during the BIM implementation period. |
| 3. | The Supplier’s designated BIM coordinator must coordinate the project's folder and file structure in the BEP document. |  |

| 19. Responsibilities and accountability for the building information model |  |  |
| --- | --- | --- |
| No. | Responsibilities for building information modelling in the project | Accountability for building information modelling in the project |
| 1. | 2 | 3 |
| 1. | Customer's representatives – Project manager, Building construction technical supervisor, Building user, etc. | - Visual BIM model review, - BIM model and project comments submission - Approval of design solutions, - Other BIM process-related responsibilities. |
| 2. | Customer-designated Information Manager (BIM manager) | - Coordinate and approve the BIM implementation plan prepared by the Supplier, - Provide comments and suggestions for the BIM execution process, - Check the geometric and informational BIM model verifications carried out by the Supplier's BIM coordinator and provide feedback to the Supplier's designated BIM coordinator, - Approve the suitability of the final BIM models and the fulfilment of other Customer’s requirements for BIM preparation, - Ensure timely communication with the Customer’s representatives (Project manager, Building construction technical supervisor, Building user, etc.) regarding the progress of BIM models and other requirements fulfilment. |
| 3. | The Supplier's designated project participants – Project managers, Project section managers, Construction managers, etc. | - Visual BIM model review, - BIM model and project comments submission - Collaboration within the project's CDE environment, - Other BIM process-related responsibilities. |
| 4. | The Supplier's designated BIM coordinator | - Create and coordinate the BIM implementation process for the project, distribute BIM tasks, control project quality, and periodically provide current situation/progress reports to the Customer's designated Information Manager (BIM Manager), - Preparation, coordination with the Customer's designated Information Manager (BIM Manager), execution, and control of the BIM execution plan (BEP) and other BIM documents with the execution of other project participants, - Ensure the compatibility and quality of the BIM model and its individual parts by performing geometric, informational, logical, visual, and other types of BIM model checks and providing feedback to project participants, - Ensure the submission of reports on performed checks (visual, clashes, model integrity, etc.) to the Customer's designated Information Manager (BIM Manager) at least once a month. The report format – .bcfzip, .bcf, or other formats allowing the visualisation of clashes, - The BEP document must include the principle matrix for performing clash checks, - Administer the CDE environment and ensure the timely collection, storage, sharing, and transfer of project data within the CDE environment. Ensure that all project team members can share information; - Organise and control information management processes, - Prepare BIM execution reports for the Customer's designated Information Manager (BIM Manager). - Advise the project team on BIM matters, - Ensure the suitability of the final BIM models and the fulfilment of other Customer’s requirements for BIM preparation, - Provide timely updates to the Customer's designated Information Manager (BIM Manager) regarding the progress of BIM models and other requirements fulfilment, - Coordinate roles and responsibilities, particularly the responsibility for coordinating various design disciplines during the design phase, - Establish the order of listing, - Agree on the creation and maintenance of specific project codes. |

| 20. Transfer of work and completion of the SGC phase |  |  |
| --- | --- | --- |
| No. | Transfer of completed work to the Customer | Formats for transferring completed work |
| 1. | 2 | 3 |
| 1. | After preparing the relevant project for the stage of the building's lifecycle (S2, S3, S4, S5), the Supplier’s designated BIM coordinator shall transfer the BIM models to the Customer's Information Manager (BIM Manager) on CD or DVD media with full rights to use the information within the building's lifecycle, i.e. the transferred rights will ensure the continuity and use of the created information in subsequent project phases (e.g., publishing in Customer's procurements, such as announcing a contracting competition, etc.). This transfer of rights to use the created BIM models in no way implies the transfer of the Project's copyright. | - The BIM models of the relevant project section and the combined BIM model of all sections in IFC format with full geometry, attribute, and attached information in no less than IFC 2x3 version format. - The BIM models of the relevant project section in original formats (native software format) (DGN, RVT, PLN, etc.), the non-graphical part of the information model (DBF or XLSX format), the textual part (PDF and DOCX or other similar formats). |