Annex 1

TECHNICAL SPECIFICATION

FOR AIRSPACE DESIGN SOFTWARE, ITS UPDATES, SUPPORT AND TRAINING

- GENERAL INFORMATION

  - Contracting authority: public limited liability company Oro Navigacija, the sole provider of air navigation services, comprising aeronautical information services (AIS), air traffic services (ATS), airspace management (ASM) services also communications, navigation and surveillance (CNS) services at Vilnius flight information region (FIR).

Address: Balio Karvelio St. 25, LT-02184 Vilnius, Lithuania, VAT payer code LT100604610. Data collected and stored at the Register of Legal Entities, code of the company 210060460.

- DEFINITIONS AND ABBREVIATIONS

  - This technical specification contains the following definitions and abbreviations:

Supplier – A tenderer whose tender for the purchase is awarded to the winner in accordance with the procedure specified in the purchase conditions and with whom the Contract will be signed;

Customer – public limited liability company ORO NAVIGACIJA

Contract – a contract of purchase and sale of products and/or services which is signed between the Supplier and the Customer;

Software – Software for airspace design.

- OBJECTIVE

  - Products and services to be purchased – the installation of Airspace Design software, updates and support, including training for Customers’ personnel.

- GENERAL REQUIREMENTS

  - Supplier has to confirm that Airspace Design software does not contain any additional or malicious code, which is not needed for a Software to run.

  - If malicious code is identified in the provided Software, this will be treated as non-compliance and non-compliance with contract terms. In that case Supplier has to cover costs associated with the material damage suffered by the customer.

  - The Supplier shall guarantee compliance with the organisational and technical cyber security requirements established under Resolution of the Government of the Republic of Lithuania No. 818 of 13 August 2018 (latest version) (818 Dėl Lietuvos Respublikos kibernetinio saugumo įstatymo įgyvendinimo; hereinafter referred to as the Requirements), applicable to the cyber security subjects.

- SOFTWARE REQUIREMENTS

  - The Airspace Design Software should be AIXM 5.1(or latest version by delivery time) compatible and operate under AIXM/GIS environment, also shall provide a tool for visual creation and modification of airspaces including airways

  - The Airspace Design Software must be fully interoperable with flight procedure design software system PANDA and the charting software system SIGMA (Oro Navigacija’s existing flight procedure design software).

  - The Airspace Design Software shall access the aeronautical database through an AIXM interface and view the data in 2D and in 3D Viewer of the software. Also, airspace(s)/airway(s) data shall be available in AIXM 5.1 format, including Meta Data.

  - The navigation database should be able to enter Lithuanian airspace data by AIS staff and store them as the following data item:

    - En-route waypoints

    - En-route airways

    - Airport/Heliport

    - Restricted airspace

    - En-route airway restrictions

    - Preferred routes and Route Segment

    - NAV AIDs

    - Enroute and FIR boundaries

    - Airspace and Airspace Volume

    - TMA, CTR and other ATS airspace

    - Topographic information compatible with eTOD area 1 requirements

    - Designated Point

  - The Airspace Design Software shall have the following features using the source data:

    - Designing Area

    - Designing Buffer

    - Designing_Point

    - Designing_Segment

  - The software shall be able to create and edit the En-route segments and ensure their storage in AIS Database in compliance with AIXM 5.1 data exchange model

  - The software shall be able to develop and visualize elementary and composite airspace structures

  - The software shall be able to overlap nominal trajectories and protection areas of flight procedures with airspace structures

  - The software shall be able to construct protection areas for en-route, terminal and holding procedures and buffer zones for airspace structures in order to determine required airspace volume and aircraft separation planning

  - The software shall be able to perform navigational calculations and geometric constructions for airspace reconstruction and optimization purposes

  - The software shall be able to store information with a geographical location and level (3 dimensions)

  - The exchange format of Airspace Design Software is AIXM 5.1 (or latest version by delivery time) Snapshot. The software shall also allow textual format for presenting airspace (in accordance with AIP) for regeneration of Airspace Volume geometry

  - Browsing data shall be provided in different ways to meet the requirements of different operators or user groups:

    - graphical browsing with search and filter functions

    - navigation according to a semantically grouping of the various AIXM 5.1(or latest version by delivery time) data types with a pre-defined and customizable entry point

    - direct access to certain, well-defined data or data types by selection with textual input

All three navigation types shall be interconnected and usable together. This shall enable the operator to use each type for certain steps in navigating the requested information.

  - The graphical browsing shall be provided for all AIXM 5.1(or latest version by delivery time) data types that contain geographic information about their positioning. The operator shall be able to restrict the displayed data by filtering for the type and by filtering for a specific geographic area

  - All the calculations in Airspace Design Software are done on the ellipsoid WGS-84. Calculation functionalities are based on computing algorithms.

  - Accuracy of the calculations depends on the initial accuracy of aeronautical data

  - The Airspace design environment shall be fully interoperable with flight procedure design software and use the same data including aeronautical, terrain and topographic data

  - The software shall allow the airspace manager to accomplish the following tasks:

    - Displaying aeronautical situation.

    - Establishment of fixed and temporary restrictions to the airspace, according to Issued regulations

  - The software shall allow creating and verifying the following aeronautical data:

    - Latitude and longitude

    - CTA/CTZ boundary points

    - En-route navaids and fixes

    - Bearing

    - Airway segment

    - En-route and terminal fix formations

    - Terminal arrival/departure route segments

    - Instrument approach procedure fix formations (as a part of flight procedure design functionality)

    - Length/distance/dimension

    - Airway segments length

    - En-route fix formations distance

    - Terminal arrival/departure route segments length

    - Terminal and instrument approach procedure fix formations distance (as a part of flight procedure design functionality)

    - The Airspace Design software shall provide the following functionalities

      - Waypoint design

      - Route segment design

      - Airspace Design

      - Airspace visualization

      - Route visualization

      - Geodetic Calculator (Direct & Inverse, Distance/Distance, Course/Course, Course/Distance)

      - Create buffer (Airspace, Enroute, Route segment).

- DESCRIPTION OF SERVICES AND SCOPE OF PURCHASE

  - The software must start working within 72 h from the date specified in the contract.

  - Software support services shall commence immediately after the installation of the Software. After the software installation, support has to be provided by phone, e-mail, and a remote connection. Support is provided by the Software manufacturer. Software support enables Customer to use the latest Software version, updates and documentation (user manual, changes list and etc.) for the whole Support period mentioned in the contract. Support must be provided on weekdays during business hours.

  - Software updates, technical support and troubleshooting has to be performed during the whole Support period mentioned in the contract. During the whole Software support period mentioned in the contract, in case of Software failures, the Supplier has to ensure support in accordance with response and resolution times mentioned in Table 1.

Table 1. Software failure severity, severity levels, supplier response time and resolution period.

| Failure ID | Description | Severity | Supplier response time | Resolution time |
| --- | --- | --- | --- | --- |
| 1 | Software failure or malfunctioning, which has critical impact on Customer’s business (software is not functioning) | Critical | 4 hours | 1,5 business days |
| 2 | Software failure or malfunctioning, which has a high impact on Customer’s business (the software is functioning, but not delivering some of its important functions) | High | 1 business day | 3 business days |
| 3 | Software malfunctioning, which has a medium impact on charting businesses (the software is functioning, but not delivering some of its non-important functions) | Medium | 2 business days | 14 business days |
| 4 | No significant impact on charting business, but may improve effectiveness of the work process by a minor improvement | Normal | 1 week | 3 months |
| 5 | No significant impact on charting business but may improve effectiveness of the work process by an improvement that is not considered as significant modifications. | Low | 1 month | 6 months |

  - Supplier has to provide Software training for a group of maximum 3 responsible employees from Plc ORO NAVIGACIJA, lasting at least 2 business days or at least 16 hours. Upon successful completion of the training, the participants have to obtain certificates from the Supplier attesting to their participation.

- DOCUMENTS

  - Supplier has to submit a form of Acceptance of the transfer of the products and services of the Software at the time the products and services are about to be provided.