Annex 2

TECHNICAL SPECIFICATION

FOR A FLIGHT PROCEDURE DESIGN SOFTWARE PANDA VALIDATION MODULE – SOFTWARE INSTALLATION, SUPPORT AND TRAINING

- GENERAL INFORMATION

  - Contracting authority: public limited liability company Oro Navigacija, the sole provider of air navigation services, comprising aeronautical information services (AIS), air traffic services (ATS), airspace management (ASM) services also communications, navigation and surveillance (CNS) services at Vilnius flight information region (FIR).

Address: Balio Karvelio St. 25, LT-02184 Vilnius, Lithuania, VAT payer code LT100604610. Data collected and stored at the Register of Legal Entities, code of the company 210060460.

- DEFINITIONS AND ABBREVIATIONS

  - This technical specification contains the following definitions and abbreviations:

Supplier – A tenderer whose tender for the purchase is awarded to the winner in accordance with the procedure specified in the purchase conditions and with whom the Contract will be signed;

Customer – public limited liability company ORO NAVIGACIJA

Contract – a contract of purchase and sale of products and/or services which is signed between the Supplier and the Customer;

Validation Module – a set of software tools, enabling the User to assess the correctness of designed flight procedures and their following functions: geographical information, graphic tools, recording, archiving and reports according to PANS-OPS requirements.

- OBJECTIVE

  - Products and services to be purchased – the installation of a software for a validation module, updates and support, including training for Customers’ personnel.

- GENERAL REQUIREMENTS

  - Supplier has to confirm, that Software does not contain any additional or malicious code, which is not needed for a Software to run.

  - If malicious code is identified in the provided Software, this will be treated as non-compliance and non-compliance with contract terms. In that case Supplier has to cover costs associated with the material damage suffered by the customer.

  - The Supplier shall guarantee compliance with the organisational and technical cyber security requirements established under Resolution of the Government of the Republic of Lithuania No. 818 of 13 August 2018 (latest version) (818 Dėl Lietuvos Respublikos kibernetinio saugumo įstatymo įgyvendinimo; hereinafter referred to as the Requirements), applicable to the cyber security subjects.

- SOFTWARE REQUIREMENTS

  - The Validation Module must be fully interoperable with flight procedure design system PANDA (Oro Navigacija’s existing flight procedure design software).

  - Regarding reporting requirements, the Validation Module must provide the User with the following capabilities:

    - Record the initial criteria and the degree to which the software covers these criteria;

    - Store test characteristics (dates, names of individuals who conducted the tests, etc.);

    - Include characteristics of the software environment used (e.g., GIS, database management system, etc.) in the validation reports.

  - The Validation Module must enable the User to assess the correctness of the following functions and aspects of the used environment

    - Geographical information: integration of coordinate reference systems, WGS-84 calculations, conversions between various reference systems, cartographic projections, etc.

    - Graphic tools: creation and organization of graphical objects (lines, curves, texts, etc.), two-dimensional (2-D) or three-dimensional (3-D) representation of geographic information.

    - Recording and archiving of the developer's activities for subsequent analysis.

    - Reports on validation of developed procedures

  - The Validation Module must ensure that the User has confidence in ability of the conventional and RNAV/PBN Design Modules to evaluate the quality characteristics of aeronautical data integrated into the procedure design environment.

The Validation Module must provide the capability to implement procedures that ensure the quality of the source data meets the requirements for procedure design in terms of the following data quality characteristics:

    - Data Accuracy

    - Data Resolution

    - Data Integrity

    - Data Traceability

    - Data Timeliness

    - Data Completeness

    - Data Format

  - The Validation Module must enable the User to verify the correct integration into the conventional and RNAV/PBN Design Modules of all data that may be used during the procedure development process, such as:

    - Navigation aids

    - Landing systems

    - Aerodromes

    - Runway characteristics

    - Obstacles

    - Airspace characteristics

    - Waypoints, intersections, fixes, reporting points

  - Verification must be accompanied by metadata validation, at a minimum for the following elements:

    - Data source

    - Horizontal reference system (e.g., WGS-84)

    - Vertical reference system (e.g., mean sea level)

    - Units of measurement

  - The Validation Module must enable the User to evaluate the correctness of the output data functions, which allow designers to obtain specific final results. The Validation Module must provide verification of the following functions related to the generation and storage of design results:

    - Two-dimensional or three-dimensional display of the geometry of the constructed procedures, including tolerance areas and obstacle consideration zones

    - Output data file containing all calculation results

    - Design logic and the decisions made

    - Graphical representation of the procedures (from design mode to aeronautical chart)

    - Procedure coding (ARINC 424 and AIXM 5.1)

  - The Validation Module must allow the User to perform validation of the flight procedure design function in the following aspects:

    - Integration of ICAO parameters for calculations

    - Modeling of applicable criteria

    - Design of conventional and RNAV/PBN procedures with protection areas for holding, departure, arrival, and approach procedures

    - Conventional and RNAV/PBN procedure calculations to ensure compliance with the following requirements:

      - Obstacle clearance altitude/height (OCA/H)

      - Required gradient of the procedure

      - Descent angle or descent rate

      - Minimum safe altitudes – procedure absolute altitude

      - Other parameters, including indicated airspeed (IAS), entry and exit altitude in a sector, bank angle, etc.

  - The use of the Validation Module must enable the User to determine whether the methods and concepts implemented in the conventional and RNAV/PBN Design Module comply with regulatory criteria (ICAO provisions).

The Validation Module must ensure transparency in the operation of the conventional and RNAV/PBN Design Modules, allowing an examination of how its software interprets and applies the regulatory criteria associated with each element.

Note: This will support the decision-making process regarding the suitability of the methodology used by the application for the evaluated element (acceptability = yes/no).

The level of transparency must be sufficient to provide adequate and satisfactory information about each method, as well as to assess potential differences between the method and the regulatory provision.

Such information must be made available by the Validation Module through digital and printed documentation, and via specialized interfaces within the validation software application—both step-by-step and in full.

  - The use of the Validation Module must enable the User to determine whether the data being used is appropriate for its intended purpose.

To achieve this, the Validation Module must provide the capability to analyze all types of input values, which may include:

    - Proposed fixed values

    - Controlled data input fields, i.e., inputs that are subject to consistency/validity checks

    - Uncontrolled data input fields

  - The use of the Validation Module must allow the User to determine whether the output data is applicable in relation to the input data or whether it is not provided (and therefore cannot be evaluated).

To this end, the Validation Module must enable the User to compare the output data with data that meets quality requirements.

Note: If such a result is missing for the item being assessed, the evaluation level cannot be considered valid.

  - The use of the Validation Module must enable the User to determine whether the constructed procedure complies with the original criteria.

To achieve this, the Validation Module must allow graphic verification through comparison of specific values, such as:

    - Angular tolerance value at the fix

    - Tolerance length at the fix

    - Angular value of zone width change

    - Surface of the given protection area

  - The Validation Module must allow the application of the described validation methodology to both conventional navigation and performance-based navigation (PBN) in the following areas (aspects) of flight procedure design:

    - En-route

    - Arrival

    - Holding procedures

    - Reversal procedures and racetrack procedures (conventional navigation only)

    - Initial approach segment

    - Intermediate approach segment

    - Final approach segment for Non-Precision Approach (NPA)

    - Approach with vertical guidance (RNAV only)

    - Precision segment

    - Missed approach

    - Circling approach (conventional navigation only)

    - Departures

    - Transitions between segments

  - This function refers to the administrative and descriptive control over the lifecycle of a flight procedure within the validation system. The Validation Module should support:

    - Updating descriptive information — such as procedure designator, communication failure, missed approach instruction and instructions

    - Editing attributive data — such as altitude and speed interpretations\

    - Decommission of procedure — making sure that after decommissioning of procedure there will be no missing links and all related features can be also decommissioned

  - Aeronautical Data Tabulation

Validation module must allow users to generate aeronautical data tabulation, which includes:

    - Title: Summary of the procedure type, runway, and final approach segment.

    - Fix-Point Table: Contains names and coordinates of critical waypoints (e.g., IF, IAF, FAF, MAPt, runway threshold).

    - Final approach descent angle: Must match the calculated vertical path.

Validation checks:

    - Coordinate format (DDMMSS.S or DDMMSS.SS) must be consistent.

    - Fix types and sequence must align with ARINC 424 leg types.

    - Descent angle must align with calculated VPA and height constraints.

  - Tabular Description

Validation module must allow users to generate tabular description, which includes:

    - Serial number and path descriptors (e.g., IF, TF, CA)

    - Waypoint identifiers

    - Course (True and Magnetic) and variation

    - Distance (NM) between legs

    - Turn direction, altitude (FT), speed (KT), and VPA (°)

    - Navigation specification (e.g., RNAV 1, RNP APCH)

Validation checks:

    - Course values must be consistent with procedure geometry.\

    - VPA (Vertical Path Angle) must match vertical profile calculation.

  - DME Profile Data Tabulation

Validation module must allow user to generate DME profile data tabulation, which includes:

    - Distance from threshold (NM)

    - Corresponding altitudes in FT and meters

Validation checks:

    - Altitudes must correspond to the published vertical path angle.

    - Height consistency across both units (FT/m).

    - DME distances must reflect distances from RWY THR or MAPt.

  - Routing

Validation module must allow the user to generate routing, which includes:

Validation checks:

    - The sequence of waypoints and leg transitions must match graphical and tabular data.

    - Altitudes and constraints (e.g., A2500+, K230) must reflect actual design.

    - ARINC and AIXM-based output must be consistent.

  - Waypoint List

Validation module must allow user to generate waypoint list, which includes:

    - Waypoint identifiers

    - Coordinates in two formats (DDMMSS.S and DDMMSS.SS)

Validation checks:

    - Coordinates must match the tabular and graphical data.

    - Format and notation should be ICAO/AIP-compliant.

    - No mismatches between main and alternate coordinate notations.

- DESCRIPTION OF SERVICES AND SCOPE OF PURCHASE

  - The software must start working within 72h from the date specified in the contract.

  - Software support services shall commence immediately after the installation of the Software. After the software installation, support has to be provided by phone, e-mail, and a remote connection. Support is provided by the Software manufacturer. Software support enables Customer to use the latest Software system version, updates and documentation (user manual, changes list and etc.) for the whole Support period mentioned in the contract. Support must be provided on weekdays during business hours.

  - Software updates, technical support and troubleshooting has to be performed during the whole Support period mentioned in the contract. During the whole Software system support period mentioned in the contract, in case of Software system failures, the Supplier has to ensure support in accordance with response and resolution times mentioned in Table 1.

Table 1. Software system failure severity, severity levels, supplier response time and resolution period.

| Failure ID | Description | Severity | Supplier response time | Resolution time |
| --- | --- | --- | --- | --- |
| 1 | Software system failure or malfunctioning, which has critical impact on Customer’s business (system is not functioning) | Critical | 4 hours | 1,5 business days |
| 2 | Software system failure or malfunctioning, which has a high impact on Customer’s business (the system is functioning, but not delivering some of its important functions) | High | 1 business day | 3 business days |
| 3 | Software system malfunctioning, which has a medium impact on charting businesses (the system is functioning, but not delivering some of its non-important functions) | Medium | 2 business days | 14 business days |
| 4 | No significant impact on charting business, but may improve effectiveness of the work process by a minor improvement | Normal | 1 week | 3 months |
| 5 | No significant impact on charting business but may improve effectiveness of the work process by an improvement that is not considered as significant modifications. | Low | 1 month | 6 months |

  - Supplier has to provide Software training related with the installed Modules for a group of maximum 3 responsible employees from Plc ORO NAVIGACIJA lasting at least 2 business days or at least 16 hours. Upon successful completion of the training, the participants have to obtain certificates from the Supplier attesting to their participation.

- DOCUMENTS

  - Supplier has to submit a form of Acceptance of the transfer of the products and services of the Software system at the time the products and services are about to be provided.