ID-3 formato asmens dokumentų blankų gamybos paslaugos (PPR-562)

TECHNINĖ SPECIFIKACIJA

1. SANTRUMPOS

ADIS – Asmens dokumentų išdavimo informacinė sistema.

BAC – bazinė prieigos kontrolė (angl. Basic Access Control).

CLI / MLI – sudėtinio arba kintamo lazerinio vaizdo elementas (angl. Changeable Laser Image, Multiple Laser Image).

CSCA – šalies sertifikavimo įstaiga (angl. Country Signing Authority).

DS – dokumentą pasirašanti įstaiga (ang. Document Signer).

CVCA – šalies patikros sertifikavimo įstaiga (angl. Country Verifying Certificate Authority).

DPA – skirtuminio galingumo analizė (angl. Differential Power Analysis).

DVCA – dokumentų patikros sertifikavimo įstaiga (angl. Document Verifying Certificate Authority).

EAC – išplėstinė prieigos kontrolė (angl. Extended Access Control).

IEC – Tarptautinė elektrotechnikos komisija (angl. International Electrotechnical Commission).

ISO – Tarptautinė standartizacijos organizacija (angl. International Standarts Organization).

LDS – loginė duomenų struktūra (angl. Logical Data Structure).

MRZ – mašininio nuskaitymo zona (angl. Machine Readable Zone).

PA – pasyvus atpažinimas (angl. Passive Authentication).

ICAO – Tarptautinė civilinės aviacijos organizacija (angl. International Civil Aviation Organisation).

2. ĮŽANGA

2.1. Perkančioji organizacija (toliau – Migracijos departamentas)

Migracijos departamentas prie Lietuvos Respublikos vidaus reikalų ministerijos

Sapiegos g. 1, LT- 10312 Vilnius, Lietuva

Tel. +370 707 67000

Įmonės kodas 188610666

2.2. Esama situacija

2.2.1. Vadovaujantis 2004 m. gruodžio 13 d. Europos Tarybos reglamentu (EB) Nr. 2252/2004 [1] dėl valstybių narių išduodamų pasų ir kelionės dokumentų apsauginių savybių ir biometrikos standartų reikalavimų (toliau – Reglamentas 2252/2004) ir 2018 m. lapkričio 30 d. Komisijos įgyvendinimo sprendimu Nr. C(2018) 7774, kuriuo nustatomos valstybių narių išduodamų pasų ir kelionės dokumentų apsaugos priemonių ir biometrinių duomenų standartų techninės specifikacijos ir panaikinami sprendimai K(2006) 2909 ir K(2008) 8657 (toliau – Komisijos sprendimas K(2018) 7774) [2], Lietuvos Respublikoje išduodami Lietuvos Respublikos pasai (toliau – pasai) ir kelionės dokumentai su nekontaktinėje elektroninėje laikmenoje nustatytu būdu įrašytais ir atitinkamu būdu apsaugotais asmens dokumento turėtojo biometriniais duomenimis – veido atvaizdu ir pirštų atspaudais.

2.2.2. Pasai ir kiti kelionės bei asmens tapatybės dokumentai išrašomi centralizuotai Migracijos departamente, nuolat atnaujinamomis ADIS priemonėmis. ADIS apima pasų ir kitų kelionės dokumentų bei ID-1 formato dokumentų, vizualinio išrašymo, nekontaktinės ir kontaktinės elektroninių laikmenų įrašymo, prašymų registravimo ir biometrinių duomenų nuskaitymo ir kitas posistemes. Asmens dokumentai išrašomi naudojant modernią modulinę išrašymo techninę įrangą ,,Datacard PB 6500“ – knygelės tipo (ID-3 formato) dokumentams, „Datacard MX2100“ – kortelės tipo (ID-1 formato) dokumentams.

2.2.3. Migracijos departamente įdiegta ir funkcionuoja elektroninių laikmenų ir biometrinių duomenų apsaugai nuo klastojimo bei neteisėto nuskaitymo reikalinga Viešojo rakto infrastruktūra; realizuoti prieigos kontrolės prie dokumentų elektroninėje laikmenoje įrašytų duomenų mechanizmai: pirminės prieigos kontrolės (BAC), išplėstinės prieigos kontrolės (EAC). Yra įdiegti CSCA, DS, CVCA, DVCA funkcionalumai.

2.2.4. Paslaugų teikėjas (toliau – Tiekėjas), siekdamas, kad jo pasiūlymas būtų pripažintas tinkamu, rengdamas pasiūlymą gali susipažinti su Migracijos departamento turima programine ir technine įranga, įvertinti perkančiosios organizacijos turimos įrangos racionaliausio panaudojimo būdus. Tiekėjas privalės užtikrinti, kad jo tiekiamuose blankuose išrašomi asmens dokumentai galėtų būti išrašomi Migracijos departamento turima programine ir technine įranga. Esant tiekiamų blankų fizinių savybių ar blankuose integruotų elektroninių laikmenų operacinės sistemos nesuderinamumui su pasų, tarnybinių pasų, diplomatinių pasų, užsieniečio pasų, asmens be pilietybės kelionės dokumentų, pabėgėlio kelionės dokumentų blankais ar Migracijos departamento turima įrašymo į elektronines laikmenas technine ar programine įranga, keisti Migracijos departamento turimą įrangą tiekėjas turės savo sąskaita, įskaitant derinimui ir testavimui naudojamas reikalingas medžiagas.

2.2.5. Bet kurios įrangos diegimo metu negalės būti pažeistas esamas ADIS funkcionalumas ir negalės būti sutrikdytas nė vienos rūšies asmens dokumentų išrašymas ir išdavimas.

2.3. Pirkimo tikslas ir pirkimo objektas

2.3.1. Šio pirkimo tikslas yra įsigyti Diplomatinio paso blankų, atitinkančių Lietuvos Respublikos užsienio reikalų ministro 2018 m. lapkričio 12 d. įsakymu Nr. V-299 „Dėl Lietuvos Respublikos diplomatinio paso pavyzdžio (dokumento blanko privalomosios formos) patvirtinimo“ patvirtintą Diplomatinio paso blanko privalomąją formą, Lietuvos Respublikos paso blankų, atitinkančių Lietuvos Respublikos vidaus reikalų ministro 2006 m. gruodžio 28 d. įsakymu Nr. 1V-498 „Dėl Lietuvos Respublikos paso formos ir Lietuvos Respublikos paso formos aprašymo patvirtinimo“ patvirtintą Lietuvos Respublikos paso formą, Tarnybinio paso blankų, atitinkančių Lietuvos Respublikos vidaus reikalų ministro 2003 m. rugsėjo 10 d. įsakymu Nr. 1V-332 „Dėl tarnybinio paso formos patvirtinimo“ patvirtintą Tarnybinio paso formą, Lietuvos Respublikos asmens be pilietybės kelionės dokumento ir Lietuvos Respublikos pabėgėlio kelionės dokumento blankų, atitinkančių Lietuvos Respublikos vidaus reikalų ministro 2008 m. liepos 28 d. įsakymu Nr. 1V-289 „Dėl Lietuvos Respublikos asmens be pilietybės kelionės dokumento blanko privalomosios formos ir Lietuvos Respublikos pabėgėlio kelionės dokumento blanko privalomosios formos patvirtinimo“ patvirtintas Lietuvos Respublikos asmens be pilietybės kelionės dokumento ir Lietuvos Respublikos pabėgėlio kelionės dokumento blankų formas, Užsieniečio paso blankų, atitinkančių Lietuvos Respublikos vidaus reikalų ministro 2004 m. lapkričio 10 d. įsakymu Nr. 1V-357 „Dėl Užsieniečio paso formos patvirtinimo“ patvirtintą Užsieniečio paso formą,  blankų gamybos paslaugą.

2.3.2. Pirkimo objektą sudaro ID-3 formato blankų gamybos paslauga, kurios kiekiai ir terminai nurodyti 1 lentelėje.

1 lentelė

| Blanko pavadinimas | Blankų kiekis (vnt.) | Numatoma blankų tiekimo data |
| --- | --- | --- |
| Lietuvos Respublikos paso blankas | 1 000 000 | 2026-2028 |
| Tarnybinio paso blankas | 1 000 | 2026-2028 |
| Asmens be pilietybės kelionės dokumento blankas | 1 000 | 2026 |
| Pabėgėlio kelionės dokumento blankas | 1 000 | 2026 |
| Diplomatinio paso blankas | 1 000 | 2026 |

2.3.3. Esant poreikiui, sutarties galiojimo laikotarpiu ar pratęsus sutartį, perkančioji organizacija gali papildomai užsakyti 2 lentelėje nurodytus blankų kiekius, apmokant už juos pagal pirkimo sutartyje nurodytus įkainius.

2 lentelė

| Blanko pavadinimas | Blankų kiekis (vnt.) |
| --- | --- |
| Lietuvos Respublikos paso blankas | 1 000 000 |
| Tarnybinio paso blankas | 1 000 |
| Užsieniečio paso blankas | 2 000 |
| Asmens be pilietybės kelionės dokumento blankas | 1 000 |
| Pabėgėlio kelionės dokumento blankas | 2 000 |
| Diplomatinio paso blankas | 2 000 |

3. ID-3 FORMATO ASMENS DOKUMENTŲ BLANKŲ TECHNINĖ SPECIFIKACIJA

| Blankų specifikacijos |  |
| --- | --- |
| 3.1. Bendrieji reikalavimai |  |
| Forma | Blankų formos turi atitikti patvirtintas privalomąsias formas. Blankai turi būti gaminami pagal Valstybės dokumentų technologinės apsaugos tarnybos prie Finansų ministerijos (toliau – VDTAT) parengtus atitinkamų blankų grafinius projektus. Blankai turi būti gaminami vadovaujantis Lietuvos Respublikos saugiųjų dokumentų ir saugiųjų dokumentų blankų gamybos įstatymu. |
| Formatas | Blankų formatas turi atitikti ID-3 formatą pagal ICAO dokumentą 9303 [3]. Blanko knygelė turi visiškai užsidaryti ją užskleidžiant. |
| Emblema | Turi atitikti ICAO dokumente 9303 elektroninių pasų ir kelionės dokumentų emblemai nustatytus reikalavimus. |
| Numeravimas | Blankai turi būti numeruoti. Numeracija turi būti nuosekli, be tarpų nustatytame intervale, pradedant perkančiosios organizacijos nurodytais pradiniais intervalų numeriais. |
| Duomenų lapo savybės | Duomenų lapas turi būti vientisas, pagamintas iš 100 proc. polikarbonato. Duomenų lapo storis turi atitikti ICAO rekomendacijas ir būti ne didesnis kaip 0,9 mm. Siekiant užtikrinti suderinamumą rekomenduojama, kad storis atitiktų šiuo metu Migracijos departamento išrašomų blankų storį. Duomenų lapo įsiuvimo į blanko knygutę elementas turi būti integruotas į duomenų lapą tokių būdu, kad su duomenų lapo medžiaga sudarytų ištisinį darinį ir jų nebūtų galima atskirti nesuardant duomenų lapo. Duomenų lapo savybės turi užtikrinti aukštos kokybės tikrosios pilkų spalvų gamos išgavimą graviruojant lazeriu. Turi leisti graviruojant išgauti „reljefinį“ vaizdą (angl. tactile engraving). Turi leisti, esant sprendimui, formuoti kitas norimas saugos priemones (pvz., ImagePerf©, paslėptą vaizdą pvz. IPI©). |
| Duomenų lapo ir / ar viršelio ir / ar vidinių puslapių pagrindiniai spaudos ir technologinės apsaugos reikalavimai | Blankuose įdiegtos technologinės apsaugos savybės turi atitikti perkančiosios organizacijos ir VDTAT suderintą technologinės apsaugos priemonių sąrašą. |
| Elektroninės laikmenos integravimas | Elektroninė laikmena ir antena turi būti integruota pasų ir kelionės dokumentų duomenų lape. Elektroninė laikmena ir antena turi susiardyti bandant juos išimti iš duomenų lapo. Elektroninė laikmena ir antena turi būti išdėstyti duomenų puslapyje tokiu būdu, kad nepersidengtų su CLI / MLI ar skaidraus langelio zona, jei tokios numatytos, ir neįtakotų MRZ zonos skaitomumo. |
| Papildomi reikalavimai | Asmens dokumento išrašymo procesas neturi paveikti jame integruotų elektroninių laikmenų funkcionalumo. Perkančiajai organizacijai turi būti pateikta visa informacija, reikalinga elektroninės laikmenos išrašymui ir sąlygos blankų sandėliavimui. |
| Blankų su elektroninėmis laikmenomis transportavimas | Blankai su elektroninėmis laikmenomis turi būti transportuojami užrakinus elektronines laikmenas unikaliais transporto raktais. |
| Garantija | Blankas turi atlaikyti ne mažiau kaip 10 tūkst. atvertimų įrišimo zonoje. Blankams turi būti suteikiama ne mažesnė kaip 10 (dešimt) metų garantija; garantiniu laikotarpiu blankų savybės ir išvaizda neturi pasikeisti. Garantinis laikotarpis pradedamas skaičiuoti nuo perdavimo-priėmimo akto pasirašymo. |
| 3.2. Reikalavimai bekontaktei elektroninei laikmenai |  |
| Bendros sąlygos | Mikrokontroleris su vienu ar daugiau mikroprocesoriais ir atmintimi, su viena ar kelių antenų sistema, kuri veikia iki 10 cm atstumu 13.56 MHz radijo dažniu ir atitinkantis ISO/IEC14443 [5] ar lygiavertį standartą. |
| Elektroninės laikmenos charakteristikos | Ne mažiau kaip vienas papildomas kriptografinėms funkcijoms vykdyti dedikuotas procesorius. Gamybos technologinis procesas - ne daugiau kaip 90 nm; Patikimo duomenų laikmenoje saugojimo terminas - ne mažiau kaip 10 metų. Tikrųjų atsitiktinių skaičių generavimas. Laikmena turi veikti nuo -25 iki +50 laipsnių Celsijaus aplinkos temperatūroje. |
| Reikalavimai operacinei sistemai | Laikmena turi būti su gamintojo įdiegta operacine sistema ir ICAO aplikacija (įdiegtos ROM arba Flash atmintyje). |
| Elektroninės laikmenos atmintis | Turi būti saugi („rakinama“ gamintojo ir vartotojo), programuojama ir netrinama. Bendras atminties dydis prieinamas ICAO aplikacijai turi būti ne mažesnis nei 100 kilobaitų (KB). |
| Bekontaktė sąsaja | A arba B tipo bekontakčio ryšio sąsaja pagal ISO 14443 2-4 d. ar lygiavertį standartą, atitinkanti ICAO rekomendacijas. Sąsaja turi užtikrinti standartinį duomenų perdavimo greitį 106 kbp/s pagal ISO 14443 arba lygiavertį standartą, o taip pat ir didesnius duomenų perdavimo greičius iki 848 kbp/s. |
| ICAO aplikacijos bazinis funkcionalumas | ICAO aplikacija turi būti skirta pasų ir kelionės dokumentų funkcionalumui (pagal [1], [2] ir [9] nustatytus reikalavimus) užtikrinti. Aplikacija ir operacinė sistema turi tenkinti šiuos reikalavimus: - turi būti galimybė skaityti tiek visą LDS struktūrą, tiek dalimis; - turi užtikrinti šių apsaugos priemonių funkcionavimą: - pasyvus atpažinimas (angl. Passive authentication) (PA); - PACE v2 prieigos kontrolė, nenaudojant BAC; - išplėstinė prieigos kontrolė (angl. Extended Access Control) (EAC), į kurią įeina elektroninės laikmenos autentiškumo patikrinimas (chip authentication) ir terminalo autentiškumo patikrinimas (terminal authentication). |
| ICAO aplikacijos vykdomos kriptografinės funkcijos | ICAO aplikacija turi vykdyti visas rekomendacijose numatytas kriptografines funkcijas, susijusias su PA, SAC (PACEv2) ir EAC mechanizmų įgyvendinimu, turi būti realizuota eliptinių kreivių (Elliptic Curves) kriptografija. |
| ICAO aplikacijos duomenų apsauga | ICAO aplikacijai skirta atmintis turi būti užrakinama. Pasibaigus įrašymo procesui, jokie aplikacijos atminties turinio pakeitimai turi būti neįmanomi užrakinus atmintį. |
| ICAO aplikacijos veikimo greitis | Veido atvaizdas (15 KB) turi būti nuskaitomas ne lėčiau kaip per 3 sek. |
| Saugumo reikalavimai | Elektroninė laikmena turi būti sertifikuota ne žemesniu nei EAL 5+ saugos lygiu (pagal Common Criteria). Operacinė sistema kartu su elektronine laikmena ir ICAO aplikacija turi būti sertifikuota ne žemesniu nei EAL 5+ saugos lygiu pagal naujausius apsaugos profilius (BSI-CC-PP-0068-V2-2011 (PACE PP) [6], BSI-CC-PP-0056-V2-2012 (EAC PP) [7]. Tiekėjas privalo savo sąskaita užtikrinti atitiktį aukščiau nurodytiems reikalavimams ne trumpesniam kaip 4 metų laikotarpiui, terminą skaičiuojant nuo blankų pateikimo momento. |
| Atitikties vertinimas | ICAO aplikacijos funkcinių parametrų veikimui įvertinti turi būti naudojami atitinkami standartai, kaip numatyta Komisijos sprendimo K(2018) 7774) [2] 6.3 skyriuje. Kartu su pirmu blankų tiekimu Tiekėjas privalo pateikti akredituotos testavimo laboratorijos atliktų fizinio atsparumo testo pagal ISO/IEC 18745-2 [8], analoginio testo pagal ISO/IEC 18745-2, skaitmeninio ir protokolo testo pagal ISO/IEC 18745-2 ir aplikacijos veikimo testo su EACv1 ataskaitas tokiai aplikacijos konfigūracijai, kuri atitinka šiuo metu išduodamų pasų ir kelionės dokumentų ICAO aplikacijos profilį, įskaitant naudojamus PACE, CA (Chip Authentication), TA (Terminal Authentication), PA (Passive Authentication) algoritmus bei raktų tipus ir dydžius. Sutarties vykdymo metu pasikeitus elektroninei laikmenai, turi būti pateikti nauji sertifikatai. |
| 3.3. Kiti reikalavimai |  |
| Veiksmai ekstremalios situacijos ar paskelbtos mobilizacijos metu | Turėti galimybę suteikti Perkančiajai organizacijai visą infrastruktūrą ir specialistų pagalbą, reikalingą saugiesiems ID-3 formato dokumentams (pasai, tarnybiniai pasai, diplomatiniai pasai, užsieniečio pasai, asmens be pilietybės kelionės dokumentai, pabėgėlio kelionės dokumentai) išrašyti, t. y. patalpas, kurių geografinė vieta turi būti suderinta su Perkančiąja organizacija, reikalingą techninę įrangą ir kt. reikalingą infrastuktūrą. Taip pat užtikrinti saugų pagamintų blankų pristatymą Perkančiajai organizacijai jos nurodytu adresu. Rezervinis dokumentų išrašymas ir pagamintų blankų pristatymas turi būti įvykdytas per 72 val. nuo Perkančiosios organizacijos prašymo arba per kitą su Perkančiąja organizacija suderintą terminą. |

4. NORMATYVINĖS NUORODOS

Pasų ir kelionės dokumentų blankai turi atitikti šių teisės aktų ir standartų reikalavimus:

- Tarybos reglamentas (EB) Nr. 2252/2004 dėl valstybių narių išduodamų pasų ir kelionės dokumentų apsauginių savybių ir biometrikos standartų;

- 2018 m. lapkričio 30 d. Komisijos įgyvendinimo sprendimas Nr. C(2018) 7774, kuriuo nustatomos valstybių narių išduodamų pasų ir kelionės dokumentų apsaugos priemonių ir biometrinių duomenų standartų techninės specifikacijos ir panaikinami sprendimai K(2006) 2909 ir K(2008) 8657;

- International Civil Aviation Organization (ICAO), Machine Readable Travel Documents, Doc 9303, 8th Edition, 2021;

- BSI, TR-03110, Advanced Security Mechanisms for Machine Readable Travel Documents, Version 2.10, 2012;

- ISO/IEC 14443, Tapatybės kortelės – Bekontakčio(-ių) integrinio(-ių) grandyno(-ų) kortelės. Nuotolinės kortelės;

- Common Criteria Protection Profile. Machine Readable Travel Document using Standard Inspection Procedure with PACE (PACE PP) BSI-CC-PP-0068-V2-2011, Version 1.0, 2nd November 2011;

- Common Criteria Protection Profile. Machine Readable Travel Document with "ICAO Application", Extended Access Control with PACE, BSI-CC-PP-0056-V2-2012 (EAC PP);

- ISO/IEC 18745-2:2021 Test methods for machine readable travel documents (MRTD) and associated devices — Part 2: Test methods for the contactless interface;

- 2021 m. birželio 4 d. Komisijos įgyvendinimo sprendimas Nr. C(2021) 3741, kuriuo iš dalies keičiamas Komisijos įgyvendinimo sprendimo C(2018) 7774 priedas, normatyvinių nuorodų atžvilgiu.

_______________

UNOFFICIAL TRANSLATION

Services for the production of ID-3 format personal document forms (PPR- 562)

TECHNICAL SPECIFICATION

1. ABBREVIATIONS

ADIS - Information system for issuing personal documents.

BAC - Basic Access Control.

CLI / MLI - Changeable Laser Image or Multiple Laser Image element.

CSCA - Country Signing Authority.

DS - Document Signer.

CVCA - Country Verifying Certificate Authority.

DPA - Differential Power Analysis.

DVCA - Document Verifying Certificate Authority.

EAC - Extended Access Control.

IEC - International Electrotechnical Commission.

ISO - International Standards Organization.

LDS - Logical Data Structure.

MRZ - Machine Readable Zone.

PA - Passive Authentication.

ICAO - International Civil Aviation Organisation.

2. INTRODUCTION

2.1. Contracting Authority (hereinafter referred to as the Migration Department)

Migration Department under the Ministry of the Interior of the Republic of Lithuania

Sapiegos St. 1, LT- 10312 Vilnius, Lithuania

Phone: +370 707 67000

Company code 188610666

2.2. The current situation

2.2.1. In accordance with Council Regulation (EC) No 2252/2004 [1] of the European Parliament and of the Council of 13 December 2004 on standards for security features and biometrics in passports and travel documents issued by Member States (hereinafter referred to as Regulation 2252/2004) and Commission Implementing Decision No  C(2018) 7774 of 30 November 2018 laying down technical specifications for the security features and biometric data standards for passports and travel documents issued by Member States and repealing Decisions C(2006) 2909 and C(2008) 8657 (hereinafter referred to as the Commission Decision C(2018) 7774) [2], Passports of the Republic of Lithuania (hereinafter referred to as passports) and travel documents issued in the Republic of Lithuania contain the biometric data of the holder of the document, i.e. a facial image and fingerprints, recorded in a contactless electronic medium in the manner specified and protected in the appropriate manner.

2.2.2. Passports and other travel and identity documents are issued centrally at the Migration Department, using ADIS tools that are updated on a regular basis. ADIS includes subsystems for passports and other travel documents and ID-1 format documents, visual issuance, contactless and contact recording of electronic medium, registration of applications and biometric data scanning and other sub-systems. Personal documents are issued using modern modular issuing hardware “Datacard PB 6500” for booklet-type documents (ID-3 format), “Datacard MX2100” for card-type documents (ID-1 format).

2.2.3. The Public Key Infrastructure necessary for the protection of electronic medium and biometric data against forgery and unauthorised reading has been installed and is functioning at the Migration Department; mechanisms for controlling access to data stored on electronic medium have been implemented: basic access control (BAC) and extended access control (EAC). CSCA, DS, CVCA, DVCA functionalities are installed.

2.2.4. In order for its tender to be considered eligible, the service provider (hereinafter referred to as the Provider) may, when preparing its tender, familiarise itself with the software and hardware available at the Migration Department and assess the most rational ways of using the equipment available at the disposal of the Contracting Authority. The Provider will have to ensure that the personal documents issued on the forms the Provider supplies can be issued using the software and hardware available at the Migration Department. In case of incompatibility of the physical characteristics of the provided forms or the operating system of the electronic medium integrated into the forms with passports, official passports, diplomatic passports, alien passports, travel documents of stateless persons, refugee travel document forms or the hardware or software used by the Migration Department for recording data on electronic medium, the Provider shall replace the equipment owned by the Migration Department at its own expense, including the materials necessary for adjustment and testing.

2.2.5. During the installation of any equipment, the existing ADIS functionality must not be compromised and the issuance and delivery of any type of personal documents must not be disrupted.

2.3. Purpose and object of the procurement

2.3.1. The purpose of this procurement is to acquire diplomat passport forms that comply with the mandatory form of diplomat passport forms approved by Order No V-299 of the Minister of Foreign Affairs of the Republic of Lithuania of 12 November 2018 “On the approval of the model of the diplomatic passport of the Republic of Lithuania (mandatory document form)”, passport forms of the Republic of Lithuania that comply with the passport form of the Republic of Lithuania approved by Order No 1V-498 of the Minister of the Interior of the Republic of Lithuania of 28 December 2006 “On the approval of the passport form of the Republic of Lithuania and the description of the passport form of the Republic of Lithuania”, official passport forms that comply with the official passport form approved by Order No 1V-332 of the Minister of the Interior of the Republic of Lithuania of 10 September 2003 “On the approval of the form of official passports”, travel document forms for stateless persons of the Republic of Lithuania and travel document forms for refugees of the Republic of Lithuania that comply with the mandatory form of travel document forms for stateless persons of the Republic of Lithuania approved by Order No 1V-289 of the Minister of the Interior of the Republic of Lithuania of 28 July 2008 “On the mandatory form of travel document forms for stateless persons of the Republic of Lithuania”, alien passport forms that meet the requirements of the alien passport form approved by Order No 1V-357 of the Minister of the Interior of the Republic of Lithuania of 10 November 2004 “On the approval of the alien passport form”, and the service of producing such forms.

2.3.2. The object of the procurement is the service for the production of ID-3 format forms, the quantities and terms of which are indicated in Table 1.

Table 1

| Name of the form | Quantity of forms (pcs.) | Estimated date of delivery of forms |
| --- | --- | --- |
| Passport form of the Republic of Lithuania | 1 000 000 | 2026-2028 |
| Official passport form | 1 000 | 2026-2028 |
| Stateless person travel document form | 1 000 | 2026 |
| Refugee travel document form | 1 000 | 2026 |
| Diplomatic passport form | 1 000 | 2026 |

2.3.3. If necessary, during the term of the contract or upon its extension, the Contracting Authority may order additional quantities of the forms referred to in Table 2, paying for them in accordance with the prices specified in the procurement contract.

Table 2

| Name of the form | Quantity of forms (pcs.) |
| --- | --- |
| Passport form of the Republic of Lithuania | 1 000 000 |
| Official passport form | 1 000 |
| Alien passport form | 2 000 |
| Stateless person travel document form | 1 000 |
| Refugee travel document form | 2 000 |
| Diplomatic passport form | 2 000 |

3. TECHNICAL SPECIFICATIONS FOR PERSONAL DOCUMENT FORMS IN D-3 FORMAT

| Specifications of forms |  |
| --- | --- |
| 3.1. General requirements |  |
| Form | The forms shall conform to the approved mandatory forms. The forms shall be produced in accordance with the graphic designs of the relevant forms drawn up by the State Service of Technological Security of State Documents under the Ministry of Finance (hereinafter referred to as the STSSD). The forms shall be produced in accordance with the Law on the Production of Security Documents and Security Document Forms of the Republic of Lithuania. |
| Format | The format of the forms shall conform to the ID-3 format in accordance with ICAO document 9303 [3]. The form booklet must close completely when folded. |
| Emblem | It shall comply with the requirements for the emblem of the electronic passports and travel documents set out in ICAO document 9303. |
| Numbering | The forms must be numbered. The numbering shall be consecutive, without gaps, within a specified range, starting with the initial range numbers indicated by the Contracting Authority. |
| Data sheet properties | The data sheet shall be a solid sheet made of 100% polycarbonate. The thickness of the data sheet shall comply with ICAO recommendations and shall not exceed 0.9 mm. To ensure compatibility, it is recommended that the thickness is the same as the thickness of the forms currently issued by the Migration Department. The data sheet stitching element shall be integrated into the data sheet in such a way that it forms a continuous unit with the data sheet material and cannot be separated without damaging the data sheet. The characteristics of the datasheet shall ensure high quality extraction of the true grey gamut by laser engraving. It must allow tactile engraving. It must allow, if decided, the formation of other desired safety measures (e.g. ImagePerf©, hidden image, e.g. IPI©). |

| Basic printing and technological protection requirements for the data sheet and/or cover and/or inner pages | The technological protection features of the forms shall comply with the list of technological protection measures agreed between the Contracting Authority and the STSSD. |
| --- | --- |
| Integration of electronic medium | The electronic medium and antenna must be integrated in the passports and travel documents data sheet. The electronic medium and antenna must break when you try to remove them from the data sheet. The electronic medium and antenna shall be positioned on the data page in such a way that it does not overlap with the CLI/MLI or clear cell zone, if provided, and does not affect the readability of the MRZ. |
| Additional requirements | The process of issuing an identity document shall not affect the functionality of the electronic medium integrated in it. The Contracting Authority must be provided with all the information necessary for the issue of the electronic medium and the conditions for storage of forms. |
| Transportation of forms with electronic medium | Forms containing electronic medium must be transported with the electronic medium locked with unique transport keys. |
| Warranty | The form shall withstand at least 10 000 openings in the binding zone. The forms must have a warranty of at least 10 years; the characteristics and appearance of the forms must not change during the warranty period. The warranty period starts from the signing of the handover certificate. |
| 3.2. Requirements for contactless electronic medium |  |
| General conditions | A microcontroller with one or more microprocessors and memory, with one or more antennas, operating up to a distance of 10 cm at a radio frequency of 13.56 MHz and complying with ISO/IEC14443 [5] or equivalent standard. |
| Characteristics of the electronic medium | At least one additional processor for the performance of cryptographic functions. Production technological process - not exceeding 90 nm; The term for reliable data storage on a data medium - at least 10 years. Generation of real random numbers. The storage medium shall be able to operate at an ambient temperature of -25 to +50 degrees Celsius. |
| Operating system requirements | The storage medium must have the operating system installed by the manufacturer and the ICAO application (installed in ROM or Flash memory). |
| Electronic medium memory | It must be secure (“lockable” by the manufacturer and the user), programmable and non-erasable. The total amount of memory available for the ICAO application shall be at least 100 kilobytes (KB). |
| Contactless interface | Type A or B contactless interface in accordance with ISO 14443 d. 2-4 or equivalent standard, complying with ICAO recommendations. The interface shall provide a standard data rate of 106 kbp/s according to ISO 14443 or equivalent standard, and higher data rates up to 848 kbp/s. |
| Basic ICAO application functionality | The ICAO application shall be designed to ensure the functionality of the passports and travel documents (according to the requirements set out in [1], [2] and [9]). The application and operating system must meet the following requirements: - it must be possible to read both the entire LDS structure and parts of it; - must ensure the functioning of the following protective measures: - passive authentication (PA); - PACE v2 access control without BAC; - Extended Access Control (EAC), which includes chip authentication and terminal authentication. |
| Cryptographic functions performed by ICAO applications | The ICAO application shall implement all the cryptographic functions specified in the Recommendations related to the implementation of the PA, SAC (PACEv2) and EAC mechanisms, and Elliptic Curves cryptography shall be implemented. |
| ICAO application data protection | The memory designed for the ICAO application must be lockable. Once the recording process has finished, no changes to the contents of the application memory must be possible after locking the memory. |
| ICAO application speed | The facial image (15 KB) must be scanned in at least 3 seconds. |
| Security requirements | The electronic medium must be certified to a security level of at least EAL 5+ (according to Common Criteria). The operating system, together with the electronic medium and the ICAO application, shall be certified to a security level of at least EAL 5+ in accordance with the latest security profiles (BSI-CC-PP-0068-V2-2011 (PACE PP) [6], BSI-CC-PP-0056-V2-2012 (EAC PP) [7]. The Provider must ensure compliance with the above requirements at its own expense for a period of at least 4 years from the date of submission of the forms. |
| Conformity assessment | Appropriate standards shall be used to assess the performance of the functional parameters of the ICAO application, as provided for in Chapter 6.3 of Commission Decision C(2018) 7774) [2]. Together with the first delivery of forms, the Provider must submit a physical resistance test performed by an accredited testing laboratory in accordance with ISO/IEC 18745-2 [8], an analogue test in accordance with ISO/IEC 18745-2, a digital and protocol test in accordance with ISO/IEC 18745-2 and an application performance test with EACv1 reports for such application configuration, which complies with the ICAO application profile for passports and travel documents currently issued, including the PACE, CA (Chip Authentication), TA (Terminal Authentication) and PA (Passive Authentication) algorithms used, as well as key types and sizes. New certificates must be submitted if the electronic medium changes during the performance of the contract. |
| 3.3. Other requirements |  |
| Actions in case of emergency or declared mobilisation | Be able to provide the Contracting Authority with all the infrastructure and assistance of specialists necessary for the issuance of secure ID-3 format documents (passports, official passports, diplomatic passports, alien passports, travel documents for stateless persons, refugee travel documents), i.e. facilities, the geographical location of which must be agreed with the Contracting Authority, necessary technical equipment and other necessary infrastructure. Ensure the safe delivery of the produced forms to the Contracting Authority at the address specified by the Customer. Reserve issuance of documents and delivery of produced forms must be completed within 72 hours of the Contracting Authority’s request or within another period agreed with the Contracting Authority. |

4. NORMATIVE REFERENCES

Passport and travel document forms must comply with the requirements of the following legal acts and standards:

- Council Regulation (EC) No 2252/2004 on standards for the security features and biometrics of passports and travel documents issued by Member States;

- Commission Implementing Decision No C(2018) 7774 of 30 November 2018 laying down technical specifications for the security features and biometric data standards for passports and travel documents issued by Member States and repealing Decisions C(2006) 2909 and C(2008) 8657;

- International Civil Aviation Organization (ICAO), Machine Readable Travel Documents, Doc 9303, 8th Edition, 2021;

- BSI, TR-03110, Advanced Security Mechanisms for Machine Readable Travel Documents, Version 2.10, 2012;

- ISO/IEC 14443, Identity cards - Contactless integrated circuit card(s). Remote cards;

- Common Criteria Protection Profile. Machine Readable Travel Document using Standard Inspection Procedure with PACE (PACE PP) BSI-CC-PP-0068-V2-2011, Version 1.0, 2nd November 2011;

- Common Criteria Protection Profile. Machine Readable Travel Document with “ICAO Application”, Extended Access Control with PACE, BSI-CC-PP-0056-V2-2012 (EAC PP);

- SO/IEC 18745-2:2021 Test methods for machine readable travel documents (MRTD) and associated devices — Part 2: Test methods for the contactless interface;

- Commission Implementing Decision No C(2021) 3741 of 4 June 2021 amending the Annex to Commission Implementing Decision C(2018) 7774 as regards normative references.

_______________