TECHNINĖ SPECIFIKACIJA (LT)

Pirkimas apima smėlio dalelių srauto intensyvumo jutiklių, skirtų smėlio ir kietųjų dalelių stebėsenai ir vėjo sukeltai smėlio pernašai Baltijos jūros krantuose nustatyti (matuoti) įrangos pirkimą.

Pirkimo kiekiai:

1) smėlio srauto jutikliai: 6 vnt.;

2) Laikančiosios konstrukcijos (stovai) jutiklių montavimui: 3 vnt.

| Eil. Nr. | Techninė charakteristika | Reikalavimai | Tiekėjo siūlomos įrangos parametrai ir jų reikšmės pagal 2 ir 3 stulpelių reikalavimus |
| --- | --- | --- | --- |
| Reikalavimai smėlio dalelių srauto intensyvumo jutikliams - (6 vnt.) |  |  |  |
| 1 | Matavimo principas | akustinis, be judančių mechaninių dalių |  |
| 2 | Matavimo parametras | vėjo pernešamų kietųjų dalelių (pvz., smėlio) srauto intensyvumo matavimai (g/m²/s) |  |
| 3 | Tikslumas: | ne didesnė kaip ±5 % paklaida esant tolygiai paskirstytam dalelių srautui. |  |
| 4 | Išvestys ir sąsajos | analoginiai signalai (nuolatinė arba impulsinė įtampa); skaitmeniniai protokolai: SDI-12, TTL, RS-232, RS-485 (Modbus RTU). USB sąsaja konfigūravimui |  |
| 5 | Darbinė temperatūra | –40 °C iki +80 °C |  |
| 6 | Atsparumas aplinkos poveikiui: | atsparumas dulkėms ir drėgmei (IP67 ar lygiavertis) |  |
| 7 | Maitinimas | Įtampa: 6–30 V DC (SDI-12 režime – 9.6 V arba 16 V). Srovė: <1 mA budėjimo režime, iki 20 mA matavimo metu. Tipinis režimas: 2.1 mA (10% darbo ciklas). |  |
| 8 | Montavimas | turi būti galimybė tvirtinti prie įvairių konstrukcijų naudojant universalius laikiklius. Jutikliai turi būti tiekiami kartu su tvirtinimo laikikliais |  |
| 9 | Matavimo aukštis | kuo arčiau žemės paviršiaus |  |
| 10 | Programinė įranga | turi būti galimybė konfigūruoti jutiklį per kompiuterį naudojant nemokamą programinę įrangą ir USB sąsają |  |
| Laikančiosios konstrukcijos (stovai) – (3 vnt.) |  |  |  |
| 11 | Aukštis | Reguliuojamas nuo 2,5 m iki 3,0 m |  |
| 12 | Konstrukcija | Teleskopinė arba segmentinė |  |
| 13 | Tvirtinimo žiedo diametras | Suderinamas su jutikliais |  |
| 14 | Medžiaga | Nerūdijantis plienas arba cinkuotas aliuminis |  |
| 15 | Stabilizavimas | Tvirtinimas su žemės inkarais arba trikoju pagrindu; papildomai – galimybė naudoti įtempimo lynus |  |
| 16 | Montavimo įrankiai | Komplektuojamas su reikalingais veržliarakčiais ar tvirtinimo elementais |  |

TECHNICAL SPECIFICATION (EN)

The procurement includes the purchase of equipment for the detection (measurement) of sand particle flow intensity sensors for sand and particulate matter monitoring and wind-induced sand transport on the shores of the Baltic Sea.

Purchase quantities:

1) Sand flow sensors: 6 units;

2) Load-bearing structures (stands) for the installation of sensors: 3 units.

| Second. No. | Technical characteristics | Requirements | Parameters of the equipment offered by the supplier and their values according to the requirements of columns 2 and 3 |
| --- | --- | --- | --- |
| Requirements for sand particle flow intensity sensors - (6 units) |  |  |  |
| 1 | Measurement principle | acoustic, without moving mechanical parts |  |
| 2 | Measurement parameter | Measurements of the flow intensity of wind-borne particulate matter (e.g. sand) (g/m²/s) |  |
| 3 | Accuracy: | an error of not more than ±5 % with an evenly distributed particle stream. |  |
| 4 | Outputs and interfaces | analog signals (constant or pulsed voltage); Digital protocols: SDI-12, TTL, RS-232, RS-485 (Modbus RTU). USB interface for configuration |  |
| 5 | Operating Temperature | –40 °C to +80 °C |  |
| 6 | Resistance to environmental influences: | Dust and moisture resistance (IP67 or equivalent) |  |
| 7 | Power | Voltage: 6-30 V DC (9.6 V or 16 V in SDI-12 mode). Current: <1 mA in standby mode, up to 20 mA during measurement. Typical mode: 2.1 mA (10% duty cycle). |  |
| 8 | Installation | must be able to attach to various structures using universal brackets. Sensors must be supplied with mounting brackets |  |
| 9 | Measuring height | As close as possible to the earth's surface |  |
| 10 | Software | it must be possible to configure the sensor via a computer using free software and a USB interface |  |
| Load-bearing structures (stands) – (3 units) |  |  |  |
| 11 | Height | Adjustable from 2.5 m to 3.0 m |  |
| 12 | Design | Telescopic or segmental |  |
| 13 | Mounting ring diameter | Compatible with sensors |  |
| 14 | Substance | Stainless steel or galvanized aluminum |  |
| 15 | Stabilization | Fastening with ground anchors or tripod base; In addition, the possibility of using tension ropes |  |
| 16 | Installation tools | Comes with the necessary wrenches or fasteners |  |