Annex No 1 to the Special Conditions of Purchase

TECHNICAL SPECIFICATION

- TERMS AND ABBREVIATIONS/ GENERAL INFORMATION

  - Buyer / Contracting Authority – Vilnius University.

  - Supplier – economic entity – a natural person, a private or public legal entity, another organisation and a subdivision thereof, or a group of such persons, including temporary associations of economic entities, with which the Buyer will conclude this Purchase Agreement.

  - Contract – the Procurement Contract concluded between the Supplier and the Buyer regarding the object of this Procurement.

- PURCHASE OBJECT

  - The object of the procurement is a dilution gas refrigeration system with delivery, installation and training (hereinafter referred to as the goods).

  - The Procurement Object is not divided into parts of the Procurement Object, therefore the Supplier must submit a bid for the entire volume and/or quantity of the Procurement Object specified below.

  - Place of delivery: Vilnius University, Faculty of Physics, Saulėtekio al. 3, LT-10257 Vilnius.

  - Volume and/or quantity of goods:

Table 1.

| Second. No. | Name of goods | The quantity of goods and measures per piece. | Placing orders |  | Term of delivery of goods from the entry into force of the Agreement (k.d.) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Yes (mark if orders for goods will be placed on demand, periodically, etc.) | No (mark if the full quantity of goods purchased will be delivered at the specified time) |  |
| 1. | Diluted Gas Cooling System with delivery, installation, and training | 1 pc. | ☐ | ☒ | no later than within 16 months. |

  - The quantity and/or volume of goods indicated in the table above is accurate and will not change during the performance of the Agreement.

  - Order submission procedure:

    - orders are not placed during the period of validity of the Agreement. The goods must be delivered immediately after the date of entry into force of the Contract within the time limit set out in Table 1.

- PRODUCT REQUIREMENTS

3.1. If specific models or sources, specific processes or trademarks, patents, types, specific origin or production, etc., are used in the procurement documents, they may be replaced by equivalents.

Table 2.

| Second. No. | Setting | Required Value** | Technical parameters of the proposed equipment, precise references to technical documents (to be completed by the supplier) |
| --- | --- | --- | --- |
| Diluted gas refrigeration system |  |  |  |
| 1. | Manufacturer, model | Specifies the proposed item/ model/ name. A reference to the manufacturer or equivalent website or a copy of the technical documentation containing information on the characteristics of the product proposed must be provided. |  |
| 2. | Lowest Base Temperature | Less than 10 mK |  |
| 3. | Cooling power | More than 12W at 20mK More than 250W at 100mK More than 360W at 120mK |  |
| 4. | Temperature Meter | Not less than 8 channels for temperature measurement With touch screen |  |
| 5. | Helium-3 gas in the system | Purity not less than 99.9% Not less than 18 liters |  |
| 6. | Heater at 4K flange | Power not less than 40W |  |
| 7. | Main circulation pump | Not less than 250l/min |  |
| 8. | Vacuum Pump | Not less than 420l/s |  |
| 9. | Condensing compressor | Not less than 50l/min |  |
| 10. | Cryocooler Compressor | Electrical power: not less than 12kW Cooling capacity: not less than 2.7 W at 4.2 K not less then 55 W at 45 K |  |
| 11. | Magnets | Inner diameter at least 96mm Weight no more than 41kg Burnout protection |  |
| 12. | Coaxial wires | SMA type or equivalent, not less than 18GHz, not less than 3 pcs. |  |
| 13. | Vacuum housing | - Not less than 3 parts |  |
| 14. | GHS Advanced Feature Package | The system must have: Valve position reading sensor Water Flow Meter UPS (uninterruptible power) >1 min. |  |
| 15. | Ports for experimental wires | Not less than 7 SMA connectors in the KF40 flange. |  |
| 16. | Gas Management System | The system must have: - PLC controller or equivalent - Maintenance-free circulation pump - Self-diagnosis and full automatic control |  |
| 17. | Guarantee* | The warranty period offered by the supplier or applied by the manufacturer of the goods is established for the goods, but in any case not less than 24 (twenty-four) months. |  |

**NOTE: The supplier must provide together with the offer the technical parameters of the proposed equipment, except for the marked *, reliable supporting documents (e.g. manufacturer's product description, online link to the manufacturer's page). or other equivalent document).

3.2. The price of the goods must include delivery, full installation at the workplace and 12 (twelve) academic hours training of the Buyer's staff for 2 (two) persons to work with the equipment at the Buyer's premises.

- ENVIRONMENTAL REQUIREMENTS

4.1. The Environmental Protection Criteria shall be applied to the procurement   in accordance with the Order of the Minister of the Environment of the Republic of Lithuania No. D1-401 "On the List of Products to Which the Environmental Protection Criteria Are Applicable to Public Procurement and Procurement, the Environmental Protection Criteria and the Environmental Protection Criteria to be Applied by Contracting Authorities and Contracting Entities of 28 June 2011 when purchasing goods, services or works,  Chapter II, Sub-paragraph 4.4.4.1 of the Description of the Description of the Application of the Procedure for the Application of Environmental Protection Criteria in the Implementation of Green Procurement. The detailed requirements are specified in Clause 13.1 of the Special Terms and Conditions of the Agreement.

An equivalent object is considered to be a procurement object the characteristics of which are not inferior (i.e. the same or better) than the requirements for the object to be purchased in the procurement documents, and the proposed equivalent object of purchase can be used for its intended purpose without any limitation (including, but not limited to:

• without making additional changes to the interacting elements;

• the use will not affect faster wear and tear of the interacting elements, breakdowns and/or loss of warranty;

• the envisaged service period is not shorter;

• There is no lower level of technical excellence.

When proposing an equivalent procurement object, it is mandatory to submit documents proving compliance with the requirements set for the procurement object. Such documents could be a research report or certificate of a conformity assessment body established in the Republic of Lithuania, as well as a certificate issued by equivalent conformity assessment bodies established in other countries. If the Supplier is unable to obtain the specified certificates or test reports due to circumstances beyond the Supplier's control and proves by objective, written evidence that the proposed equivalent procurement object complies with the requirements or criteria specified in the Technical Specification, the criteria for the evaluation of tenders or the conditions for the performance of the procurement contract, the Buyer also recognises other appropriate measures. However, the Supplier's self-declaration without specific, technical evidence is not considered to be appropriate measures. The Buyer reserves the right to carry out a Hazard Risk Assessment if the documents provided for the equivalence of the proposed product are insufficient.