ACQUISITION OF VNO PERIMETER AND NO-FLY ZONE ANTI-DRONE PROTECTION SYSTEM

TECHNICAL SPECIFICATION

Contents

1.	PIRKIMO OBJEKTO APRAŠYMAS	2

1.1.	Objetives and scope of the project	3

2.	GENERAL REQUIREMENTS FOR THE SYSTEM (AAS) "SAKALAS"	3

3.	DETAILED REQUIREMENTS FOR THE SYSTEM	3

1.	Requirements for (AAS) "Sakalas" passive radio sensor module:	3

3.2.	Requirements for (AAS) "Sakalas" Centralized Threat Detection and Management Module:	3

3.3.	Requirements for the (AAS) "Sakalas" camera module:	4

3.4.	Requirements for the (AAS) "Sakalas" UAV takeover module:	4

3.5.	Requirements for stems:	4

3.6.	Requirements for the (AAS) "Sakalas" UAV "Sakalas" module:	4

3.7.	Requirements for (AAS) "Sakalas" for the "Nests" module:	6

4.	SUPPORT AND MAINTENANCE	7

5.	OTHER REQUIREMENTS	7

DESCRIPTION OF THE OBJECT OF PROCURAMENT

Joint Stock Company Lithuanian Airports (hereinafter referred to as the Buyer) plans to procure stationary Vilnius Airport Anti-drone protection system (AAS) "Sakalas" is designed to automatically detect (active) in the protected airspace of the airport (in the territory and its accesses in the no-fly zone 0AGL) unmanned aircraft (UAV or drone) and/or its operator according to the radio emission emitted by them (control, telemetry or video signals), to determine the coordinates and altitude of UAV by triangulation and to remove UAV from the protected space.

Objetives and scope of the project

(AAS) "Sakalas" determines the UAVs in the protected airspace without permission according to the U-SPACE data received by AB Oro navigacija (ON), which are integrated into (AAS) "Sakalas" according to the application programming interface in Annex No. 4 of the TS.

(AAS) "Sakalas" removes UAVs in the protected airspace without permission by using electronic-cyber (taking control, blocking or landing access) (without the use of radio wave suppression (jamming)) or kinetic impact (aerodynamic disturbance) means that interrupt the flight of the UAV in the protected airspace (without the use of weapons or explosives).

(AAS) "Sakalas" also uses the means of influence integrated in the (AAS) "Sakalas" – UAV "Sakalas" – for the identification of UAV in the protected airspace of the airport without permission (type, model, flight direction and route, coordinates, altitude and danger) and its kinetic removal.

UAV "Sakalas" operates at up to 10 km from the farthest (AAS) "Sakalas" control antenna (repeater). It reacts to the detected violations and the decision to activate them (flies) within 3 minutes.

(AAS) "Sakalas" manages its integrated UAV "Sakalas" in automated, mission and "first person view" (FPV) modes. For the activated UAV "Sakalas", data (coordinates and altitude) about the thread (the UAV in the protected airspace without permission – “Duck”) are provided and updated automatically and continuously. The (AAS) "Sakalas" is controlled by the operator, who can select the control mode of the UAV "Sakalas".

UAV "Sakalas" are additionally used for patrol and threat checking function around the aerodrome perimeter.

(AAS) "Sakalas" is intended for continuous operation (24/7 principle).

GENERAL REQUIREMENTS FOR THE SYSTEM (AAS) "SAKALAS"

The system must be installed  in accordance with the requirements of INTERNATIONAL AMMUNITION TECHNICAL GUIDELINES (IATG) 05.60 "Hazards of electromagnetic radiation" or an equivalent standard.

Must comply with the  requirements of the EMC Directive 2014/30/EU and the Minimum Health and Safety Directive ETF Directive 2013/35/EU,

The set of system modules must ensure automatic detection of unauthorized UAV, sensor correlation, real-time detection and transmission of UAV coordinates, altitude to the operator and UAV "Sakalas", and removal of the target in the entire protected space.

The system must not threaten civil aviation, disrupt traffic operations, but act in their favor.

The system consists of:

Passive Radio Sensor Module;

Centralized threat detection and management module;

Camera module;

UAV Takeover module.

UAV "Sakalas" module.

Quick launch sites "Nests" modules.

DETAILED REQUIREMENTS FOR THE SYSTEM

- 1.	Requirements for (AAS) "Sakalas" passive radio sensor module:

The module must operate passively, without emitting radio waves. Must have at least 3 direction-determining antennas;

The antennas of the module must have at least 6 radio analyzers (sensors) operating in the specified frequency range, each analyzer has a scanning speed of at least 1 THz/s, with the possibility of connecting additional analyzers if necessary without additional integration;

Sensors must not have moving parts – passive cooling;

The system shall support the following satellite positioning systems: GPS, GLONASS, Galileo, BeiDou;

Operating temperature range – at least -30 C to +55 C;

Environmental resistance – at least IP 66 or equivalent;

System components (radio elements, antennas, antenna storage structures, cables) that are installed outdoors must be adapted to work in the outdoor climate conditions of Lithuania;

Provision must be made for the possibility to operate of (AAS) „Sakalas“ idle GNSS;

The module shall operate in a frequency band not narrower than 400 MHz to 6 GHz;

The module shall provide at least 360 coverage of  the entire no-fly zone of the airport, including runway accesses. Must monitor the full frequency range in all directions with signal direction setting. Suitable for use in urban areas;

The detection distance at runway approaches (straight) must be at least 10 km (in open space, from the nearest detector);

The module must be capable of detecting, tracking, and identifying (Detect – Track – Identify) DJI drones that transmit encrypted or protected telemetry using only the system's own software;

The module must be able to track at least 3 objects at the same time;

The module must be able to detect UAV swarms (at least 20 UAV at a time);

The module must identify devices that send analog and digital standard signals;

If a conventional WiFi connection is used for UAV control or video transmission, the module must be able to detect the signals sent by UAV based on the network address lists of UAV manufacturers stored in the database.

The module must be able to analyze the received signals and identify the technologies and protocols used by the UAV and submit them to the (AAS) "Sakalas" operator. Determine type, model, serial number;

The module must support the Remote Drone ID function;

If standard UAV radio protocols (e.g. DJI) must also identify the control panel of the UAV.

The altitude of the UAV, the location of the UAV and the UAV operator shall be determined by the principle of triangulation using at least three frequency sensors installed at different locations of the protected object. The system must be capable of detecting and continuously tracking the precise location of the drone in real time, ensuring GPS-level accuracy, regardless of Remote ID data;

The sensors of the module must be correlated with each real object (AAS) "Sakalas" to display the operator as one, despite the fact that it is being monitored by several sensors;

Azimuth tracking accuracy of not less than ±1,5° using triangulation and GPS accuracy using protocol decoding;

The time, coordinates of the UAV and UAV operator, altitude and trajectory of movement must be continuously displayed on a digital map (GIS) for use by the (AAS) "Sakalas" operator.

Requirements for (AAS) "Sakalas" Centralized Threat Detection and Management Module:

All system elements and modules must be integrated and controlled from a single software, including but not limited to UAV (“Duck”) detection, its interception, and the management and configuration of the drone (for the neutralization, protection and patrolling) UAV "Sakalas";

The system must operate at the protocol level by triangulation by determining the location of the control panel of all the most popular drone manufacturers and UAV location. The minimum mandatory list of protocols for UAV and other low-flying aircraft is provided in Annex 1 to the TS;

The system must determine the location of homemade and other types of UAV by triangulation. The minimum mandatory list of protocols is set out in Annex 2 to the TS;

The system must integrate data from the radio sensor modules. The system shall automatically recognize UAV when UAV customized communication protocols or video transmission are used.

The system must have the function of decoding the detected FPV type UAV broadcast video signal, render the decoded image.

The module must store the entire collected history of events: (type, model, serial number, flight  direction, and route, coordinates, altitude, frequency, time, user ID). The data must be stored for at least 90 days. The data must be exported in the following formats: an Excel file with all the system actions and a KMZ file that provides the location of the system, drone detections and neutralization actions suitable for viewing in a Google Earth 3D environment;

The system must be able to include detected UAVs in the system database, identify repeated violations with a warning from the (AAS) "Sakalas" operator.

The system must have the ability for the user to upload their offline maps. There must be the possibility of using 2D and 3D maps;

The user graphical interface must be adapted to the work of a single operator;

The representation of the sensors' field of view must be displayed on the map. A perimeter marking function must be implemented that allows you to define an unlimited number of protected areas without limiting the shape of the territory. The operator must be informed by an audible and visual signal (alarm) about UAV detection in the protected area;

(AAS) "Sakalas" must display the integrated U-SPACE data (TS Annex No. 4 by AB Oro Navigacija) in accordance with the application programming interface, which must be displayed in the system and marked accordingly. They must contain at least:

All available information from the ON issued permit to fly enables (AAS) "Sakalas" to verify – compare the consistency of the established data with U-SPACE about UAV and UAV control panels) and when they are detected and marked.

All information about the flight (planned/operational flight) time, place, altitude, destination.

UAV Autoresponder (transponder) data, which would ensure its accurate verification in the system (note: without its data, LTOU will not coordinate permission to fly in its protected airspace).

The pilot's personal data and contact details shall be available for immediate contact;

All planned future protected area UAV flights with detailed (especially contact) information about the flight and the pilot.

The sensor-correlated information must be displayed on the map. The coordinates, altitude and trajectory of the UAV and UAV operator shall be permanently displayed on a digital map.

There must be an ability to control the map (rotating, drawing different zones);

The system must have automatic target tracking with continuous transmission of information to UAV "Sakalas";

The system must contain an audible and visual signal (alarm) in the event of detection of UAV “Duck” in the tracking area, except for the UAV "Sakalas", which must be recognizable on the map;

The operator of the (AAS) "Sakalas" shall be given the possibility to mark drones in the system as "friendly" in real time or according to a pre-established list, and the system shall automatically recognise them each time they enter the detection area.

The system must be able to control UAV "Sakalas" with a joystick (FPV controls, glasses) or with the help of a keyboard and mouse.

There must be an opportunity to monitor the parameters (telemetry) of UAV "Sakalas“.

There must be an opportunity to watch the video broadcast by UAV "Sakalas" in real time (up to 3 at a time) and record it.

There must be an opportunity to connect to another operator, to work in the network (observer, controller or UAV "Sakalas" in FPV operator mode);

The User's software must be installed on a computer with an operating system adapted to the operation of the software and ensuring cybersecurity in accordance with the Lithuanian airports policy.

The computer platform must be Intel vPro or equivalent;

Memory – at least 16 GB DDR5;

2x23 LED monitors with brackets: resolution – at least 1920x1080, refresh rate – at least 60 Hz, response time – no more than 8 ms, IPS technology, screen brightness – at least 250 cd/m

Must have a keyboard and a professional video surveillance joystick/console (FPV controls, glasses).

Requirements for the (AAS) "Sakalas" camera module:

On pan tilt installed, automatic/controlled, fully integrated (interactive, helping to point and focus cameras on detected UAV and track, zoom in for initial threat assessment and capture) to the central control module day-night pair of cameras.

The module consists of an optical camera and thermal imager;

Automated direction and AI-based visual target tracking (artificial intelligence or digital image analysis algorithm, computer vision) must be implemented.

The targeting of the detected UAV must be ensured from 7 km, and its hazard assessment must be ensured from 3 km to the module (camera);

Optical cameras with image stabilization and autofocus resolution not less1920x1080 (HD);

Thermal imager resolution at least 1280x720;

The focal length of the optical camera is not less than 15-250 mm;

The field of view of the optical camera is not less than 1.5 to 25 degrees;

Thermal imager focal length 75-900 mm;

The field of view of the thermal imager is not less than 0.1 to10 degrees;

The module must operate in 24/7 mode and meet the requirements of IP67 certification;

Operating ambient temperature from -20C to +45C.

Requirements for the (AAS) "Sakalas" UAV takeover module:

The system must be able to take control of the detected drone by disconnecting it from the pilot and pointing it to a predetermined safe landing point;

The system is controlled by the operator but must also be able to operate autonomously – automatically, without the operator, select to activate UAV neutralization measures.

The operator must be able to configure several safe landing zones to which the intercepted drone can be directed. The system must automatically select the nearest possible landing point, taking into account the location of the drone at the time of the takeover;

The effective neutralization operating distance shall be 5 km from the nearest antenna for interception;

The system must be able to use "whitelists" to detect, but not to take neutralisation actions against whitelisted UAVs.

All cyber neutralization functions must be controlled from a unified user interface – the workstation of the (AAS) "Sakalas" operator.

The system must ensure that the neutralization actions do not interfere with other UAV "Sakalas" operating in the same operational area, or others with UAV permits.

The system must be able to take over the management of the UAV of the most popular manufacturers, landing them safely in the chosen place, the minimum mandatory UAV and the list of protocols is provided TS Annex No. 3

Requirements for stems:

Altitude – the location of the object is flat and aerodrome The project for the installation of radio sensor modules must be coordinated with Lithuanian airports and ON, must take into account the peculiarities of the aerodrome and the established installation requirements.

For installation and preparation work, professional structures and fasteners that are resistant to environmental effects during the warranty period must be used;

The type of supports supplied must be openwork.

Requirements for the (AAS) "Sakalas" UAV "Sakalas" module:

UAV "Sakalas":

10-inch quadcopter type drone, VTOL (vertical take-off and landing);

Maximum speed: up to 150 km/h;

Effective operating distance from the "Nest": 10 km one way (or 20 km in total);

Maximum flight time: up to 60 min (or up to 30 min with payload/kinetic net trap). The flight time can be summed up if the "Sakalas" change each other in an autonomous automatic way;

Integration with (AAS) "Sakalas", which provides coordinates and altitude of targets in real time, ensuring the ability of UAV "Sakalas" to carry out automated response.

Automated target finding (automatic mission) by receiving altitude, coordinates and updating them in real time.

Ability to visually identify/evaluate the target (“Duck”) (type, model and determine its hazardous) and, depending on the action chosen, track or neutralize it:

When firing at the UAV – the target's net.

By disrupting UAV – the aerodynamics of the target by kinetic effects.

Compatible with other (in addition to (AAS) Sakalas) open standards data reporting systems (using APIs or similar data exchange methods/protocols).

Transmission of UAV "Sakalas" telemetry (name and destination, ID, position, altitude, speed, etc.) to detection and control systems (transponder).

Weather restrictions for the use of UAV "Sakalas" up to:

Maximum wind speed: up to 12 m/s (with gusts up to 18 m/s);

Air temperature: from -20 °C to +45 °C;

Relative humidity: up to 90%, non-condensing;

Maximum operating altitude over sea level: up to 3000 m (AMSL);

UAV "Sakalas" must have protection of electronic components (covered with a special material (conformal coating)) against precipitation and have additional housing details that direct water access from the electronics.

The control and retransmission means of the UAV "Sakalas" must be equipped with directional or multidirectional antennas (Ground Station). They are installed in the same places as the elements of the passive radio sensor module of (AAS) "Sakalas“.

Automatic tracking mechanisms (antennas with servo mechanisms) and fixed directional systems must be supported.

For the control of the UAV "Sakalas", a stationary antenna (stationary repeater) installed in the airport must be used.

Communication between communication antennas and repeaters may be provided by wire or radio communication, depending on operational needs and capabilities.

UAV "Sakalas" control must use interference-resistant in difficult electromagnetic conditions FHSS technology (frequency hop scattering spectrum). Main parameters:

Frequency band: 570-2100 MHz (depending on configuration);

Configurable channels: It must be possible to select and harmonize frequencies according to the requirements of the user or services, taking into account the requirements of the Communications Regulatory Authority of the Republic of Lithuania and aviation security;

Frequency spikes must be performed according to a secure algorithm, reducing the likelihood of interception or blocking of the connection.

UAV "Sakalas" inhibition measures must enable UAV "Sakalas" AI to function.

Autonomous functions of UAV "Sakalas":

Autonomous ascent / descent to the "Nest";

Autonomous route execution according to (AAS) "Sakalas" automatic coordination, which allows to obtain the location of the target in real time.

Autonomous return mode: In case of loss of connection or completion of a task, UAV "Sakalas" automatically returns to the take-off point (or other designated point) using GNSS-based navigation.

Autonomous return with battery reserve assessment and the ability to adjust the route according to the course of the mission;

(AAS) "Sakalas" and UAV "Sakalas" must have an integrated capability to perform semi-automatic or fully autonomous targeting in the neutralization phase to increase the accuracy of hitting against moving or static targets:

UAV Sakalas' automated target tracking and neutralization (Auto-Tracking/Auto-Targeting) system must use image analysis algorithms (AI, or digital image analysis algorithm, computer vision) to detect, capture and track a moving target in real time. When the auto-aim/approach mode is activated, UAV "Sakalas" independently adjusts the trajectory according to the movement of the object until the collision/net shooting.

The manual targeting of the UAV "Sakalas" is carried out by the operator, it can take control at any moment, perform the application using either the image on the screen or the real-time FPV mode (its visual and control tools). In this way, making it possible to carry out complex tasks or adjust the trajectory in specific situations;

In hybrid mode, the UAV "Sakalas" must be able to start the mission in automatic autonomous mode and switch to manual mode (or vice versa), thus giving the operator more flexibility and control in emergency situations.

UAV "Sakalas" target neutralization methods:

Trapping with a net. The UAV "Sakalas" must have a special net ejection module, which allows to neutralize the UAV – the target by ejecting the net into it at a close distance (at 3-5 m). The net must be fired automatically or by the operator's command. The network capsule at the launch site must be able to be replaced within 3 minutes.

Physical kinetic neutralization of a target – UAV (“Duck”) by disrupting its aerodynamics with kinetic effects. Where rapid and more aggressive neutralization is necessary, it must be possible to choose direct disruptive contact with the target by airflow or direct force.

UAV "Sakalas" control is carried out directly from the (AAS) "Sakalas" operator's console or in manual first-person mode (FPV) (control panel and glasses).

UAV "Sakalas" navigation must be based on coordinating task scenarios (missions) (in FPV mode when navigation is not working).

UAV "Sakalas" must be sent to specific GPS coordinates received from (AAS) "Sakalas" or to the targets transmitted by it to automatically and autonomously perform the return function based on route points.

UAV "Sakalas" sensors (two sets):

Intelligence UAV "Sakalas" are equipped with a gimbal:

Stabilized gimbal with dual sensor package (day and IR camera);

Long-distance optical zoom (7-15x);

Low-light or thermal imaging sensor that allows it to operate in poor visibility or night conditions;

Real-time video transmission to the operator via the control console;

Ability to capture the exact coordinates of the object during reconnaissance (integration with coordinate presentation systems).

Patrol UAV "Sakalas":

Ability to install and control the network exhaust module;

Type: Digital day-night camera;

Daytime camera: Resolution1080p (1920 × 1080);

Thermal imager – resolution 640x512 (or equivalent), which allows you to recognize objects at a distance of at least 300 meters;

Possibility to integrate additional cameras (modular structure) e.g. higher resolution thermal imager or day care camera with optical and/or digital zoom;

Video transmission – 720p/1080p resolution video broadcast to the (AAS) "Sakalas" control module.

Requirements for (AAS) "Sakalas" for the "Nests" module:

UAV "Sakalas" "Nests" (containers) are installed. Drone-in-a-box, this autonomous operation solution allows the drone to automatically take off and land at a specially adapted station (“Nest”). This container is designed to accelerate response, automate missions, improve readiness, simplify logistics, and maintain continuous drone (UAV “Sakalas”) readiness without human intervention:

UAV "Sakalas" must be able to autonomously take off from and descend to "Nest“.

The "Nest" must be IP65 or higher in resistance to protect the drone (UAV “Sakalas”) and all the electronics it contains from precipitation, dust, cold and heat. Suitable for continuous outdoor use in an operating temperature range of at least -30 C to +55 C;

Automatic battery charging must be carried out after landing in the "Nest". The UAV "Sakalas" battery is automatically charged through a contact or induction system.

The operational readiness status of the UAV "Sakalas" must be monitored in real time (charging level, GPS connection, functionality of cameras, sensors), and the information is transmitted to the operator to (AAS) "Sakalas“.

The "Nests" must be made of resistant metal or composite;

"Nests" must be installed in several locations at a distance from each other in order to ensure expediency at the ends of the runway and in the centre;

In the "Nests" (in each) there should be at least 4 places for UAV "Sakalas".

SUPPORT AND MAINTENANCE

The Supplier must train users (up to 20 persons), technical personnel and instructors (up to 5 persons) who will perform first-level maintenance and repair of equipment at its own expense within 3 months from the date of signing the act of acceptance-transfer.

The Procurement Object must be provided  with a guarantee of at least 60 months, calculated from the date of signing the act of its delivery to the Procurement Entity.

The price of the procurement object must include the periodic technical maintenance required for the functioning of the procurement object systems, the technical and licensing support of the required software for at least 60 months.

The price of the purchase object must include timely (when necessary - immediate) updates to the database of drones detected by the system, thus enabling the system to recognize new UAVs and their data transmission protocols that have appeared on the market. Updates must be provided at least 4 times a year and for at least 60 months.

OTHER REQUIREMENTS

The supplier of goods, its sub-supplier, the economic entity whose capacities are relied upon, or the producer and the person controlling them cannot be registered in the country (if the producer or the person controlling it is a natural person – permanently resident or having citizenship of that country), which by the Resolution of the Government of the Republic of Lithuania No. 280 of 30 March 2022 "On Article 92 13 of the Republic of Lithuania Law on Public Procurement,  implementation of paragraphs 14 and 15' is not included in the approved list of States.

The supplier can only offer manufactured and commercially available equipment, CE marking and equipment with the necessary functionality. The CE marking is mandatory only for those product categories that are subject to the European Union directives requiring such marking.

The goods, services or works offered by the supplier must not pose a threat to national security.

The system must be protected from unauthorized login, control, or reconfiguration. It shall not include the possibility for the supplier to use the "Kill switch" function. There must be a system restore function from a backup. All devices that can be connected to via an internal computer network or directly must be protected with passwords.

Within the scope of the project, the Supplier's invoice must be received and submitted to the Buyer by all permits and licenses and certificates necessary for the introduction of the provided equipment into Lithuania or its operation, as well as taxes paid.

The equipment of the systems must be new, not previously used, suitable for use for its intended purpose. Used or refurbished equipment cannot be offered.

The supplier of goods, its sub-supplier, the economic entity whose capacities are relied on must:

Provide all software, configuration files, and backups;

To submit a detailed implementation project of the procurement object, reflecting the principles of the installation of the system being developed and presenting the diagram of the connection of the equipment, the power supply diagram and explaining the essential technical solutions, the parameters of the configuration of the system being developed and the characteristics of the equipment;

To provide relevant system user instruction in Lithuanian;

Provide relevant instructions to the system administrator in Lithuanian;

All installation, installation and configuration of the supplied equipment must be carried out within the scope of the project, including but not limited to the assembly, installation, assembly of the equipment and all other necessary works and materials and tools required for this purpose. The scope of supply must include all necessary connecting cables and other equipment and materials;

Within the scope of the Project, electrical power supply equipment (with uninterruptible power supplies ensuring the operation of the system for at least 4 hours) must be installed to connect all the equipment supplied and installed during the Project to the existing connection points;

All outdoor installation equipment must have adequate grounding, surge protection and installed lightning protection solutions;

The location of the equipment must be chosen in such a way as to ensure maximum coverage of the monitored area and its accesses when installed.

The UAV detection system must be able to be integrated into the existing security system by transmitting, but not limited to, "Alarm" and system fault signals.

The system must have the ability to transmit information about UAV`s detections in the following formats: Excel file with all system actions and KMZ file in open standard format.

The system must be integrated with the U-SPACE system installed by AB Oro navigacija according to the application programming interface in Annex No. 4 of the TS (provided upon demand).