Annex 1

to the Procurement Conditions

TECHNICAL SPECIFICATION OF

REMOTE EXPLOSIVE DETONATION SYSTEMS

### 1. Introduction This technical specification defines the requirements for a remote explosive detonation system (hereinafter referred to as the System). The System is designed to initiate explosive detonation remotely. CPV code – 35700000-1 Military Electronic Systems.

### 2. Functional Requirements 2.1 The complete System must be brand new, unused, and delivered in original manufacturer's packaging. 2.2 The System shall support both group and individual explosive detonations. 2.3 The System must operate and be ready for detonation using only a transmitter and receiver, without additional system components. 2.4 The System must control at least ten receiver-initiators. 2.5 The System must be capable of detonating explosives at the following minimum distances: 2.5.1 In urban terrain – no less than 2 km; 2.5.2 In open terrain – no less than 3 km; 2.5.3 In line of sight – no less than 10 km. 2.6 In standby mode, the transmitter and receiver-initiator must remain passive and emit no signal. 2.7 The System must have water and dust resistance protection rated at least IP65 (as per IEC60529 standard). 2.8 Communication between components must be encrypted at AES128 security level or higher. 2.9 The System must include functionality to verify correct connections between components and prevent misfire. It must be able to inform the user of each initiator’s status: - Safe mode; - Armed mode. 2.10. Communication frequency must comply with the spectrum approved by the Lithuanian Communications Regulatory Authority. 2.11. The System must be powered by standard household batteries (including rechargeable variants), which must be easily replaceable. In case of rechargeable or integrated batteries, charging must be ensured via USB-C connection and/or charger. 2.12 The System must operate within the temperature range from A1 to C1, as defined in STANAG 4866 AECTP-230. 2.13 The System must be delivered with all necessary components and materials required for full internal/external and peripheral connections to ensure full functionality. 2.14 In accordance with Article 40, paragraph 9 of the Public Procurement Law of the Republic of Lithuania for Defence and Security, the System must not pose any threat to national security.

### 3. Other Requirements 3.1 A minimum 24-month warranty must be provided for the System equipment. During the warranty period, the Supplier shall rectify any equipment failures at their own expense, except those caused by user misuse. 3.2 Warranty repair duration must not exceed 6 months from the date of delivery to the manufacturer. 3.3 System package must include: 3.3.1 At least one transmitter; 3.3.2 At least five receiver-initiators with spare batteries for each; 3.3.3 Shock-resistant, sealed equipment case with at least IP67 protection (IEC60529). The case must contain internal padding with cutouts matching the shape of equipment and accessories. 3.4 The System must be delivered with all necessary components and materials required for full integration of internal/external and peripheral devices. 3.5 Transmitter and receiver-initiator casings must have matte dark color selected from the following RAL palette: RAL 6002, RAL 6003, RAL 6004, RAL 6005, RAL 6006, RAL 6007, RAL 6008, RAL 6009, RAL 6012, RAL 6014, RAL 6015, RAL 6020, RAL 6022, RAL 6031, RAL 6035, RAL 6036, RAL 7002, RAL 7008, RAL 7009, RAL 7012, RAL 7013, RAL 7016, RAL 7021, RAL 7022, RAL 7024, RAL 7026, RAL 8011, RAL 8014, RAL 8015, RAL 8016, RAL 8027, RAL 2017, RAL 8019, RAL 8022, RAL 8028, RAL 9004, RAL 9005, RAL 9011, RAL 9017. Metal components must be coated with corrosion-resistant paint per STANAG 4360. 3.6 To initiate detonation, the System must incorporate safety mechanisms preventing unintentional use (e.g. special key, combination of at least two buttons, delay timer, additional safeguard component). 3.7 The System must be assigned a NATO Stock Number (NSN). 3.8 The Supplier must train System operators and technical maintenance personnel. 3.9 User manual must be provided in Lithuanian and/or English. ### 4. Terminology Transmitter – control device that sends commands to receiver-initiator remotely. Receiver-Initiator – device receiving commands from the transmitter and passing signal to detonator. Detonator – charge that explodes upon receiving an impulse, triggering the main explosive. Charger – device used to charge rechargeable batteries.