Appendix 2 to Annex 2 to the Purchase Conditions (In case of differences in interpretation the Lithuanian text shall prevail)

TECHNICAL SPECIFICATIONS FOR THE PROPOSED BALLISTIC PROTECTIVE GLASSES FOR DUST

(applicable to Part 2)

By submitting a tender for a purchase, the supplier certifies that, the object to be purchased will meet the following requirements:

| Technical specification item | Required technical characteristics of the goods | Possible value | Specific values for the product indicators offered by the supplier* | Supporting documents. |
| --- | --- | --- | --- | --- |
| I. | GENERAL PROVISIONS |  |  |  |  |
| 2. | The glasses are designed to provide eye protection for soldiers on combat missions. | Yes or no |  | - |
| 4. | The glasses shall be manufactured to comply with the ballistic protection and optical class standard MIL-PRF-32432A or equivalent for Class 2 protective glasses. | Class 2 (goggles) |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 5. | The term of the product quality guarantee shall be not less than 12 (twelve) months under conditions of active operation starting from the date of issue of the goods from the Buyer’s warehouse, 24 (twenty-four) months starting from the date of the signing of the documents of acceptance of the goods into warehouse | Specify the exact duration of the guarantee |  | - |
| II. | TECHNICAL REQUIREMENTS |  |  |  |  |
| 6. | Glasses must provide good visibility and the lenses must be made of a highly transparent, durable, shatterproof material (polycarbonate or equivalent in terms of functionality). | Specify |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 7. | The glasses must be compatible with Advanced Combat Helmet (ACH), Modular Integrated Communications Helmet (MICH), Future Assault Shell Technology (FAST) ballistic helmets used in the Lithuanian Armed Forces. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 8. | The frame of the glasses shall be resistant to harsh and extreme environmental conditions and shall be made of black or yellowish-brown (colour to be agreed at the time of the working sample), durable, flexible thermoplastic polyurethane (TPU) or equivalent material. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 9. | No additional tools or equipment shall be required for the replacement or removal of spectacle components such as lenses, nose pads, locking straps, as specified in clause 3.6.4 of standard MIL-PRF-32432A, or equivalent. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 10. | The colour transmittance of glasses shall comply with the limits specified in accordance with clause 3.8.5.4 of standard MIL-PRF-32432A or equivalent. | Specify x; y =? |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.9.4) provide a reference to the document, page, clause |
| 11. | The lenses of glasses shall not be colour-fused as specified in clause 3.8.5.8 of standard MIL-PRF-32432A or equivalent. | Yes or no |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.9.8) provide a reference to the document, page, clause |
| 12. | Glasses shall not break or otherwise change shape when subjected to the temperatures specified in clause 3.9.5.1 of standard MIL-PRF-32432A or equivalent. | 72 h (-60F°; 160 F°) |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 13. | Glasses shall not break or otherwise change shape when exposed to sunlight as specified in clause 3.9.5.2 of standard MIL-PRF-32432A or equivalent. | 200 h |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 14. | Glasses shall not break or otherwise change shape when exposed to moisture as specified in clause 3.9.5.3 of standard MIL-PRF-32432A or equivalent. | 240 h |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 15. | Glasses shall not break or otherwise change shape, although they do not have adequate support points as specified in clause 3.9.5.4 of standard MIL-PRF-32432A or equivalent. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 16. | Glasses shall be flame retardant at a rate higher than 76 mm per minute and shall meet the ignition/flame retardancy requirements of ANSI Z87.1 2015 or equivalent standards as defined in clause 3.14.1 of standard MIL-PRF-32432A, or equivalent standard. | ≤76 mm/min |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.18) provide a reference to the document, page, clause |
| 17. | Textile products supplied with protective glasses shall be resistant to mould for at least 6 months of active use or 60 months of storage and shall comply with clause 3.8.2 of standard MIL-PRF-32432A or equivalent. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 18. | Glasses shall provide eye protection against ballistic fragments, splinters, detonation of explosive charges, mechanical eye damage, dust, wind, liquids, in all components of glasses, as specified in clause 3.8.4 of MIL-PRF-32432A, or equivalent standard. | Yes or no |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.8.5) provide a reference to the document, page, clause |
| 19. | Glasses shall provide good visibility in all climatic conditions and the lenses shall be coated with a rust-resistant coating in accordance with the requirements of clause 3.8.5.11 of MIL-PRF-32432A, or equivalent standards. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 20. | The protective glasses system shall have an elastic adjustable glasses fitting/locking strap for effective and comfortable use by the soldier. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 21. | Glasses shall intercept not less than 99,9 % of harmful ultraviolet (UV-A, UV-B) rays in accordance with the requirements of clause 3.8.5.9 of MIL-PRF-32432A, or equivalent standard. | ≥ 99.9% |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.9.9) provide a reference to the document, page, clause |
| 22. | Glasses shall be resistant to acids and alkalis and shall comply with clause 3.8.3 of MIL-PRF-32432A or equivalent standard | Yes or no |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.7.3) provide a reference to the document, page, clause |
| 23. | Glasses shall have maximum scratch resistance in accordance with clause 3.8.5.10 of standard MIL-PRF-32432A or equivalent. | Specify % |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.10) provide a reference to the document, page, clause |
| 24. | Soldiers with visual impairments shall be able to insert a (Rx) optical corrective lens holder into their glasses that is compatible with the glasses and shall have all necessary adaptors. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 25. | Glasses shall ensure a wide field of vision in all directions when lying down and standing, in accordance with the requirements of clause 3.8.5.7 of MIL-PRF-32432A, or equivalent standard. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 26. | The glasses shall be comfortable to wear without discomfort in accordance with clause 4.15.4 of MIL-PRF-32432A or equivalent standard. | Yes or no |  | - |
| 27. | The glasses must be new and without any defects. | Yes or no |  | - |
| 28. | The methods for verifying the requirements are defined in Table VI of standard MIL-PRF-32432A, in accordance with clause 4.2, or its equivalent. | Yes or no |  | - |
| 29. | The user guide for the glasses must be available in Lithuanian or English. | Specify |  | - |
| 30. | Protective sunglasses set: - glasses frame; - elastic, adjustable glasses strap; - protective clear lens; - protective dark lens; - adjustable nose support; - zipped storage and transport case for glasses; - cleaning product (wipe) for protective lenses; - user manual for the protective goggles (in Lithuanian or English). | Specify |  | - |
| 31. | Glasses shall be Type C and shall comply with clause 3.5.1.6 of standard MIL-PRF-32432A or equivalent. | Style C |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 32. | The glasses shall be fitted with a clear lens with a photopic transmission of light of at least 75 % and shall comply with clause 3.8.5.1 of standard 3.8.5.1 of MIL-PRF-32432A or equivalent standard. | ≥ 75% |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.9.1/4.9.2) provide a reference to the document, page, clause |
| 33. | The glasses shall be fitted with a dark lens with a photopic transmittance of not less than 12-18 % measured in the critical optical areas, and shall comply with clause 3.8.5.2 of standard 3.8.5.2 of MIL-PRF-32432A, or its equivalent. | 12 - 18% |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.9.1/4.9.2) provide a reference to the document, page, clause |
| 34. | The ballistics of the lenses shall be such that they will pass tests when a 0,22 calibre (0,05588 mm) diameter, 17 (± 0,5) grams (1,10158 grams) weight, T37 shaped fragment of 17 (± 0,5) grams (1,10158 grams) in weight, with a vertical velocity of 580-590 feet per second (176,78-179,83 metres per second), is struck once on the right side of the base side of the lenses of the glasses and once on the left side of the base side of the lenses. The third impact shall be made to the main side of the lenses at the centre of the vertical centreline in accordance with clause 4.8.5 of MIL-PRF-32432A or equivalent standard. | Specify |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.8.5) provide a reference to the document, page, clause |
| 35. | The lenses of the glasses must be double-layered (double-coated) to create a thermal bridge. | Yes or no |  | - |
| 36. | To protect against impact, harsh and extreme environmental conditions, the main part of the spectacle frame must be solid, strong but flexible. | Yes or no |  | - |
| 37. | The glasses shall have a removable/interchangeable frame component to interface with the face. | Yes or no |  | - |
| 38. | The interface between the main part of the frame and the face must fit snugly. | Yes or no |  | - |
| 39. | The protective glasses for dust system shall have an elastic adjustable glasses strap for effective and comfortable use by the soldier and shall allow the safe replacement of the lenses of the glasses without the need for additional instruments and equipment. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 40. | In order to provide eye protection when wearing the glasses on the face, the adjustable elastic strap shall be flame retardant and shall have holes on both sides of the spectacle frame to accommodate the strap. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 41. | When not in use and when the glasses are worn with the helmet up, the protective sleeve of the glasses shall protect against possible reflection that gives away the location of the military personnel, as well as protect the glasses against scratching, dust, dirt, and harsh outdoor conditions. | Yes or no |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 42. | The protective glasses sleeve and other textile components shall be flame retardant and shall comply with MIL-PRF-32432A 3.14.2 or equivalent. | <10 cm |  | provide a test report from a laboratory accredited according to international standards (tests according to MIL-PRF-32432A 4.19) provide a reference to the document, page, clause |
| 43. | Protective glasses for transport, wearing and protection against dust and dirt shall be supplied with a soft case made of a durable material and be fastened. It shall have a fastening capability. The fitted case shall hold securely during active movement. | Yes or no |  | - |
| III. | OPTICAL CORRECTION LENS HOLDER |  |  |  |
| 44. | The holder must be sturdy and the frame durable | Yes or no |  | - |
| 45. | The field of vision of the holder shall be wide. The colour of the frame of the holder shall not be distracting or distort the view. | Yes or no |  | - |
| 46. | Must be suitable for lenses of at least ± 6 dioptres made of polycarbonate or other resistant material. | Specify |  | provide a product description and/or the manufacturer's technical documentation and/or a declaration/confirmation from the manufacturer of the glasses provide a reference to the document, page, clause |
| 47. | The holder must be suitable for glasses for dust and be fixed. | Yes or no |  | - |
| 48. | The corrective lenses must be safely and securely pressed into place. | Yes or no |  | - |
| IV. | ACCEPTANCE AND LABELLING |  |  |  |
| 49. | Instructions for use and care must be included in Lithuanian or English in each package supplied. | Specify |  | - |
| 50. | Each product is labelled where indicated: the manufacturer’s name or trademark; for imported goods, the country of origin of the goods, if this is not the same as the country in which the manufacturer is registered; the product name, article or model; protective glasses components such as the frame and lenses are marked with markings that meet the required standards; date of manufacture; the labelling particulars shall be of sufficient size to permit easy reading and understanding of the information provided. | Yes or no |  | - |
| 51. | Dust goggle kits are packed in cardboard boxes. The packaging shall comply with the minimum environmental criteria set out in Annex 2, Chapter II 'Packaging' of Annex 2 to the 'Description of the Procedure for the Application of Environmental Criteria in Green Procurement' approved by Order No D1-508 of the Minister of the Environment of the Republic of Lithuania on 28 June 2011. | Yes or no |  | - |
| 52. | Packages shall be labelled with the following particulars: manufacturer’s name, address (for goods made in Lithuania); for imported goods, the country of origin of the goods, if this is not the same as the country in which the manufacturer is registered; the importer’s name or trade mark and address for imported goods; product name, article or model; batch and consignment indexes; size, colour (if more than one colour is supplied); quantity; date of manufacture; contract No. and date; NSN code issued by the Lithuanian Armed Forces or the manufacturer's existing NSN code if the products have one. | Yes or no |  | - |
| 53. | Products shall be accepted on the basis of an agreed and approved working sample. Two protective glasses shall be submitted for approval of the working samples, together with documentation (descriptions and test reports of the materials used by the manufacturer, test reports from certified laboratories). | Yes or no |  | - |

*The supplier must indicate the specific values of the technical characteristics to which the goods offered conform and which are indicated in the laboratory test reports or other copies of documents submitted with the tender.

_________________________