TECHNICAL SPECIFICATION

  - Purchaser, Customer – AB Amber Grid.

  - Supplier – an economic entity – a natural person, private legal entity, public legal entity, other organizations and their divisions or a group of such persons with whom the Purchaser, Customer enters into a Contract.

  - Contract – a contract entered into between the Supplier and the Purchaser regarding the Procurement Object.

  - MGP – main gas pipeline.

  - GDS – gas distribution station.

  - CS – compressor station.

- PROCUREMENT OBJECT

  - Non-contact diagnostics of main gas pipelines using the magnetic tomography method, stress concentration tomography, current magnetometry inspection, or an equivalent method (hereinafter - the Services).

- SCOPE/CHARACTERISTICS OF THE PROCUREMENT OBJECT

| No | Name of the main gas pipeline (MGP) | Preliminary length** (km) | Nominal diameter (DN), mm | Start coordinates | End coordinates |
| --- | --- | --- | --- | --- | --- |
| 1st year of the contract |  |  |  |  |  |
| 1 | MGP branch to Vilnius GDS | 0.25 | 700 | x: 54.6784404, y: 25.1857891 | x: 54.6769783, y: 25.1885312 |
| 2 | MGP branch to Vievis GDS | 0.17 | 10 | x: 54.7968094, y: 24.7729098; | x: 54.7980356, y: 24.772897 |
|  |  |  |  | x: 54.7966912, y: 24.7730748 |  |
| 3 | MGP branch to A. Paneriai GDS line 1 | 0.35 | 200 | x: 54.6496506, y: 25.1311165 | x: 54.6490298, y: 25.1351021 |
| 4 | MGP branch to Rudamina GDS | 1.77 | 300 | x: 54.5874219, y: 25.2796388 | x: 54.5885048, y: 25.3053609 |
| 5 | MGP branch to Žiežmariai GDS | 0.5 | 10 | x: 54.8187427, y: 24.4645296 | x: 54.8146328, y: 24.46185 |
| 6 | MGP branch to Širvintos GDS | 1.21 | 200 | x: 54.9992959, y: 24.9755535 | x: 54.9918681, y: 24.9688928 |
| 7. | MGP branch to Jauniūnai CS | 2.79 | 100/500/700/1000 | x: 54.956438 y: 24.988806 | x: 54.9569533, y: 24.9914825 |
|  |  |  |  | x: 54.9569533, y: 24.9914825 | x: 54.9616986, y: 24.9964872 |
|  |  |  |  | x: 54.9568858, y: 24.9892664 | x: 54.9629628, y: 24.9960912 |
|  |  |  |  | x: 54.9569713, y: 24.9950412 | x: 54.9617343, y: 24.996411 |
| 8. | MGP branch to Kaišiadorys GDS | 1.27 | 200 | x: 54.8498938, y: 24.3665562 | x: 54.8551122, y: 24.3812341 |
| 9 | MGP branch to Šalčininkai GDS | 0.25 | 10 | x: 54.3053247, y: 25.4073553 | x: 54.3068549, y: 25.4048071 |
| 10 | MGP branch to Praviena GDS | 0.24 | 150 | x: 54.880536, y: 24.2191412 | x: 54.8822257, y: 24.2193672 |
| 11 | MGP branch to Jašiūnai GDS | 0.28 | 100 | x: 54.4478439, y: 25.3360842 | x: 54.4460233, y: 25.3330702 |
| 12 | MGP branch to Birštonas GDS | 9.34 | 150/300 | x: 54.568634, y: 24.0329197 | x: 54.5852405, y: 24.0345823 |
|  |  |  |  | x: 54.5611426, y: 24.144362 | x: 54.5686344, y: 24.0329198 |
| 13. | MGP branch to Batniava GDS | 2.11 | 150 | x: 54.9691761, y: 23.6509432 | x: 54.9833328, y: 23.6356185 |
| 14 | MGP branch to Lekėčiai GDS | 2.09 | 15 | x: 54.961765, y: 23.488497; | x: 54.9731414, y: 23.5008117 |
|  |  |  |  | x: 54.961758, y: 23.488531 |  |
| 15 | MGP branch to Šakiai GDS | 0.25 | 200 | x: 54.9254624, y: 23.0626005; | x: 54.9277286, y: 23.0615781 |
|  |  |  |  | x: 54.925486, y: 23.0627525 |  |
| 16 | MGP branch to Baltoji Vokė GDS | 0.21 | 150 | x: 54.6038072, y: 25.1633552 | x: 54.6028662, y: 25.1646815 |
| 17 | MGP branch to Maišiagala GDS | 1.62 | 200 | x: 54.8896588, x: 25.0622143 | x: 54.8813097, y: 25.0449048 |
|  | Total | 24.74 | - | - | - |
| 2nd year of the contract |  |  |  |  |  |
| 18 | MGP branch to Pabradė GDS | 18.49 | 200/400 | x: 54.8940879, y: 25.6174902 | x: 54.9651142, y: 25.7943427 |
| 19 | MGP branch to Vilkaviškis GDS | 9.6 | 150 | x: 54.6985396, y: 23.1825155 | x: 54.6605153, y: 23.0555707 |
|  | Total | 28.09 | - | - | - |
| 3rd year of the contract |  |  |  |  |  |
| 20 | MGP branch to Paneriai 2 GDS | 0.32 | 500 | x: 54.6492704, y: 25.1309287 | x: 54.6482347, y: 25.1355608 |
| 21 | MGP branch to Švenčionėliai GDS | 0.28 | 10 | x: 55.143333, y: 26.096737 | x: 55.1428998, y: 26.1013523 |
| 22 | MGP branch to Nemenčinė GDS | 0.367 | 10 | x: 54.8939975, y: 25.5576179 | x: 54.8922493, y: 25.5542894 |
| 23 | MGP branch to Jurbarkas GDS | 2.65 | 30 | x: 55.0996867, y: 22.8335159 | x: 55.09858, y: 22.8242524 |
| 24 | MGP branch to Taujėnai GDS | 0.28 | 150 | x: 55.386136, y: 24.755112 | x: 55.3852655, y: 24.751561 |
| 25 | MGP branch to Raguva GDS | 0.24 | 10 | x: 55.5682175, y: 24.6452386 | x: 55.5679413, y: 24.6422228 |
| 26 | Technical connection (loop) to Panevėžys GDS | 1.34 | 500 | x: 55.7891443, y: 24.3929109 | x: 55.7822416, y: 24.4066477 |
| 27 | MGP branch to Panevėžys GCS | 1.98 | 70 | x: 55.7902127, y: 24.3901983 | x: 55.7878334, y: 24.381865 |
|  |  |  |  | x: 55.7900916, y: 24.3902168 | x: 55.7875833, y: 24.381761 |
| 28. | MGP branch to Gegužinė GCS | 0.46 | 150 | x: 55.8554234, y: 24.3524504 | x: 55.8546209, y: 24.3461205 |
| 29 | MGP branch to Pasvalys GCS | 0.15 | 15 | x: 56.074469, y: 24.342508 | x: 56.0749226, y: 24.3454171 |
| 30 | MGP branch to Pajiešmeniai GCS | 10.3 | 300/250 | x: 56.1056315, y: 24.3374351 | x: 56.1214826, y: 24.4881905 |
|  | Total | 18.37 | - | - | - |
| 31 | Undescribed MGP services according to individual orders throughout the contract period | 150* | - | - | - |

*Preliminary quantity

** Preliminary MGP lengths are provided. The Purchaser shall not be obligated to purchase the specified quantity of the Services or any part thereof. The final scope of the Services shall be determined based on the Services actually provided, but during the term of the contract, the Services shall be purchased without exceeding the maximum contract value of 500,000.00 EUR exclusive of VAT. This does not constitute an obligation on the part of the Purchaser to pay the successful Supplier the specified amount during the term of the Contract. Payment will be made according to the final actual length of the MGP after the final investigation report has been submitted. The actual amount of the Services provided will be paid for in accordance with the rates specified in the Contract. The Supplier will only be paid for the actual amount of the Services provided after the final report has been prepared. More detailed information on the technical parameters of the MGP will be provided together with the order.

Characteristics:

2.1. Non-contact MGP diagnostics using magnetic tomography or stress concentration tomography, or a working magneto-metric inspection method, or an equivalent method, in order to determine the technical condition of the MGP (metal defects) and ensure safe operation of the MGP.

2.2. Based on the diagnostic results, prepare a report on the technical condition of the MGP, identified defects, and the criticality of the defects. After identifying the MGP defects, prepare a list of possible anomalies indicating their locations.

- PLACE OF PERFORMANCE OF CONTRACTUAL OBLIGATIONS

  - Services shall be provided within the territory of the Republic of Lithuania. The results (reports, conclusions) must be submitted by e-mail as specified in the Contract.

- REQUIREMENTS FOR THE PURCHASE OBJECT

  - General requirements

4.1.1. The Supplier shall perform the Services using its own tools, equipment, and materials and shall use its own transport, including special means (e.g., a boat or other means of transport) if they are necessary to provide the Services across water bodies or marshy areas.

4.1.2. If the Supplier offers an equivalent method, it must submit a justification of equivalence together with the tender – a document (e.g., technical specification, technical assessment, or other act confirming equivalence) stating that the proposed method meets the functional and safety requirements for the object of the procurement. The document must be signed by a competent technical specialist or an independent accredited third party (e.g., a certification or testing body) authorised to perform such assessments. In the event of non-compliance, an explanation must be provided to justify that these differences do not adversely affect the functionality and safety of the service.

4.1.3. The equipment used for measurements and processing of results must be certified.

4.1.4. Identified defects and all MGP information must have GPS coordinates and must be presented in the LKS-94 coordinate system.

4.1.5. Before starting to provide the Services at an operating natural gas transmission system facility, the Supplier must obtain the Purchaser‘s consent to provide the Services at operating natural gas transmission system facilities (equipment) and/or their protection zone (hereinafter - the Consent). Without the Consent, the Supplier shall undertake not to provide any Services at an operating natural gas transmission system facility or in its protection zone. The Consent issued by the Purchaser shall be valid for the entire duration of the provision of all Services at the specified facilities and territories. The Consent shall be issued by the Purchaser for the entire term of the Contract. Request for Consent to provide Services/perform works at the Purchaser‘s facilities – Request for consent to perform works | Amber Grid.

  - Description of the procurement object

4.2.1. Before starting work, the Supplier shall agree with the Purchaser on a schedule for the performance of services in accordance with the order submitted by the Purchaser.

4.2.2. The Supplier shall independently perform all necessary measurements in accordance with the information on individual gas pipelines provided with the order.

4.2.3. The Supplier shall identify the axis of the underground MGP using measuring instruments.

4.2.4. The Supplier shall use a non-contact magnetic tomography or stress concentration tomography method, or a working magneto-metric inspection method, or an equivalent method to detect MGP defects and locations of increased stress (MPa) or increased stress conditions.

4.2.5. Measurements must identify defects such as (but not limited to): metal loss, mechanical defects, weld seam defects, stressed elbows, etc.

4.2.6. For each identified defect, the stress concentration (MPa) must be measured, the degree of danger determined, the maximum permissible pressure, the recommended direct assessment period, and other parameters important for the safe operation of the MGP must be established.

4.2.7. After performing all necessary ground measurements, processing the measurement data, and noticing indications of critical defects, the Supplier shall notify the MGP thereof, providing all necessary information.

4.2.8. After the ground measurements have been performed, if required by the Supplier‘s procedures, the Supplier shall select points for direct assessment and non-destructive testing. Direct assessment and necessary non-destructive testing shall be performed by the Supplier. Preparatory work, such as pipe excavation and removal of protective coatings, shall be performed by the Purchaser.

4.2.9. The Supplier shall provide the Purchaser with the information obtained during the direct assessment, i.e., the description of defects, measurements, non-destructive test reports (including radiographic test strip photographs), immediately, but no later than within 3 calendar days.

4.2.10. After performing a direct assessment of the selected defects (if necessary), the Supplier shall evaluate the information obtained and prepare an investigation report.

4.2.11. During the term of the Contract, the Purchaser shall have the right, upon agreement with the Supplier, to submit additional Orders for other MGP diagnostic services not specified in the technical specifications. Additional Services shall be performed at the unit (km) rate provided by the Supplier.

4.2.12. The volume of a single order submitted by the Customer shall not be less than 20 km, unless otherwise agreed in writing with the Supplier.

  - Procedure and terms for the performance of contractual obligations

4.3.1. The Supplier shall submit a draft schedule for the provision of Services (hereinafter - the Schedule) within 5 calendar days of the date of submission of each order. The Purchaser and the Supplier shall agree on the final Schedule within 21 calendar days of the date of submission of the draft Schedule. The Services shall be performed in accordance with individual orders and the agreed Schedule.

4.3.2. The test report shall be submitted as soon as possible, but no later than within 30 calendar days from the date of the measurements or, if necessary, from the date of the direct assessment of defects at the end of the Services.

4.3.3. The period of provision of the Services shall be 36 months or until the Services are purchased for the amount specified in the Contract, whichever occurs first.

4.3.4. In performing this Contract, the Supplier shall undertake to comply with the following environmental requirements: to reduce paper consumption, to refrain from unnecessary copying and printing of documents, i.e. the services shall be provided in electronic format, by electronic means. All information and/or documents necessary for the performance of the Contract shall be provided and coordinated in electronic format by electronic means.

  - Documentation to be submitted during the performance of the Contract

4.4.1. After completing the measurements, the Supplier shall prepare a report on the Services provided.

4.4.2. A separate investigation report must be submitted for each MGP.

4.4.3. The investigation report shall include (but is not limited to):

- general information about the MGP and the Services performed;

- conclusions on the technical condition of the MGP and further operation and repair deadlines, taking into account the identified defects, recommended safe gas pressure in the MGP;

- a list of identified anomalies with GPS coordinates and information about the defect;

- after performing a direct assessment, the report must include photographs, diagrams, descriptions of defects, as well as non-destructive testing reports (if any) and photographs (including photographs of radiographic test strips);

- recommendations for subsequent non-destructive testing or testing intervals.

4.4.4. A descriptive section for each calculated parameter must be provided separately or included in the test report, indicating how it was calculated, according to which document and according to which formula.

4.4.5. The test report and all other documents shall be submitted to the Purchaser in Lithuanian or English.

4.4.6. The test reports submitted by the Supplier must be signed by the responsible person and/or approved by the institution or laboratory that performed the test.

- PURCHASER‘S OBLIGATIONS

5.1. Issue consent, in accordance with the submitted request (point 4.1.5), to perform work at the Purchaser‘s natural gas transmission system facilities where the Services will be provided.

5.2. To provide available historical data on intensive measurements (if the information is required by the Supplier).

5.3. Clear vegetation (e.g., trees, bushes) from the locations where the Services are to be provided, if it interferes with the provision of the Services.

5.4. Assign an accompanying person during the provision of field Services.

5.5. Reduce the working pressure of the object under investigation (MGP) as necessary for the duration of the measurements.

5.6. Perform MGP excavation, protective cover removal, and pipe surface preparation work when performing a direct assessment of selected defects.

5.7. Accept the properly provided Services in accordance with the acceptance and transfer act and pay for them in accordance with the procedure and terms specified in the Contract.

5.8. Submit additional parameters of the MGP under investigation together with the order.