TECHNICAL SPECIFICATION

(PU-14145/25) [ITP25] High-capacity brine production and dispensing equipment

- TERMS AND ABBREVIATIONS

1.1. Purchaser – AB Kelių priežiūra.

1.2. Supplier – an economic entity – a natural person, private legal entity, public legal entity, other organizations and their divisions, or a group of such persons with whom the Purchaser enters into a Contract.

1.3. Contract – a contract concluded between the Supplier and the Purchaser regarding the Subject of the Procurement.

1.4. Name of the Subject of the Procurement – High-capacity brine production and dispensing equipment (hereinafter – the Goods/Item or the Equipment).

- DESCRIPTION, SCOPE, AND REQUIREMENTS OF THE SUBJECT OF THE PROCUREMENT

  - The Subject of procurement is not divided into parts.

  - Description of the Subject of the Procurement:

2.2.1. The Goods must comply with the technical requirements set out in the annexes to these Technical Specifications.

2.2.2. The Goods must be accompanied (no later than on the date of delivery of the Goods) by: operating and safety instructions in Lithuanian or English, electrical installation diagrams, piping diagrams, spare parts catalogues, and an EC declaration of conformity.

2.3. Scope of the Procurement:

| Ser. No | Name of the item | Unit of measurement | Quantity |
| --- | --- | --- | --- |
| 1. | High-capacity brine production and dispensing equipment | set | 1 |

  - The Purchaser purchases the Goods without delivery. The Purchaser will collect the Goods at the Point of Collection specified by the Supplier.

2.5. The Supplier must be either the Manufacturer of the proposed Item or provide a power of attorney or another document confirming the Supplier’s right to supply and service this equipment.

- PROCEDURE AND TERMS FOR THE FULFILLMENT OF CONTRACTUAL OBLIGATIONS

3.1. The goods must be ready for collection from the supplier's designated collection point no later than

100 calendar days from the date of entry into force of the contract.

4. ENVIRONMENTAL REQUIREMENTS

The Purchaser seeks to ensure that its and the Supplier‘s actions have as little impact on the environment as possible, therefore:

4.1.  During the public procurement and contract performance, communication between the Supplier and the Purchaser shall be carried out only by electronic means (CPP IS means, telephone, e-mail, etc.);

4.2.  All documentation related to the performance of the Contract shall be provided to the Purchaser and the Supplier by electronic means (e-mail, etc.);

4.3. The Contract shall be signed only by electronic means (electronic signature).

4.4. The Supplier shall undertake to reduce paper consumption, refrain from unnecessary copying and printing of documents, and if office supplies are used, they must be made from recycled raw materials or suitable for recycling.

4.5.  If the purchased Goods are to be delivered or transferred in secondary packaging, they must comply with the minimum environmental protection criteria set for packaging, unless this is contrary to hygiene standards. Packaging must be considered recyclable packaging in accordance with the provisions of the Republic of Lithuania Law on Environmental Pollution Tax.

5. ANNEXES

Annex 1 – Technical parameters

__________

All references in the procurement document to a standard, technical certificate or general technical specifications mean that the contracting authority will accept evidence of equivalent measures from other participants. If the specification refers to a specific model or source of supply, a specific process characteristic of the goods or services supplied by a particular supplier, or a trademark, patent, types, specific origin or production, which would give certain entities or certain products an advantage or lead to their rejection, an equivalent object to that specified may be submitted. If an equivalent object is offered, the supplier undertakes to submit/justify/prove its equivalence to the object specified in the document.

Annex 1 to the Technical Specification

TECHNICAL PARAMETERS

(PU-14145/25) [ITP25] High-capacity brine production and dispensing equipment

The following must be submitted:

Technical documentation from the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations from the manufacturer of the Item/Equipment (if the manufacturer’s technical documentation does not fully reflect the compliance of the offered Item/Equipment with the technical specification requirements) and other documents specified in the technical specifications or other equivalent documents proving the compliance of the offered Item/Equipment with the technical requirements. In these documents, the Supplier must indicate (i.e., clearly mark – highlight in colour and/or indicate with arrows, and/or underline, etc.) the specific locations in the documents provided where the required technical characteristics are described and indicate which point of the technical requirements they comply with.

OR

links to a publicly accessible website where the Purchaser can verify the authenticity of the data provided, i.e., the compliance of the offered Item/Equipment with technical requirements. If the information provided on the specified website does not correspond to the data declared by the Supplier, the tender must be accompanied by a declaration from the manufacturer of the Item/Equipment or other equivalent documents confirming that the offered Item/Equipment comply with the technical specification requirements.

The Supplier‘s declaration shall not be considered equivalent documentation, except in cases where the Supplier is the official representative of the manufacturer of the offered Item/Equipment.

In the event that the Supplier (who is not the manufacturer) submits technical documentation and/or declarations prepared by representatives of the manufacturer who have the relevant rights, the Supplier must also submit documents proving the relevant rights of the manufacturer‘s representative together with the offer (authorization granted by the manufacturer of the Item/Equipment to prepare/approve technical documentation for the Item/Equipment).

If the technical documentation for the Item/Equipment is prepared by a Supplier who is not the manufacturer and who has not been granted the manufacturer‘s authorization to prepare/approve technical documentation, in such a case the technical documentation prepared by the Supplier itself does not meet the criteria of objectivity, reliability, traceability of information, and other criteria, therefore, during the evaluation of the tender, the Purchaser will consider that the Supplier has only provided declarative information that is not substantiated by objective data and will reject such a tender.

## Documents (technical documentation of the manufacturer of the Item/Equipment (catalogues, brochures) and/or declarations by the manufacturer of the Item/Equipment or other equivalent documents proving that the offered Item/Equipment comply with the technical characteristics) may be submitted in Lithuanian and/or English. When evaluating the Suppliers’ tenders and at the Purchaser’s request, the Suppliers shall be required to submit the specified documents or parts thereof, translated into Lithuanian, and a confirmation of the translation.

## All specified specific materials, solutions and/or specific names or standards of the Item/Equipment are subject to the “or equivalent” clause. A Supplier offering the Item/Equipment with equivalent characteristics must additionally indicate this in the technical specification and/or tender and prove by reliable means that the offered Item/Equipment is equivalent and fully complies with the requirements set out in the technical specification.

| Item No. | Parameter / characteristic | Purchaser's requirement | The supplier must confirm compliance with the technical requirement by indicating: yes/no and, where required, enter the exact value of the Item offered*. |
| --- | --- | --- | --- |
| 1. GENERAL REQUIREMENTS |  |  |  |
| 1.1 | Purpose of the Equipment | High-capacity salt brine production and dispensing equipment. | Yes/No (delete as appropriate). |
| 1.2 | Year of production | The fully assembled and operation-ready equipment must be manufactured no earlier than the year 2025, must comply with the technical requirements set by the Republic of Lithuania and/or the EU, must have a CE product certificate and marking. | Yes/No (delete as appropriate). |
| 1.3 | Electrical part compatibility | The electrical part of the equipment must be compatible with a 230 V ±10%, 50 Hz (single-phase) or 400 V ±10%, 50 Hz (three-phase system) electrical network | Electrical parameters: _______________ (enter parameters) |
| 1.4 | Warranty | Not less than 12 months. | Proposed warranty : _______________ (enter parameters) |
| 1.5 | Spare parts and service | The Supplier must ensure the supply of spare parts and remote technical support. | Yes/No (delete as appropriate). |
| 1.6 | Remote access | Capability to connect, view parameters and control the equipment via LAN, Wi-Fi or other mobile communication compatible at the equipment usage location (in Lithuania) is mandatory. | Remote access capabilities: _______________ (enter parameters) |
| 1.7 | Equipment preparation for transportation | The Supplier shall prepare and pack the equipment so that it can be transported by road, rail, sea, or air transport. The packaging must withstand loading, unloading, and transshipment operations. The entire equipment set must be packed in a single package. | Dimensions and weight of the equipment prepared for transportation _______________ (enter parameters) |
| 1.8 | Documentation if the goods are prepared for collection in a non-EU country. | The Supplier must prepare and submit all documents required for export and import procedures, which include: Invoice, Packing List, goods description for the customs declaration, and any other required certificates, permits, or additional documents. | Yes/No (delete as appropriate). |
| 1.9 | Serial production | The device model must be a standard series-produced product (cannot be a unique, custom-made product). A manufacturer's declaration confirming the serial production of the proposed model must be submitted. | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference ________. |
| 2. BRINE PRODUCTION EQUIPMENT |  |  |  |
| 2.1 | Production type | Continuous brine production. | Yes/No (delete as appropriate). |
| 2.2 | Brine production capacity | Not less than 20 000 l/h. | Equipment capacity : _______________ (enter parameters) Title of document submitted as evidence - _________ and page No ___________ or reference ________. |
| 2.3 | Salt used for production | Sodium chloride. | Yes/No (delete as appropriate). |
| 2.4 | Concentration of produced brine | The brine production equipment must have an automated system for maintaining the set level of salt concentration (mandatory concentration level adjustment range 20 - 23% (Equipment with a wider adjustment range may be offered) with an interval step of at least 0.1% | Yes/No (delete as appropriate). Title of document submitted as evidence - _________ and page No ___________ or reference ________. |
| 2.5 | Process control | Fully automated, controlled via a color touch screen installed in the control cabinet. | Yes/No (delete as appropriate). |
| 2.6 | Salt hopper | The equipment must have a salt hopper with a capacity of at least 3.0 m³ and no more than 4.0 m³ installed and integrated into the salt solution preparation process. | Hopper capacity : _______________ (enter parameters) |
| 2.7 | Salt hopper dimensions | Length from 3.2 m to 3.5 m. Height from 1.1 m to 1.3 m. Width from 0.8 m to 1.0 m. | Hopper dimensions : _______________ (enter parameters) |
| 2.8 | Salt hopper protection | Installed | Yes/No (delete as appropriate). |
| 2.9 | Data logging, export | Integrated data logging with export function. | Yes/No (delete as appropriate). |
| 2.10 | Diagnostics | Built-in self-diagnostics. | Yes/No (delete as appropriate). |
| 2.11 | Clean-out system (Large solids) | Quick-opening access panel, not less than 0.3 m x 0.2 m (0.06 m²). | Yes/No (delete as appropriate). |
| 2.12 | Clean-out system (Small solids) | Not smaller than a 2" valve with fresh water flush capability. | Yes/No (delete as appropriate). |
| 2.13 | Construction | Stainless steel tanks with stiffening ribs. | Yes/No (delete as appropriate). |
| 2.14 | Pump housing material | Stainless steel. | Yes/No (delete as appropriate). |
| 2.15 | Pump power | Power from 2 to 4 kW. | Pump power : _______________ (enter parameters) |
| 2.16 | Pump capacity | Not less than 600 l/min. | Pump capacity : _______________ (enter parameters) |
| 2.17 | Water connection diameter | Connection diameter 2". | Yes/No (delete as appropriate). |
| 2.18 | Brine discharge pipe diameter | Discharge pipe diameter 2". | Yes/No (delete as appropriate). |
| 2.19 | Transportation | Integrated forklift pockets. | Yes/No (delete as appropriate). |
| 2.20 | Production equipment dimensions | Length from 4.0 m to 5.0 m. Height from 1.5 m to 1.8 m. Width from 1.30 m to 1.6 m. | Equipment dimensions: _______________ (enter parameters) |
| 2.21 | Production equipment weight | Not less than 1000 kg, not more than 1500 kg. | Equipment weight: _______________ (enter parameters) |
| - BRINE DISPENSING STATION |  |  |  |
| 3.1 | Purpose | Automated brine dispensing station for trucks. Can operate as a separate system or together with the brine production/storage tank system. | Yes/No (delete as appropriate). |
| 3.2 | Dispensing capacity | Variable flow rates from 100–700 l/min (equipment with a wider range is acceptable). | Yes/No (delete as appropriate). |
| 3.3 | Brine return | Capability to off-load to transfer liquids from the truck back to storage tanks. | Yes/No (delete as appropriate). |
| 3.4 | Connection of possible brines to the dispensing station | Capability to stack not less than 2 different products. | Yes/No (delete as appropriate). |
| 3.5 | Agitation | Equipped. | Yes/No (delete as appropriate). |
| 3.6 | Construction | Stainless steel frame with integrated forklift pockets. | Yes/No (delete as appropriate). |
| 3.7 | Dimensions | Width from 1.2 m to 1.50 m. Height from 1.5 m to 2 m. Depth from 0.8 m to 1.0 m. | Equipment dimensions: _______________ (enter parameters) |
| 3.8 | Weight | Not less than 300 kg, but not more than 400 kg. | Equipment weight: _______________ (enter parameters) |
| 3.9 | Brine connection pipe diameter | Connection diameter 3". | Yes/No (delete as appropriate). |
| 3.10 | Brine discharge pipe diameter | Discharge pipe diameter 3". | Yes/No (delete as appropriate). |
| 3.11 | Electric motor power | Not less than 3 kW. | Electric motor power _________. (enter parameters) |
| 3.12 | Pump housing material | Stainless steel or equivalent. | Yes/No (delete as appropriate). |
| 3.13 | Control valves. | Electric actuated, protection class IP68. | Yes/No (delete as appropriate). |
| 3.14 | Flow meter | Electromagnetic or equivalent. | Yes/No (delete as appropriate). |
| 3.15 | Control | Touch screen installed in the control cabinet. | Yes/No (delete as appropriate). |
| 3.16 | Control cabinet | Stainless steel cabinet with window, protection class IP66. | Yes/No (delete as appropriate). |
| 3.17 | Data logging | Dispensing and mixture data, in CSV format, can be collected via LAN or USB. | Yes/No (delete as appropriate). |
| 3.18 | User access | Up to 100 users with individual PIN codes. | Yes/No (delete as appropriate). |
| 3.19 | Data security | Password protection. | Yes/No (delete as appropriate). |

* The Supplier must confirm compliance with the technical requirement by indicating: Yes/No (delete as appropriate) and, where required, enter the exact parameter/value of the proposed Item.

Where required, the Supplier must enter the name of the document proving the parameters and/or a reference to the source confirming the proposed parameters.

_____________________              ________________________                      ______________________

(Title)	                     (signature)	(Name, Surname)

If the tender is signed by a person empowered by the Supplier, a document (power of attorney) granting the specified person the right to sign on behalf of the Supplier must be submitted together with the tender.