1. OBJECT OF the procurement

  - ASPHALT ROUGHNESS CONTROL MEASUREMENT EQUIPMENT (PROFILOMETER)

Asphalt Roughness Control Measurement Equipment (profilometer)

  - DELIVERY ADDRESS: LIDERYSTĖS G. 4, BIRULIŠKĖS, KAUNAS DISTRICT MUNICIPALITY, LITHUANIA

  - CODES OF THE COMMON PROCUREMENT VOCABULARY:

38900000-4 (Miscellaneous measuring or testing instruments);

38636100-3 (Lasers);

34933000-6 (Navigation equipment);

38651600-9 (Digital cameras);

30200000-1 (Computer equipment and supplies);

48420000-8 (Equipment Management Software Packages and Software Packages);

  - terms

    - Asphalt roughness control measurement equipment (profilometer) - a set of road asphalt pavement roughness measurement equipment, that consist of laser sensors with integrated accelerometers, distance measuring instrument, imaging and geographical positioning equipment, including measurement and data post-process software equipment, control and processing computers, and is prepared for installation in the base of a Toyota Hilux vehicle.

- DESCRIPTION OF TECHNICAL REQUIREMENTS TO BE MET BY THE PURCHASED GOODS/SERVICES

2.1. STANDARDS, TECHNICAL CERTIFICATES OR GENERAL TECHNICAL SPECIFICATIONS

2.1.1. Asphalt roughness control measurement equipment (profilometer) shall meet requirements of the most recent version of international standard EN 13036-6 or equivalent standard.

2.1.2. Asphalt roughness control measurement equipment (profilometer) classification according to international standard EN 13036-6 shall be not worse than 1L1111 or equal classification specified in equal standard.

2.1.3. Algorithms of data collection and post-process software for calculation of International Road Roughness Index IRI of Asphalt roughness control measurement equipment (profilometer) shall meet requirements of the most recent version of international standard EN 13036-5 or equivalent standard.

2.1.4. Pavement texture parameter measurement of asphalt roughness control measurement equipment (profilometer) shall meet requirements of the most recent version of international standard EN ISO 13473-1 or equivalent standard.

2.2. PROPERTIES OF THE OBJECT OF THE PROCUREMENT, FUNCTIONAL REQUIREMENTS OR/AND DESIRED RESULT

2.2.1. Requirements of the asphalt roughness control measurement equipment (profilometer) components.

| No. | Technical parameters of the goods indicated in the technical specification | Technical parameters of the offered goods |
| --- | --- | --- |
| 1. | Point lasers |  |
| 1.1. | Quantity | not less than 2 (two) |
| 1.2. | Frequency of data collection | not less than 32 kHz |
| 1.3. | Measurement range | not less than 200 mm |
| 1.4. | Environmental protection class according to IP | not less than IP65 |
| 2. | Accelerometers |  |
| 2.1. | Quantity | not less than 2 (two) |
| 2.2. | Measuring range | not less than ± 4 G |
| 2.3 | Environmental protection class according to IP | not less than IP65 |
| 3. | Distance measuring device |  |
| 3.1. | Resolution | at least 2000 signals per revolution |
| 3.2. | Environmental protection class according to IP | not less than IP65 |
| 4. | Front-view camera |  |
| 4.1. | Quantity | not more than 1 (one) |
| 4.2. | Resolution | at least 1,4 MP, or at least 1360x1024 px |
| 4.3. | Image format | 4:3 or 16:9 |
| 4.4. | Frames per second | at least 30 FPS at maximum resolution |
| 5. | Geographical positioning equipment |  |
| 5.1. | Satellite system type | GPS, GLONASS or equivalent |
| 5.2. | Signal reception | GPS L1/L2, GLONASS L1/L2 or equivalent |
| 5.3. | Signal receiving channels | at least 220 (two hundred and twenty) |
| 5.4. | Position update rate | not less than 10 Hz |
| 5.5. | Position correction service | DGPS or xFill or RTK |
| 5.6. | Correction receiving/broadcasting formats | CMR and/or RTCM or equivalent |
| 5.7. | Position determination horizontal accuracy | 30 cm or better |
| 5.8. | Position determination vertical accuracy | 50 cm or better |

2.3. OTHER CHARACTERISTICS OF THE OBJECT OF PURCHASE

OTHER PROPERTIES OF THE OBJECT OF THE PROCUREMENT.

2.3.1. Asphalt roughness control measurement equipment (profilometer) shall measure longitudinal road profile and longitudinal roughness in both wheel paths simultaneously. The measurement equipment shall measure pavement texture in both wheel paths simultaneously. The equipment shall be suitable to measure various types of pavements: asphalt pavement, concrete pavement, and small element (concrete block, cobblestone, etc.) pavement. The equipment shall acquire data for determination of following parameters: longitudinal profile, IRI (International Roughness Index), MPD (Mean Profile Depth), depth of longitudinal profile under 3 m straightedge or equivalent method. The equipment shall be able to measure roughness continuously when stopped and started moving again, that is, shall have "stop & go" functionality or equivalent.

2.3.2. Asphalt roughness control measurement equipment (profilometer)shall consist of not less than 2 (two) laser sensors and not less than 2 (two) accelerometers and a distance measuring device. Laser sensors and accelerometers must be equipped with lightweight, i.e. aluminium or equivalent material brackets or frames, suitable for mounting on the front or rear of a Toyota Hilux base vehicle. The distance measuring device must be equipped with magnetic holders or an equivalent that can be attached, but not screwed onto, to the specified vehicle body.

2.3.3. Imaging equipment shall consist of not more than 1 (one) front view camera, dedicated cables, and camera holders. Camera shall be connected to computer via cables that assure rapid and uninterrupted data connection. View capturing equipment shall be synchronized with data acquisition and post-processing of the mobile road survey equipment. Camera shall capture colourful views.

2.3.4. The front-view camera shall capture images of driving direction of the vehicle. The camera shall be mounted on the vehicle roof using magnetic or other non-screw mounts, in the middle of vehicle's width. The camera shall be tilted forward to capture more than 50 % of horizon without any parts of the vehicle. Camera shall be put into fitting rainfall-proof holster. Camera position shall be known regarding to GPS system position.

2.3.5. Geographical positioning equipment shall consist of GPS antenna, receivers, cables, computer for acquisition of position data. Geographical positioning equipment data acquisition and data post-processing shall be synchronised with data acquisition and post-processing of the asphalt roughness control measurement equipment (profilometer). GPS system shall be complete, ready to use. The GPS antenna and other external devices must be mounted on the vehicle roof using magnetic or other non-screw mounts. System shall be suitable for Lithuanian coordinate system LKS94 and provide x, y, z positioning in LKS94 and WGS projections.

2.3.6. Data acquisition software shall work by in advance prepared informational files of measurement sections or by entering information of measurement section in the initial window of data acquisition software start-up window. Informational files shall consist or in the start-up window shall be entered the following, but not limited to, information about measurement site:

- serial number of the measurement site, not less than five digits, e. g. 30031;

- identification number of the measurement site, not less than ten digits and not more than two characters, e.g. 0001111310l;

- road number or street name, not less than twenty characters and/or digits, e.g. A1, 0230, Architektu_gatve;

- start and end of measurement site in metres, not less than six digits;

- length of measurement site in metres, not less than six digits;

- start and end of measurement site coordinates in Lithuanian projection LKS94;

- lane of measurement site, not less than four character and/or digits, e.g. D1, K3, Z0, KL1, DG2;

- Information of measurement site start and end landmark, not less than twenty character and/or digits, e.g. 10, 1|0, z|2 D1|D2, miesto_ribozenklis.

2.3.7. Data acquisition software shall be able to display the following, but not limited to, information and data:

- planned length;

- start and end point;

- number of the road or street name

- measurement site serial number;

- measurement site lane;

- distance;

- GPS information;

- real time IRI, texture;

- image capture systems views;

- GPS maps, satellite information;

- road profile information;

2.3.8 Data acquisition software shall simultaneously mange all data acquisition sensors, receive and record all acquired data into main computer or into removable hard drives. Data acquisition software may be used for equipment testing and calibration purposes. Data collection program shall be able to start, switch measurement sections, and end the measurement according to pre-defined coordinates during the measurement when the files described in paragraph 2.3.6. are used to conduct measurements.

2.3.9. Data post-processing software shall process and analyse all collected data in detail, have comprehensive reporting tools, and be able to export data to txt, csv, or other equivalent format, and shall be compatible with Windows 11 or another equivalent operating system. Data processing software shall be regularly updated, at least once a year, to function properly on the latest versions of operating systems.

2.3.10. Data post-processing software shall be able to generate reports of user-specified all data and/or individual data at user-specified intervals:

- IRI, m/km;

- MPD, mm;

- profile depth under a 3 m straightedge in the longitudinal direction of the profile or equivalent method, mm;

- coordinates in LKS94 projection and altitude, m;

- distance, m;

- speed, km/h;

- name of the current interval photo with directory;

- start and end point;

- number of the road or street name

- measurement site serial number;

- measurement site lane.

2.3.11. Data post-processing software shall be able to generate reports of user-specified all data and/or individual data at user-specified intervals:

- 1 m;

- 5 m;

- 10 m;

- 20 m;

- 50 m;

- 100 m.

2.3.12. Asphalt roughness control measurement equipment (profilometer) shall be complete with all the wires, cables, computers, monitors, keyboards necessary for the full functioning of this equipment. Devices, that not specified in other parts of this technical specification and will be mounted outside the vehicle shall have body-frame degree of protection not worse than IP65 (according to IP Code) or equal. Every device shall be new and unused.

2.3.13. The components of asphalt roughness control measurement equipment (profilometer), specified in previous points of this technical specification, shall be controlled by a single, unified software and data shall be processed by a single, unified data post-process software. It also shall be provided the third party controlling or data post-processing software of each specified component.

2.3.14. Components of asphalt roughness control measurement equipment (profilometer)shall be calibrated and ready to use and the equipment itself and its collected data parameters shall be approved and certified by competent independent third-party organisation.

2.3.15. Asphalt roughness control measurement equipment (profilometer) shall include all necessary tools for lasers' and other components' calibration or testing.

2.3.16. The supplier shall provide to the customer a guarantee for the quality of the delivered goods. Supplier shall ensure that the delivered goods shall be new and unused. The warranty period shall be not less than 24 months. Warranty period begins on the day of equipment delivery to the customer. The supplier shall perform the system support throughout system warranty period without any extra charge. The supplier shall perform the system service, including certification, throughout system warranty period without any extra charge. The supplier shall perform the system's diagnostics before end of warranty period, but not less than 60 days. The supplier shall eliminate any occurred malfunction without extra charge.

2.3.17. The supplier shall train to manage the system and introduce the system composition to at least 5 employees of the costumer. The training shall consist of operating of the system, maintenance, data collection, data post-processing.

2.3.18. The supplier must have produced at least one equivalent road measurement equipment during last three years. System maintenance centre shall be located not further than 1500 km from the costumer.

2.3.19. The goods transferred by the supplier and the guarantee must meet the minimum environmental protection criteria according to the 2022 Decree of the Minister of the Environment of the Republic of Lithuania. December 13 order no. D1-401 "Approval of the procedure description for the application of environmental protection criteria in green procurement" 4.4.4.4.4. point, that is:

- the product is sturdy, durable, functional;

- the product or its components are suitable for multiple uses and/or are easily repaired and/or replaced.

2.3.20. The contracting organization, while carrying out procurements for object, which includes goods or services of the CPV codes specified in the list provided for in Article 92(13) of the Law on Public Procurement of the Republic of Lithuania, shall consider that the goods or services pose a threat to national security when:

1) the manufacturer of the goods or the person controlling it is registered (if the manufacturer or the person controlling it is a natural person - permanently residing or having citizenship) in the countries or territories specified in the list provided for in Article 92(14) of the Law on Public Procurement of the Republic of Lithuania;

2) the provision of services would be carried out from the countries or territories specified in the list provided for in Article 92(14) of this Law.

Also, when carrying out procurements, the object of which includes goods or services of the CPV codes specified in the list provided for in Article 92, Part 13 of the Law on Public Procurement of the Republic of Lithuania, the contracting authority shall consider that the supplier has interests that may pose a threat to national security and shall prohibit the participation in the procurement of suppliers, their sub-suppliers or economic entities whose capacities are relied upon, who themselves or the persons controlling them are registered (if the supplier, its sub-supplier, economic entity whose capacities are relied upon, or the person controlling them is a natural person - permanently residing or having citizenship) in the countries or territories specified in the list provided for in Article 92, Part 14 of the Law on Public Procurement of the Republic of Lithuania.

The lists were approved by Resolution No. 280 of the Government of the Republic of Lithuania of 30 March 2022.

3. Documents required to evaluate technical properties and quality of the object of THE PROCUREMENT

3.1. DOCUMENTS TO BE SUBMITTED WITH THE TENDER

3.1.1. The supplier shall provide an in-depth summary of offered goods by highlighting each component parameter specification of offered goods.

3.2. DOCUMENTS TO BE SUBMITTED WITH GOODS, WHEN TRANSFERRING THE PROVIDED SERVICES PROVIDED

3.2.1. The supplier shall issue a certificate of the quality of the system and a certificate of offered goods compliance to the international standards specified in paragraph 2.1. of this Technical Specification.