Annex 1

to the Procurement Conditions

TECHNICAL SPECIFICATION OF A PAPER DETECTOR TO DETECT CHEMICAL WARFARE AGENTS

- Paper detector (hereinafter referred to as the detector) is intended to detect the presence of dispersed chemical warfare agents of groups G, V and H in a liquid aggregate state.

- The detection of chemical warfare agents groups shall be based on the change in colour on a paper sheet following a chemical reaction when droplets of a liquid chemical warfare agent fall on it.

- The paper sheets of the detector should turn green when exposed to V group substances, red by H group substances and yellow by G group substances.

- The detector shall contain instructions for using the detector, as well as the date of manufacture and expire date of the detector.

- The paper sheets of the detector shall be perforated or stapled into a booklet with at least 10 paper sheets.

- The paper sheets of the detector shall be designed to be affixed to a different surfaces (metal, wood, rubber, fabric, plastic, etc.).

- The detector shall have a description regarding the groups of chemical warfare agents and their respective colours, how the paper sheets are coloured when the corresponding group of agents falls on it.

- The detector shall detect the groups of chemical warfare agents in no more than 45 seconds.

- The detector shall be suitable for use in an environment from at least - 20 °C to at least + 40 °C.

- The paper sheet of the detector shall be at least 60 x 80 mm in size.

- The detector paper sheets shall be easily unpacked, torn off and attached to various surfaces while wearing protective butyl rubber gloves.

- Each detector shall be individually packed in a waterproof packaging.

- The storage time of the detector in a packaging shall be at least 5 years from the date of delivery.

- The detector shall be manufactured in the year of supply.

_____________________________________