1. GENERAL INFORMATION and objectives

  - GENERAL INFORMATION

    - Object of procurement - Mobility data provision service (further – Service).

Via Lietuva (hereinafter referred to as the “Contracting Authority”) administers approximately 4,349 road sections with varying traffic volumes on national roads. Data on the number of vehicles, classifications, travel directions, and average speeds are collected annually. Using both permanent and temporary counting stations, traffic is monitored in about 1,260 of these sections. As a result, traffic volumes must be estimated for the remaining 3,089 sections where no traffic census will be conducted this year.

However, the current segmentation of road sections is homogeneous, meaning that many sections contain one or more intersections with national or municipal roads. This approach does not accurately reflect actual traffic flows. Therefore, it is necessary to re-segment the existing homogeneous road sections into junction-to-junction segments. The Contracting Authority has carried out the re-segmentation by dividing main and support roads at intersections with all national roads, as well as at selected intersections with municipal roads. Meanwhile, secondary roads were divided at intersections with all support and national roads. The newly formed number of national road sections is 8,112. For segments without measured traffic flow data, the AADT (Annual Average Daily Traffic) values should be estimated to ensure more accurate traffic information on national roads.

- The expected output of this procurement is an application/solution designed to achieve the goal of estimating the AADT values for newly re-segmented national road sections.

- The solution combines traffic census data available to the Contracting Authority with an OD matrix based on mobile network operators’ data.

- The solution must be provided as an application intended for internal use by the Contracting Authority.

- Open-source transparency must be ensured so that data preprocessing remains traceable and changeable if necessary.

- The price must include training for two people from the Contracting Authority to work with the application/solution being developed.

    - In order to fulfil the assignment, the Contracting Authority has put together and consolidated the data describing the traffic census that falls within the defined scope. The Supplier must take into account that the Contracting Authority has gathered the relevant data to the best extent possible, in the most suitable form, with reasonable coverage and detail. The Supplier must understand that the data may not be complete, however, and may contain errors. The afore-mentioned limitations cannot be used as the reason for not completing the services or its parts.

      - The Contracting Authority shall provide the following data as input for solving the task:

1.1.2.1.1.Historical data of the Annual Average Daily Traffic (AADT) on national roads for the last 5 years.

        - Raw data from the counting points, which includes:

          - Permanent counting points – measuring traffic frequencies 24/7/365 (approx. 450 units).

          - Temporary enumeration points – measuring different road sections with a duration of 1 day – 1 week each year (approx. 800 units).

        - Transportation zones – 400 transportation zones provided as a shapefile.

        - The newly segmented road network, 8112 road sections, provided in SHP format, where each road section is identified by the road number and the start kilometre and the end kilometre.

  - OBJECTS OF PROCUREMENT

    - ANALYSIS OF THE HISTORICAL DATA OF ANNUAL AVERAGE DAILY TRAFFIC

Report on the analysis of the historical data of annual average daily traffic (AADT) and the trends observed over the last 5 years on national roads in Lithuania.

    - ORIGIN-DESTINATION (OD) MATRICES

The Origin-Destination (OD) matrices represent general daily movements between zones. The OD encompasses all movements associated with stays at different locations as well as the travel between them. They are derived from mobile network operators’ data.

    - ANNUAL AVERAGE DAILY TRAFFIC MODELLING APPLICATION

The annual average daily traffic (AADT) modelling application is a GIS solution or command line script that models the daily traffic frequencies for the road sections where traffic flow intensity has not been measured during the census year.

    - MODELLED ANNUAL AVERAGE DAILY TRAFFIC FOR THE YEAR 2026

Modelled average annual daily traffic (AADT) for the year 2026 provides the estimated traffic flows for the missing road sections on a junction-to-junction basis, including all intersections between national roads and intersections with municipal roads.

  - SERVICE PROVISION STAGES AND PAYMENT PROCEDURE

    - STAGES OF PROVISION OF THE SERVICE

| Stage | Deadline for completion |
| --- | --- |
| 1. Historical annual average daily traffic analysis | 2 months |
| 2. Origin-Destination (OD) matrices | 6 months |
| 3. Annual average daily traffic modelling application | 2 months |
| 4. Modelled annual average daily traffic for the year 2026 | 2 months |

    - PAYMENT PROCEDURE

- The total period for providing the services specified in paragraph 1.2. of the Technical Specification shall not exceed 12 months from the date of entry into force of the Contract.

- Payments shall be made upon completion of each stage specified in paragraph 1.3.1. of this specification.

- The Supplier shall perform the Mobility Data Provision Services in the stages, sequence, and within the time limits specified in paragraph 1.3.1. of this specification.

- When submitting a tender, the Supplier must include all costs and taxes that shall be incurred in the provision of the afore-mentioned services.

2. tECHNICAL SPECIFICATIONS

  - REPORT ON THE ANALYSIS OF THE HISTORICAL DATA OF THE AADT

    - The analysis of the historical data of the AADT must cover at least 5 years of the latest AADT data provided by the Contracting Authority.

    - The analysis report should be presented in both PDF and DOCX format and should contain, but not be limited to, the following:

      - Long-term traffic flow trends (by segments and road numbers).

        - Changes in the AADT over the past 5 years for each road segment, road number and road categories: showing both absolute and percentage values.

        - Growth or decline trends: is the traffic consistently increasing/decreasing or does it fluctuate cyclically?

        - Emerging or developing traffic corridors: where is consistent growth being observed?

      - Territorial and functional differentiation (by road category, vehicle classification, and location).

        - Differences in traffic changes by road category (main, support and secondary roads): is traffic growth proportional across all categories?

        - Differences in traffic changes by vehicle classification: what differences are observed between vehicle classes A, B and C?

        - Differences in traffic changes by territory: comparison across regions, municipalities, or zones (e.g. urban peripheries, border areas, or provided zones)

      - Comparison of historical and modelled AADT flows.

        - Does modelled AADT match historical trends?

        - If any, where and by how much does the modelled AADT deviate from historical trends?

      - Visualisations.

        - Appropriate graphical representation to support the above analyses (e.g. maps, trend charts, comparative graphs).

  - OD MATRICES

    - The OD matrix data must be based on mobile subscribers’ data, and the Supplier must ensure that the sample coverage reaches at least 30% of the total population of Lithuania. The Supplier must also provide the percentage of the Lithuanian population covered by the selected mobile network operator covers.

    - The OD matrix data should be derived using a model that calculates stays and movements for the entire duration the mobile subscriber appears in the dataset.

    - A stop of more than 30 minutes shall start a new trip.

    - The final OD matrix output should reflect all travellers from each origin zone to each destination zone and be presented in the following structure and sample. Trips starting and ending in the same zone must also be included in the OD matrix.

      - The OD matrix output structure:

Date: date of the start of the movement (DD/MM/YYYY);

(Hour: hour of the start of the movement);

O_code: zone code of the origin zone;

D_code: zone code of the destination zone;

From_A_to_B: number of travellers from zone A to zone B.

      - Example of the OD matrix output:

| Date | O_code | D_code | From_A_to_B |
| --- | --- | --- | --- |
| 15/05/2026 | 222 | 223 | 856 |
| 16/05/2026 | 222 | 223 | 932 |
| 17/05/2026 | 222 | 223 | 156 |
| 18/05/2026 | 222 | 223 | 842 |

    - It is sufficient to provide the desired OD matrices of a representative data period totalling three (3) months, i.e. February, April and July of the year 2026

    - The OD matrices must be derived on two temporal accuracies: Daily (summed 24 hours) and Hourly (OD matrix of each hour: 00, 01, 02, 03 … 23).

    - It is required that the OD matrices of both temporal accuracies are provided for each day of the 3-month data period (89 days) and as an average April workday, where an average workday considers all Tuesdays, Wednesdays, and Thursdays of April excluding public and school holidays.

    - Spatial accuracy of 400 transportation zones provided by the Contracting Authority based on a shapefile (can be transformed into GeoJSON or any geo format).

    - If required, transportation zones can be consolidated by the Supplier in accordance with the Contracting Authority, provided there is good reasoning, and should be reflected in all the resulting OD matrices.

    - To protect individual privacy, each combination of zone movements is represented at least ten times (K-anonymity: 10).

  - AADT MODELLING APPLICATION

    - The AADT modelling application is a GIS solution or a command line script where the final output must generate a table in CSV format with modelled daily traffic frequencies for the road sections that weren’t measured during the census year.

    - Traffic frequencies must be distinguishable by at least 3 main vehicle groups as provided in the following table.

| Classification scheme | A | B | C |
| --- | --- | --- | --- |
| CA7 | Classes 1+7 | Classes 2+6 | Classes 3+4+5 |
| Class 1 | Passenger vehicle & passenger vehicle + trailer |  |  |
| Class 2 |  | Small van & small bus |  |
| Class 3 |  |  | Truck |
| Class 4 |  |  | Truck + trailer |
| Class 5 |  |  | Articulated truck |
| Class 6 |  | Bus |  |
| Class 7 | Motorcycles & other vehicles not included in other classes |  |  |

    - The resulting CSV format table must read:

Road number, Road name, Start km, End m, AADT vehicles/day, A/day, B/day, C/day, A%, B%, C%, A accuracy, B accuracy, C accuracy, International transit %, Local transit %, Season, Census year.

Where for each road section:

Road number – associated road number.

Road name – associated road name.

Start km – start of the road section in km.

End km – end of the road section in km.

AADT – average annual daily traffic in vehicles per day.

A/B/C/Day – the number of vehicles per day of class A/B/C.

A/B/C% - the percentage of vehicles of class A/B/C of total AADT.

A/B/C/accuracy – the estimation accuracy of class A/B/C.

International transit% - the percentage of trips, where either origin, destination or both are outside of Lithuania.

Local transit% - the percentage of trips lasting longer than 1 hour with both origin and destination inside of Lithuania.

Season – representative season of AADT estimation.

Census year – year of the data used.

    - The output should reflect the seasonality of traffic, at least during the following three periods.

      - School periods – September, October, November, March, April, May.

      - Winter period – December, January, February.

      - Summer holiday period – June, July, August.

    - The model should incorporate an understanding of the zoning principle of entry-exit gates of foreign countries to ensure valid results on border-crossing roads.

    - The modelling application code must be written in a clear, well-documented, and modular manner to ensure maintainability, reproducibility, and traceability of model logic and assumptions.

    - The developed model should be adaptable for future use, allowing traffic counters may change in both location and number.

  - MODELLED ADDT FOR THE YEAR 2026

    - Average annual daily traffic flows for the year 2026 must be modelled using the developed AADT modelling application.

    - The modelling results must be presented as described in section 2.3.3.

    - Modelling results must cover all national road sections, following a junction-to-junction principle including all junctions with national and municipal roads, where traffic counts were not conducted during the modelled year. However, if required, the Supplier can consolidate junction-to-junction road sections that are split only by the municipal roads, to ensure data quality criteria.

    - The resulting AADT frequencies must meet the following allowed difference criteria according to road categories:

      - Main roads +/- 8%.

      - Support roads +/- 15%.

      - Secondary roads +/- 30%.

      - Out-of-range anomalies are assessed in absolute numbers and on a case-by-case basis and must be identifiable.

    - The generated results must be uploaded in an electronic format that can be opened and processed using standard office software or freeware.

  - REQUIREMENTS FOR SERVICE PROVISION:

    - The Supplier must provide training of two persons from the Contracting Authority to work with the developed application/solution.

    - Final output must include the Report of historical AADT analysis, the OD matrices (the OD matrices of each day and each hour of the 3-month period mentioned in paragraph 2.2.5., the OD matrix of an average workday in April and for each hour of the average workday in April, where the workday is described in paragraph 2.2.7.), the AADT modelling application, modelled AADT for the year 2026 and relevant Documentation.

    - The relevant documentation must include, but is not limited to, the following.

      - User guide to system setup, preparation and execution.

      - Description of the methodology:

        - Description of the input data.

        - Quality assurance of input data.

        - Methods for OD generation.

        - Method for routing.

        - Methods for AADT calculations and result validations.

      - Results and discussion on model limitations, including results of the validation and reasoning for anomalies.

    - The Supplier must ensure open-source transparency of data processing to be traceable and changeable if necessary.

    - Signature of the transfer and acceptance certificate under each stage shall not release the Supplier from the obligation to correct errors in the performed works, including correcting errors of which the Contracting Authority was not aware when signing the transfer and acceptance certificate. The Supplier shall correct the errors that have appeared at his own expense to eliminate the identified deficiencies within 30 days of receiving the written notice and within a reasonable time, bear the costs related to the installation of the new version, document and hand over the corrected version (corrected software code with updated documentation) to the Contracting Authority.

    - Upon completion of services under each stage, the Supplier shall present and comment on the finished work at a meeting called by the Contracting Authority, after which the presentation is shared with the Contracting Authority.

    - The application, which is the output of the public procurement, shall be tested before accepting the work and the results obtained are validated. The results are jointly tested in the Contracting Authority’s system and the results are compared with the results obtained using the Supplier’s application.

    - Changes to transportation zones and road sections must be justified and reviewed on an individual basis.

    - The raw telecommunications data must be accessible for specific purposes, such as auditing, under strict guidance by the data controller.

  - SERVICE PROVISION STAGES AND PAYMENT PROCEDURE

    - STAGES OF PROVISION OF THE SERVICE

| Stage | Deadline for completion |
| --- | --- |
| 1. Historical annual average daily traffic analysis | 2 months |
| 2. Origin-Destination (OD) matrices | 6 months |
| 3. Annual average daily traffic modelling application | 2 months |
| 4. Modelled annual average daily traffic for the year 2026 | - months |

  - PAYMENT PROCEDURE

- The total period for providing the services specified in paragraph 1.2. of the Technical Specification shall not exceed 12 months from the date of entry into force of the Contract.

- Payments shall be made upon completion of each stage specified in paragraph 2.6.1. of this specification.

- The Supplier shall perform the Mobility Data Provision Services in the stages, sequence, and within the time limits specified in paragraph 2.6.1. of this specification.

- The final delivery of services shall be considered completed after all stages specified in subsection 2.6.1 have been completed and the handover certificate has been signed following submission of the results of Stage Four (“Modelled Average Annual Daily Traffic for the Year 2026”).

- When submitting a tender, the Supplier must include all costs and taxes that shall be incurred in the provision of the afore-mentioned services.

3. ADDITIONAL DOCUMENTS TO BE SUBMITTED TOGETHER WITH THE TENDER

The following additional documents must be submitted together with the tender submitted for the procurement.

  - Proof of access to mobile network data covering at least 30% of Lithuanian population, e.g. collaboration agreement with a mobile network operator in Lithuania.