Annex 2

to the Procurement Conditions

PROTECTIVE IMPERMEABLE DECONTAMINATION CLOTHING

TECHNICAL SPECIFICATION

2024 April        No.

- Protective impermeable decontamination clothing (hereinafter – decontamination clothing) is a set of multiple-use decontamination clothing designed to protect the body against chemical warfare agents, biological agents and radioactive particles (hereinafter – hazardous substances), when conducting decontamination tasks with high-pressure decontamination systems.

- The decontamination clothing shall be manufactured (only one of the 3 options should be offered):

  - either as a one-piece overall whose trousers, a jacket with an integral hood and protective boots are integrated and combined;

  - or as a partially one-piece overall whose trousers and a jacket with an integral hood are integrated and combined, and the protective boots are added separately;

  - or as a costume consisting of 3 separate parts: a blouse with an integral hood, trousers, protective boots.

- The decontamination clothing shall be made of material, which shall be suitable for multiple use and shall be resistant to spraying of high-pressure liquids.

- It shall be possible to decontaminate the decontamination clothing with liquid decontaminants.

- The decontamination clothing shall be manufactured in at least 3 different sizes from 170 cm to 190 cm.

- Seams of the decontamination clothing (if any) shall be glued or covered by other means to protect against external impacts.

- The construction of the decontamination clothing shall be such as to prevent the penetration of hazardous substances through its zippers, seams, flaps and closures.

- The decontamination clothing shall bear labels on the inside, indicating at least the following information: product size, date of manufacture, maintenance instructions (standard and understandable symbols may be used).

- The hood of the decontamination clothing shall:

- be sealed elastically on the frame of the gas-mask and so that no hazardous substances can penetrate through the connection.

- be suitable for use with different models of gas masks (Avon Protection FM-50, 3M Scott Safety M-95);

- have an elastic grommet, which shall fit on the frame of the gas mask without gaps and wrinkles and would adhere to the rubber of the gas mask;

- have a hook and loop (velcro) fastener (for adjustment) in the chin area, which shall allow the hood to be fixed on the frame of the gas mask together with the rubber grommet.

- The design of the cuffs of the decontamination clothing jacket shall ensure tightness and protection to prevent penetration of hazardous substances between the protective gloves and the cuffs of the jacket.

- The design of trousers and a jacket of the decontamination clothing (if presented as separate parts) shall ensure tightness and protection to prevent penetration of hazardous substances between the trousers and a jacket.

- The design of trousers and boots of the decontamination clothing (if presented as separate parts) shall ensure tightness and protection to prevent penetration of hazardous substances between the trousers and the boots. If additional multiple-use protective socks are required to ensure tightness, they shall be included in the set.

- The material of the decontamination clothing shall be tested with HD chemical warfare agent droplets and provide protection against the penetration of droplets of HD for at least 24 hours.

- The tensile strength of the decontamination clothing material (except protective boots) shall not be less than 700 N according to ISO 1421 or equivalent standard.

- The boots of the decontamination clothing shall be:

  - black (RAL 6007 or equivalent), brown grey (RAL 7013 or equivalent), or bottle green (RAL 6007 or equivalent) colour (outer side);

  - tested with HD chemical warfare agents drops and ensure at least 8-hour protection from HD material penetration;

  - resistant to flexing, i. e. it shall be no cracking before 125000 flex cycles in accordance with EN ISO 20344 or equivalent standard;

  - produced in at least 5 different sizes with an interval from 270 mm to 310 mm in accordance with the length of the foot (insole);

  - with reinforced (thicker) soles. The sole shall be slip-resistant, rugged and have a large pattern.

- The decontamination clothing and the protective boots shall be suitable for use at temperatures from 0℃ till +40℃.

- The weight of the maximum size of the decontamination clothing without boots shall not exceed 8 kg.

- The outer side of the decontamination clothing shall be of brown grey (RAL 7013 or equivalent), or bottle green (RAL 6007 or equivalent), or black (RAL 9005 or equivalent) colour.

- A multi-use means of transport (storage, safekeeping) shall be provided together with the decontamination clothing, made of a durable material and suitable for outdoor use (e.g. a case, plastic or metal box, bag or equivalent means of transport that cannot be made of paper or cardboard).

- A multi-use means of transport shall be of brown grey (RAL 7013 or equivalent), or bottle green (RAL 6007 or equivalent), or black (RAL 9005 or equivalent) colour.

- The decontamination clothing shall be accompanied by the manufacturer’s recommendations (in Lithuanian and English) on daily and periodic maintenance, decontamination methods and decontaminants to be used after contamination with hazardous substances.

- The decontamination clothing shall have a shelf life period of at least 10 years, under which the protective characteristics specified in this Technical Specification shall be ensured and the protective properties shall be maintained intact.

- The decontamination clothing shall have a warranty period of at least 2 years.

- During the 10-year period, the decontamination clothing shall retain the protective characteristics specified in this Technical Specification without additional conditions and without additional maintenance or service.

- The warranty period shall be calculated from the date of signature of the acceptance and delivery act between the Buyer and the Seller. The Seller shall undertake to eliminate the deficiencies that have arisen during the warranty period within a period not exceeding 30 working days. During the warranty period, the supplier shall carry out repairs at its own expense, including transportation costs.

- The manufacturer or supplier shall organise training in Lithuania on how to use the decontamination clothing no later than four weeks after delivery of the goods, as well as to issue training certificates to the participants of the training (up to 20 participants).

________________________________________